3GPP TSG-SA4 Telco on FS_5GMS_Multicast	S4aI201081

# 29th October, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TR 26.802 | CR | – | rev | – | Current version: | 0.0.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | pCR to TR26.802 on related multicast and broadcast work in 3GPP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TELUS |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5GMS_Multicast |  |  |  |  |  | Date: |  |  | 2020-10-29 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification). |  |  |  |  |  |  |  |  |  |
| Reason for change: |  | Add related multicast and broadcast work in 3GPP |  |  |  |  |  |  |  |  |
| Summary of change: |  | Related work TR23.757, MB2, xMB, etc. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Study Item will be lack of context from other related work in 3GPP |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Changes against baseline document TR 26.802 v0.0.2 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

## 3.1 	Terms

For the purposes of the present document, the terms given in TR 21.905 [5], TS 26.501 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 or TS 26.501.

Multimedia Broadcast/Multicast Service (MBMS): A unidirectional point-to-multipoint service in which data is transmitted from a single source entity to a group of users in a specific area. The MBMS has two modes: Broadcast mode and Multicast mode.

Broadcast session: A session to deliver the broadcast communication service. A broadcast session is characterised by the content to send and the geographical area where to distribute it.

Multicast session: A session to deliver the multicast communication service. A multicast session is characterised by the content to send, by the list of UEs that may receive the service and optionally by a multicast area where to distribute it.

MBS session: A multicast session or a broadcast session.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [5] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905.

5GMS	5G Media Streaming

ABR	Adaptive Bit Rate

AL-FEC	Application-Level Forward Erasure Correction

IPTV	Internet Protocol Television

MBS				Multicast/Broadcast Service

TMGI				Temporary Mobile Group Identity

NEXT CHANGE

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 26.501: "5G Media Streaming (5GMS); General description and architecture”".

[2]	IETF RFC 2236: "Internet Group Management Protocol, Version 2".

[3]	IETF RFC 4604: "Using Internet Group Management Protocol Version 3 (IGMPv3) and Multicast Listener Discovery Protocol Version 2 (MLDv2) for Source-Specific Multicast".

[4]	IETF RFC 3376: "Internet Group Management Protocol, Version 3".

[5]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[6]	3GPP TS 23.246: "MBMS Architecture and functional description".

[7]	3GPP TR 23.757: “Study on architecture enhancements for 5G multicast-broadcast services".

[8]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G system".

[9]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[10]	3GPP TS 23.502: "System architecture for the 5G System (5GS)".

[11]	3GPP TS 23.503: "System architecture for the 5G System (5GS)".

[12]	DVB BlueBook A176 (Second edition): "Adaptive media streaming over IP multicast", Internet Available https://dvb.org/wp-content/uploads/2020/03/A176_Adaptive-Media-Streaming-over-IP-Multicast_Mar-2020.pdf

[13]	CableLabs: "IP Multicast Adaptive Bit Rate Architecture Technical Report", Internet Available https://specification-search.cablelabs.com/ip-multicast-adaptive-bit-rate-architecture-technical-report

[14]	ETSI TS 103 285: "Digital Video Broadcasting (DVB); MPEG-DASH Profile for Transport of ISO BMFF Based DVB Services over IP Based Networks".

[15]	3GPP TS 26.348: "Multimedia Broadcast/Multicast Service (MBMS); Content ingest at xMB".

[16]	3GPP TS 26.346: " Multimedia Broadcast/Multicast Service (MBMS); Protocols and Codecs", Release 16.

[17]	ATSC A/331: "ATSC Standard: Signaling, Delivery, Synchronization, and Error Protection".

[18]	3GPP TS 29.468, “Group Communication System Enablers for LTE (GCSE_LTE); MB2 Reference Point; Stage 3”.

[19]	3GPP TS 23.468, “Group Communication System Enablers for LTE (GCSE_LTE); Stage 2”.

[20] 	RFC 6733, “Diameter Base Protocol”.

NEXT CHANGE

## 4.2		Related 5G multicast and broadcast work in 3GPP

This clause provides a brief summary of multicast and broadcast standardization work in 3GPP. Clause 4.2.1 reviews SA2 workgroup’s activities on enhanced 5G multicast-broadcast architecture. Clauses 4.2.2 and 4.2.3 review xMB interface and MB2 interface, respectively. For group communications related services, the standard interface to and from BM-SC is MB2, specified in TS 29.468 [18] and TS 23.468 [19]. For services other than Group Communications, the standard reference point between the content provider and the BM-SC is defined in TS 26.348 [15].

### 4.2.1 	Architectural enhancements for 5G multicast-broadcast

3GPP SA2 workgroup has been exploring potential solutions to enhance 5G multicast-broadcast functionalities in TS 23.757 [7]  The study assumes the sequence to eatablish and deliver an Multicast Broadcast (MBS) session as follows:

1.	Optional delivery of 5G MBS service info from application/service layer to 5GC.

2.	UEs participate in receiving MBS flow i.e. UE requests to join an MBS session (for Multicast session).

3.	Establishment of MBS flow transport. This step may happen before step 2 for individual UEs joining an MBS session which is already started.

4.	MBS data delivery to UEs.

5.	UEs stop receiving MBS flow (for Multicast session).

6.	Release of MBS flow transport (what used to be session stop).

Multiple delivery methods may be used to deliver MBS traffic in the 5GS from a single data source to multiple UEs. TR 23.757 [7] further described delivery methods in 5G CN and RAN.  Two delivery methods are possible from 5G CN point of view:

-	5GC Individual MBS traffic delivery method: 5G CN receives a single copy of MBS data packets and delivers separate copies of those MBS data packets to individual UEs via per-UE PDU sessions.

-	5GC Shared MBS traffic delivery method: 5G CN receives a single copy of MBS data packets and delivers a single copy of those MBS packets packet to a RAN node, which then delivers them to one or multiple UEs

From  RAN point of view, in the case of the shared delivery, two delivery methods are available for the transmission of MBS packet flows over radio:

-	Point-to-Point (PTP) delivery method: a RAN node delivers separate copies of MBS data packet over radio to individual UE.

-	Point-to-Multipoint (PTM) delivery method: a RAN node delivers a single copy of MBS data packets over radio to a set of UEs.

A RAN node may use a combination of PTP/PTM to deliver an MBS packet to UEs.

NOTE : The shared MBS traffic delivery, individual MBS traffic delivery, the PTP and PTM delivery methods are defined in SA and RAN WGs respectively, and they are listed here for reference only.

In Figure 4.2.1-1, Shared PTP or PTM delivery method and Individual delivery method may be used at the same time for a 5G MBS session depending on selected solution.

Figure 4.2.1-1: Overview of User Plane for a multicast session

A set of interim requiremetns are agreed in the conclusion section of TR 23.757 [7]

-	For multicast solutions, signalling from the UE to the network to join a multicast session shall be supported by UE and network. Join/leave operation via Control Plane (NAS) signalling shall be supported.

-	For N3 transport of the shared delivery method, GTP-U tunnelling using a transport layer IP multicast method and shared N3 (GTP-U) Point-to-Point tunnel shall be supported with support for QoS.

-	Both 5GC Shared MBS traffic delivery method and 5GC Individual MBS traffic delivery method shall be standardized for multicast data delivery.

-	The network shall be able to prepare and start the multicast traffic transmission for a MBS session after MBS service is started.

-	The network shall support selection of MB-SMF or SMF (depending on solution) at session join.

-	For N3 transport of the 5GC shared MBS delivery method, for unicast transport there shall be 1-1 mapping between MBS Session and GTP-U tunnel towards a RAN node, and for multicast transport there shall be 1-1 mapping between MBS Session and the GTP-U tunnel.

A reference architecture is provided in Annex A.3 of [7], reproduced as Figure 4.2.1-2 here

Fig. 4.2.1-2:	5G MBS Reference Architectrue from TR 23.757

The MBSF performs the following functions:

-	Service level functionality to support MBS, and interworking with LTE MBMS

-	Interacting with AF and MB-SMF for MBS session operations, and transport.

-	Selection of MB-SMF for MBS Session

-	Controlling MBSU if the MBSU is used.

The MBSF-U and MBSF-C could be a part of 5GMS AS and 5GMS AF respectively in 5G Media Streaming General Architecture as depicted in Figure 4.4-1.

4.2.2 		xMB Interface

In EPS, an MBMS bearer service architecture in Figure 4.2.2-1 is used to transpot data for one or more MBMS download, streaming, transparent, or Group Communications sessions. The MBMS User Service interfaces to the MBMS system via three entities, i.e. the BM-SC, MBMS-GW, and the UE. The MBMS Bearer Service is identified by TMGI.

Figure 4.2.2-1: MBMS nework architecture model for EPS

TS26.348 [15]  specified xMB interface between Content Provider and BM-SC. Using the xMB reference point, content provider can invoke procedures supported by BM-SC to  setup and manage MBMS user service from BMSC to the MBMS client, as shown in Figure 4.2.2-2. The BM-SC handles all MBMS related complexity. The Content Provider can use four user plance procedures to ingest content to BM-SC, as specified in Clause 5.5 of TS 26.348, data transfer modes: file push, file pull, RTP streaming, and transport.

In file ingestion with Pull, the content provider provides the file URLs to the BM-SC and the BM-SC fetches the files using HTTP GET. In file ingestion with Push, the Content Provider pushes the files using HTTP PUT.

Figure 4.2.2-2: The xMB reference model

RTP streaming mode and transport mode are more relevant to multicast live streaming are

- In RTP streaming mode, the BM-SC establishes an RTP session to the content provider and starts the streaming session to relay media streams. The content provider shall support PSS server functionality according to PSS as described in Clause 5.3 of 3GPP TS 26.234.

Figure 4.2.2-3: MBMS Streaming with RTP

- In transport mode, the BM-SC listens on one IP address and one port number to receive UDP packets.

The transparent delivery methods shall be used by the BM-SC to transmit downstream service content received over xMB-U from the content provider, when the Session Type property is set to “Transport-Mode”.

The transparent delivery method delivers application dta units as part of UDP or IP flows over an MBMS bearer to the UE. This delivery method complemetns the download delivery method and streaming delivery methodand is particularly useful for multicast and broadcast of IP-based services for which the media codecs and application protocols are defined outside of this spec.There are two xMB-U options for Transparent Delivery Mode.

In Transparent delivery with proxy mode, as depicted in Figure 4.2.2-4, the payload of UDP streams is opaque to the MBMS session and and MBMS client is expected to make the UDP payloads available to an applicatioin, without further knowledge on the content. BM-SC will change the IP to multicast.

Figure 4.2.2-4: Transparent Delivery with Proxy mode

The following Session Properties allow the configuration of this xMB-U mode:

-	Session Type is set by the Content Provider to Transport-Mode.

-	Delivery Mode Configuration for user plane (Session Type specific property) is set by the Content Provider to Proxy.

-	Session Description Parameters for User Plane (Session Type specific property) is set by the Content Provider and contains the UDP flow mapping descriptions.

-	When Session Announcement Mode (Session Type specific property) is set by the Content Provider to SACH, the BM-SC will add according session description into the SACH. In this case the MBMS Client (cf. TS 26.347) will offer the service to an application.

-	When Session Announcement Mode (Session Type specific property) is set by the Content Provider to Content Provider then the Content Provider is responsible to announce services to UEs (e.g. using GC1). The BM-SC provides at least the TMGIs as value of the Delivery Session Description Parameters property.

In Transparent Delivery with Forward-Only mode in Figure 4.2.2-5, the transport protocol on top of IP is opaque to the MBMS session and an MBMS client is expected to make the UDP payloads available to an application. In EPS, this is like the GTP tunnel IP is multicast

Figure 4.2.2-5: Transparent Delivery with Forward-Only

The following Session Properties allow the configuration of this xMB-U mode:

-	Session Type is set by the Content Provider to Transport-Mode.

-	Delivery Mode Configuration for user plane (Session Type specific property) is set by the Content Provider to Forward-only.

-	Session Description Parameters for User Plane (Session Type specific property) is set by the Content Provider and contains the UDP flow mapping descriptions.

-	When Session Announcement Mode (Session Type specific property) is set by the Content Provider to SACH, the BM-SC will add according session description into the SACH. In this case the MBMS Client (cf. TS 26.347) will offer the service to an application.

-	When Session Announcement Mode (Session Type specific property) is set by the Content Provider to Content Provider then the Content Provider is responsible to announce services to UEs (e.g. using GC1). The BM-SC provides at least the TMGIs as value of the Delivery Session Description Parameters property.

4.2.3 		MB2 Interface

MB2 interface, specified in TS 29.468 [18] and TS 23.468 [19], is used when providing Group Communication Services (such as MCPTT) delivery to the UE [16], as shown in Figure 4.2.3-1.

NOTE: for services other than Group Communications, the standard reference point between the content provider and the BM-SC is defind in TS 26.348, and reviewed in section 4.2.2

Figure 4.2.3-1: MBMS network architecture model for GCS Delivery

The MBMS multicast service activation procedure registers the user in the network to enable the reception of data from a specific multicast MBMS bearer service. The activation is a signalling procedure between the UE and the network. The procedure establishes MBMS UE contexts in UE, SGSN and GGSN and Iu mode BSC/RNC for each activated multicast MBMS bearer service comparable to regular PDP contexts.

MB2 interface carries both control and user plane data and provides a standardized way for external entity, e.g. GCS AS to connect to BM-SC. A high level reference model of the architectural elements relevant to understand the MB2 reference point is shown in Figure 4.2.3-2 [18]. More complete reference models for GCSE are contained in TS 23.468 [19].

Figure 4.2.3-2: Reference model for MB2 reference point

For MBMS delivery, the MB2 interface provides

-	MB2-C procedures defined in TS 23.468 [19], for requesting the BM-SC to activate, deactivate, modify an MBMS bearer, allocate/deallocate TMGI.

-	Forwarding of data to be delivered via an MBMS bearer to the BM-SC via the MB2-U reference point.

The MBMS session is identified by TMGI and Flow Identifier, which are assigned by TMGI upon request of the AS function.

MB2-C protocol: Diameter based protocol as defined in RFC 6733 [20]. BM-SC is the Diameter server in the sense that it is the network element that handles action requests and sends notifications. The AS function shall act as the Diameter client in the sense it is the network element requesting actions and handles notification from the BM-SC. Transport protocol of Diameter messages over MB2-C interfaces shall make use of SCTP or TCP.

MB2-U Protocol stack is specified in Section 7 of TS 29.468 [18], as depicted in Figure 4.2.3-3

Figure 4.2.3-3: 	The user plane protocol stack

END OF CHANGES