3GPP TSG-S4 Meeting #112e	S4aI201116

# Online, Still safe at home,  21st January 2021<Start_Date> - <End_Date>

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.802 | CR | <CR#> | rev | <Rev#> | Current version: | 0.1.8 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New Key Issue on “Collaboration and deployment scenarios” |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5GMSA-Multicast |  |  |  |  |  | Date: |  |  | <Res_date> |
| Category: | <Cat> |  |  |  |  |  | Release: |  |  | <Release> |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The discussion on collaboration and deployment discussion has started in S4aI201094. This document contains a text proposal to be inserted into TR 26.802 |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new Key Issue around collaboration and deployment scenarios depict the various combinations of function deployments |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 5.x	Key Issue #X: Collaboration and deployment scenarios

### 5.x.1	Description

In the following, four different deployment models are presented. The key guiding assumption here is, that the MBSF contains key IP Multicast related BM-SC functions such as a FLUTE Sender (which belongs to the “MBMS Download and Streaming Delivery Function”).

The existing 5GMSA APIs M1d, M2d, M4d and M5d maybe be extended during 3GPP Release 17 with 5MBS (and other) functions.

It is further assumed that MB-M1 is an evolution of xMB-C and MB-M2 an evolaution of xMB-U.

A general assumption for all the collaboration scenarios is that the 5GMSd functions are used for unicast content distribution, e.g. CDN functionality for DASH streaming is used.

Collaboration A depicts a deployment, where all 5MBS and 5GMSd functions are deployed inside of the trusted DN. Three different variants are depicted.

The 5GMSd AF and AS are responsible for unicast content distribution (e.g. CDN), i.e. M5d and M4d are exposed by the 5GMSd functions.

The MBSF and MSBU functions are for 5MBS distribution. The MBSF is the control and interacts with the MB-SMF using Nmbsmf.

-	A0: The MBSF is integrated within the 5GMSd AF

-	A1: Fully separated functions.

-	A2: Integrated control and user plane functions

Collaboration A0 describes a model, where the MBSF function is integrated into the 5GMSd AF and the MBSU function is still stand-alone. Background here is that the user plane functions are more specialized, i.e. optimized HTTP servers for unicast and optimized Multicast delivery functions for multicast. The 5GMSd AF uses the newly developed MB-M3 (Nmbsu) API to configure and control the multicast delivery functions. The 5GMSd AS might be extended to cut-though any push ingest into the MB-M2.

Figure 5.x.1-1: Collaboration A0: MBSF integrated within the 5GMSd AF

Figure 5.x.1-2: Collaboration A1: Fully separated functions

Figure 5.x.1-3: Collaboration A2: Integrated Control and User Plane functions

Collaboration B depicts a mixed deployment, where only the 5MBS related functions are deployed in the trusted DN. Configuration B in Figure A.3.2-2 (TR 23.757) indicates, that an external AF uses the NEF as control plane entry point. It is assumed that the MB-M1 interface is passed through the NEF and the NEF add security related functions transparently.

Like in Collaboration A (and C), the 5GMSd functions are used for unicast content distribution, e.g. CDN functionality for DASH streaming is in an external DN. The functions in the trusted DN are leveraged to prepare the content for 5MBS delivery. Here is it assumed, that unicast functions like unicast content reception (e.g. DASH) and features like file repair are offered by the 5GMSd AS from the external DN.

Also for Collaboration B, three different variants are depicted.

-	B0: The MBSF is presented in the trusted DN for service management

-	B1: The MBSF is absent and only an MBSU is used

-	B2: Only 5MBS functions, without 5GMSA functions

Figure 5.x.1-4: Collaboration B0: Mixed external and trusted DN functions

Figure 5.x.1-5: Collaboration B1: Mixed external and trusted DN functions

Figure 5.x.1-6: Collaboration B2: Mixed external and trusted DN functions

Collaboration C depicts a deployment, where all media related functions are deployed in an external DN and the 5G System offers only connectivity services, i.e. either unicast connectivity or 5MBS transport-only connectivity.

Figure 5.x.1-1: Collaboration C: All media functions in external DN

One could wonder, “why 3GPP should consider this deployment option?”. The consideration here is that an 5GMSA client (including a new 5MBS Client) in the UE can still be leveraged as receiver, supporting reception of 3GPP defined “DASH over 5MBS”, generic file delivery and RTP streaming. An MBSF-U like function would generate a bit stream, which is compliant to TS 26.346.  An Application Function (AF) may use Nmbsmf (via NEF) to activate a Transport-Only type of delivery into the MB-UPF (according to Configuration 1 in Figure A.3.2-2 of TR 23.757).

Collaboration D depicts a deployment similar to Collaboration #4 In TS 26.501. Here, the media plane does not follow 3GPP specifications. An Application Function (AF) may use Nmbsmf (via NEF) to activate a Transport-Only type of delivery into the MB-UPF (according to Configuration 1 in Figure A.3.2-2 of TR 23.757). Still, a 3GPP defined Media Session Handler is interacting with a 3GPP defined 5GMSd AF.

Figure 5.x.1-1: Collaboration D: Usage of Transport only with not 3GPP defined M4d-mb protocols.

### 5.x.2	Identified gaps

Editor’s Note: Gaps to be identified