SA4-e (AH) MBS SWG post 111-e	S4aI201118

# 21st January 2-21

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Draft CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.512 | CR | - | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on Procedures and APIs for Downlink and Uplink Streaming |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, BBC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS3 |  |  |  |  |  | Date: |  |  | 2020-12-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There are a number of bugs in the terminology and description of the procedures and APIs for downlink and uplink streaming in various sections of TS 26.512. Notably, very little normative text is currently present in the document on uplink streaming related interface procedures and associated APIs. It can be shown that most of the procedures/APIs currrently attributed to downlink streaming are also applicable to uplink streaming, and can be quite easily adapted for that purpose. The focus of this CR is on the M1 and M5 interfaces and modifying the associated descriptions in clauses 4, 7 and 11. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrections of bugs and modifications to descriptions of existing downlink-specifc interface procedures and APIs to also support uplink streaming. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inaccuracies and missing descriptions in TS 26.512 on uplink streaming |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4, 4.1, 4.2, 4.3, 5.2, 6.4.4, 7, 8, 11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| 56 |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st CHANGE: Correction to clause 4 and its various sub-clauses

# 4	Common and Downlink Streaming specific Procedures

## 4.1	General

This clause defines all procedures common for use between Downlink Streaming and Uplink Streaming, as well as procedures specific to Downlink Streaming, using the different 5G Media Streaming Reference Points.

## 4.2	APIs relevant to Downlink Streaming

Table 4.2-1 summarises the APIs used to provision and use the various downlink streaming features specified in TS 26.501 [2].

Table 4.2-1: Summary of APIs relevant to downlink streaming features

| 5GMSd feature | Abstract | Relevant APIs |  |  |
| --- | --- | --- | --- | --- |
|  |  | Interface | API name | Clause |
| Content Hosting | Content is ingested, hosted and distributed by the 5GMSd AS according to a Content Hosting Configuration associated with a Provisioning Session. | M1 | Provisioning Sessions API | 7.2 |
|  |  |  | Server Certificates Provisioning API | 7.3 |
|  |  |  | Content Preparation Templates Provisioning API | 7.4 |
|  |  |  | Content Protocols Discovery API | 7.5 |
|  |  |  | Content Hosting Provisioning API | 7.6 |
|  |  | M2d | HTTP-pull based content ingest protocol | 8.2 |
|  |  |  | DASH-IF push based content ingest protocol | 8.3 |
|  |  | M4d | DASH [4] or 3GP [37] | 10 |
|  |  | M5d | Service Access Information API | 11.2 |
| Metrics reporting | The 5GMSd Client uploads metrics reports to the 5GMSd AF according to a provisioned Metrics Reporting Configuration it obtains from the Service Access Information for its Provisioning Session. | M1 | Provisioning Sessions API | 7.2 |
|  |  |  | Metrics Reporting Configuration API | 7.8 |
|  |  | M5 | Service Access Information API | 11.2 |
|  |  |  | Metrics Reporting API | 11.4 |
| Consumption Reporting | The 5GMSd Client provides feedback reports on currently consumed content according to a provisioned Consumption Reporting Configuration it obtains from the Service Access Information for its Provisioning Session. | M1 | Provisioning Sessions API | 7.2 |
|  |  |  | Consumption Reporting Provisioning API | 7.7 |
|  |  | M5 | Service Access Information API | 11.2 |
|  |  |  | Consumption Reporting API | 11.3 |
| Dynamic Policy invocation | The 5GMSd Client activates different traffic treatment policies selected from a set of Policy Templates configured in its Provisioning Session. | M1 | Provisioning Sessions API | 7.2 |
|  |  |  | Policy Templates Provisioning API | 7.9 |
|  |  | M5 | Service Access Information API | 11.2 |
|  |  |  | Dynamic Policies API | 11.5 |
| Network Assistance | The 5GMSd Client requests bit rate recommendations and delivery boosts from the 5GMSd AF. | M5 | Service Access Information API | 11.2 |
|  |  |  | Network Assistance API | 11.6 |

## 4.3	Procedures of the M1 (5GMS Provisioning) interface

### 4.3.1	General

A 5GMSd Application Provider (5GMSu Application Provider) may use the procedures in this clause to provision the network for downlink (uplink) media streaming sessions that are operated by the 5GMSd Application Provider (5GMSu Application Provider). For downlink streaming, these sessions may be DASH streaming sessions, progressive download sessions, or any other type of media streaming or distribution (e.g. HLS) sessions. For uplink streaming, the content format and delivery protocol is defined by the 5GMSu Application Provider, but which typically employ adaptive HTTP streaming file formats such as DASH, HLS and CMAF, delivered over TCP.

The M1 interface offers three different sets of procedures:

-	For downlink streaming, configuration of content ingest at M2d for onward distribution over M4d by the 5GMSd AS: designed as an API that is equivalent to the functionality of a public CDN. For uplink streaming, configuration of content egest at M2u for the media content received by the 5GMSu AF from the 5GMSu Client over M4u . The resource types involved in content hosting configuration are provisioning session (see clause 4.3.2), content hosting procedures (see clause 4.3.3), ingest protocols (see clause 4.3.4), content preparation template (see clause 4.3.5), and server certificates (see clause 4.3.6) .

-	Configuration of dynamic policies: allows the configuration of Policy Templates at M5 that can be applied to M4 downlink/uplink sessions.

-	Configuration of reporting: permits the MNO to collect, at M5, QoE metrics and consumption reports about M4 downlink sessions, as well as permits the MNO to collect, at M5, QoE metrics reports about M4 uplink sessions.

A 5GMSd Application Provider (5GMSd Application Provider) may use any of these procedures, in any combination, to support its downlink (uplink) media streaming sessions.

### 4.3.2	Provisioning Session procedures

#### 4.3.2.1	General

Prior to configuring content hosting, dynamic policies, or reporting, the 5GMSd/5GMSu Application Provider shall create a new Provisioning Session. The following CRUD operations are used to manage a provisioning session.

#### 4.3.2.2	Create Provisioning Session

For downlink streaming (uplink streaming), this procedure is used by the 5GMSd Application Provider (5GMSu Application Provider) to create a new Provisioning Session. The 5GMSd/5GMSu Application Provider shall use the HTTP POST method to create a new Provisioning Session. Upon successful creation, the 5GMSd/5GMSu AF shall respond with a 201 (Created) response message that includes the resource identifier of the newly created Provisioning Session in the body of the reply and the URL of the resource, including its resource identifier, shall be returned as part of the HTTP Location header field.

#### 4.3.2.3	Read Provisioning Session properties

For downlink streaming (uplink streaming), this procedure is used by the 5GMSd Application Provider (5GMSu Application Provider) to obtain the properties of the Provisioning Session from the 5GMSd AF (5GMSu AF). The 5GMSd/5GMSu Application Provider uses the GET method for this purpose.

#### 4.3.2.4	Update Provisioning Session properties

The Update operation is not allowed on Provisioning Sessions.

#### 4.3.2.5	Delete Provisioning Session

For downlink streaming (uplink streaming), this procedure is used by the 5GMSd Application Provider (5GMSu Application Provider) to delete a Provisioning Session. The 5GMSd/5GMSu AF will release any associated resources, purge any cached data, delete all QoS and reporting configurations associated with this Provisioning Session. The 5GMSd/5GMSu AF shall use the HTTP DELETE method for this purpose.

### 4.3.3	Content Hosting Configuration procedures

#### 4.3.3.1	General

For downlink streaming (uplink streaming), these procedures are used by the 5GMSd Application Provider (5GMSu Application Provider) and the 5GMSd AF (5GMSu AF) on M1 to configure the content hosting feature for downlink (uplink) streaming.

#### 4.3.3.2	Create Content Hosting Configuration

This procedure is used by the 5GMSd Application Provider to create a new Content Hosting Configuration. The 5GMSd Application Provider shall use the HTTP POST method for this purpose and the request message body shall include a ContentHostingConfiguration resource, as specified in clause 7.6.3.1.

If the Content Hosting Configuration uses the Push-based content ingest method, i.e. the pull attribute is set to False, then the path and entryPoint properties are read-only and shall not be set by the 5GMSd Application Provider. In this case, the canonicalDomainName property is also read-only and shall be assigned by the 5GMSd AF.

If the procedure is successful, the 5GMSd AF shall generate a resource identifier representing the new Content Hosting Configuration. In this case, the 5GMSd AF shall respond with a 201 (Created) HTTP response message and shall provide the URL to the newly created resource in the Location header field. The response message body may include a ContentHostingConfiguration resource (see clause 7.6.3.1) that represents the current state of the Content Hosting Configuration, including any fields set by the 5GMSd AF.

If the procedure is not successful, the 5GMSd AF shall provide a response code as defined in clause 6.3.

#### 4.3.3.3	Read Content Hosting Configuration properties

For downlink streaming (uplink streaming), this procedure is used by the 5GMSd Application Provider (5GMSu Application Provider) to obtain the properties of an existing Content Hosting Configuration resource from the 5GMSd AF (5GMSu AF). The HTTP GET method shall be used for this purpose.

If the procedure is successful, the 5GMSd/5GMSu AF shall respond with a 200 (OK) response message that includes the ContentHostingConfiguration resource in the response message body.

If the procedure is not successful, the 5GMSd/5GMSu AF shall provide a response code as defined in clause 6.3.

#### 4.3.3.4	Update Content Hosting Configuration properties

For downlink streaming (uplink streaming), the update operation is invoked by the 5GMSd Application Provider (5GMSu Application Provider) to modify the properties of an existing ContentHostingConfiguration resource. All writeable properties except domainNameAlias may be updated. The HTTP PATCH or HTTP PUT methods shall be used for the update operation.

If the procedure is successful, the 5GMSd/5GMSu AF shall respond with a 200 (OK) and provide the content of the resource in the response, confirming the successful update operation.

If the procedure is not successful, the 5GMSd/5GMSu AF shall provide a response code as defined in clause 6.3.

#### 4.3.3.5	Delete Content Hosting Configuration

For downlink streaming (uplink streaming), this operation is used by the 5GMSd Application Provider (5GMSu Application Provider) to destroy a Content Hosting Configuration resource and to terminate the related distribution. The HTTP DELETE method shall be used for this purpose. As a result, the 5GMSd/5GMSu AF will release any associated network resources, purge any cached content, and delete any corresponding configurations.

If the procedure is successful, the 5GMSd/5GMSu AF shall respond with a 200 (OK) response message.

If the procedure is not successful, the 5GMSd/5GMSu AF shall provide a response code as defined in clause 6.3.

### 4.3.4	Content Protocols procedures

#### 4.3.4.1	General

The set of content ingest (egest) protocols supported by the 5GMSd AS (5GMSu AS) at interface M2d (M2u) is described by the ContentProtocols resource at M1, as specified in clause 7.5.3.1.

#### 4.3.4.2	Create Content Protocols

The Create operation is not permitted for the ContentProtocols resource.

#### 4.3.4.3	Read Content Protocols

For downlink streaming (uplink streaming), this procedure is used by the 5GMSd Application Provider (5GMSu Application Provider) to retrieve a list of content ingest (egest) protocols supported by the 5GMSd AS (5GMSu AS). The HTTP GET method shall be used for this purpose.

If the procedure is successful, the 5GMSd/5GMSu AF shall respond with a 200 (OK) response that includes a ContentProtocols resource in the response message body, as specified in clause 7.5.3.1. If the procedure is not successful, the 5GMSd/5GMSu AF shall provide a response code as defined in clause 6.3.

#### 4.3.4.4	Update Ingest Protocols

The Update operation is not permitted for the ContentProtocols resource.

#### 4.3.4.5	Delete Ingest Protocols

The Delete operation is not permitted for the ContentProtocols resource.

### 4.3.5	Content Preparation Template procedures

#### 4.3.5.1	General

For downlink streaming, the 5GMSd AS may be required to process content ingested at interface M2d before serving it on interface M4d. For uplink streaming, the 5GMSu AS may be required to process content it receives from the 5GMSu Client before passing it to the 5GMSu Application Provider on the egest interface M2u. The downlink or uplink streaming related content processing information are defined in the form of a Content Preparation Template as described in clause 7.4.1. The content processing operations are specified in a Content Preparation Template resource, as specified in clause 7.4.2.

#### 4.3.5.2	Create Content Preparation Template

For downlink streaming (uplink streaming), this procedure is used by the 5GMSd Application Provider (5GMSu Application Provider) to register a new Content Preparation Template with a Provisioning Session. The 5GMSd/5GMSu Application Provider shall use the HTTP POST method to upload a new Content Preparation Template resource. The MIME content type of the Content Preparation Template shall be supplied in the Content-Type HTTP request header.

Upon successful creation, the 5GMSd/5GMSu AF shall respond with a 201 (Created) response message and the URL of the newly created resource, including its resource identifier, shall be returned as part of the HTTP Location header field.

If the MIME content type indicated in Content-Type is not understood by the 5GMSd AF/5GMSu AF, the creation of the Content Preparation Template resource shall fail with HTTP error response status code 422 (Unprocessable entity).

If the 5GMSd/5GMSu AF is unable to provision the resources indicated in the supplied Content Preparation Template, the creation operation shall fail with an HTTP response status code of 503 (Service Unavailable).

If the procedure is not successful, the 5GMSd/5GMSu AF shall provide a response code as defined in clause 6.3.

#### 4.3.5.3	Read Content Preparation Template

For downlink streaming (uplink streaming), this procedure is used by the 5GMSd Application Provider (5GMSu Application Provider) to download a copy of a Content Preparation Template resource from the 5GMSd AF (5GMSu AF). The 5GMSd/5GMSu Application Provider shall use the GET method for this purpose.

If the procedure is successful, the 5GMSd/5GMSu AF shall respond with 200 (OK) and shall provide the requested resource in the HTTP message response body. The Content-Type response header shall have the same value as that supplied when the Content Preparation Template was created.

If the procedure is not successful, the 5GMSd/5GMSu AF shall provide a response code as defined in clause 6.3.

#### 4.3.5.4	Update Content Preparation Template

For downlink streaming (uplink streaming), the update procedure is used by the 5GMSd Application Provider (5GMSu Application Provider) to modify or replace an existing Content Preparation Template resource. The HTTP PATCH or HTTP PUT methods shall be used for the update operation.

If the procedure is successful, the 5GMSd/5GMSu AF shall respond with a 200 (OK) and provide the content of the resource in the response, reflecting the successful update operation.

If the procedure is not successful, the 5GMSd/5GMSu AF shall provide a response code as defined in clause 6.3.

#### 4.3.5.5	Delete Content Preparation Template

For downlink streaming (uplink streaming), this operation is used by the 5GMSd Application Provider (5GMSu Application Provider) to delete a Content Preparation Template resource. The HTTP DELETE method shall be used for this purpose.

If the procedure is successful, the 5GMSd/5GMSu AF shall respond with a 200 (OK) response message.

If the procedure is not successful, the 5GMSd/5GMSu AF shall provide a response code as defined in clause 6.3. If the Content Preparation Template is in use as part of a Content Hosting Configuration, the procedure shall fail with HTTP error response status code 409 (Conflict).

### 4.3.6	Server Certificate procedures

#### 4.3.6.1	General

For downlink streaming (uplink streaming), each X.509 server certificate [8] presented by the 5GMSd AS (5GMSu AS) at interface M4d (M4u) is represented by a Server Certificate resource at M1. The Server Certificates Provisioning API as specified in clause 7.3 enables a Server Certificate resource to be created within the scope of a Provisioning Session, and subsequently referenced by a Content Hosting Configuration created in the scope of the same Provisioning Session. That API supports two alternative provisioning methods for Server Certificate resources: one in which a certificate is generated by the 5GMS System operator on behalf of the 5GMSd/5GMSu Application Provider; the other in which a certificate is generated by the 5GMSd/5GMSu Application Provider from a Certificate Signing Request solicited from the 5GMSd/5GMSu AF. Both methods shall be supported by implementations of the 5GMSd/5GMSu AF.

#### 4.3.6.2	Create Server Certificate

For downlink streaming (uplink streaming), this procedure is used by the 5GMSd Application Provider (5GMSu Application Provider) to request that the 5GMS System generates a new X.509 certificate on its behalf within the scope of a Provisioning Session. In this case, the certificate's Common Name (CN) is assigned in a domain under the control of the 5GMSd/5GMSu System operator.

The 5GMSd/5GMSu Application Provider shall use the HTTP POST method to create a new Server Certificate resource. Upon successful creation, the 5GMSd/5GMSu AF shall respond with a 201 (Created) response message and the URL of the resource, including its resource identifier, shall be returned in the HTTP Location header. The response message body may optionally include a copy of the X.509 certificate corresponding to the newly created Server Certificate resource, as specified in clause 7.3.3.2.

NOTE:	The X.509 certificate corresponding to the newly created Server Certificate resource may not be available immediately for interrogation and use. See clause 4.3.6.4 below for more details.

If the procedure is not successful, the 5GMSd/5GMSu AF shall provide a response code as defined in clause 6.3.

#### 4.3.6.3	Reserve Server Certificate

For downlink streaming (uplink streaming), this procedure is used by the 5GMSd Application Provider (5GMSu Application Provider) to solicit a Certificate Signing Request from the 5GMSd AF (5GMSu AF) for the purpose of generating an X.509 certificate independently of the 5GMSd System. In this case, the certificate's Common Name (CN) is assigned in a domain under the control of the 5GMSd/5GMSu Application Provider itself, or that of a third party acting on its behalf. The 5GMSd/5GMSu Application Provider shall separately arrange for the FQDN carried in the Common Name of the certificate, or that of a Subject Alternative Name (subjectAltName) extension in the same certificate (see section 4.2.1.6 of RFC 5280 [20]), to resolve to the address of a 5GMSd/5GMSu AS in the target 5GMS System.

The 5GMSd/5GMSu Application Provider shall use the HTTP POST method to create a new Server Certificate. Upon successful creation of the resource, the 5GMSd/5GMSu AF shall respond with a 201 (Created) response message and the URL of the resource, including its resource identifier, shall be returned in the HTTP Location header. The Content-Type response header and the body of the HTTP response message shall be as specified in clause 7.3.3.1.

If the procedure is not successful, the 5GMSd/5GMSu AF shall provide a response code as defined in clause 6.3.

#### 4.3.6.4	Retrieve Server Certificate

For downlink streaming (uplink streaming), this procedure is used by the 5GMSd Application Provider (5GMSu Application Provider) to download a Server Certificate resource for inspection. The HTTP GET method shall be used for this purpose. If the requested resource exists and is populated with an X.509 certificate, the 5GMSd/5GMSu AF (shall respond with 200 (OK) and shall return the requested Server Certificate in accordance with clause 7.3.3.2.

In the case where the X.509 certificate was provisioned by the 5GMSd System (5GMSu System) on behalf of the 5GMSd Application Provider (5GMSu Application Provider) according to clause 4.3.6.2 above, the HTTP response 503 (Service Unavailable) shall be returned until such time as the X.509 certificate is generated and available for download. The optional HTTP response header Retry-After should be included in such a response, indicating when the certificate is expected to become available for inspection and use.

In cases where the X.509 certificate is to be generated by the 5GMSd/5GMSu Application Provider from a Certificate Signing Request obtained according to clause 4.3.6.3 above, the HTTP response 404 (Not Found) shall be returned until such time as the X.509 certificate has been uploaded using the procedure specified in clause 4.3.6.5 below.

#### 4.3.6.5	Upload Server Certificate

For downlink streaming (uplink streaming), this procedure is used by a 5GMSd Application Provider (5GMSu Application Provider) to upload an X.509 certificate that it has generated in response to a Certificate Signing Request solicited using the reservation procedure specified in clause 4.3.6.3 above. The HTTP PUT method shall be used for this purpose. The Content-Type request header and the body of the HTTP request message shall be as specified in clause 7.3.3.2.

Before accepting the supplied X.509 certificate, the 5GMSd/5GMSu AF shall verify that the party originating the upload is the same party that reserved the Server Certificate resource using the procedure specified in clause 4.3.6.3 above. If there is a mismatch, the HTTP response 403 (Forbidden) shall be returned.

Attempting to upload an X.509 certificate to a Server Certificate resource that has not been reserved shall elicit a 404 (Not Found) HTTP response.

#### 4.3.6.6	Update Server Certificate

Updating a previously uploaded Server Certificate is not permitted for security reasons. Any attempt to do so using the PUT method shall result in the HTTP response 405 (Method Not Allowed).

To supply a replacement X.509 certificate, for example when a previously supplied certificate is shortly due to expire, the 5GMSd/5GMSu Application Provider should instead use one of the procedures specified in clause 4.3.6.2 or 4.3.6.3 above to create or reserve a new Server Certificate resource and, once the certificate is available for use, update the Content Hosting Configuration to reference it.

#### 4.3.6.7	Delete Server Certificate

This procedure is used to remove a Server Certificate from a Provisioning Session. The HTTP DELETE method shall be used for this purpose. On success, the HTTP response 200 (OK) or 204 (No content) shall be returned and afterwards the identifier of the Service Certificate resource is no longer valid.

Only the party that created (see clause 4.3.6.2) or reserved (see clause 4.3.6.3) the Server Certificate resource is permitted to delete it. Any attempt by another party to delete a Server Certificate resource shall elicit the HTTP response 405 (Method Not Allowed).

The HTTP response 409 (Conflict) shall be returned if an attempt is made to delete a Server Certificate resource that is currently referenced by a Content Hosting Configuration.

Attempting to destroy a Server Certificate resource that has been reserved but never uploaded shall elicit a 200 (OK) HTTP response. In this case, the 5GMSd AF should release any resources associated with the reservation.

If the procedure is not successful, the 5GMSd/5GMSu AF shall provide a response code as defined in clause 6.3.

### 4.3.7	Dynamic Policy Configuration procedures

#### 4.3.7.1	General

These procedures are used by the 5GMS Application Provider to configure the Policy Templates for streaming sessions of a particular Provisioning Session.

Figure 4.3.7.1-1 below is a state diagram showing the life-cycle of a Policy Template.

Figure 4.3.7.1-1: Policy Template State Diagram

Since Policy Templates require 5GMS System operator verification, a Policy Template that is newly created cannot be used immediately. Upon creation, a Policy Template shall be in the pending state. Once all mandatory properties are provided, the 5GMS AF triggers validation. If the Policy Template is not deemed to be valid by the operator of the 5GMS System, it shall move to the invalid state, from where it can be updated to remedy the defect. Once it has been successfully validated by the 5GMS System operator, a Policy Template shall take the ready state, indicating that it may be applied to streaming sessions. If it is subsequently updated by the 5GMS Application Provider, a Policy Template shall return to the pending state, awaiting revalidation by the operator of the 5GMS System. Finally, a Policy Template may be suspended by the 5GMS System operator, e.g. in case of a violation of the usage terms or for some other reasons, which renders it unusable. The update of any property moves the state into pending and triggers revalidation. A Policy Template may be destroyed when it is in any of the abovementioned states.

The 5GMSd/5GMSu AF shall verify the status of a Policy Template prior to allowing a Dynamic Policy Instance to instantiate it. Only Policy Templates in the ready state are eligible to be instantiated in this way.

#### 4.3.7.2	Create Policy Template

This procedure is used by the 5GMS Application Provider to create a new Policy Template. The HTTP POST method shall be used for this purpose.

If the procedure is successful, the 5GMSd/5GMSu AF shall generate a resource identifier to uniquely identify the newly created Policy Template. In that case, it shall respond with a 201 (Created) HTTP response message and provide the URL to the newly created resource in the Location header field.

If the procedure is not successful, the 5GMSd/5GMSu AF shall provide a response code as defined in clause 6.3.

The default state of a newly created Policy Template is pending. If all mandatory property values have been provided, the Policy Template is eligible for validation.

#### 4.3.7.3	Read Policy Template

This procedure is used by the 5GMS Application Provider and other 5GMSd/5GMSu AFs to query the properties of an existing Policy Template resource from the 5GMSd/5GMSu AF. The HTTP GET method shall be used for this purpose.

If the procedure is successful, the 5GMSd/5GMSu AF shall respond with a 200 (OK) response that includes the Policy Template in the response message body.

If the procedure is not successful, the 5GMSd/5GMSu AF shall provide a response code as defined in clause 6.3.

#### 4.3.7.4	Update Policy Template

The update operation is invoked by the 5GMS Application Provider to modify the properties of an existing Policy Template. All available properties except state may be updated. The HTTP PATCH or HTTP PUT methods shall be used for the update operation.

Any update to the Policy Template resource will change its state back to pending, which makes it temporarily unusable. If all mandatory property values have been provided, the Policy Template is eligible for revalidation.

If the procedure is successful, the 5GMSd/5GMSu AF shall respond with a 200 (OK) response message that includes the Policy Template in the response message body. Modifications to read-only properties, such as changes to the state of a Policy Template, shall be rejected with a 403 (Forbidden) HTTP response.

If the procedure is not successful, the 5GMSd/5GMSu AF shall provide a response code as defined in clause 6.3.

#### 4.3.7.5	Delete Policy Template

This operation is used by the 5GMS Application Provider to delete a Policy Template resource. The HTTP DELETE method shall be used for this purpose. As a result, the 5GMSd/5GMSu AF will remove the Policy Template from any Provisioning Sessions that reference it.

Currently active streaming sessions using the deleted Policy Template, if any exist, shall be stopped by the removal of the Policy Template.

If the procedure is successful, the 5GMSd/5GMSu AF shall respond with a 200 (OK) response message.

If the procedure is not successful, the 5GMSd/5GMSu AF shall provide a response code as defined in clause 6.3.

---- <snipped> ----

### 4.3.9	Metrics Reporting Configuration procedures

#### 4.3.9.1	General

##### For downlink streaming (uplink streaming), these procedures are used by the 5GMSd Application Provider (5GMSu Application Provider) to configure QoE metrics reporting functionality associated with downlink streaming (uplink streaming). This clause defines the basic procedures. More details are provided in clause 7.8.

##### A given instance of a Metrics Reporting Configuration is identified by the metricsReportingConfigurationId property of the MetricsReportingConfiguration resource, The properties of that resource, as described in clause 7.8.3.1, pertain to metrics collection and reporting by the Media Session Handler to the 5GMS AF.

#### 4.3.9.2	Create Metrics Reporting Configuration

For downlink streaming (uplink streaming), this procedure is used by the 5GMSd Application Provider (5GMSu Application Provider) to create a Metrics Reporting Configuration for a particular Provisioning Session. The 5GMSd/5GMSu Application Provider shall use the HTTP POST method for this purpose and the request message body may include a MetricsReportingConfiguration resource, as specified in clause 7.8.3.1. Upon successful operation, the 5GMSd/5GMSu AF shall respond with a 201 (Created) response message and the resource URL for the newly-created Metrics Reporting Configuration shall be returned in the Location header field. If the procedure is unsuccessful, the 5GMSd/5GMSu AF shall provide a response code as defined in clause 6.3.

This procedure may be perfomed multiple times to provision different Metrics Reporting Configurations in the scope of a particular Provisioning Session. Each such configuration is represented by a different value of metricsReportingConfigurationId.

#### 4.3.9.3	Read Metrics Reporting Configuration

For downlink streaming (uplink streaming), this procedure is used by the 5GMSd Application Provider (5GMSu Application Provider) to obtain the properties of an existing Metrics Reporting Configuration resource from the 5GMSd AF (5GMSu AF). The 5GMSd/5GMSu Application Provider shall use the GET method for this purpose. If successful, the 5GMSd/5GMSu AF shall respond with a 200 (OK) and the requested MetricsReportingConfiguration resource (see clause 7.8.3.1) shall be returned in the body of the HTTP response message. If the procedure is unsuccessful, the 5GMSd/5GMSu AF shall provide a response code as defined in clause 6.3.

#### 4.3.9.4	Update Metrics Reporting Configuration

For downlink streaming (uplink streaming), this update operation is invoked by the 5GMSd Application Provider (5GMSu Application Provider) to initially upload the Metrics Reporting Configuration resource, or in the case of an existing Metrics Reporting Configuration resource, to entirely replace or modify certain properties of that resource. All available properties may be updated. The HTTP PATCH or HTTP PUT methods shall be used for the update operation.

If the procedure is successful, the 5GMSd/5GMSu AF shall respond with a 200 (OK) reflecting the successful update operation. If the procedure is unsuccessful, the 5GMSd/5GMSu AF shall provide a response code as defined in clause 6.3.

#### 4.3.9.5	Delete Metrics Reporting Configuration

This operation is used by the 5GMSd Application Provider (5GMSu Application Provider) to destroy a Metrics Reporting Configuration resource and to terminate the related metrics reporting procedure. The HTTP DELETE method shall be used for this purpose. As a result, the 5GMSd/5GMSu AF should release any associated resources, discard any pending metrics reports, and delete any corresponding configurations.

##### If the procedure is successful, the 5GMSd/5GMSu AF shall respond with a 200 (OK) response message. If the procedure is unsuccessful, the 5GMSd/5GMSu AF shall provide a response code as defined in clause 6.3.

---- <snipped> ----

## 4.7	Procedures of the M5 (Media Session Handling) interface

### 4.7.1	Introduction

The M5 APIs are used by a Media Session Handler within a 5GMS Client to invoke downlink streaming (uplink streaming) related services at the 5GMSd AF (5GMSu AF).

### 4.7.2	Procedures for Service Access Information

#### 4.7.2.1	General

Service Access Information is the set of parameters and addresses needed by the 5GMSd Client to activate reception of a downlink streaming session. Typically, the 5GMSd Client receives via M8 a media entry point (e.g. a URL to a DASH MPD or a URL to a progressive download file) that can be consumed by the Media Player and is handed to the Media Player through M7. In addition, the media entry point URL may trigger the Media Session Handler to fetch the Service Access information from the 5GMSd/5GMSu AF for this streaming session.

This clause specifies the procedures whereby the 5GMSd/5GMSu Client fetches the Service Access Information from the 5GMSd/5GMSu AF.

#### 4.7.2.2	Create Service Access Information

The Create operation is not allowed on Service Access Information.

#### 4.7.2.3	Read Service Access Information properties

This procedure shall be used by the Media Session Handler to acquire Service Access Information from the 5GMSd/5GMSu AF. The Media Session Handler uses the GET method for this purpose.

The downlink or uplink streaming session for which the Media Session Handler is requesting data is identified by a unique reference contained in the path of the URL, as specified in clause 11.2.2.

Once it has obtained an initial set of Service Access Information, the Media Session Handler shall periodically check for updated Service Access Information by issuing a conditional HTTP GET request containing either:

- an If-None-Match request header with the value of the entity tag (ETag) that was returned with the most recently acquired ServiceAccessInformation resource; or else

- an If-Modified-Since request header with the Last-Modified value of that most recently acquired resource.

The periodicity of polling for updated Service Access Information shall be guided by the value of the Expires and/or Cache-control: max-age headers that shall be included along with every response message for this procedure.

#### 4.7.2.4	Update Service Access Information properties.

The Update operation is not allowed on Service Access Information.

#### 4.7.2.5	Delete Service Access Information properties

The Delete operation is not allowed on Service Access Information.

### 4.7.3	Procedures for dynamic policy invocation

For either downlink streaming or uplink streaming, this procedure is used by a Media Session Handler to manage Dynamic Policy Instance resources via the M5 interface. A dynamic policy invocation consists of a Policy Template Id, flow description(s), a 5GMSd/5GMSu Application Service Configuration Id and potentially other parameters, according to TS 26.501 clause 5.7.

A Policy Template Id identifies the desired Policy Template to be applied to an application flow. A Policy Template includes properties such as specific QoS (e.g. background data) or different charging treatments. The 5GMSd/5GMSu AF combines the information from the Policy Template with dynamic information from the Media Session Handler to gather a complete set of parameters to invoke the N33 or N5 API call. The Policy Template may contain for example the AF identifier.

The flow description allows the identification and classification of the media traffic, such as the packet filter sets given in  clause 5.7.6 of [2].

In order to instantiate a new dynamic policy, the Media Session Handler shall first create a resource for the Dynamic Policy Instance on the 5GMSd/5GMSu AF. When the Media Session Handler needs several dynamic policies, it repeats the step as often as needed.

The Media Session Handler creates a new Dynamic Policy Instance by sending an HTTP POST message to the 5GMSd/5GMSu AF. The body of the HTTP POST message shall include a Provisioning Session Id, the Policy Template Id and the traffic descriptor. The traffic descriptor identifies the actual application flow(s) to be policed according to the Policy Template. If the operation is successful, the 5GMSd/5GMSu AF creates a new resource URL representing the Dynamic Policy Instance. In this case, the 5GMSd/5GMSu AF shall respond to the Media Session Handler with a 201 Created HTTP response message, including the URL for the newly created Dynamic Policy Instance resource as the value of the Location header field.

Editor's Note: At minimum, the N5 and N33 API requires the UE IP Address at time of API invocation. The full Flow Description is an optional element, when more fine-grained traffic flow identification is required. It needs to be studied, how to enable usage of other traffic filtering parameters, such as an application id.

The Media Session Handler can modify the parameters of an existing Dynamic Policy Instance resource using either the HTTP PUT or PATCH methods, as appropriate to the desired update. The 5GMSd/5GMSu AF shall trigger the appropriate actions towards other Network Functions like PCF or NEF when all information is set.

Editor's Note: It is not clear what triggers the 5GMSd/5GMSu AF to start the PCF/NEF interactions.

The Media Session Handler can delete a Dynamic Policy Instance resource using the HTTP DELETE method. As a result, the 5GMSd/5GMSu AF shall trigger the appropriate actions towards other Network Functions like PCF or NEF to remove the associated PCC rule.

Editor's Note: Notification subscription will be added in the next version of the pCR.

---- <snipped> ----

### 4.7.5	Procedures for metrics reporting

The M5 procedures for QoE metrics reporting pertain to the combination of the provisioning of metrics collection and reporting in the Media Session Handler using relevant Service Access Information, and the sending of collected metrics by the Media Session Handler to the 5GMSd/5GMSu AF in accordance with the configured metrics scheme(s). A metrics scheme may be 3GPP-defined or non-3GPP-defined.

When the metrics collection and reporting feature is activated for a downlink streaming session, one or more metrics configuration sets, each associated with a metrics scheme, may be provided to the 5GMSd/5GMSu Client. A given metrics configuration set contains information such as the 5GMSd/5GMSu AF address(es) to which metrics are to be sent by the Media Session Handler, metrics reporting interval, target percentage of streaming sessions for which reports should be sent, and the set of metrics to be collected and reported. See TS 26.501 [2] for additional details.

For progressive download and DASH streaming services, the listed metrics in a given metrics configuration set are associated with the 3GPP metrics scheme and shall correspond to one or more of the metrics as specified in clauses 10.3 and 10.4, respectively, of TS 26.247 [4].

Details of the metrics reporting API are provided in clause 11.4, and for 3GP-DASH based downlink streaming services, the 3GPP-defined metrics reporting scheme and metrics report format are defined in clause 11.4.3.

### 4.7.6	Procedures for network assistance

For downlin streaming (uplink streaming), this procedure is used by the 5GMSd Client (5GMSu Client) to request Network Assistance from the 5GMSd AF (5GMSu AF).

The 5GMSd/5GMSu Client first creates a Network Assistance Session. It provides information that will be used by the Network Assistance function to request QoS from the PCF and to recommend a bit rate to the 5GMSd/5GMSu Client.

The 5GMSd/5GMSu Client may also request a delivery boost to be provided.

After the Network Assistance Session resource is provisioned, the 5GMSd/5GMSu Client uses the Network Assistance Session identifier when requesting a bit rate recommendation.

In order to terminate a Network Assistance Session, the 5GMSd/5GMSu Client deletes the Network Assistance session resource.

END OF 1st CHANGE

2nd CHANGE: Corrections to clause 5 and its sub-clauses

# 5	Procedures for Uplink Streaming

## 5.1	General

Uplink streaming functional entities in the 5GMS System include the 5GMSu Application Provider, 5GMSu AF, 5GMSu AS and the UE. To make use of these other entities, the UE includes a 5GMSu-Aware Application that is provided by the 5GMS Application Provider and a 5GMSu Client comprising the Media Session Handler and the Media Streamer.

The M1 Provisioning API enables the 5GMSu Application Provider to establish and manage the uplink media session handling and streaming options of the 5GMSu system.

The M2u Egest interface enables uplink streaming content sent by the 5GMSu Client to the 5GMSu AS over interface M4u to be subsequently delivered to the 5GMSu Application Provider. Uplink streaming media transfer from the 5GMSu AS to the 5GMSu Application Provider may be either pull-based and initiated by the 5GMSu Application Provider using the HTTP GET method, or push-based and initiated by the 5GMSu AS using the HTTP PUT method. The resource identifier of the 5GMSu Application Provider for push-based streaming content delivery is provided to the 5GMSu AS by the 5GMSu AF over the M3u interface, as part of the M1 Provisioning Session.

The 5GMSu AF, having acquired M1 Provisioning information, sets up the M5 interface that the 5GMSu Client can use for uplink streaming session management, remote control, metrics reporting, network assistance and request for policy and/or charging treatment. Certain types of configuration and policy information accessed over M5 by the Media Session Handler, such as uplink metrics reporting, QoS policy, or support for AF-based network assistance are further passed to the Media Streamer via the M7u API.

Based on the configuration information received on M5 and a request from the Media Streamer received over the M6u interface, the Media Session Handler sets up an uplink streaming session with the 5GMSu AF. Upon successful session establishment, the Media Session Handler triggers the Media Streamer to begin uplink streaming of media content to the 5GMSu AS over the M4u interface.

Subscription to status and other event notification services are offered by the Media Session Handler to the 5GMSu-Aware Application and to the Media Streamer via the M6u APIs exposed by the Media Session Handler.

Subscription to status and other event notification services are also offered by the Media Streamer to the 5GMSu-Aware Application and to the Media Session Handler via the M7u APIs exposed by the Media Player.

## 5.2	APIs relevant to Uplink Streaming

Table 5.2-1 summarises the APIs used to provision and use the various uplink streaming features specified in TS 26.501 [2]. The procedures that are applicable to both downlink streaming and uplink streaming are Provisioning Session, Content Hosting Configuration, Content Protocols, Content Preparation template, Server Certificate Provisioning, Dynamic Policy Configuration and Metrics Reporting Configuration, as described in clauses 4.3.2, 4,3,3, 4.3.4, 4.3.5, 4.3.6, 4.3.7 and 4.3.9, respectively.

Table 5.2-1: Summary of APIs relevant to uplink streaming features

| 5GMSu feature | Abstract | Relevant APIs |  |  |
| --- | --- | --- | --- | --- |
|  |  | Interface | API name |  |  | Clause |
| Content Hosting | Media content streamed from the Media Player to the 5GMSu AS is subsequently egested to the 5GMSu Application Provider in accordance to a Content Hosting Configuration associated with a Provisioning Session. | M1 | Provisioing Session API |  |  | 7.2 |
|  |  |  | Server Certificates Provisioning API |  |  | 7.3 |
|  |  |  | Content Preparation Templates Provisioning API |  |  | 7.4 |
|  |  |  | Content Protocols Discovery API |  |  | 7.5 |
|  |  | M4u | Uplink streaming protocol (TBD) | 10.X |
|  |  | M2u |  | Content egest protocol | 8.2 |  |
| Metrics Reporting | The 5GMSu Client uploads metrics reports to the 5GMSu AF according to a provisioned Metrics Reporting Configuration it obtains from the Service Access Information for its Provisioning Session. | M1 | Provisioning Sessions API | 7.2 |
|  |  |  | Metrics Reporting Configuration API | 7.8 |
|  |  | M5 | Service Access Information API | 11.2 |
|  |  |  | Metrics Reporting API | 11.4 |
| Dynamic Policy invocation | The 5GMSu Client activates different traffic treatment policies selected from a set of Policy Templates configured in its Provisioning Session. | M1 | Provisioning Sessions API | 7.2 |
|  |  |  | Policy Templates Provisioning API | 7.9 |
|  |  | M5 | Service Access Information API | 11.2 |
|  |  |  | Dynamic Policies API | 11.5 |
| Network Assistance | The 5GMSu Client requests bit rate recommendations and delivery boosts from the 5GMSu AF. | M5 | Service Access Information API | 11.2 |
|  |  |  | Network Assistance API | 11.6 |

END OF 2nd CHANGE

3rd CHANGE: Additional enumerated data type under clause 6.4.4

### 6.4.4	Enumerated data types

#### 6.4.4.1	CellIdentifierType enumeration

The data model for the CellIdentifierType enumeration which indicates the type of cell identifier as defined in TS 23.003 [7], is specified in Table 6.4.4.1-1 below:

Table 6.4.4.1-1: Definition of CellIdentifierType enumeration

| Enumeration value | Description |
| --- | --- |
| CGI | Cell Global Identification. |
| ECGI | E-UTRAN Cell Global Identification. |
| NCGI | NR Cell Global Identity. |

#### 6.4.4.2	SdfMethod enumeration

The data model for the SdfMethod enumeration is specified in Table 6.4.4.2-1 below:

Table 6.4.4.2-1: Definition of SdfMethod enumeration

| Enumeration value | Description |
| --- | --- |
| 5Tuple | The Media Session Handler shall use 5-Tuples for Service Data Flow descriptions. The 5-Tuple shall not contain a wildcard. |
| 2Tuple | The Media Session Handler shall use a 2-Tuple of UE IP and Server IP as Service Data Flow Description. |
| typeOfServiceMarking | The Media Session Handler shall apply Type of Service (ToS) marking to the Service Data Flow. |
| flowLabel | The Media Session Handler shall apply IPv6 flow label marking and provide the IPv6 flow label of the Service Data Flow. |
| domainName | The Media Session Handler shall provide the domain name of the 5GMSd AS. |

#### 6.4.4.3	ProvisioningSessionType enumeration

The data model for the ProvisioningSessionType enumeration is specified in Table 6.4.4.3-1 below:

Table 6.4.4.3-1: Definition of ProvisioningSessionType enumeration

| Enumeration value | Description |
| --- | --- |
| downlink | Downlink streaming |
| uplink | Uplink streaming |

END OF 3rd CHANGE

4th CHANGE: Corrections to sub-clauses of clause 7

# 7	Provisioning (M1) APIs

## 7.1	General

This clause defines the provisioning API used by a 5GMS Application Provider to configure downlink or uplink 5G Media Streaming services.

## 7.2	Provisioning Sessions API

### 7.2.1	Overview

The Provisioning Sessions API is used by the 5GMS Application Provider to instantiate and manipulate Provisioning Sessions in the 5GMS System, as described in clause 4.3.2. Having created a Provisioning Session, the 5GMS Application Provider can then go on to provision other 5GMS features in the context of that Provisioning Session, using the APIs specified in clause 7.3 et seq. Certain of these features are only applicable to the type of Provisioning Session created.

### 7.2.2	Resource structure

The Provisioning Sessions API is accessible through the following URL base path:

{apiRoot}/3gpp-m1/v1/provisioning-sessions/

Table 7.4.2-1 specifies the operations and the corresponding HTTP methods that are supported by this API. In each case, the sub-resource path specified in the second column of the table shall be appended to the above URL base path.

Table 7.2.2-1: Operations supported by the Provisioning Sessions API

| Operation | Sub-resource path | Allowed HTTP method(s) | Description |
| --- | --- | --- | --- |
| Create Provisioning Session |  | POST | Used to create a new Provisioning Session resource. If the operation succeeds, the URL of the created Provisioning Session resource shall be returned in the Location header of the response. |
| Retrieve Provisioning Session | {provisioningSessionId} | GET | Used to retrieve a Provisioning Session resource for inspection. |
| Destroy Provisioning Session |  | DELETE | Used to destroy an existing Provisioning Session resource. |

### 7.2.3	Data model

#### 7.2.3.1	ProvisioningSession resource

The data model for the ProvisioningSession resource is specified in Table 7.2.3.1-1 below. Different properties are present in the resource depending on the type of Provisioning Session indicated in the provisioningSessionType property, and this is specified in the Applicability column.

Table 7.2.3.1-1: Definition of ProvisioningSession resource

| Property name | Type | Cardinality | Usage | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| provisioningSessionId | String | 1..1 | C: R R: RO | A unique identifier for this Provisioning Session. | All types. |
| provisioningSession‌Type | Provisioning‌Session‌Type | 1..1 | C: RW R: RO U: – | The type of Provisioning Session. | All types. |
| aspId | AspId | 0..1 | C: W R: RO | The identity of the Application Service Provider responsible for this Provisioning Session, as specified in clause 5.6.2.3 of TS 29.514 [34]. | All types. |
| serverCertificateIds | Array(String) | 0..1 | C: – R: RO | A list of Server Certificate identifiers currently associated with this Provisioning Session. | downlink, uplink |
| contentPreparation‌TemplateIds | Array(String) | 0..1 | C: – R: RO | A list of Content Preparation Template identifiers currently associated with this Provisioning Session. | downlink, uplink |
| contentProtocols | String | 1..1 | C: R R: RO | The Content Protocols resource identifier. Fixed value specified as the sub-resource path in table 7.5.2-1. | downlink, uplink |
| contentHosting‌ConfigurationId | String | 0..1 | C: – R: RO | The Content Hosting Configuration identifier currently associated with this Provisioning Session, if any. | downlink, uplink |
| consumptionReporting‌ConfigurationId | String | 0..1 | C: – R: RO | The Consumption Reporting Configuration identifier currently associated with this Provisioning Session, if any. | downlink |
| metricsReporting‌ConfigurationIds | Array(String) | 0..1 | C: – R: RO | A list of Metrics Reporting Configuration identifiers currently associated with this Provisioning Session. | downlink, uplink |
| policyTemplateIds | Array(String) | 0..1 | C: – R: RO | A list of Policy Template identifiers currently associated with this Provisioning Session. | downlink, uplink |

## 7.3	Server Certificates Provisioning API

### 7.3.1	Overview

The Server Certificates Provisioning API is used to provision X.509 [8] server certificates that can be referenced by a Content Hosting Configuration and subsequently presented a) for downlink streaming, by the 5GMSd AS when it distributes content to 5GMSd Clients at interface M4d using Transport Layer Security [12], and b) for uplink streaming, by the 5GMSu AS for reception of uplink content sent by 5GMSu Clients at interface M4u. Server Certificate resources are provisioned within the scope of an enclosing Provisioning Session.

### 7.3.2	Resource structure

The Server Certificates Provisioning API is accessible through the following URL base path:

{apiRoot}/3gpp-m1/v1/provisioning-sessions/{provisioningSessionId}/

Table 7.3.2-1 specifies the operations and the corresponding HTTP methods that are supported by this API. In each case, the Provisioning Session identifier shall be substituted into {provisioningSessionId} in the above URL template and the sub-resource path specified in the second column shall be appended to the URL base path.

Table 7.3.2-1: Operations supported by the Server Certificates Provisioning API

| Operation | Sub-resource path | Allowed HTTP method(s) | Description |
| --- | --- | --- | --- |
| Create Server Certificate | certificates | POST | Invoked on the Server Certificates collection associated with a Provisioning Session to request that the 5GMS System creates a new Server Certificate on behalf of the 5GMSd/5GMSu Application Provider. The request message body shall be empty. If the operation succeeds, the URL of the created Server Certificate resource shall be returned in the Location header of the response and this shall comply with the sub-resource path specified below for manipulating Server Certificate resources in the collection. The body of the response message may include a copy of the created X.509 certificate, as specified in clause 7.3.3.2 below. |
| Reserve Server Certificate | certificates?csr | POST | Invoked on the Server Certificates collection associated with a Provisioning Session to solicit a Certificate Signing Request for a new Server Certificate. The request message body shall be empty. If the operation succeeds, the URL of the reserved Server Certificate resource shall be returned in the Location header of the response and this shall comply with the sub-resource path specified below for manipulating Server Certificate resources in the collection. The body of the response shall be a PEM-encoded X.509 Certificate Signing Request, as specified in clause 7.3.3.1 below. |
| Retrieve Server Certificate | certificates/{certificateId} | GET | Used to retrieve a previously created or uploaded Server Certificate. If a Server Certificate resource has been reserved but not yet uploaded, this operation shall return 404 (Not Found). |
| Upload Server Certificate |  | PUT | Used by the 5GMSd/5GMSu Application Provider to supply a new Server Certificate in response to a solicited Certificate Signing Request. The body of the request message shall be a PEM-encoded X.509 certificate signed with the public key of the Certificate Signing Request, as specified in clause 7.3.3 below. The 5GMSd/5GMSu AF shall associate the Server Certificate with the private key it generated alongside the Certificate Signing Request. Attempting to update a previously uploaded Server Certificate is an error. |
| Destroy Server Certificate |  | DELETE | Removes the specified Server Certificate from the set of certificates associated with the Provisioning Session. |
| NOTE: The Server Certificate resource identifier {certificateId} differs from the serial number of the X.509 certificate. |  |  |  |

### 7.3.3	Data model

#### 7.3.3.1	Certificate Signing Request

The Certificate Signing Request shall comply with the Privacy-Enhanced Mail (PEM) textual format specified in RFC 7468 [17], i.e. a Base64-encoded DER certificate request or certificate, including leading and trailing encapsulation boundary lines.

The MIME content type shall be application/x-pem-file.

#### 7.3.3.2	Server Certificate resource

The Server Certificate resource shall comply with the Privacy-Enhanced Mail (PEM) textual format specified in RFC 7468 [17], i.e. a Base64-encoded DER certificate request or certificate, including leading and trailing encapsulation boundary lines. The resource shall include only the public parts of the X.509 certificate. In particular, the private key shall not be included.

The MIME content type shall be application/x-pem-file.

### 7.3.4	Operations

Under no circumstances shall the 5GMSd AF (5GMSu AF) reveal the private key associated with the Certificate Signing Request to the 5GMSd Application Provider (5GMSu Application Provider).

## 7.4	Content Preparation Templates Provisioning API

### 7.4.1	Overview

For downlink streaming (uplink streaming), Content Preparation Templates are used to specify manipulations applied by a 5GMSd AS (5GMSu AS) to media resources ingested at interface M2d (M2u) for distribution at ingest interface M4d (contribution at egest interface M4u). The Content Preparation Templates API is used to provision a Content Preparation Template within the scope of a Provisioning Session that can subsequently be referenced from a Content Hosting Configuration.

### 7.4.2	Resource structure

The Content Preparation Templates Provisioning API is accessible through the following URL base path:

{apiRoot}/3gpp-m1d/v1/provisioning-sessions/{provisioningSessionId}/

Table 7.4.2-1 specifies the operations and the corresponding HTTP methods that are supported by this API. In each case, the Provisioning Session identifier shall be substituted into {provisioningSessionId} in the above URL template and the sub-resource path specified in the second column shall be appended to the URL base path.

Table 7.4.2-1: Operations supported by the Content Preparation Templates Provisioning API

| Operation | Sub-resource path | Allowed HTTP method(s) | Description |
| --- | --- | --- | --- |
| Create Content Preparation Template | content-preparation-templates | POST | Invoked on a Content Preparation Templates collection when supplying a new Content Preparation Template resource. If the operation succeeds, the URL of the newly created Content Preparation Template resource shall be returned in the Location header of the response and this shall comply with the sub-resource path specified below for manipulating Content Preparation Templates. |
| Retrieve Content Preparation Template | content-preparation-templates/‌{contentPreparationTemplateId} | GET | Used to retrieve a Content Preparation Template resource. |
| Update Content Preparation Template |  | PUT, PATCH | Used to modify an existing Content Preparation Template resource. |
| Destroy Content Preparation Template |  | DELETE | Used to destroy an existing Content Preparation Template resource. |

### 7.4.3	Data model

The data model of the Content Preparation Template resource shall be determined by its MIME content type.

### 7.4.4	Operations

The operations shall be determined by the MIME content type of the Content Preparation Template resource.

## 7.5	Content Protocols Discovery API

### 7.5.1	Overview

The Content Protocols Discovery API is used by a 5GMS Application Provider to find out which content ingest or egest protocols are supported by the 5GMS AS(s) associated with a 5GMS AF. One of the supported ingest protocols is subsequently indicated in a Content Hosting Configuration for downlink streaming.

### 7.5.2	Resource structure

The Content Protocols Discovery API is accessible through the following URL base path:

{apiRoot}/3gpp-m1/v1/provisioning-sessions/{provisioningSessionId}/

Table 7.5.2-1 below specifies the operations and the corresponding HTTP methods that are supported by this API. In each case, the Provisioning Session identifier shall be substituted into {provisioningSessionId} in the above URL template and the sub-resource path specified in the second column of the table shall be appended to the URL base path.

Table 7.5.2-1: Operations supported by the Ingest Protocols Discovery API

| Operation | Sub-resource path | Allowed HTTP method(s) | Description |
| --- | --- | --- | --- |
| Fetch list of supported content protocols | protocols | GET | This operation is used to retrieve a list of supported content protocols. |

### 7.5.3	Data model

#### 7.5.3.1	ContentProtocols resource

The data model for the ContentProtocols resource is specified in table 7.5.3.1-1 below:

Table 7.5.3.1-1: Definition of ContentProtocols resource

| Property name | Data Type | Cardinality | Description |
| --- | --- | --- | --- |
| downlinkIngestProtocols | Array(Content‌Protocol‌Descriptor) | 0..1 | An array of ContentProtocolDescriptor objects, as specified in clause 7.5.3.2, each one uniquely identifying a content ingest protocol supported at interface M2d by the 5GMSd AS(s) associated with the corresponding 5GMSd AF. |
| uplinkEgestProtocols | Array(Content‌Protocol‌Descriptor) | 0..1 | An array of ContentProtocolDescriptor objects, as specified in clause 7.5.3.2, each one uniquely identifying a content egest protocol supported at interface M2u by the 5GMSu AS(s) associated with the corresponding 5GMSu AF. |
| geoFencingLocatorTypes | Array(URI String) | 0..1 | An array of fully-qualified term identifiers, each one indicating a content geo-fencing locator type supported by the 5GMS System. Every 5GMS System shall support at least the locator type urn:3gpp:5gms:locatortype:iso3166. |

#### 7.5.3.2	ContentProtocolDescriptor type

The data model for the ContentProtocolDescriptor type is specified in table 7.5.3.2-1 below:

Table 7.5.3.2-1: Definition of ContentProtocolDescriptor type

| Property name | Data Type | Cardinality | Description |
| --- | --- | --- | --- |
| termIdentifier | URI String | 1..1 | A fully-qualified term identifier from the controlled vocabulary urn:3gpp:5gms:content-protocol, as specified in clause 8. |
| descriptionLocator | URL String | 0..1 | The location of a description of the content protocol, for example the public web URL of its specification. |

## 7.6	Content Hosting Configuration API

[Editor’s note: This clause and its sub-clauses should be modified to describe, besides its use for downlink streaming, the ContentHostingConfiguration resource with regards to uplink streaming – e.g., replace IngestConfiguration and DistributionConfigurations which pertain to downlink streaming to EgestConfiguration and ContentProcessingConfiguration which pertain to uplink streaming. Details of such modifications exercise might be better deferred to the next release of this specification.]

## 7.7	Consumption Reporting Provisioning API

### 7.7.1	Overview

The Consumption Reporting Provisioning API is a RESTful API that allows a 5GMSd Application Provider to configure the downlink streaming related Consumption Reporting Procedure for a particular Provisioning Session at interface M1d. The different procedures are described in clause 4.2.5. The Consumption Reporting Configuration is represented by a ConsumptionReportingConfiguration, the data model for which is specified in clause 7.7.3 below. The RESTful resources for managing the Consumption Reporting Configuration is specified in clause 7.7.2.

---- <snipped> ----

## 7.8	Metrics Reporting Configuration API

### 7.8.1	Overview

The Metrics Reporting Configuration API allow an 5GMS System operator or a 5GMS Application Provider to configure the Metrics Collection and Reporting procedure pertaining to a downlink or uplink streaming service for a particular Provisioning Session at interface M1.

### 7.8.2	Resource structure

The Metrics Reporting Configuration API is accessible through the following URL base path:

{apiRoot}/3gpp-m1/v1/provisioning-sessions/{provisioningSessionId}/

Table 7.8.2-1 below specifies the operations and the corresponding HTTP methods that are supported by this API. In each case, the Provisioning Session identifier shall be substituted into {provisioningSessionId} in the above URL template and the sub-resource path specified in the second column of the table shall be appended to the URL base path.

Table 7.8.2-1: Metrics Reporting Configuration resource

| Operation | Sub-resource path | Allowed HTTP method(s) | Description |
| --- | --- | --- | --- |
| Create Metrics Reporting Configuration | metrics-reporting-configuration | POST | Create and optionally provide a configuration. If the operation succeeds, the URL of the created Metrics Reporting Configuration resource shall be returned in the Location header of the response. |
| Read Metrics Reporting Configuration | metrics-reporting-configuration/ {metricsReportingConfigurationId} | GET | Retrieve the values of an existing Metrics Reporting Configuration. |
| Update Metrics Reporting Configuration |  | PUT, PATCH | Provide initial upload of a new configuration, or either the modification of, or replacement to an existing configuration. |
| Delete Metrics Reporting Configuration |  | DELETE | Delete a configuration, disables reporting. |

### 7.8.3	Data model

#### 7.8.3.1	MetricsReportingConfiguration resource

The data model for the MetricsReportingConfiguration resource is specified in Table 7.8.3-1 below:

Table 7.8.3-1: Definition of MetricsReportingConfiguration resource

| Property name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| metricsReportingConfigurationId | String | 1..1 | An identifier for this Metrics Reporting Configuration that is unique within the scope of the enclosing Provisioning Session. |
| scheme | Array(URI String) | 0..1 | The scheme associated with this Metrics Reporting Configuration. A scheme may be associated with 3GPP or with a non-3GPP entity. For downlink streaming, if not specified, the 3GPP metrics scheme urn:‌3GPP:‌ns:‌PSS:‌DASH:‌QM10 from TS 26.247 shall apply. For uplink streaming, if not specified, the implication is that no associated uplink metrics reporting shall be performed. |
| dataNetworkName | String | 0..1 | The Data Network Name (DNN) which shall be used when sending metrics reports. If not specified, the default DNN shall be used. |
| reportingInterval | DurationSec | 0..1 | The time interval between successive metrics reports. If not specified, a single final report shall be sent after the streaming session has ended. |
| samplePercentage | Percentage | 0..1 | The proportion of streaming sessions for which metrics shall be reported. If not specified, reports shall be sent for all sessions. |
| urlFilters | Array(String) | 0..1 | A non-empty list of content URL patterns for which metrics shall be reported. If not specified, reporting shall be done for all URLs. |
| metrics | Array(String) | 0..1 | A non-empty list of metrics which shall be collected and reported. In the case of downlink streaming and for the 3GPP scheme urn:‌3GPP:‌ns:‌PSS:‌DASH:‌QM10 the listed metrics shall correspond to one or more of the metrics as specified in clauses 10.3 and 10.4, respectively, of TS 26.247 [7], and the quality reporting scheme and quality reporting protocol as defined in clauses 10.5 and 10.6, respectively, of [7] shall be used. If not specified, the complete (or default if applicable) set of metrics associated with the specified scheme shall be collected and reported. |

## 7.9	Policy Templates Provisioning API

### 7.9.1	Overview

The Policy Templates Provisioning API allow a 5GMS Application Provider to configure a set of Policy Templates within the scope of a Provisioning Session that can subsequently be applied to downlink or uplink media streaming sessions belonging to that Application Provider using the Dynamic Policies API specified in clause 11.5. A Policy Template is used to specify the traffic shaping and charging policies to be applied to these media streaming sessions.

A Policy Template, identified by its policyTemplateId, represents a set of PCF/NEF API parameters which defines the service quality and associated charging for the corresponding downlink or uplink media streaming session(s). The Policy Template is configured as part of the provisioning procedures with the 5GMS AF and is then used by the 5GMS AF to request specific QoS and charging policies for that session from the PCF or NEF.

The state of a Policy Template can be:

-	pending: The Policy Template is awaiting validation, potentially because not all required parameters have yet been provided. This is the default state after Policy Template creation.

-	invalid: One or more of the Policy Template's properties failed validation by the 5GMS AF.

-	ready: After successful validation by the 5GMS AF the Policy Template moves into this state.

-	suspended: The 5GMS AF may move a Policy Template into this state under certain conditions defined within the Service Level Agreement.

When the Policy Template is used for QoS Flows, the qoSSpecification object (of type M1QoSSpecification) shall be present:

-	The qosReference value is obtained with the Service Level Agreement. See TS 23.502 for detailed usage.

-	The maxBtrUl and maxBtrDl properties define the maximal bit rate which can be used for QoS Flows. This value is defined by the 5G System.

-	The maxAuthBtrUl and MaxAuthBtrDl properties define the maximal authorized bit rate values which can be requested by a Media Session Handler. Higher bit rate values are not authorized for use by the 5GMS Application Provider.

-	The minPacketLossRateDl and minPacketLossRateUl properties define the minimal authorized packet loss rate, which can be requested by a Media Session Handler.

When the Policy Template is used for differential changing the chargingSpecification property shall be present.

The ApplicationSessionContext Object is a mandatory object, which contains at least the aspId property.

-	The aspId identifies the API invoker.

-	The dnn property contains the Data Network Name of the data network, in which the 5GMS AF is hosted.

-	When Network Slicing is used, the sliceInfo property contains information about the network slice, which is serving the UE.

### 7.9.2	Resource structure

The Policy Template Provisioning API is accessible through the following URL base path:

{apiRoot}/3gpp-m1/v1/provisioning-sessions/{provisioningSessionId}/

Table 7.9.2-1 below specifies the operations and the corresponding HTTP methods that are supported by this API. In each case, the Provisioning Session identifier shall be substituted into {provisioningSessionId} in the above URL template and the sub-resource path specified in the second column shall be appended to the URL base path.

Table 7.9.2-1: Operations supported by the Policy Template Provisioning API

| Operation | Sub-resource path | Allowed HTTP method(s) | Description |
| --- | --- | --- | --- |
| Create a new Policy Template | policy-templates | POST | Used to create a new Policy Template resource. If the operation succeeds, the URL of the created Policy Template resource shall be returned in the Location header of the response. |
| Fetch a Policy Template | policy-templates/‌{policyTemplateId} | GET | Used to retrieve an existing Policy Template resource. |
| Update a Policy Template |  | PUT, PATCH | Used to modify the configuration of an existing Policy Template. |
| Delete a Policy Template |  | DELETE | Used to delete an existing Policy Template resource. |

### 7.9.3	Data model

#### 7.9.3.1	PolicyTemplate resource

The data model for the PolicyTemplate resource is specified in table 7.9.3-1 below:

Table 7.9.3-1: Definition of PolicyTemplate resource

| Property | Type | Cardinality | Usage | Visibility | Description |
| --- | --- | --- | --- | --- | --- |
| policyTemplateId | String | 1..1 | C: RO R: RO U: RO |  | Unique identifier of this Policy Template within the scope of the Provisioning Session. |
| state | Enumeration of Strings | 1..1 | C: RO R: RO U: RO |  | A Policy Template may be in the pending, ready, or suspended state. Only a Policy Template in the ready state may be instantiated as a Dynamic Policy Instance and applied to streaming sessions. |
| apiEndPoint | String | 1..1 | C: RW R: RO U: RW | MNO Admin | The API endpoint that should be invoked when activating a Dynamic Policy Instance based on this Policy Template. |
| apiType | Enumeration of Strings | 1..1 | C: RW R: RO U: RW | MNO Admin | N5: Npcf Policy Authorization Service. N33: AsSessionWithQoS or CHargableParty. |
| externalReference | String | 1..1 | C: RW R: RO U: RW |  | Additional identifier for this Policy Template, unique within the scope of its Provisioning Session, that can be cross-referenced with external metadata about the streaming session. |
| qoSSpecification | M1QoSSpecification | 0..1 | C: RW R: RO U: RW |  | Specifies the network quality of service to be applied to streaming sessions at this Policy Template. |
| ApplicationSession‌Context | Object | 1..1 |  |  | Specifies information about the application session context to which this Policy Template can be applied. |
| afAppId | AfAppId | 0..1 |  | Read-Only | As defined in clause 5.6.2.3 of TS 29.514 [34]. |
| sliceInfo | Snssai | 0..1 |  |  |  |
| dnn | Dnn | 0..1 |  |  |  |
| aspId | AspId | 0..1 |  |  |  |
| chargingSpecification | ChargingSpecification | 0..1 |  |  | Provides information about the charging policy to be used for this Policy Template. |

Editor's Note: The parameter externalReference is for further study. It may be a provisioning parameter of the Media Player/Media Streamer and/or a Media Session Handler to assist mapping of external references to a policyTemplateId.

Editor's Note: The ChargingSpecification object may contain any charging related information, such as sponId or afChargeId

END OF 4th CHANGE

5th CHANGE: Corrections to sub-clauses of clause 11

# 11	Media Session Handling (M5) APIs

## 11.1	General

This clause defines the Media Session Handling APIs used by the Media Session Handler to access resources exposed by the 5GMS AF at interface M5.

NOTE:	While the entirety of the Media Session Handling APIs apply to downlink streaming, only a subset is applicable to uplink streaming. Specifically, the Consumption Reporting API is not applicable to uplink streaming.

## 11.2	Service Access Information API

### 11.2.1	General

The Service Access Information API is used by the Media Session Handler to obtain configuration information from the 5GMS AF that enables it to use the other Media Session Handling APIs specified in clause 11.3 et seq.

### 11.2.2	Resource structure

The Service Access Information API is accessible through the following URL base path:

{apiRoot}/3gpp-m5/v1/service-access-information/

The operations and the corresponding HTTP methods in Table 11.2.2-1 are supported. In each case, the sub-resource path specified in the second column shall be appended to the URL base path.

Table 11.2.2-1: Operations supported by the Service Access Information API

| Operation | Sub-resource path | Allowed HTTP method(s) | Description |
| --- | --- | --- | --- |
| Fetch Service Access Information | {provisioningSessionId} | GET | Used to acquire the Service Access Information resource for the specified Provisioning Session. The {provisioningSessionId} uniquely identifies the Service Access Information Resource and is allocated by the 5GMS AF during creation of a Provisioning Session. |

### 11.2.3	Data model

#### 11.2.3.1	ServiceAccessInformation resource type

The data model for the ServiceAccessInformation resource is specified in Table 11.2.3.1-1 below. Different properties are present in the resource depending on the type of Provisioning Session from which the Service Access Information is derived (as indicated in the provisioningSessionType property) and this is specified in the Applicability column.

Table 11.2.3.1-1: Definition of ServiceAccessInformation resource

| Property name | Type | Cardinality | Usage | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| provisioningSessionId | String | 1..1 | RO | Unique identification of the M1 Provisioning Session. | All types |
| StreamingAccess | Object | 0..1 | RO |  | downlink |
| mediaPlayerEntry | URL String | 0..1 | RO | A document or a pointer to a document that defines a media presentation e.g. MPD for DASH content or URL to a video clip file. |  |
| ClientConsumptionReporting‌Configuration | Object | 0..1 | RO |  | downlink |
| reportingInterval | DurationSec | 0..1 | RO | The time interval, expressed in seconds, between consumption report messages being sent by the Media Session Handler. The value shall be greater than zero. When this property is omitted, a single final report shall be sent immediately after the streaming session has ended. |  |
| serverAddresses | Array(URL String) | 1..1 | RO | A list of 5GMSd AF addresses (URLs) where the consumption reporting messages are sent by the Media Session Handler. (Opaque URL, following the 5GMS URL format.) |  |
| locationReporting | Boolean | 1..1 | RO | Stipulates whether the Media Session Handler is required to provide location data to the 5GMSd AF in consumption reporting messages (in case of MNO or trusted third parties). |  |
| samplePercentage | Percentage | 1..1 | RO | The percentage of streaming sessions that shall send consumption reports, expressed as a floating point value between 0.0 and 100.0. |  |
| DynamicPolicyInvocationConfiguration | Object | 0..1 | RO |  | downlink, uplink |
| serverAddresses | Array(URL String) | 1..1 | RO | A list of 5GMSd AF addresses (URLs) which offer the APIs for dynamic policy invocation sent by the Media Session Handler. (Opaque URL, following the 5GMS URL format.) |  |
| validPolicyTemplateIds | Array(String) | 1..1 | RO | A list of Policy Template identifiers which the 5GMS Client is authorized to use. |  |
| sdfMethods | Array(SdfMethod) | 1..1 | RO | A list of recommended service data flow description methods (descriptors), e.g. 5-Tuple, ToS, 2-Tuple, etc, which should be used by the Media Session Handler to describe the service data flows for the traffic to be policed. |  |
| externalReferences | Array(String) | 0..1 | RO | Additional identifier for this Policy Template, unique within the scope of its Provisioning Session, that can be cross-referenced with external metadata about the streaming session. Example: "HD_Premium". |  |
| ClientMetricsReportingConfigurations | Array(Object) | 0..1 | RO |  |  |
| serverAddresses | Array(URL String) | 1..1 | RO | A list of 5GMS AF addresses to which metrics reports shall be sent. (Opaque URL, following the 5GMS URL format.) | downlink, uplink |
| dataNetworkName | String | 0..1 | RO | The DNN which shall be used when sending metrics reports. If not specified, the name of the default DN shall be used. | downlink, uplink |
| reportingInterval | DurationSec | 0..1 | RO | The time interval, expressed in seconds, between metrics reports being sent by the Media Session Handler. The value shall be greater than zero. When this property is omitted, a single final report shall be sent immediately after the streaming session has ended. | downlink, uplink |
| samplePercentage | Percentage | 1..1 | RO | The percentage of streaming sessions that shall report metrics, expressed as a floating point value between 0.0 and 100.0. | downlink, uplink |
| urlFilters | Array(String) | 0..1 | RO | A non-empty list of URL patterns for which metrics reporting shall be done. The format of each pattern shall be a regular expression as specified in [5]. If not specified, reporting shall be done for all sessions. | downlink, uplink |
| metrics | Array(String) | 1..1 | RO | A list of metrics which shall be reported. | downlink, uplink |
| NetworkAssistanceConfiguration | Object | 0..1 | RO |  | downlink, uplink |
| serverAddress | URL String | 1..1 | RO | Address of the 5GMS AF that offers the APIs for 5GMS AF-based Network Assistance, for access by the 5GMSd Media Session Handler. This address shall be an opaque URL, following the 5GMS URL format. |  |

### 11.2.4	Operations

This clause defines the behaviour that is expected from the 5GMS AF when a Service Access Information resource is acquired by the Media Session Handler. The main operation that is performed is to look up or generate the Service Access Information corresponding to the requested Provisioning Session.

## 11.3	Consumption Reporting API

### 11.3.1	General

The Consumption Reporting API allows the Media Session Handler to report downlink media consumption to the 5GMSd AF. The API defines data models, resources and the related procedures for the creation and management of the consumption reporting procedures. This procedure is configured by the ServiceAccessInformation resource, as defined in clause 11.2.3.

---- <snipped> ----

## 11.4		Metrics Reporting API

### 11.4.1	General

The Metrics Reporting API allows the Media Session Handler to send metrics reports to the 5GMS AF. This procedure is configured by the ServiceAccessInformation resource, as defined in clause 11.2.3. Note that multiple metrics configurations can be active at the same time, each identified by a unique metricsReportingConfigurationId.

### 11.4.2	Reporting procedure

Metrics reports related to a specific metricsReportingConfigurationId shall be submitted to one of the URLs selected from the ClientMetricsReportingConfiguration.serverAddresses array of the ServiceAccessInformation resource (see clause 11.2.3). The path of the URL should conform to the following general format:

{apiRoot}/3gpp-m5/v1/metrics-reporting/{provisioningSessionId}/{metricsReportingConfigurationId}

where {provisioningSessionId} shall be substituted by the 5GMS Client with the relevant Provisioning Session identifier and {metricsReportingConfigurationId} shall be substituted with the relevant Metrics Reporting Configuration identifier.

The only HTTP method supported by this endpoint is POST.

### 11.4.3	Report format

Metrics reports shall be submitted by the Media Session Handler in a format specified by the metrics reporting scheme in question. The Content-Type HTTP request header shall be set in accordance with the relevant metrics reporting scheme specification.

NOTE:	For downlink streaming, TS 26.247 [7] clauses 10.6.1 and 10.6.2 specify the required MIME content type and metrics report format for the 3GPP urn:‌3GPP:‌ns:‌PSS:‌DASH:‌QM10 metrics reporting scheme.

## 11.5	Dynamic Policies API

### 11.5.1	Overview

The Dynamic Policies API allows the Media Session Handler to request a specific policy and charging treatment to be applied to a particular application data flow of a downlink or uplink streaming session by invoking RESTful operations on the 5GMS AF at interface M5. The API defines a set of data models, resources and the related procedures for the creation and management of the dynamic policy request.

### 11.5.2	Resource structure

The Dynamic Policies API is accessible through the following URL base path:

{apiRoot}/3gpp-m5/v1/dynamicpolicies/

Table 11.5.2-1 below specifies the operations and the corresponding HTTP methods that are supported by this API. The sub-resource path specified in the second column shall be appended to the URL base path.

Table 11.5.2-1: Operations supported by the Dynamic Policies API

| Resource name | Sub-resource path | Allowed HTTP methods | Description |
| --- | --- | --- | --- |
| Dynamic Policies | policies | POST | Create a new Dynamic Policy resource. If the operation succeeds, the URL of the created Dynamic Policy Instance resource shall be returned in the Location header of the response. |
| Dynamic Policy | policies/{dynamicPolicyId} | GET | Read a Dynamic Policy resource. |
|  |  | PUT | Replace an existing Dynamic Policy resource. |
|  |  | PATCH | Modify an existing Dynamic Policy resource. |
|  |  | DELETE | Delete an existing Dynamic Policy resource. |

---- <snipped> ----

### 11.5.4	Operations

This clause defines the behaviour that is expected when activating a Dynamic Policy Instance. The policyTemplateId uniquely identifies the Policy Template, to which the Dynamic Policy Instance is associated. The provisioningSessionId associates the Dynamic Policy Instance to a Provisioning Session.

The Dynamic Policy resource contains a serviceDataFlowDescription property which contains the service data flow template according to TS 23.503. The ServiceDataFlowDescription shall contain one of:

-	a flowDescription Object (incl. 5-Tuples, Type of Service, Security Parameter Index, etc.).

-	a domainName.

When the Media Session Handler activate a QoS-related Dynamic Policy Template, then the qosSpecifcation property shall be present and it shall contain the following properties:

-	marBwDlBitRate or marBwUlBitRate, indicating the maximum requested bit rate by the Media Session Handler.

-	mirBwDlBitRate or mirBwUlBitRate, indicating the minimum requested bit rate by the Media Session Handler.

-	minDesBwDlBitRate or minDesBwUlBitrate, indicating the minimum bit rate desired by the Media Session Handler.

When the 5G System employs a traffic enforcement function to ensure that the traffic is complying a certain traffic policy, the Dynamic Policy resource may contain the following two properties:

-	an enforcementMethod, indicating the type of enforcement method (like leaky bucket).

-	an enforcementBitrate property, indicating the maximal permitted bit rate.

## 11.6	Network Assistance API

### 11.6.1	Overview

If AF-based Network Assistance is supported, then the Network Assistance API component of interface M5, as defined in the present sub-clause, is first used to provision a Network Assistance Session resource. The Network Assistance Resource can then be used to obtain bit rate recommendations and to issue delivery boost requests during the ongoing media streaming session.

### 11.6.2	Resource structure

The Network Assistance API is accessible via the following URL base path:

{apiRoot}/3gpp-m5/v1/network-assistance/

Table 11.6.2-1 below specifies the operations and the corresponding HTTP methods that are supported by this API. In each case, the sub-resource path specified in the second column of the table shall be appended to the URL base path.

Table 11.6.2-1: Operations supported by the Network Assistance API

| Operation | Sub-resource path | Allowed HTTP method(s) | Description |
| --- | --- | --- | --- |
| Create Network Assistance Session resource |  | POST | Provision a new Network Assistance Session. If the operation succeeds, the URL of the created Network Assistance Session resource shall be returned in the Location header of the response. |
| Fetch a Network Assistance Session resource | {naSessionId} | GET | Fetch the properties of an existing Network Assistance Session. |
| Update a Network Assistance Session resource | {naSessionId} | PUT, PATCH | Update the properties of an existing Network Assistance Session. |
| Request a bit rate recommendation | {naSessionId}/recommendation | GET | Obtain a bit rate recommendation for the next recommendation window. |
| Request a delivery boost | {naSessionId}/boostRequest | POST | Request a delivery boost for the next recommendation window. |
| Terminate Network Assistance Session | {naSessionId} | DELETE | Terminate a Network Assistance session. |

### 11.6.3	Data model

#### 11.6.3.1	NetworkAssistanceSession resource

The NetworkAssistanceSession resource is specified in table 11.6.3.1-1 below.

Table 11.6.3.1-1: Definition of NetworkAssistanceSession resource

| Property name | Type | Cardinality | Usage | Description |
| --- | --- | --- | --- | --- |
| naSessionId | String | 1..1 | C: RO R: RO U: RO | Unique identifier for this Network Assistance Session. |
| serviceDataFlowInformation | Array(ServiceDataFlowDescription) | 0..1 | C: RW R: RO U: RW | Identification of the application flows for the streaming session for which Network Assistance is to be used, e.g. 2-tuple (IP addresses) or 5-tuple (IP Addresses, protocol and ports). |
| policyTemplateId | String | 0..1 | C: RW R: RO U: RW | Identification of the policy that is in force for the streaming session. |
| requestedQoS | M5QoSSpecification | 0..1 | C: RW R: RO U: RW | The requested QoS parameters. |
| recommendedQoS | M5QoSSpecification | 0..1 | C: RO R: RO U: RO | The QoS parameters currently recommended by the 5GMS AF. |
| notficationURL | String | 0..1 | C: RO R: RO U: RO | A URL to the MQTT channel over which notifications are to be sent by the 5GMS AF for this session. When set, the Media Session Handler shall subscribe to this channel. The notification messages shall be in the form of the M5QoSSpecification data type. |

### 11.6.4	Operations

The 5GMS Client uses the POST method to create a Network Assistance session with the 5GMS AF. The AF returns the Network Assistance session identifier if session setup was successful, otherwise an error code is returned without a Network Assistance session identifier.

The 5GMS Client uses the Network Assistance session resource identifier (naSessionId) provided by the AF to refer all subsequent API calls to the AF applicable to that Network Assistance session.

The 5GMS AF populates the Network Assistance session resource with the service data flow information and optionally the policy template id that are valid for the streaming session for which Network Assistance operations are to be performed. The AF uses this information to execute Network Assistance operations in the 5GC.

The 5GMS Client uses the GET method with the Network Assistance Session resource identifier to retrieve a Network Assistance Session resource from the 5GMS AF. The AF returns the Network Assistance Session resource if retrieval was successful, otherwise an appropriate error code is returned without the session resource in case of failure.

The 5GMS Client uses the GET method with the sub-resource path specified in Table 11.6.2-1 to request a bit rate recommendation from the 5GMS AF. The 5GMS AF shall return the recommended bit rate in an HTTP response body of type M5QoSSpecification if a bit rate recommendation could be obtained, otherwise an appropriate HTTP error code shall be returned with no response body. For a downlink streaming session, the recommended minimum and maximum bit rates shall be indicated in the properties mirBwDlBitRate and marBwDlBitRate, respectively. For an uplink streaming session, the recommended minimum and maximum bit rates shall be indicated in the properties mirBwUlBitRate and marBwUlBitRate, respectively. If a unique recommendation is given by the 5GMSd AF then this recommended bit rate shall be set in both of these properties. The optional properties minDesBwDlBitRate, minDesBwUlBitRate, desLatency and desLoss shall not be included in the response.

The 5GMS Client uses the POST method with the sub-resource path specified in Table 11.6.2-1 to request a delivery boost from the 5GMS AF. The 5GMS AF shall respond with the OperationSuccessResponse data type indicating whether or not the delivery boost will be attempted by the network within an upcoming nominal time period.

The 5GMS Client uses the PUT or PATCH methods to replace the existing steaming session parameters with new settings. The 5GMS AF returns the NetworkAssistanceSession resource with settings resulting from the PUT or PATCH update operation.

The 5GMS Client uses the DELETE method to terminate the indicated Network Assistance session. The 5GMS AF returns an appropriate response code. If the termination was successful, then any subsequent calls referring to the terminated session will result in the error 404 (Not Found).

END OF 5h CHANGE