3GPP TSG- ad hoc Meeting #Post 112-e ad hoc	S4aI201122

# Online,  , 23rd February–25th March 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TR 26.802 | CR | — | rev | — | Current version: | 0.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Mapping of DVB-MABR and CableLabs MABR to 5MBS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BBC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5GMS_Multicast |  |  |  |  |  | Date: |  |  | 2021-02-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |
| Reason for change: |  | Initial solution to Key Issue #1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Mapping of DVB-MABR and CableLabs M-ABR logical functions to the 5MBS reference model - Mapping of DVB-MABR and CableLabs logical reference points to the 5MBS reference model |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Failure to progress the study of this Key Issue and requirements will not be fed into other aspects of solution design. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

(SNIPPED)

[13]	CableLabs OC-TR-IP-MULTI-ARCH-C01: "IP Multicast Adaptive Bit Rate Architecture Technical Report", October 2016. Internet Available https://www.cablelabs.com/specifications/ip-multicast-adaptive-bit-rate-architecture-technical-report

LAST CHANGE

## 5.2	Key Issue#1: Support of multicast ABR in 5G Media Streaming Architecture

(SNIPPED)

### 5.2.6	Mapping of DVB-MABR and CableLabs MABR reference architectures to 5MBS reference architecture for Scenario #1

#### 5.2.6.1	Introduction

With reference to Multicast ABR operation of a 5MBS-enhanced 5GMS Systerm as described in Scenario #1 (see clause 5.2.2), it is useful to map the logical functions and reference points of the DVB-MABR and CableLabs ABR reference models into the proposed 5MBS and 5GMS combined functional architecture that is summarised in clause 4.4 of the present document.

#### 5.2.6.2	Mapping of logical functions

Table 5.2.6.1 below provides a mapping of the DVB-MABR logical functions [12] reproduced in Figure 4.3.1.1-1, and those of the CableLabs reference model described in [13] to the proposed 5MBS and 5GMS combined functional architecture.

Table 5.2.6.1: Mapping of DVB-MABR and CableLabs M-ABR logical functions
to the 5MBS reference model

| DVB-MABR logical function | CableLabs M-ABR function | 3GPP 5MBS function | 3GPP 5GMS function |
| --- | --- | --- | --- |
| Content Provider control | Multicast controller |  | 5GMS Application Provider |
| Content Provider metrics capture |  |  |  |
| Content preparation | Packager |  |  |
| Content hosting | Origin/CDN |  | 5GMS AS (see NOTE) |
| Provisioning | BSS, NMS |  | 5GMS AF |
| Multicast server | Multicast server | MBSF, MBSTF |  |
| Content playback | IP STB | Media player (5GMS-enabled UE only) | Media player |
| Multicast rendezvous service | Transparent sessions handled using channel mapping technique. | MBSF Client? | Media Session Handler? |
| Multicast gateway | Embedded Multicast Gateway inside Gateway | 5MBS Client (incorporating MBSF Client and MBSTF Client) |  |
| Unicast repair | Multicast server (optional support for negative acknowledgements) | Packet-level unicast repair not supported for 5G Media Streaming. |  |
| NOTE: Also supports HTTP-based unicast repair. |  |  |  |

Packet-level unicast repair of lost multicast/broadcast packets (e.g. an RTP retransmission server) is not within the current scope of the SA2 architecture proposal in TR 23.757 [7]. In the above mapping, it is assumed that the 5MBS Client is able to perform efficient object-level unicast repair either directly or indirectly with the 5GMS AS.

The following open questions remain concerning this mapping:

1.	Which function provides the equivalent mechanism to the Multicast rendezvous service to support transparent session bootstapping to a 5GMS-Aware Application that is unaware of 5MBS operation. This function could be provided by the Media Session Handler or by the MBSF Client.

#### 5.2.6.2	Mapping of logical reference points

Table 5.2.6.2 below provides a mapping of the reference points defined by DVB-MABR logical reference model [12] reproduced in Figure 4.3.1.1-1, and those of the CableLabs reference model [13] to the proposed 5MBS and 5GMS combined functional architecture.

Table 5.2.6.2: Mapping of DVB-MABR and CableLabs logical reference points
to the 5MBS reference model

| DVB-MABR reference point |  | CableLabs M-ABR reference point | 3GPP 5MBS reference point | 3GPP 5GMS reference point |
| --- | --- | --- | --- | --- |
| CCP | Content provider provisioning | mc-pkg | (Nmbsmf, Npcf) | M1 |
| CMS | Multicast server configuration | mc-ms | Nmbsu (based on xMB-C) |  |
| CMR | Multicast gateway configuration | mc-emc | MBS-5 |  |
| Pin | Content ingest by Content hosting from Content preparation (unicast) | pkg-cdn |  | M2 |
| Pin′ | Push-based content ingest by Multicast server from Content preparation (unicast) | pkg-ms | MB2-U (see NOTE) |  |
| Oin | Pull-based content ingest by Multicast server from Content hosting (unicast) | cdn-ms |  | M2 or M4 clone |
| M | Multicast transport | ms-emc | MBS-4(-MC) |  |
| — | Multicast rendezvous configuration | Implicit | MBS-5 Session Announcement? | M5 configuration of Media Session Handler? |
| L | Content playback retrieval (unicast) | cpe |  | M7 |
| B | Session bootstrap (unicast) | Implicit |  | M6 or M7? |
| U | Packed-based repair (unicast) | ms-emc | Not supported for 5G Media Streaming |  |
| A | File-based repair (unicast) | cdn-gw | MBS-4(-UC) or M40 | M40 |
| RPM | Metrics reporting from Content playback | Out of scope |  | M6, M5 |
| RCP | Metrics reporting from Multicast gateway | mc-emc | MBS-5? |  |
| NOTE: Not applicable to 5G Media Streaming. |  |  |  |  |

A number of open questions arise from this initial mapping exercise:

1.	The reference point used by the MBSTF to ingest content from the 5GMS AS needs to be named. If it resembles the media ingest interface, supporting both push- and pull-based operating modes, it could be a clone of M2; if it more closely resembles the media consumption interface, it could instead be a clone of M4.

2.	How is the session bootstap mechanism (Multicast rendezvous service) configured to enable transparent operation? It could be configured by means of Session Annoumcements delivered via an MBS-5 Session Announcement Channel, or the Session Announcements could be delivered via unicast means (e.g. via M5 to the Media Session Handler).

3.	How is transparent operation achieved in practice? Which client-side function is able to intercept the initial presentation manifest request at the start of a streaming session and to determine that 5MBS operation is available? Is it necessary to modify the presentation manifest to achieve this goal, or is it possible to realise the desired transparency using HTTP redirects alone?

4.	Is the file-based repair mechanism part of the 5MBS architecture, an extended use of the 5GMS AS or does it straddle both architectures?

5.	Does the MBSF report 5MBS-related metrics back to the MBSF via MBS-5?

END OF CHANGES