3GPP TSG-S4 MBS Call	S4aI201140

# 4th March 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.803 | CR |  | rev |  | Current version: | 0.5.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EMSA call flow updates |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_EMSA |  |  |  |  |  | Date: |  |  | 04-03-2021 |
| Category: | <Cat> |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

### 6.3.2	Client-driven edge discovery

Figure 8 outlines a detailed call flow for client-driven session establishment.

Figure 8: Client-driven session establishment

The Edge Computing Provisioning phase is a provisioning phase, that may be repeated several times (e.g. to extend edge processing coverage to new geographical areas or to increase the capacity of an already provisioned area). All steps in this phase are optional and performed on need basis. The steps are:

1.	Spawn ECS: In this step, a new ECS is instantiated to manage new or increased demand for edge processing.

2.	Spawn 5GMS AF: In this step, a new 5GMS AF that is edge-enabled is instantiated to handle new or increased demand for media sessions with edge processing.

3.	EES Configuration: The EES is configured for a specific Edge Data Network.

4.	EES Registration with ECS: The EES registers with the ECS that is in authority over the target EDN.

The 5GMS Application Provider Provisioning phase is performed prior to the establishment of any related media streaming sessions by the 5GMS Application Provider. Subsequent updates to the provisioning session are possible.

5.	Create Provisioning Session: In this step, the 5GMS Application Provider creates a new provisioning session.

6.	Provision 5GMS features: In this step, the 5GMS Application Provider may create different configurations such as Content Hosting, Reporting, Edge Processing, etc.

During the UE Edge Computing Discovery phase, the UE discovers an EAS that is able to serve 5G media streaming session.

7.	Application Initialization: The user launches the 5GMS-Aware Application. The application performs any required initialization steps.

8.	Locate EAS/5GMS AS: The Application Client requests the location of a suitable EAS/5GMS AS that will be able to serve the application.

9.	Locate local EES: The EEC queries the ECS for a suitable EES.

10.	Locate EAS/5GMS AS: The EEC contacts the EES to query for an EAS/5GMS AS that can serve the session.

The optional subflow is for provisioning an additional 5GMS AS instance if a suitable EAS/5GMS AS cannot be located. The steps are:

11.	Check resource template: The 5GMS AF checks the provisioned edge processing resource template for the related application to determine the requirements of the application.

12.	Instantiate new EAS/5MGS AS: The 5GMS AF requests the MnS to instantiate a new EAS/5GMS AS with the specified requirements.

13.	Spawn 5GMS AS instance: The MnS creates a new instance of the EAS/5GMS AS with the requested placement and resources.

14.	EAS configuration: The newly instantiated EAS/5GMS AS is configured.

15.	Register EAS with EES: The newly instantiated EAS registers itself with the triggering EES.

16.	Configure provisioned features: This may include configuring and launching the server-side application in the 5GMS AS.

Completion of UE Edge Computing Discovery phase:

17.	5GMS AS location response: The EES/5GMS AF responds to the EEC/MSH with the location of the EAS/5GMS AS.

18.	5GMS AS location response: The location of the EAS/5GMS AS is passed back to the application.

After successful discovery, the actual streaming session may start in the 5GMS Session phase:

19.	Start session: The 5GMS-Aware Application invokes the Media Streamer with appropriate streaming access parameters (e.g. a Media Player Entry such as a DASH MPD URL).

20.	Session starting event: The application informs the Media Session Handler about the start of a new 5GMS session.

21.	Retrieve service access information: The Media Session Handler retrieves Service Access Information from the 5GMS AF appropriate to the 5GMS session.

22.	Media transfer: The 5GMS-Aware Application connects to the selected EAS/5GMS AS and the streaming starts.

23.	Method calls and notifications: Supporting information about the 5GMS session is passed from the Media Stream Handler to the Media Session Handler.

24.	Reporting, network assistance, and dynamic policy: The Media Session Handler exchanges supporting information about the 5GMS session with the 5GMS AF.

25.	End session: the 5GMS-Aware Application informs the Media Session Handler that the 5GMS session has ended.

26.	Session ending event: The Media Streamer informs the Media Session Handler about the end of the 5GMS session.

27.	Final reporting: The Media Session Handler performs any final reporting to the 5GMS AF.

Second Change

### 6.3.3	AP-driven management of 5GMS edge processing

Figure 9 outlines a detailed call flow for the AP-driven management of edge processing. In the previous sequence, the optional provisioning of an additional 5GMS AS instance occurs in response to an explicit call from the Application Client to an on-board EEC, whereas in this sequence it occurs, if needed, as part of and in response to Application Provider provisioning. It may also be repeated at any time during the lifetime of the 5GMS Provisioning Session in response to satisfy demand levels (not illustrated for brevity).

Figure 9: AP-driven management of 5GMS edge processing

The Edge Computing Provisioning phase is a provisioning phase, that may be repeated several times (e.g. to extend edge processing coverage to new geographical areas or to increase the capacity of an already provisioned area). All steps in this phase are optional and performed on need basis. Steps 1–4 are identical to those described in clause 6.3.2 above.

The 5GMS Application Provider Provisioning phase is performed prior to the establishment of any related media streaming sessions by the 5GMS Application Provider. Subsequent updates to the provisioning session are possible. Steps 5–6 are identical to those described in clause 6.3.2 above.

The optional sub-flow to provision an additional 5GMS AS instance may be repeated multiple times on need basis to add new capacity or to increase existing capacity for edge processing. The edge processing capacity is tailored for the specific 5GMS Application Provider based on the information in the Provisioning Session. Steps 7–12 are identical to steps 11–16 described in clause 6.3.2 above.

After successful discovery, the actual streaming session may start in the 5GMS Session phase. Steps 13–21 are identical to steps 19–27 described in clause 6.3.2 above.

NOTE:	In this call flow, the Application Client (AC) and EEC are not used to discover the 5GMS AS location. Instead, a Media Player Entry may be provided to the Media Session Handler by the 5GMS AF in the Service Access Information at M5 (step 15) or otherwise the location of the 5GMS AS is provided directly to the 5GMS-Aware Application via (out of scope) interface M8.

Third Change

## 6.4	Identified Gaps in Architecture and Procedures

Editor’s Note: this section is FFS.