3GPP TSG SA4 ad hoc meeting	S4aI201145

# Online, 23rd February–25th March 2021	revision of S4aI201123

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TR 26.802 | CR | — | rev | — | Current version: | 0.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Alternative combined 5GMS/5MBS client architecture |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BBC, Enensys Technology, Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5GMS_Mulicast |  |  |  |  |  | Date: |  |  | 2021-02-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | 17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |
| Reason for change: |  | The sources believe that an alternative client architecture should be studied in addition to the one already captured in the Technical Report. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Function MBSU renamed MBSTF and reference point Nmbsu renamed Nx at SA2#134-e. - Minor adjustment to explanation of exsiting client architecture diagram for clartiy. - Additional reference architecture diagram and explanation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A relevant option will not be studied. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 4.4.1, 4.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S4aI201123 -> S4aI201145 |  |  |  |  |  |  |  |  |

FIRST CHANGE

## 3.2	Abbreviations

For the present document, the abbreviations given in TR 21.905 [5] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905.

5MBS	5G Multicast/Broadcast Service

5GMS	5G Media Streaming.

ABR	Adaptive Bit Rate.

AL-FEC	Application-Level Forward Error Correction

ATSC	Advanced Television Systems Committee

BM-SC	Broadcast-Multicast - Service Centre

CMAF	Common Media Application Format

DASH	Dynamic Adaptive Streaming over HTTP

DNS	Domain Name Service

DVB	Digital Video Broadcasting

FEC	Forward Error Correction

FLUTE	File deLivery over Unidirectional Transport

HLS	HTTP Live Streaming

HTTP	HyperText Transfer Protocol

IGMP	Internet Group Management Protocol

IPTV	Internet Protocol Television

ISO BMFF	International Standardization Organization Base Media File Format

MABR	Multicast ABR

MBMS	Multimedia  Broadcast/Multicast Service

MBS	Multicast/Broadcast Service

MBSF	Multicast/Broadcast Service Function

MBSTF	Multicast/Broadcast Service Transport Function

MLD	Multicast Listener Discovery

MPEG	Moving Picture Experts Group

OTT	Over-The-Top

RoHC	Robust Header Compression

ROUTE	Real-time transport Object delivery over Unidirectional Transport

TMGI	Temporary Mobile Group Identity

XML	Extensible Markup Language

NEXT CHANGE

### 4.4.1	Baseline Network Reference Architectures

#### 4.4.1.1	General

(SNIPPED – NO CHANGES)

#### 4.4.1.2	5GMSA functions in the Trusted DN

The following diagram illustrates a network reference architecture with all 5GMS and 5MBS functions within the Trusted DN. A 5GMS Application Provider (typically) in an External DN configures the 5GMS features via a Release 17 version of M1d interface. Two different models are considered:

1:	The usage of 5MBS for media distribution is completely hidden from the 5GMS Application Provider. The 5GMS System selects usage of 5MBS based on internal criteria.

2:	By means of 5GMS provisioning procedures at (extended) M1d, the 5GMS Application Provider explicitly controls the potential usage of 5MBS in certain areas and for certain content. For example, some content might not be authorized for 5MBS distribution by content rights owners. Or, some content might only be authorized for 5MBS distribution.

Figure 4.4.1.2-1: 5MBS architecture combined with 5GMS hosted in Trusted DN

#### 4.4.1.3	5GMSA functions in an External DN

The following diagram illustrates a network reference architecture with all 5GMS within an external DN. Only the MBSU resides inside a trusted DN. A 5GMS Application Provider (typically) in an external DN configures the 5GMS features via a Release 17 version of M1d interface.

Figure 4.4.1.3-1: 5MBS architecture combined with 5GMS hosted in External DN

NEXT CHANGE

### 4.4.2	Client Architectures

#### 4.4.2.1	Introduction

(SNIPPED – NO CHANGES)

#### 4.4.2.2	Standalone 5MBS client architecture

Figure 4.4.2.2-1 provides an architecture for which 5MBS is used independently of 5GMS. Note that the network part may have different instantations and implementation models, and is hence is not fully documented.

Figure 4.4.2.2-1: 5MBS architecture independent of 5GMS

The key aspects of the client architecture are:

1.	The existence of a 5MBS client on the UE. This client is expected to have similar functionalities of an MBMS client as defined in TS 26.346 [21].

2.	An API from the 5MBS client to the MBSF for the purpose of 5MBS control plane and service handling referred to as interface MBS-5. It is expected that this API has similar functionalities to those of the User Service Description as defined in TS 26.346 [21].

3.	An interface between the 5MBS Client and the MBSTF for the purpose of 5MBS user data exchange at MBS-4. This interface includes:

-	MBS-4-MC dealing with unidirectional and multicast delivery from the MBSTF to the 5MBS client. It is expected that this interface has similar functionalities as defined in the delivery methods defined in TS 26.346 [21].

Editor’s Note: 	Whether or not the architecture requires an interface MBS-4-UC for bidirectional and unicast-based delivery between the 5MBS AS and the 5MBS Client, and how the 5MBS AS is configured, is for further study.

4.	An interface MBS-8 between the 5MBS Application Provider and the 5MBS Aware-Application in order to announce 5MBS services.

5.	An API-based interface MBS-6 exposed by the 5MBS Client and used by the 5MBS-Aware Application to manage and control 5MBS services. It is expected that this API has similar functionalities to the control interfaces defined in clause 6 of TS 26.347 [21].

6.	An API-based interface MBS-7 exposed by the 5MBS Client and used by the 5MBS-Aware Application to receive user data information about 5MBS services. It is expected that this API has similar functionalities as the data interfaces defined in clause 7 of TS 26.347 [21].

NOTE:	In the case where the 5MBS Client is deployed in a 5G Residential Gateway and the 5MBS-Aware Application is deployed in a separate end device, reference points MBS-6 and MBS-7 span the network interface between these devices.

A further decomposition of the above client architecture is provided in Figure 4.4.2.2-2 for which the 5MBS Client is separated into two components, namely:

- A MBSF Client communicating with the MBSF function and predominantly dealing with user service description aspects. This function exposes the aforementioned MBS-6 API.

- A MBSTF Client communicating with the MBSTF function for delivery functions. This function exposes the aforementioned MBS-7 API.

Figure 4.4.2.2-2: Extended 5MBS architecture independent of 5GMS

Two new APIs are introduced, namely:

- MBS-6′ that predominantly provides an API to control and manage the delivery functions of the MBSTF Client.

- MBS-7′ that provides information logically assigned to MBS-5 delivery (user service information) through MBS-7′.

Some open question remain on details (see also clause 5.6 key issues):

- Are there deployments that do not require an MBSF Client and hence, MBS-6′ is directly exposed to the 5MBS defined application.

- Are there deployments for which no unicast is used? In this case MBS-5 is completely served through MBS-7′.

- Is it useful to separate the MBSTF Client into a multicast delivery and a unicast delivery component?

- On the MBS-4-UC requests, which unicast requests are proxied through 5MBS, for example to detect consumption, and which are served through M8?

NOTE:	The 5MBS-Aware Application itself may be a media function and may include a media player, independently of the 5GMS architecture.

#### 4.4.2.3	5GMS client architecture using 5MBS (option A)

Based on the architectures in clause 4.4.2.2, the 5GMS Client can be viewed as a 5MBS-Aware Application and the 5GMS-Aware Application is basically unaware of 5MBS usage. The network aspects are again not detailed here: it is assumed that MBSF and MBSTF exist.

Figure 4.4.2.3-1: Combined 5GMS and MBS client architecture (option A)

In this case:

-	The Media Session Handler, based on information via M6 and M5, identifies the existence of a 5GMS service that is available through 5MBS and uses this information to:

-	Initialize and control the 5MBS through MBS-6.

-	Initialize the Media Player through M7, to the extent that MBS-7 is used for service delivery.

- The Media Player, based on static and dynamic information from the 5MSB client uses MBS-7 and/or M4 for receiving media.

Some open question remain on details (see also clause 5.6 key issues):

-	Where is the manifest served from, through MBS-7, through M4, both, or either?

-	How different deployment scenarios in terms of hybrid, MooD, can be realized?

#### 4.4.2.4	5GMS client architecture using 5MBS (option B)

The alternative client architecture depicted in Figure 4.4.2.4-1 below combines the 5GMS architecture with that of 5MBS. It differs from Figure 4.4.2.3-1 in the use of an additional core function, the 5MBS AS, to terminate unicast file repair requests from the 5MBS Client at reference point MBS-4-UC. The interface at this reference point is technically identical to that at M4, i.e. unicast HTTP.------ The Media Player continues to use reference point M4 for non-5MBS media retrieval from the 5GMS AS.

Figure 4.4.2.4-1: Combined 5GMS and MBS client architecture (option B)

In practical deployments that combine 5G Media Streaming with 5MBS, the MBSF is likely to be co-located with the 5GMS AF, as described in clause 4.4.1 of the present document. In addition, the 5MBS AS is likely to be co-located with the 5GMS AS in such deployments. Figure 4.4.2.4-2 below illustrates this likely deployment architecture.

Figure 4.4.2.4-2: Combined 5GMS and MBS client architecture (option B) depicting likely co-location

In both of the above figures, the 5MBS AS exposes an interface at reference point MBS-4-UC that is technically identical to M4, and which is potentially useful to the 5MBS Client in at least the following contexts:

1.	Fast 5GMS session start-up via unicast while the 5MBS Client is waiting for initial multicast/broadcast packets to start arriving via MBS-4.

2.	Unicast recovery of the application payload data carried in multicast/broadcast packets that are not successfully received via MBS-4, in a manner that is transparent to the 5GMS Client.

3.	5GMS session continuity when multicast/broadcast service is temporarily unavailable, in a manner that is transparent to the 5GMS Client.

4.	Switching the operating mode of a 5GMS session to unicast under the direction of network-based multicast operation on demand (MooD), in a manner that is transparent to the 5GMS Client.

Relevant issues to be studied include:

-	Shared use of a single PDU Session by M4 and MBS-4-UC. (The solution to this may be trivial.)

-	Whether the presentation manifest is delivered via M4 or MBS-7.

-	How different deployment scenarios such as hybrid operation and MooD are realised.

END OF CHANGES