F3GPP TSG SA WG4  MBS SWG post 112-e	S4aI201154	revision of S4aI201147

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.804 | CR | <CR#> | rev | - | Current version: | 0.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_5GMS-EXT] Updated text for Content Preparation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Tencent |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5GMS-EXT |  |  |  |  |  | Date: |  |  | 2021-01-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The study item description identifies the key topic “Content Preparation”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add two more collaboration scenarios and associated call flows for all collaboration scenarios |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Key topic not addressed |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| 56 |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

=====  CHANGE  =====

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 26.501: " 5G Media Streaming (5GMS); General description and architecture".

[3]	3GPP TS 26.511: "5G Media Streaming (5GMS); Profiles, codecs and formats".

[4]	3GPP TS 26.512: "5G Media Streaming (5GMS); Protocols".

[V]	ISO/IEC 13818-1: "Information technology — Generic coding of moving pictures and associated audio information: Systems".

[W]	SCTE-35: "Digital Program Insertion Cueing Message for Cable".

[Z]	ISO/IEC 23000-19:2018: "Information technology — Multimedia application format (MPEG-A) — Part 19: Common media application format (CMAF) for segmented media".

[ZZ]	ISO/IEC 23009-1:2021: "Information technology — Dynamic adaptive streaming over HTTP (DASH) — Part 1: Media presentation description and segment formats".

[X]	VSF TR-06-01, RIST Simple Profile, https://www.videoservicesforum.org/download/technical_recommendations/VSF_TR-06-1_2018_10_17.pdf

[Y]	VSF TR-06-02, RIST Main Profile, https://www.videoservicesforum.org/download/technical_recommendations/VSF_TR-06-2_2020_03_24.pdf

=====  CHANGE  =====

# 4	Introduction to 5G Media Streaming

## 4.1	Introduction

## 4.2	Collaboration Scenarios

## 4.3	Architectures

## 4.4	Summary of Stage-3 enablers

=====  CHANGE  =====

# 5	Key Topics

## 5.1	Introduction

## 5.2	Content Preparation

### 5.2.1	Overview

[TS 26.501 includes the high-level concept of content preparation and/or modification within the 5G Media Streaming System. However, Content Preparation has not yet been defined in detail in TS 26.512. The extent of content preparation support (including. use- cases, functionalities and features, and resulting formats) needed in the existing 5G Media Streaming Architecture, and how these functions can be realized in with current 5GMS architecture are subject of this study.]

Content preparation may be applied in the following scenarios:

- In uplink streaming: on an uplinked stream, providing the resulting content to Application Provider.

- In downlink streaming: on an input stream from an Application Provider, providing the resulting content for distribution.

- Between uplink and downlink streaming: on an uplink stream, providing the resulting content for distribution.

Two aspects are of relevance for the discussion on Content Preparation:

- Uplink ingest content formats and protocols

- Content preparation instructions

On uplink ingest formats, a survey of existing formats is needed to understand the available options. Examples include

- MPEG-2 Transport Stream [V] with SCTE metadata [W].

- CMAF content [Z] with timed metadata tracks.

- CMAF content [Z] with a manifest such as the MPD [ZZ].

- Reliable Internet Stream Transport (RIST) is an open-source, open specification transport protocol designed for reliable transmission of video over lossy networks (including the internet) with low latency and high quality. It is currently under development under the Video Services Forum's "RIST Activity Group." To date, two open specifications have been produced:

  - TR-06-1 (RIST Simple Profile) [X], first released in 2018, defines the basic ARQ (Automatic Repeat Query) technology used to recover lost packets. It has been updated in 2020 to include an optional "RTT Echo" message that streamlines the process of configuring packet buffers.

  - TR-06-2 (RIST Main Profile) [Y], released in 2020, defines additional functions required for commercial applications, including stream encryption, sender/receiver authentication, in-band data tunneling, and bandwidth optimization.

- Secure Reliable Transport (SRT) is an open-source video transport protocol maintained by the SRT Alliance.

### 5.2.2	Gap Analysis of 26.512

TS 26.512 [4] defines a very limited set of features for content preparation:

1.	The location for content preparation template provisioning.

2.	CRUD Operation for content preparation template.

The current specification neither defines the uplink ingest formats nor the content preparation instructions.

### 5.2.3	Use-cases

#### 5.2.3.1	Basic CMAF/DASH/HLS multi-rate live streaming of user-generated content

Use Case Name Basic CMAF/DASH/HLS multi-rate live streaming of user-generated content

Description Kim is subscribed to an Application for live streaming of captured videos from her everyday life. Based on the previous number and diversity of Kim’s usual audience (e.g. close friends), the Application/Application Provider has an “audience codecs-rates” profile which represents the typical number of streams based on Kim’s previous streaming sessions and provides the corresponding Content Preparation Template to the MNO as Kim starts the session live stream session. While Kim is streaming a single stream through her device’s uplink, the MNO processes the received content based on the Content Preparation Template and provides multi-rate tracks for distribution. If new users join Kim’s streaming session which cannot be supported with the current codecs-rates, the Application/Application Provider may update the Content Preparation Template in the session, and the MNO updates the content preparation process accordingly.

Categorization Delivery: Live Streaming Device: Any device connected to the cell network

Preconditions - On the device: - A 3GPP-supported encoder is installed. - UE is 5GMS capable. - UE’s Application is installed which supports 5GMS. - On the network: - The AS may or may not have resources for running a requested content preparation with the update to the Content Preparation Template and responds accordingly.

Nominal Cost Analysis The cost of service increases linearly with the number of uplink ingest streams. The cost of service increases less than linearly with the number of download streaming clients because the encoding and caching requirements are common to a large number of viewers.

Potential Standardization Status and Needs The Content Preparation Template is expected to define uplink ingest format(s) as well as the following instructions for content preparation: 1. Input characteristics. 2, Outputs characteristics. 3. The media processes and/or functionalities applied. 4. mechanism for updating the content preparation template Content preparation for CMAF streaming when the uplink ingest format in CMAF and output is CMAF content with DASH and HLS manifests for live and on-demand services. In this case, the Content Preparation Template should include the following: - Uplink ingest characteristics such as: - Video characteristics - Audio characteristics - Format of the subtitle track(s) - Metadata tracks and presence of media track events - Ingest protocols - Instructions such as - CMAF/DASH publishing profile - Configuration of Switching sets: - Ladder of bit rate/quality - The encoder configuration for each track - Nominal and maximum segment duration - Frequency and characteristics of random access and switching points - Initialization segment characteristics - Content protection scheme and encryption mode - Metadata and annotation such as roles, languages, ratings, and accessibility - Metadata tracks associated with the switching set or individual tracks - Media track events to be included in each track - Chunk encoding for low latency streaming - Period structure, splicing opportunities, and conditions - 5GMSd AS URL exposed at M4 - Segments Addressing modes - Trick mode and thumbnail navigation tracks - Service description - Content splicing/ad opportunity signaling - Subtitle generation from the uplink ingest audio track(s) - Multi-language support (when audio tracks for two or more languages are ingested). - Manifest format, annotations, and metadata

### 5.2.4	Collaboration Scenarios

### 5.2.4.1	Content preparation before downlink streaming

In this collaboration, the 5GMSd Application Provider requests content preparation for its stream before distribution. Figure 5.2.4.1-1 shows such a scenario.

UE

5GMSd

Client

5GMSd

Aware Application

5GMSd AS

External DN

5GMSd

Application Provider

M1d

M2d

Trusted DN

M4d

5GMSd AF

M5d

PCF

M8d

N5

Figure 5.2.4.1-1: Content preparation before downlink streaming

In this case:

1.	The Application Provider defines the required content preparation and requests the 5GMSd AF to create the process using the Content Preparation Template (CPT) through M1d.

2.	The 5GMSd AF configures the 5GMSd AS according to the media transformation(s) specified in the Content Preparation Template (M3d) and responds to the Application Provider (M1d).

3.	The media is streamed from Application Provider to 5GMSd AS (M2d).

4.	The content is transformed by 5GMSd AS according to the Content Preparation Template and provided for distributions (M4d).

5.	The Application Provider may update, retrieve, or destroy the Content Preparation Template using M1d.

### 5.2.4.2 Content preparation after uplink ingest streaming

In this collaboration, the 5GMSu Application Provider or 5GMSu Aware Application requests content preparation for the uplink ingest stream and delivery to 5GMSu Application Provider. Figure 5.2.4.2-1 shows such a scenario.

UE

5GMSu

Client

5GMSu

Aware Application

5GMSu AS

External DN

5GMSu

Application Provider

M1u

M2u

Trusted DN

M4u

5GMSu AF

M5u

PCF

M8u

N5

Figure 5.2.4.2-1: Content preparation after uplink ingest streaming

In this case:

1.	The Application Provider defines the required content preparation and requests the 5GMSu AF to create the process using the Content Preparation Template (CPT) through M1u.

2.	The 5GMSu AF configures the 5GMSu AS according to the media transformation(s) specified in the Content Preparation Template (M3u) and responds to the Application Provider (M1u).

3.	The media is streamed from 5GMSu Client to 5GMSu AS (M4u).

4.	The content is transformed by 5GMSu AS according to the Content Preparation Template and egested to the Application Provider (M2u).

5.	The Application Provider/Application may update, retrieve, or destroy the Content Preparation Template using M1u/M5u.

### 5.2.4.3 Content preparation between uplink ingest and downlink streamings

In this collaboration, the 5GMSu Application Provider requests content preparation for the uplink ingest stream from one UE and before downlink streaming to other UEs. Figure 5.2.4.3-1  shows such a scenario.

UE

5GMSu Client

5GMSu Aware Application

5GMSu AS

External DN

5GMS Application Provider

M1u

Trusted DN

M4u

5GMSu AF

M5u

PCF

M8u

N5

5GMSd AS

5GMSd AF

M1d

UE

5GMSd Client

5GMSd Aware Application

M4d

M5d

M3u

M3d

I2

I1

M8d

Figure 5.2.4.3-1: Content preparation after uplink ingest streaming

In this case:

- The Application Provider defines the required content preparation and requests the 5GMSd AF to create the process using the Content Preparation Template (CPT) through M1d.

- The 5GMSd AF configures the 5GMSd AS according to the media transformation(s) specified in the Content Preparation Template (M3d) and responds to the Application Provider (M1d).

- The 5GMSu AF configures the 5GMSu AS according to the media transformation(s) specified in the Content Preparation Template (M3u) and responds to the Application Provider (M1u).

- The content is transferred to 5GMSd AS and transformed according to the Content Preparation Template (I2). I2 is an implementation specific interface and is considered out of scope of this document.

- The content is streamed to the 5GSd Client from 5GMSd AS (M4d).

- The Application Provider/Application may update, retrieve, or destroy the Content Preparation Template (M1d).

NOTE 1: In the above steps, the I1 interface is not used for the simplicity. In section 5.6, two alternative call flows are presented that use I1 interface. I1 is an implementation specific interface and is considered out of scope of this document.

NOTE 2: In the deployment of this collaboration scenario, 5GMSu AS and 5GMSd AS may be colocated and therefore the I2 interface is not exposed. Similarly, 5GMSu AF and 5GMSd AF may be colocated.

Editor’s Note: How does DistributionConfiguration relate to CPT? Does the CPT define the initial/generic formats (such as HLS and DASH) and DistributionConfiguration defines a subset of it for distribution?

### 5.2.5	Deployment Architectures

### [The deployment architectures of TS 26.501 Annex B would be also applicable to this topic with no change]

### 5.2.6	Mapping to 5G Media Streaming and High-Level Call Flows

The following sections outline the high-level call flows for various collaboration scenarios of section 5.2.4.

### 5.2.6.1 Call-flow for content preparation before downlink streaming

The call flow is shown in Figure 5.2.6.1-1.

Figure 5.2.6.1-1: High-level call flow for content preparation before downlink streaming

Steps:

- The 5GMSd Application Provider creates a Provisioning Session with the 5GMSd AF. During this request, a content preparation template is provided that defines the instructions for content preparation as well as the output formats.

- 5GMSd AF, based on the content preparation template (optionally discovers and) requests the 5GMSd AS to allocate content preparation and distribution resources. e.g. to allocate 5GMSd content ingest and distribution resources. The M3d procedures between 5GMSd AF and 5GMSd AS are not specified.

- 5GMSd AS starts the content preparation process (as well as allocating distribution resources).

- 5GMSd AS acknowledges the allocation of resources to 5GMSd AF through M3d response.

- 5GMSd AF acknowledges the allocation of content preparation as well as other distribution processes to the 5GMSd Application Provider through M1d.

The rest of the call flow is identical to the 5GMS download streaming process with the exception that the content processing is running on 5GMSd AS.

### 5.2.6.2 Call-flow for content preparation after uplink ingest streaming

The call flow is shown in Figure 5.2.6.2-1.

Figure 5.2.6.2-1: Call flow for content preparation after uplink ingest streaming

Steps:

- The 5GMSu Application Provider creates a Provisioning Session with the 5GMSu AF. During this request, a content preparation template is provided that defines the instructions for content preparation as well as the output formats.

- 5GMSu AF, based on the content preparation template (optionally discovers and) requests the 5GMSu AS to allocate content preparation and distribution resources. e.g. to allocate 5GMSu content ingest and distribution resources. The M3u procedures between 5GMSu AF and 5GMSu AS are not specified.

- 5GMSu AS starts the content preparation process (as well as allocating distribution resources).

- 5GMSu AS acknowledges the allocation of resources to 5GMSuAF through M3u response.

- 5GMSu AF acknowledges the allocation of content preparation as well as other distribution processes to the 5GMSu Application Provider through M1u.

The rest of the call flow is identical to the 5GMS uplinking ingest process with the exception that the content processing is running on 5GMSu AS.

### Call-flow for content between uplink ingest and downlink streamings

### Basic call flow for content between uplink ingest and downlink streamings

Figure 5.2.6.3-1 shows the call flow for this scenario.

Figure 5.2.6.3.1-1: High-level call flow for content preparation between uplink ingest and downlink streaming

Steps:

- The 5GMS Application Provider creates a Provisioning Session with the 5GMSd AF. During this request, a content preparation template is provided that defines the instructions for content preparation as well as the output formats.

- 5GMSd AF, based on the content preparation template (optionally discovers and) requests the 5GMSd AS to allocate content preparation and distribution resources. e.g. to allocate 5GMSd content ingest and distribution resources. The M3d procedures between 5GMSd AF and 5GMSd AS are not specified.

- 5GMSd AS starts the content preparation process (as well as allocating distribution resources).

- 5GMSd AS acknowledges the allocation of resources to 5GMSd AF through M3d response.

- 5GMSd AF acknowledges the allocation of content preparation as well as other distribution processes to the 5GMS Application Provider through M1d.

- The 5GMS Application Provider creates a Provisioning Session with the 5GMSu AF. During this request, the input of 5GMSd is defined.

- 5GMSu AF requests the 5GMSu AS to allocate 5GMSu content ingest and distributions to 5GMSd. The M3u procedures between 5GMSu AF and 5GMSu AS are not specified.

- 5GMSu AS acknowledges the allocation of resources to 5GMSuAF through M3u response.

- 5GMSu AF acknowledges the allocation of content preparation as well as other distribution processes to the 5GMS Application Provider through M1u.

10-14. The call-flow for uplink ingest streaming similar to the one in Figure 5.2.6.2-1 is executed.

15. 5GMSu streams the content to 5GMSd (I2). The interface I2 is not specified.

16-20.  The call-flow for downlink streaming similar to the one in Figure 5.2.6.1-1 is executed.

### 5.2.6.3.2 Alternative call-flows for content between uplink ingest and downlink streaming

A more compact way of the call flow for this case is when the Application provider provisions both downlink and uplink streaming at the same it. This call flow is shown in Figure 5.2.6.3.2-1.

Figure 5.2.6.3.2-1: An alternative more compact call flow for content preparation between uplink ingest and downlink streaming

Steps:

- The 5GMS Application Provider creates a Provisioning Session with the 5GMSd AF. During this request, a content preparation template is provided that defines the instructions for content preparation as well as the output formats and also provisioning for 5GMSu AF.

- 5GMSd AF, based on the content preparation template (optionally discovers and) requests the 5GMSd AS to allocate content preparation and distribution resources. e.g. to allocate 5GMSd content ingest and distribution resources. The M3d procedures between 5GMSd AF and 5GMSd AS are not specified.

- 5GMSd AS starts the content preparation process (as well as allocating distribution resources).

- 5GMSd AS acknowledges the allocation of resources to 5GMSd AF through M3d response.

- 5GMSd AF requests 5GMSu AF to provision the uplink streaming by sending the relevant templates received from the Application Provider (I1).  The procedures between 5GMSd AF and 5GMSu AF are not specified.

- 5GMSu AF requests the 5GMSu AS to allocate 5GMSu content ingest and distributions to 5GMSd. The M3u procedures between 5GMSu AF and 5GMSu AS are not specified.

- 5GMSu AS acknowledges the allocation of resources to 5GMSu AF through M3u response.

- 5GMSu AF acknowledges the allocation of content preparation as well as other distribution processes to the 5GMSd AF.

- 5GMSd AF acknowledges the allocation of content preparation as well as other uplink and distribution processes to the 5GMS Application Provider through M1d.

10-14. The call-flow for uplink ingest streaming similar to the one in Figure 5.2.6.2-1 is executed.

15. 5GMSu streams the content to 5GMSd (I2). The interface I2 is not specified.

16-20.  The call-flow for downlink streaming similar to the one in Figure 5.2.6.1-1 is executed.

In the above call-flow, the Application Server provisions the entire session using the interface to 5GMSd AF. Alternatively, the Application Server may provision the entire session using the interface to 5GMSu AF as shown in Figure 5.3.6.2-2.

Figure 5.2.6.3.2-2: Another alternative more compact call flow for content preparation between uplink ingest and downlink streaming

Note that in the alternative call flows, the interface M1d and M1u are extended to support provisioning for both uplink and downlink and therefore they must be noted as M1 interface.

### 5.2.7	Potential open issues

Editor’s Note: Identify the issues that need to be solved.

### 5.2.8	Candidate Solutions

Editor’s Note: Provide candidate solutions (including call flows) for each of the identified issues.