3GPP TSG-SA4 Post Meeting #113e	S4aI211165

# Electronic Meeting, Telco,  >14, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.805 | CR | - | rev | - | Current version: | 0.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Overview of NMOS functionality |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BBC, Ericsson LM, EBU, Sennheiser |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_NPN4AVProd |  |  |  |  |  | Date: |  |  | <Res_date> |
| Category: | <Cat> |  |  |  |  |  | Release: |  |  | <Release> |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The new study FS_NPN4AVProd includes some objectives around existing orchestration solutions. The present document introduces the Network Media Open Specification (NMOS), which provides many configuration and control procedures for Media Production workflows. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A short overview of the key conecpts of NMOS is provided. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First change ****

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]		3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]		3GPP TS 22.261: "Service requirements for the 5G system".

[3]		3GPP TS 22.263: "Service requirements for Video, Imaging and Audio for Professional Applications (VIAPA)".

[4]		3GPP TS 22.827: "Study on Audio-Visual Service Production".

[x0]	EBU: "Minimum User Requirements to Build and Manage an IP-Based Media Facility", 15 July 2020 https://tech.ebu.ch/files/live/sites/tech/files/shared/tech/tech3371.pdf.

[x1]	AMWA: "NMOS Overview", https://www.amwa.tv/nmos-overview.

[x2]	EBU: "The Technology Pyramid For Media Nodes", https://tech.ebu.ch/publications/technology_pyramid_for_media_nodes.

[x]		EBU: "Technology Pyramid Media Node Maturity Checklist", September 2021,  https://tech.ebu.ch/publications/technology-pyramid-media-node-maturity-checklist?rec=1.

[y]		AMWA: "NMOS Technical Overview", https://specs.amwa.tv/nmos/branches/main/docs/2.0._Technical_Overview.html.

[z]		AMWA: "Networked Media Systems – the Big Picture", https://static.amwa.tv/networked-media-systems-big-picture-2021-03-05.pdf.

[zz]	AMWA: "NMOS specification repository", https://specs.amwa.tv/nmos.

**** Next Change ****

# 5	Review of existing orchestration and control solutions

## 5.1 	General

Editor’s Note: This is a placeholder section for some general text around orchestration

The professional broadcast industry uses a range of legacy and proprietary approaches that have been developed over many years to provide operational control. The lack of a consistent approach to interoperability has caused complexity in the architecture and integration of broadcast facilities. The Networked Media Open Specifications [x1] have been developed as a response to this problem as the industry transistions to an all-IP approach. The set of specifications are primarily used for media orchestration and control purposes. Media prchestration refers to the procedures of instantiate needed media processing functions in virtualized environments and provide the control functionality for workflow management. The control functionality can be broken down into three main areas:

-	Discovery and registration: procedures to register and identify all available functions in the media production network and their capabilities

-	Media Routing configuration: define sources and sinks for media related traffic flows

-	Operational control: changes during configurations, like changing capture setting, etc.

## 5.2	AMWA Network Media Open Specification (NMOS)

The Networked Media Open Specifications (NMOS) [x1] is a family of specifications produced by the Advanced Media Workflow Association (AMWA) and are related to networked media for professional applications. They were created to help enable automation in live IP-based architectures through control plane APIs that are built on typical patterns used for web services (REST, publish-subscribe). NMOS specifications are increasingly being adopted for applications using SMPTE ST 2110, and are part of the EBU’s Technology Pyramid for Media Nodes [x2][x0].

Editor’s Note: EBU has acknowledge the use of the figure in the TR.

Figure 5.2-1: EBU’s Technology Pyramid for Media Nodes

AMWA has defined a system template containing several building blocks in [y]. The system template contains four distinct layers, namely Media & Infrastructure, Control, Monitoring and Security. Figure 5.2-2 depicts Figure 3 from [y] for convenience.

Editor’s Note: AMWA has acknowledge the use of the figure in the TR.

Figure 5.2-2: Networked Media Systems Template – Showing the Roles of NMOS (Figure 3 from [y])

The Control layer contains:

-	Provisioning, containing Discovery and Registration, Device Configuration and System Parameters.

-	Media Routing, containing Flow Connection, Audio Channel Mapping and Network Routing.

-	Operational Control functions (Service Control and Event & Tally).

The Media layer is subdivided into Production, Service and Flows. For the present study, the content within the Flows box is mostly of interest. Flows are configured and controlled using the Flow Connection tool from the Control layer.

Further details of NMOS can be found at [x1], and the specifications are documented at [zz].

The most relevant NMOS specifications are depicted in also Figure 5.2-2:

-	AMWA IS-04 allows media nodes (i.e. networked media devices) to register themselves, along with what they are (or are capable of) sending or receiving, and allows control applications to query this information.

-	AMWA IS-05 allows control applications to set up and remove connections between media nodes.

-	AMWA IS-07 provides a publish-and-subscribe channel for sending time-based events such as tally information.

-	AMWA IS-08 specifies how to handle audio channels in NMOS APIs.

-	AMWA BCP-003 suite of specifications (including IS-10) covers secure communication and authorisation of NMOS APIs.

-	AMWA BCP-004-01_...

To date NMOS has mostly been used with ST 2110 [] uncompressed multicast video and audio streams within wired facilities. However, NMOS can be used with other types of streams, including unicast. There is growing interest in other areas, such as professional audiovisual applications using compressed video, and where media is streamed between facilities over WAN connections (VSF WAN group).

**** Last Change ****