3GPP TSG-SA4 meeting # post 113-e	S4al211170

# Electronic Meeting, April- May, 2021	revision of S4-210623

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TR 26.803 | CR | – | rev | – | Current version: | 1.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | General Call flow for EAS relocation based on the EMSA architecture |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei Technologies Co. Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_EMSA |  |  |  |  |  | Date: |  |  | 2021-4-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification). |  |  |  |  |  |  |  |  |  |
| Reason for change: |  | The general call flow for the EAS relocation based on the EMSA architecture is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the EAS relocation analysis and generic call flows. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

## 6.4	Call Flows for EAS relocation

### 6.4.1	Introduction

Under certain circumstances, it is necessary to relocate an EAS instance to a different edge location. In such cases, it may also be necessary to relocate the application context currently associated with the EAS instance to support service continuity.

The EAS relocation procedure may be triggered for the following reasons:

- UE mobility, including predictive or expected UE mobility.

- Overload situations in the EAS or EDN.

- Maintenance aspects, such as graceful shutdown of an EAS.

- Temporal edge resource requirements, for example, the application need an EAS with new capability which is not available in the current one.

According to the procedures for service continuity defined by SA6 in clause 8.8 of TS 23.558 [3], three principal actors are involved in Application Context Relocation (ACR):

-	ACR detection entity

-	ACR decision entity.

-	ACR execution entity.

Depending on the different functional entities responsible for detection, decision and execution, there are five different scenarios for the EAS relocation procedure.

The complete set of call flows are depicted together in figure 6.4.1-1 below.

In all cases, the edge-enabled 5G Media Streaming sessions are established via the call flow defined in clause 6.3.

Figure 6.4.1-1: EAS relocation scenarios in 5GMS edge processing

End of CHANGE