3GPP SA4-e (AH) MBS SWG post 114-e	S4aI211196

2 Jun – 3 Aug 2021 	revision of S4-210764

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.804 | CR | <CR#> | rev | - | Current version: | 0.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_5GMS-EXT] Content Preparation gap analysis: address translation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Tencent |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5GMS-EXT |  |  |  |  |  | Date: |  |  | 2021-01-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The study item description identifies the key topic “Content Preparation”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding more challenging potential open issues: address translation |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Key topic not addressed |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| 56 |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

===== CHANGE 1 =====

-

#### 5.2.7.1	Open issues in collaboration scenario 1: Content preparation before downlink streaming

##### Open issue 1: Content Preparation Template information

TS 26.512 only mentions that the format of the Content Preparation Template is identified by its MIME type. At least one format needs to be defined for the instruction in Content Preparation Template for common services such as multi-rate streaming.

##### Open issue 2: Support of other protocols

The current specification only supports HTTP pull and DASH-IF ingest. Other protocols may need to be added.

Open issue 3: Address translation for complex pull requests

5GMSd AS

Content Hosting

M2d

Content Preparation

PathRewriteRules

Cache

M4d

5GMSd Client

5GMSd Application Provider

Request for Segment 246

Segment 246

Request for Segment 246

Segment 246

5GMSd AS

Content Hosting

M2d

Content Preparation

PathRewriteRules

Cache

M4d

5GMSd Client

5GMSd Application Provider

Request for Segment 246

Segment 246

Segment 247

Request for Segment 247

Segment 123

Request for Segment 123

Figure 5.2.7.2-1 shows an example in which the content preparation takes segments n and n+1 from the upstream 5GMSd Application Provider and combines them into one segment with double duration for delivery to the 5GMSd Client. The goal could be to achieve higher compression efficiency for distribution.

5GMSd AS

Content Hosting

M2d

Content Preparation

Cache

M4d

5GMSd Client

5GMSd Application Provider

Request for Segment 246

Segment 246

Segment 247

Request for Segment 247

Segment 123

Request for Segment 123

Figure 5.2.7.2-1: An example of a media distribution by pull

5GMSd AS

Content Hosting

M2d

Content Preparation

PathRewriteRules

Cache

M4d

5GMSd Client

5GMSd Application Provider

Request for Segment 246

Segment 246

Segment 247

Request for Segment 247

Segment 123

Request for Segment 123

Address Translator

Steps:

1.	The 5GMSd Client requests a media segment http://cdn.com/segment123.mp4 through M4d.

2.	The requested URL is mapped according to the PathRewriteRules in the Content Hosting distribution configuration  (“cdn.com” => “originprovider.com/video/” and transformation of request URL using the rule {n} => {n, n+1}). Two separate requests are made via M2d to the 5GMSd Application Provider for the following media segments:

-	http://originprovider.com/video/segment246.mp4

-	http://originprovider.com/video/segment247.mp4

3.	The 5GMSd Application Provider provides the two requested media segments to the Content Preparation subfunction, which in turn merges the two segments into one, delivers the output segment to the Cache.

4.	The Cache subfunction delivers the merged media segment to the UE via an M4d response.

NOTE: It is assumed that the initialization segment is available for initialization of the decoder in the content preparation module.

Table 7.6.3.1-1 in TS 26.512 [16] defines the content hosting configuration resource. Each Content Hosting Configuration may include one or more Distribution Configurations, each one with an optional Content Preparation Template. However, the Content Hosting Configuration is currently allowed to include only one Ingest Configuration.

The Ingest Configuration defines the input format for content preparation. Currently, two ingest protocols are specified in clause 8.2 of [16]: HTTP pull-based ingest and DASH-IF push-based. In the case of HTTP pull, if a request is received in M4d that can’t be satisfied, an HTTP pull request is made through M2. The PathRewriteRules currently defined in clause 7.6.3 of [16] do not address step 2 of the above example (i.e. transforming one request into two different requests).

===== CHANGE 2 =====

#### 5.2.8.2	Address translation for complex pull requests

Open issue 3 of 5.2.7.1 provide a gap in address translation. There are three possibilities for addressing this gap:

1.	Keep PathRewriteRules as is, and therefore exclude content preparation cases with more than one input and/or with more complex translation to be used with push protocol(s) only.

2.	Extend the current request URL transformation capabilities of the 5GMSd Content Hosting feature.

3.	Keep PathRewriteRules as is and include an additional (and more sophisticated) address translation function as a part of the Content Preparation subfunction of the 5GMSd AS.

Figure 5.2.7.2-2 shows the architecture for the third option above.

5GMSd AS

Content Hosting

M2d

Content Preparation

PathRewriteRules

Cache

M4d

5GMSd Client

5GMSd Application Provider

Request for Segment 246

Segment 246

Segment 247

Request for Segment 247

Segment 123

Request for Segment 123

Address Translator

Figure 5.2.7.2-2: A candidate solution for the example of media distribution by pull

Steps:

1. The 5GMSd Client requests a media segment http://cdn.com/segment123.mp4 through M4d.

2. The requested URL is mapped according to the PathRewriteRules in the Content Hosting distribution configuration and provided to the address translator of the Content Preparation module.

3. The address translator module transforms the received request into two separate requests via M2d to the 5GMSd Application Provider for the following media segments:

-	http://originprovider.com/video/segment246.mp4

-	http://originprovider.com/video/segment247.mp4

4.	The 5GMSd Application Provider provides the two requested media segments to the Content Preparation subfunction, which in turn merges the two segments into one, delivers the output segment to the Cache.

5.	The Cache subfunction delivers the merged media segment to the UE via an M4d response.

NOTE: It is assumed that the initialization segment is available for initialization of the decoder in the content preparation module.