3GPP SA4-e (AH) MBS SWG post 114-e	S4aI211199

2 Jun – 3 Aug 2021 	revision of S4aI211198

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.804 | CR | <CR#> | rev | - | Current version: | 0.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to and summary text on Network Event Usage |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5GMS-EXT |  |  |  |  |  | Date: |  |  | 2021-06-08 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | An incoming LS from SA2 to SA4 on UE data collection, reviewed during SA4#114-e, provides additional information that should be documented under the “Network Event Usage” topic of the FS_5GMS_EXT study. TS 23.288 has adopted the QoE metrics defined in SA4 specifications, as additional observed service experience data to be made availble via the AF event exposure service. In addition, the EVEX Work Item was agreed during SA$#114-e. It outlines stage 2.5/3 tasks to be conducted as follow-up to the study phase on data collection and event exposure. A conclusion section is proposed for the Network Event usage section of this TR to document the closure of this topic in the study item. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) Update to clause 5.8.6.3 on recent changes in TS 23.288 specification of QoE metrics, based on SA4 specifications, as an additional service data type associated with observed service experience information. 2) New clause 5.8.7 “Conclusions and Next Steps”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Non up-to-date text on AF event exposure service associated with QoE metrics as service experience information. Absence of conclusions and recommended next steps text to document closure of the Network Event usage topic of the study item. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.6.3, (new) 5.8.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| 56 |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st CHANGE: clause 5.8.6.3

#### 5.8.6.3		Candidate media-related information for Event Exposure

As indicated previously, the stage 2 and stage 3 specifications for the Naf_EventExposure service are provided by TS 23.502 [24] and TS 29.517 [25], respectively. Up through Release 17, the categories of UE data at the AF available for event information subscription by NF consumers are the following:

-	Service Experience information for an application;

-	UE mobility information;

-	UE communication information; and

-	Exceptions information.

Comparison of the above categories with QoE metrics (as defined in TS 26.247 [40] for progressive download and DASH streaming services) and service experience and UE communication information types eligible for Naf event exposure (as defined in TS 23.288 [48]) shows that one common information attribute is throughput. “Average throughput” is a defined quality metric in TS 26.247 for both progressive download and 3GP-DASH services, and “Throughput” is a defined type of performance data from the AF in TS 23.288. In addition, TS 23.288 [48] in clause 6.4.2 defines, as service experience information, QoE metrics based on TS 26.247 definitions for progressive download and DASH streaming) for collection by the 5GMS AF and subsequent event exposure to the NWDAF.

NOTE: TS 23.288 in clause 6.4.2 also specifies QoE metrics as defined in other SA4 specifications – namely, MTSI in TS 26.114 [51], VR in TS 26.118 [53], and MBMS in TS 26.346 [54].

The definition of application event types, data components and formats relating to media streaming should be coordinated by SA4 with SA2 and CT3 in the production of the associated stage 2 and stage 3 specifications of Naf_EventExposure services.

END OF 1st CHANGE

2nd CHANGE: new clause 5.8.7

### 5.8.7	Conclusions and Next Steps on Event Exposure by AF

#### 5.8.7.1	Conclusions

The following conclusions and motivation for follow-up stage 2 and stage 3 work on the key topic “Network Event usage” of the FS_5GMS_EXT study item, pertaining to data to be exposed by the 5GMS AF, are summarized below.

- The 5G System currently defines a set of event exposure services that are offered by the 5G Network Functions to different consumers. For instance, the NWDAF collects and exposes a wide range of data for analytics purposes. The NWDAF collects its data from different data sources. It uses that information to build statistical and/or predictive analytics information that are exposed to analytics data consumers.

- The 5GMS AF supports media streaming sessions by providing functionality such as CDN access logging, QoS and charging policy modifications, network assistance, and tracking of service consumption and QoE. In addition, the 5GMS AF can host, on behalf of Application Provider, other types of media streaming session related UE data for exposure to analytics data consumers. Consequently, the 5GMS AF collects valuable information about the media streaming sessions in the network.

- The 5GMS AF may act as data source to the NWDAF by providing collected information about media sessions. Other consumers of this data may be the Application Provider, external 3rd party analytics and application servers,  OAM, and any network function.

- The 5GMS AF will leverage the AF Event Exposure APIs to share data with interested event consumers, such as the NWDAF. The event and data formats are to be defined and the access to the data is to be controlled

#### 5.8.7.2	Recommended normative work

It is proposed that normative specification of the following functionality on AF-based UE data collection and reporting via event notifications be produced:

- Define the media related data and formats for the media session data to be exposed by the 5GMS AF. The carriage in existing AF events or the definition of new AF Events will be subject to agreements with SA2.

- Enhance the 5GMS AF data collection to support direct and indirect collection of UE data pertaining to media sessions.

- Devise mechanisms to control the access to the collected media session data.

- Define a generic architecture within which media-specific solutions for the configuration and subsequent operation of data collection and data reporting (via event exposure) by the AF can be specified.

Coordination with other 3GPP TSG WGs is expected in the course of the normative work on data collection and reporting via the AF:

-	Collaboration with SA2 on the generalization of the overall design is expected. In addition, the work will be coordinated with SA2 and CT3 as the owners of the AF Event Exposure service, stage 2 and 3. Discussion is expected with SA6 on general architecture aspects. In particular, the definition of any new events or the integration of the media related data and formats into existing AF events will be coordinated with SA2 for the stage 2 part and CT3 for the stage 3 implementation.

-	The mechanisms to limit and control access to the collected media session data will be developed in coordination with SA3.

NOTE: Adoption of the generic architecture by other 3GPP WGs is at their discretion.

[Editor’s note: Additional conclusions and recommendations on other aspects may be included.]

END OF 2nd CHANGE