3GPP TSG-S4 ad hoc post Meeting #114-e	S4aI211214

# Online,  , 2nd June–3rd August 2021	revision of S4aI211210

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TR 26.531 | CR | – | rev |  | Current version: | 0.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Reference architecture for data collection and reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BBC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | EVEX |  |  |  |  |  | Date: |  |  | 2021-07-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |
| Reason for change: |  | Documentation of reference architecture. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Logical architecture diagram (reference point notation). - Description of logical functions. - Description of reference points. - Logical architecture (service-based architecture notation). - Description of service-based interfaces. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S4-210723 -> S4aI211210 |  |  |  |  |  |  |  |  |

FIRST CHANGE

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[3]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[4]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[5]	3GPP TS 29.517: "5G System; Application Function Event Exposure Service; Stage 3".

[6]	3GPP TS 29.532: "Data Collection and Reporting; Protocols and Formats".

NEXT CHANGE

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1], TS 23.501 [2], TS 23.502 [3], TS 23.288 [4], TS 29.517 [5] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

direct reporting: method of sending a data report from the Data Collection Client to the Data Collection AF

indirect reporting: method of sending a data report from a UE Application to the Data Collection AF via an external Application Service Provider server

NEXT CHANGE

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] , TS 23.501 [2], TS 23.502 [3], TS 29.517[4] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AF	Application Function

AS	Application Server

DCAF	Data Collection AF

DN	Data Network

NEF	Network Exposure Function

NWDAF	Network Data Analytics Function

UE	User Equipment

NEXT CHANGE

# 4	Reference architecture for data collection and reporting

## 4.1	General

Clause 6.2.8 of TS 23.288 [4] envisages a set of high-level procedures by which data is collected by a Network Data Analytics Function (NWDAF) from UE Application(s) via an intermediary Application Function. This clause defines a generic reference architecture for data collection and reporting that satisfies those procedures, including the logical functions involved and the logical reference points between them. It is intended that this reference architecture be instantiated in domain-specific ways to suit the needs of different features of the 5G System. The reference architecture may be instantiated separately in different slices of a network.

## 4.2		Functional entities for data collection and reporting

Figure 4.2-1 below shows the reference architecture for data collection and reporting using reference point notation.

Figure 4.2-1: Reference architecture for data collection and reporting in reference point notation

The functional entities illustrated in the figure are described as follows:

-	Data collection and reporting may be provisioned by a trusted Application Function (labelled AF) inside the mobile operator's network or by an authorised Application Service Provider server outside the mobile operator's network acting as an External AF (by invoking the appropriate provisioning API via the NEF).

-	The Data Collection AF is responsible for managing the provisioning state for data collection and reporting, and for collecting, processing and reporting data to event notification subscribers both inside the mobile operator's network (such as the NWDAF) and outside (such as the Application Service Provider server). In the latter role, the Data Collection AF realises the Event Exposure Service as defined in clause 6.2.2.1 of TS 23.288 [4]. It is responsible for ensuring that access to UE data is controlled according to the rules indicated in its provisioning state. The Data Collection AF is also responsible for providing a data collection and reporting configuration to the Data Collection Client, Application Server (AS) instances and the External AF.

NOTE 1:	The Data Collection AF may be deployed inside or outside the operator's network. In the latter case, the NWDAF uses the procedure defined in clause 6.2.2.3 of TS 23.288 [4] to collect data from the Data Collection AF via the NEF.

NOTE 2:	The processing activities performed by the Data Collection AF include, but are not limited to, reporting format conversion, data normalisation, domain-specific anonymisation of data and (dis)aggregation of data into reports to be exposed as events.

NOTE 3:	The Data Collection AF is intended to be instantiated inside another Application Function in order to satisfy the domain-specific data collection and reporting requirements corresponding to particular features in the 5G System. As such, there may be several domain-specific Data Collection AF instances operating simultaneously in a particular 5G System.

-	When its provisioning state changes, the Data Collection AF updates the set of available NF profile(s) in the NRF using the Nnrf_NFManagement service defined in clause 5.2.7.2 of TS 23.502 [3] according to the usage defined in clause 6.2.8.2.2 of TS 23.288 [4].

NOTE 4:	If the Data Collection AF is deployed outside the operator's network, this registration occurs via the NEF, as described in clause 6.2.2.3 of TS 23.288 [4].

-	The Data Collection Client is responsible for collecting relevant data in the UE and for sending data reports to the Data Collection AF via the Ndcaf_Reporting API according to a data collection and reporting configuration that it has previously obtained from the Data Collection AF.

NOTE 5:	This method of reporting corresponds to the direct data collection procedure defined in clause 6.2.8 of TS 23.288 [4].

NOTE 6	The Data Collection Client function is intended to be instantiated inside other UE functions in order to satisfy the domain-specfic data collection and reporting requirements corresponding to particular features of the 5G System. As such, there may be several domain-specific Data Collection Client instances operating simultaneously on a given UE, each one performing different roles. The realisation of these logical functions is implementation-dependent.

-	Other Application Server (labelled AS) instances inside or outside the operator's network may also collect data and report it to the Data Collection AF via the Ndcaf_Reporting API, according to a data collection and reporting configuration obtained from the Data Collection AF.

NOTE 7:	In the case where the Application Server is in a different trust domain than the Data Collection AF, it invokes the required APIs via the NEF.

NOTE 8:	The data collection and reporting requirements for such Application Servers is domain-specific and beyond the scope of the present document.

-	An authorised Application Service Provider server acting as an External AF may also collect data from UE Applications and send data reports to the Data Collection AF via the Ndcaf_Reporting API.

NOTE 9:	This method of reporting corresponds to the indirect data collection procedure defined in clause 6.2.8 of TS 23.288 [4].

NOTE 10:	In the case where the External AF is in a different trust domain than the Data Collection AF, it invokes the required API via the NEF.

-	The NWDAF is the primary consumer of data reports. These are published to it by the Data Collection AF via the Naf_EventExpoure API (as specified in TS 29.517 [5]) after any processing by the Data Collection AF has been performed according to its provisioning.

-	Certain data reports may also be published by the Data Collection AF to authorised Application Service Provider servers acting as external Application Functions via the Naf_EventExposure API [5]. In this case, the API is exposed via the NEF, as envisaged in clause 5.2.19 of TS 23.502 [3].

Editor’s Note: Need to check with SA2/CT3 whether the Naf_EventExpoure API can be exposed to external Application Functions via the NEF.

## 4.3		Reference points for data collection and reporting

The reference points in the functional architecture defined in clause 4.2 above are as follows:

-	R1 supports the following interactions between an Application Service Provider server (acting as an External AF) and the Data Collection AF:

-	Used by an External AF to provision data collection and reporting in the Data Collection AF. The provisioning information specifies what data is to be collected by Data Collection Clients, how it is to be processed by the Data Collection AF and how it is to be exposed to event notification subscribers. A generic provisioning envelope for data collection and reporting is defined in clause 4.5 of the present document, but this is expected to be extended by individual reporting domains.

-	Used by an External AF to obtain its data collection and reporting configuration from the Data Collection AF. A generic data collection and reporting configuration envelope is defined in clause 4.6 of the present document, but details of the configuration are specific to individual reporting domains and are specified elsewhere.

-	Used by the External AF to send data reports to the Data Collection AF using the Ndcaf_Reporting service defined in clause 4.4 of the present document. A generic data reporting envelope is defined in clause 4.7 of the present document, but details of the reporting format are specific to individual reporting domains and are specified elsewhere.

NOTE 1:	This method of reporting corresponds to the indirect data collection procedure defined in clause 6.2.8 of TS 23.288 [4].

-	Used by the External AF to subscribe to data reporting events exposed by the Data Collection AF, according to the Naf_EventExposure_Subscribe procedure defined in clause 5.2.19.1 of TS 26.502 and specified in TS 29.517 [5].

-	Used by the Data Collection AF to publish data reports to the External AF using the Naf_EventExposure_Notify procedure defined in clause 5.2.19.1 of TS 26.502 and specified in TS 29.517 [5].

Editor’s Note: Need to check with SA2/CT3 about the security aspects of event exposure to External AFs via the NEF. Exposure of events to external parties via NEF is not explicitly described in TS 23.288, although it is envisaged as a possibility in TS 26.502.

-	R2 supports the following interactions between the Data Collection Client and the Data Collection AF:

-	Used by a Data Collection Client to obtain its data collection and reporting configuration from the Data Collection AF. A generic data collection and reporting configuration envelope is defined in clause 4.6 of the present document, but details of the configuration are specific to individual reporting domains and are specified elsewhere.

-	Subsequently used by the Data Collection Client to send reports to the Data Collection AF using the Ndcaf_Reporting service defined in clause 4.4 of the present document. A generic data reporting envelope is defined in clause 4.7 of the present document, but details of the reporting format are specific to individual reporting domains and are specified elsewhere.

NOTE 2:	This method of reporting corresponds to the direct data collection procedure defined in clause 6.2.8 of TS 23.288 [4].

-	R3 supports the following interactions between trusted Application Functions inside the operator's network and the Data Collection AF.

-	Used by the trusted AF to provision data collection and reporting in the Data Collection AF. The provisioning information specifies what data is to be collected by Data Collection Clients, how it is to be processed by the Data Collection AF and how it is to be exposed to event notification subscribers. A generic provisioning envelope for data collection and reporting is defined in clause 4.5 of the present document, but this is expected to be extended by individual reporting domains.

-	R4 supports the following interactions between Application Servers inside or outside the operator's network and the Data Collection AF:

-	Used by an AS to obtain a configuration from the Data Collection AF. A generic data collection and reporting configuration envelope is defined in clause 4.6 of the present document, but details of the configuration are specific to individual reporting domains and are specified elsewhere.

-	Used by an AS to send reports to the Data Collection AF using the Ndcaf_Reporting service defined in clause 4.4 of the present document.

-	R5 supports the following interactions between the NWDAF and the Data Collection AF:

Used by the NWDAF to subscribe to data reporting events exposed by the Data Collection AF, according to the Naf_EventExposure_Subscribe procedure defined in clause 5.2.19.2.2 of TS 23.502 [3], as further elaborated in step 3a of clause 6.2.8.2.3 in TS 23.288 [4].

Used by the Data Collection AF to publish data reporting events to the NWDAF, according to the Naf_EventExposure_Notify procedure defined in clause 5.2.19.2.2 of TS 23.502 [3], as further elaborated in step 5a of clause 6.2.8.2.3 in TS 23.288 [4].

-	R6 is a client API offered by the Data Collection Client to the UE Application.

-	R7 supports data collection and reporting interactions between the UE Application and the Application Service Provider server.

NOTE 3:	Interactions at reference point R7 are beyond the scope of 3GPP standardisation.

## 4.4	Service-based architecture for data collection and reporting

Figure 4.4-1 below shows the reference architecture for data collection and reporting in service-based architecture notation.

Figure 4.4-1: Reference architecture for data collection and reporting
in service-based architecture notation

The following service-based APIs are used in connection with data collection and reporting:

1. 	The Ndcaf_Provisioning service is provided by the Data Collection AF. It is defined by the present document and specified in TS 26.532 [6].

a.	Used by trusted AF instances inside the operator's network and by the Application Service Provider server (acting as an External AF) to provision data collection and reporting in the Data Collection AF.

2. The Nnrf_NFManagement service is provided by the NRF. It is defined in clause 5.2.7.2 of TS 23.502 [3].

a.	Used by the Data Collection AF to register an available NF profile with the NRF for each set of data collection and reporting provisioning information held by the former.

NOTE:	As described in clause 6.2.8.2.2 of TS 23.288 [4] the NF profile in this case includes the External Application Identifier (used by clients when reporting data to the Data Collection AF), the Internal Application Identfier (used for event exposure to the NWDAF) and the Event ID. These NF profile parameters are in addition to those specified in clause 5.2.7.2 of TS 26.502 [3].

3.	The Ndcaf_Reporting service is provided by the Data Collection AF. It is defined by the present document and specified in TS 26.532 [6].

a.	Used by the Data Collection Client, by trusted AS instances in the operator's network and (via the NEF) by external AS instances to obtain their data collection and reporting configuration from the Data Collection AF.

b.	Subsequently used by the Data Collection Client, by trusted AS instances in the operator's network and (via the NEF) by external AS instances to send data reports to the Data Collection AF.

4.	The Naf_EventExposure service is provided by the Data Collection AF. It is defined in clause 5.2.19.2 of TS 23.502 [3] and TS 23.288 [4], and specified in TS 29.517 [5].

a.	Used by the NWDAF to subscribe to data reporting events exposed by the Data Collection AF and subsequently used by the Data Collection AF to notify these events to the NWDAF, as described in clause 6.2.2.2 of TS 23.288 [4].

b.	Used by the Application Service Provider server (acting as an External AF) to subscribe to data reporting events exposed by the Data Collection AF via the NEF and subseqently used by the Data Collection AF to notify these events to the Application Service Provider server (acting as an External AF) via the NEF, as described in clause 6.2.2.3 of TS 23.288 [4].

## 4.5	Provisioning information for data collection and reporting

Editor’s Note: Stage 2 data model used to provision data collection and reporting in the Data Collection AF. Needs to include application identifier and external application identifier.

A separate set of provisioning information shall be provided to the Data Collection AF at reference point R1 or R3 (as appropriate) for each Event ID it is to expose. When provided at reference point R1, this provisioning information embodies the Service Level Agreement between the network operator and the Application Service Provider envisaged in clause 6.2.8.1 of TS 26.288 [4]. In any case, the provisioning information shall include at least the parameters defined in table 4.5-1 below:

Table 4.5-1: Baseline provisioning information for data collection and reporting

| Parameter | Description |
| --- | --- |
| External Application Identifier | The identifier used in reports sent to the Data Reporting AF. (This needs to be mapped to the Internal Application Identifier when exposing events to the NWDAF.) |
| Internal Application Identifier | The identifier used by the NWDAF when subscribing to events in the Data Reporting AF. |
| Event ID | The identifier of an AF event that will be exposed to event consumers as a result of the provisioning. |
| Valid targets | A parameter to control whether event consumers are permitted to filter events by External UE identifier or External Group Identifier when subscribing, instead of receiving events relating to all UEs. |
| Parameters to be collected | The subset of domain-specific parameters associated with the specified Evend ID to be collected by the Data Collection AF (subject to user consent). |
| Data transformation recipe | A set of instructions that enable the Data Collection AF to transform received data reporting records into one or more exposed events. These transformations may involve aggregation/consolidation/summarisation of collected parameter values, anonymisation of collected parameters, |

## 4.6	Configuration information for data collection and reporting clients

Editor’s Note: Stage 2 data model retrieved from the Data Collection AF by its data reporting clients.

All clients of the Data Collection AF wishing to report data shall first obtain a data collection and reporting configuration from the Data Collection AF at reference point R1, R2 or R4 (as appropriate). The data collection and reporting configuration shall include at least the parameters defined in table 4.6-1 below:

| Parameter | Description |
| --- | --- |
| External Application Identifier | Identifies the UE Application to which this data collection and reporting configuration pertains. Quoted in reports sent to the Data Reporting AF. |
| Parameters to be collected | The subset of domain-specific parameters associated with the specified Evend ID to be collected by the Data Collection AF (subject to user consent). |

Editor’s Note: When requesting configuration, additional information needs to be passed across the R6 client API (then R2) to realise the "authentication information that enables the [Data Collection] AF to verify the auhtentictiy of the UE's Application that provides data" as envisaged in clause 6.2.8.1 of TS 23.288 [4].

## 4.7	Information included in data reports to the Data Collection AF

Table 4.7-1: Baseline information for data reporting

| Parameter | Description |
| --- | --- |
| External Application Identifier | Identifies the UE Application to which this data report pertains. |
| Collected parameters | The subset of domain-specific parameters associated with the specified Evend ID to be collected by the Data Collection AF (subject to user consent). |

Editor’s Note: When reporting data, additional information needs to be passed across the R6 client API (then R2) to realise the "authentication information that enables the [Data Collection] AF to verify the auhtentictiy of the UE's Application that provides data" as envisaged in clause 6.2.8.1 of TS 23.288 [4].

END OF CHANGES