Agenda item: 	4.11

Source: 	Qualcomm Inc.

Title: 	Application context relocation for media services

Document for	Discussion and Agreement

# Introduction

SA4 has sent a request to SA6 to enquire about their definition of an EAS. SA6 has issued a reply in document S6-212391 [1]. The response states that every server that the application client connects to is considered an EAS. Even if these servers are deployed as part of the same container or VM, they would still count as separate EAS(s).

The reply also states that the procedure to collect and exchange the application context is out of scope of SA6’s specification. The only supported exchange is by going through the EES, in which case the S-EAS sends its context to the S-EES, which in turn shares it with the T-EES. The T-EES will pass it to the T-EAS. This procedure is called EELManagedACR.

# Discussion

Based on the information above, we can narrow down the scope of the application context relocation assistance in 5GMS to the following points:

- Provisioning the level tolerance to context relocation

- Definition of the Media Session Handler (MSH) context

- Definition of 5GMS AF context related to a media streaming session

- Definition of the context of a Media Access Client/Media Streamer, which connects to the EAS over M4. This may cover aspects like streamer requests and re-initialization status, etc.

According to the assumptions and definitions made by SA6, it is expected that the S-EAS and T-EAS have the means to connect to each other and exchange data between themselves about the context of a session that is to be relocated. There is no need for procedures to collect the application context from the S-EAS, unless the managed ACR mode is required. Instead, we propose to focus on the role that the 5GMS client and the 5GMS AF can play in facilitating the relocation, e.g. by defining baseline procedures that implementers of an EAS can rely on during a context relocation procedure.

# Proposal

We propose to take the information in the reply from SA6 and the analysis in section 2 into account and to define the ACR procedures accordingly.

# References

[1]	3GPP SA6 Tdoc S6-212391, Reply LS on the definition of EAS and EAS context