| 3GPP TS 26.502 V0.1.0 (2021-09) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group SA; 5G multicast–broadcast services; User Service architecture (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	7

3.3	Abbreviations	7

4	Reference architecture for 5G Multicast–Broadcast User Services	7

4.1	General	7

4.2	System description	7

4.2.1	Network architecture	7

4.2.2	User Service architecture	7

4.2.3	Delivery methods	7

4.3	Functional entities	7

4.4	Reference points and interfaces	8

4.4	Domain model	8

4.5	Life-cycle model	8

4.6	QoS model	8

4.7	Security	8

5	Procedures for 5G Multicast–Broadcast User Services	8

5.1	General	8

5.2	High-level baseline procedures	8

5.3	Procedures for User Service discovery/announcement	8

5.4	Procedures for User Service initiation/termination	8

5.5	Procedures for User Service data transfer	8

5.6	Associated delivery procedures	8

6	MBS User Services Delivery Methods	9

6.1	Object Delivery Method	9

6.2	Packet/Transparent Delivery Method	9

Annex <A> (informative): Deployment and Collaboration Models	10

A.1	Group Communication	10

A.2	5G Media Streaming	10

Annex <X> (informative): Change history	11

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document defines an architecture and high-level procedures for User Services conveyed using the 5G multicast–broadcast capabilities of the 5G System defined in TS 23.501 [2], TS 23.502 [3] and TS 23.247 [5].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[3]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[4]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[5]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services; Stage 2".

[6]	3GPP TS 26.348: "Northbound Application Programming Interface (API) for Multimedia Broadcast/Multicast Service (MBMS) at the xMB reference point".

[7] 	3GPP TS 23.246: "Multimedia Broadcast/Multicast Service (MBMS); Architecture and functional description"

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1], TS 23.501 [2], TS 23.502 [3], TS 23.247 [5] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Broadcast MBS session: an MBS session to deliver the broadcast communication service, as defined in TS 23.247 [4].

delivery method: a mechanism used by the MBSTF to deliver data as part of a User Service to the MBS Client.

MBS-Aware Application: A UE-based application that consumes User Services by invoking with MBS Client APIs.

MBS Client: the UE function that consumes User Services defined in the present document.

MBS session: a multicast session or a broadcast session, as defined in TS 23.247 [4].

Multicast MBS session: an MBS session to deliver the multicast communication service, as defined in TS 23.247 [4].

Object delivery method: the delivery method supporting real-time and non-real-time distribution of discrete binary objects, including media segments, to MBS Clients as part of an MBS session.

Transparent delivery method: the delivery method supporting transparent distribution of Application Data Units to 5MBS Clients as part of an MBS session.

User Service: an abstract high-level usage of an MBS session for the purpose of supporting an application that presents a complete service offering to an MBS-Aware Application via a set of APIs that allows the MBS Client to activate and deactivate reception of the MBS session.

## 3.2	Symbols

Void.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1], TS 23.501 [2], TS 23.502 [3], TS 23.247 [4] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

MBS	Multicast–Broadcast Services

MB-SMF	Multicast–Broadcast Session Management Function

MB-UPF	Multicast–Broadcast User Plane Function

MBSF	Multicast–Broadcast Service Function

MBSTF	Multicast–Broadcast Service Transport Function

PCF	Policy and Charging Function

NEF	Network Exposure Function

UE	User Equipment

# 4	Reference architecture for 5G Multicast–Broadcast User Services

## 4.1	General

This clause defines a reference architecture for 5G Multicast–Broadcast User Services, including the logical functions involved and the logical reference points between them.

## 4.2	System description

Editor’s Note: Explanation of fundamental concepts in the MBS User Services architecture.

### 4.2.1	Network architecture

Editor’s Note: How this specification relates to the SA2 architecture in TS 23.247.

### 4.2.2	User Service architecture

Editor’s Note: Introduction to MBS User Services and how they are manifested in the MBSF and MBSTF.

### 4.2.3	Delivery methods

Editor’s Note: Explanation of what a delivery method is and what delivery methods are for.

## 4.3	Functional entities

Editor’s Note: Reference architecture for MBS User Services, including client functions.

### 4.3.1	General

In 5MBS, the MBSF and MBSTF offer service layer functionality for sending data via MBS Sessions. The MBSTF acts as a media anchor and sources the IP Multicast traffic. The MBSF offers control plane functionality while the MBSTF offers user plane functionality. A new interface Nmb2 is introduced between the control and user plane functions.

According to TS 23.247 [5]:

-	The user plane function (MBSTF) receives the traffic using the Nmb8 reference point interface, and sends the MBS data traffic using the Nmb9 reference point

-	The control plane function (MBSF) receives provisioning and control commands using either Nmb10 (direct) or Nmb5 (via NEF) reference point, controls the MBSTF if the MBSTF is used, and interacts with MB-SMF for MBS session operations via Nmb1.

-	The MBSF and MBSTF offer MB2 and xMB reference points for legacy support. Note, the MBSTF may not be present in all cases.

### 4.3.2	MBSF

The MBSF performs the following functions to support MBS:

-	Interacting with AF and MB-SMF for MBS session operations, determination of transport parameters, and session transport.

-	Determination of sender IP multicast address for the MBS session if IP multicast address is sourced by MBSTF.

-	Determination of Transport Session Identifier (TSI) for the MBS session if Object delivery Method is used.

-    Determination of the Association Delivery Procedures for the MBS session if MBS AS is used.

-    Supporting MBS session management

-    Generating the Service Announcement for the MBS session

-    Controlling the Service Announcement delivery if the Service Announcement is delivered by the MBSF（SACH）

-	Controlling MBSTF if the MBSTF is used.

-     Monitoring the status of ancillary information and controlling the In-band ancillary information delivery if ancillary information is changed and the MBSTF is used.

Editor’s Note: Usage of QoS is FFS

### 4.3.3	MBSTF

#### 4.3.3.1	General

The MBSTF performs the following functions to support MBS:

-	Media anchor for MBS data traffic if needed.

-	Sourcing of IP Multicast if needed.

-	Generic packet transport functionalities available to any IP multicast enabled application such as framing, multiple flows, packet FEC (encoding).

-	Multicast/unicast delivery of input files as object or object flows for Object Delivery method.

-	Multicast/unicast delivery of input packets as packets sequencing for Packets Delivery method.

Should provide the following functions

-	Multiplexing of In-band ancillary information delivery.

-    Carousel delivery controlling for Object Delivery method

-    Sending the notification event to MBSF, i.e cannot receive the input data, session terminated, delivery started

#### 4.3.3.2	MBSTF subfunctions to support Object Distribution

Details about the MBSTF subfunctions for supporting Object Distribution is given in Figure 4.3.3.2-1.

Figure 4.3.3.2-1 MBSTF architecture overview for Object Delivery Method

The Object Delivery Ingestion Subfunction support

-	Pull based ingest: The Object Delivery Ingestion Function fetches one or more objects using HTTPS from an application function / application server.

-	Push based ingest: The Object Delivery Ingestion function receives one or more objects using HTTPS from an application function / application server.

The File Partitioning & FEC & MBS Data Payload creation subfunction supports

-	the partitioning the received object into suitable units for MBS Session transmission

-	Optionally, creation of Application Layer FEC redundancy

The Streamer & Pacer subfunction supports

-	Creation of Object Distribution method protocol headers

-	Encapsulation and transmission of Object Distribution PDUs into Nmb9 SDUs

#### -	Pacing the outgoing data stream according to configuration.

#### 4.3.3.3	MBSTF subfunctions to support Packet Distribution

Details about the MBSTF subfunctions for supporting Packet Distribution is given in Figure 4.3.3.3-1.

Figure 4.3.3.3-1: MBSTF architecture overview for Packet Distribution Method

The Packet Delivery Ingestion Subfunction support

-	Reception of a packet sequence

The FEC & MBS Data Payload creation subfunction supports

-	Optionally, creation of Application Layer FEC redundancy

The Streamer & Pacer subfunction supports

-	Creation of Packet Distribution method protocol headers

-	Encapsulation and transmission of Packet Distribution PDUs into Nmb9 SDUs

-	Pacing the outgoing data stream according to configuration.

### 4.3.4	MBS AS

The MBS AS performs the following functions to support MBS:

-	Providing the byte-range file repair service for the Object Delivery method.

-    Providing the packet repair service for the Packets Delivery method.

Editor Notes: The Reception Reporting Service is FFS. In principle, the Reception Reporting is used by the Network Operators to analyse the packet loss rate (Block Error Rates - BLER), and the main target is to adjust the FEC redundancy level to leverage the FEC redundancy level and radio frequency usage efficiency.

### 4.3.5	MBS Client

The MBS Client performs the following functions to support MBS:

-	Reception of IP multicast data using either MBS Multicast or MBS Broadcast.

-	Handling of incoming MBS Flows.

-	Support of signalling for activate/deactivate multicast MBS session.

-	MBS services exposure towards a 5MBS-aware Application

-    Unicast recovery of the application payload data carried in multicast/broadcast packets that are not successfully received via MBS-4, if unicast repair is configured

-    Using AL-FEC repair to the received packets if FEC is used,

Editor Notes: Handling the roaming is FFS.

## 4.4	Reference points and interfaces

Editor’s Note: Description of the reference points.

In this specification, the following reference points are defined.

Nmb1:	Reference point between the MB-SMF and the MBSF.

Nmb2:	Reference point between the MBSF and the MBSTF.

Nmb5:	Reference point between the MBSF and the NEF.

Nmb8:	Reference point between the MBSTF and the AF.

Nmb9:	Reference point between the MB-UPF and the MBSTF.

Nmb10:	Reference point between the MBSF and the AF.

Nmb12:	Reference point between the MBSF and the PCF.

The 5G System Architecture contains the following additional reference points for MBSF and MBSTF

Nmb1:	Reference point between the MB-SMF and the MBSF.

Nmb9:	Reference point between the MB-UPF and the MBSTF.

Nmb12:	Reference point between the MBSF and the PCF.

Editor note: The interface to 5MBS AS is FFS.

## 4.4	Domain model

Editor’s Note: The static domain model for services and sessions.

## 4.5	Life-cycle model

Editor’s Note: State charts explaining the dynamics of MBS User Services.

## 4.6	QoS model

Editor’s Note: How MBS User Services make use of the network Quality of Service primitives defined by SA2 is TS 23.247.

## 4.7	Security

Editor’s Node: How MBS User Services makes use of the security primitives studied by SA3 in TR 33.850.

# 5	Procedures for 5G Multicast–Broadcast User Services

## 5.1	General

This clause defines the high-level procedures for 5G Multicast–Broadcast User Services.

## 5.2	High-level baseline procedures

## 5.3	Procedures for User Service discovery/announcement

## 5.4	Procedures for User Service initiation/termination

## 5.5	Procedures for User Service data transfer

## 5.6	Associated delivery procedures

# 6	MBS User Services Delivery Methods

## 6.1	Object Delivery Method

## 6.2	[Packet/Transparent] Delivery Method

# 7	MBS User Services Ingest Procedures

## 7.1 General

Editor Notes: for the AF/AS, it is better to hide the 5GC technology (for example TMGI, session, MBS bearer etc.), and just encapsulate them as the service, for example file download service, Live streaming service, Group Communication Service. The AF/CP will just configure the service(s) and service related properties to the MBSF.

## 7.1 	Provision a File Delivery service

Figure 7.1-1: File Delivery procedure

## 7.2 Provision a Live Streaming service

Figure 2: Live Streaming delivery Procedure

# 8	Network Function Services

## 8.1	General

## 8.2	MBSF Services

The following table illustrates the MBSF Services for MBS.

Table 8.2-1: NF services provided by MBSF

| Service Name | Service Operations | Operation Semantics | Example Consumer (s) |
| --- | --- | --- | --- |
| Nmbsf_ MBSService | Create | Request/Response | AF, NEF |
|  | Get | Request/Response | AF, NEF |
|  | Update | Request/Response | AF, NEF |
|  | Delete | Request/Response | AF, NEF |
| Nmbsf_ MBSSession | Create | Request/Response | AF, NEF |
|  | Get | Request/Response | AF, NEF |
|  | Update | Request/Response | AF, NEF |
|  | Delete | Request/Response | AF, NEF |
|  | StatusSubscribe | Subscribe/Notify | AF, NEF |
|  | StatusUnsubscribe |  | AF, NEF |
|  | StatusNotify |  | AF, NEF |

Editor Note 1: reuse the legacy xMB-C/MB2-C interface or design a new interface

Editor Note 2: Service exposure to AF is FFS.

### 8.2.1 Reuse xMB-C or MB2-C

Editor Notes: reuse the legacy xMB/MB2 interface is FFS.

### 8.2.2 Nmbsf MBS Service operation

#### 8.2.2.1 Nmbsf_MBSService_Create service operation

Service operation name: Nmbsf_MBSService_Create

Description: Create a new multicast service or broadcast service during MBS service configuration.

Input, Required: MBS service ID

Input, Optional: Service Name, service language, description, service type, Receive Only Mode, Service Announcement mode, MBS service area, max Bitrate, Consumption Reporting Configuration.

Output, Required: Result Indication.

Output, Optional: cause, service properties, Service Announcement fetch URL

#### 8.2.2.2 Nmbsf_MBSService_Update service operation

Service operation name: Nmbsf_ MBSService_Update

Description: Update the established multicast service or broadcast service properties.

Input, Required: MBS Service ID

Input, Optional: service properties.

Output, Required: Result Indication.

Output, Optional: Cause.

#### 8.2.2.3 Nmbsf_MBSService_Delete service operation

Service operation name: Nmbsf_MBSService_Delete

Description: Release the multicast service or broadcast service.

Input, Required: MBS Service ID.

Input, Optional: None.

Output, Required: Result Indication.

Output, Optional: Cause.

#### 8.2.2.4 Nmbsmf_MBSService_Get service operation

Service operation name: Nmbsf_MBSService_Get

Description: This service operation is used by the NF/NEF to retrieve the properties of the service.

Input, Required: None

Input, Optional: MBS Service ID.

Output, Required: Result Indication.

Output, Optional: Cause, properties of the MBS service(s)

### 8.2.3 Nmbsf MBS Session operation

#### 8.2.3.1 Nmbsf_MBSSession_Create service operation

Service operation name: Nmbsf_MBSSession_Create

Description: Create a new multicast session or broadcast session during MBS service configuration. Optionally subscribe to notifications for this MBS session.

Input, Required: MBS Service ID, MBS Session ID

Input, Optional: session start time, session stop time, MBS service area, Max Bitrate, max delay, Session Type, object delivery session, files, application session, packet delivery proxy mode session, packet delivery forward-only mode session, GC session

Output, Required: Result Indication.

Output, Optional: cause, webdavUrl for push mode in object delivery session, webdavUrl for push mode in application session, MBSTF IP address for packet delivery method.

#### 8.2.2.2 Nmbsf_MBSSession_Update service operation

Service operation name: Nmbsf_MBSSession_Update

Description: Update the established multicast session or broadcast session, e.g. prolong the MBS session duration, change files, change the MBS service area.

Input, Required: MBS Service ID, MBS Session ID

Input, Optional: Session properties

Output, Required: Result Indication.

Output, Optional: Cause.

#### 8.2.2.3 Nmbsf_MBSSession_Delete service operation

Service operation name: Nmbsf_MBSSession_Delete

Description: Release the multicast session or broadcast session.

Input, Required: MBS Service ID, MBS Session ID

Input, Optional: None.

Output, Required: Result Indication.

Output, Optional: Cause.

#### 8.2.2.4 Nmbsf_MBSSession_Get service operation

Service operation name: Nmbsf_MBSSession_Get

Description: This service operation is used by the AF/NEF to retrieve the properties of the session(s).

Input, Required: MBS Service ID

Input, Optional: MBS Session ID

Output, Required: Result Indication.

Output, Optional: Cause, properties of the MBS Session(s)

#### 8.2.2.5 Nmbsf_MBSSession_StatusSubscribe operation

The Service Operation is used by AF/NEF to subscribe to a status(s) of one service, session, and/or files.

Service operation name: Nmb2_MBSSession StatusSubscribe

Description: The Subscribe service operation is invoked by AF/NEF, towards the MBSF, when it needs to create a subscription to monitor at least one event relevant to the service, session and/or file(s). The AF may subscribe to multiple events in a subscription.

Input, Required: MBS Service ID, Event ID(s), notification target address.

Input, Optional: MBS Session ID

Output, Required: When the subscription is accepted: Subscription Correlation ID.

#### 8.2.2.6 Nmbsf_MBSSession_StatusUnsubscribe operation

The Unsubscribe service operation is invoked by MBSF, towards the MBSTF, to remove an existing subscription previously created by itself at the MBSTF.

Service operation name: Nmb2_MBSSession StatusUnsubscribe

Description: The Unsubscribe service operation is to remove an existing subscription.

Input, Required: Subscription Correlation ID.

Output, Required: Result Indication.

#### 8.2.2.7 Nmbsf_MBSSession_StatusNotify operation

The Notify service operation is invoked by the MBSTF, to send a event notification, towards the notification URI, when certain event included in the subscription has taken place.

Service operation name: Nmb2_MBSSession_StatusNotify

Description: This service operation is used by the MBSTF to notify MBSF about the status change of the session or the status of the file.

Input, Required: MBS Service ID, Event ID(s).

Input, Optional: MBS Session ID, fileUrl, Status information.

Output, Required: Result Indication.

### 8.3	MBSTF Services

The following table illustrates the MBSTF Services for MBS.

Table 8.3-1: NF services provided by MBSTF

| Service Name | Service Operations | Operation Semantics | Example Consumer (s) |
| --- | --- | --- | --- |
| Nmb2_MBSSession | Create | Request/Response | MBSF |
|  | Get | Request/Response | MBSF |
|  | Update | Request/Response | MBSF |
|  | Delete | Request/Response | MBSF |
|  | StatusSubscribe | Subscribe/Notify | MBSF |
|  | StatusUnsubscribe |  | MBSF |
|  | StatusNotify |  | MBSF |

#### 8.3.1 Nmb2_MBSSession service

##### 8.3.1.1 Nmb2_MBSSession_Create service operation

Service operation name: Nmb2_MBSSession_Create

Description: Create a new multicast session or broadcast session during MBS session configuration.

Input, Required: MBS Session ID

Input, Optional: MBS activation time, MBS termination time, QoS flow information, Bandwidth, unicast Address of MB-UPF, TSI if object delivery object is used, IP multicast address is sourced by MBSTF,

Output, Required: Result Indication.

Output, Optional: Cause, Sourcing of IP Multicast if multicast is used,

#### 8.3.1.2 Nmb2_MBSSession_Update service operation

Service operation name: Nmb2_MBSSession_Update

Description: Update a multicast session or broadcast session during MBS session configuration, for example session stop time, object delivery session, application session, packets delivery session, files, and ancillary info of the session.

Input, Required: MBS Session ID

Input, Optional: MBS termination time, unicast Address of MB-UPF list, object delivery session, application session, packetsdelivery session, ancillary info of the session.

Output, Required: Result Indication.

Output, Optional: Cause, additional info (input parameters)

#### 8.3.1.3 Nmb2_MBSSession_Delete service operation

Service operation name: Nmb2_MBSSession_Update

Description: delete a multicast session or broadcast session during MBS session configuration

Input, Required: MBS Session ID

Output, Required: Result Indication.

Output, Optional: Cause

#### 8.3.1.4 Nmb2_MBSSession_get service operation

Service operation name: Nmb2_MBSSession_Get

Description: get a multicast session or broadcast session during MBS session configuration

Input, Required: None

Input, Required: MBS Session ID

Output, Required: Result Indication.

Output, Optional: Cause, the properties of MBS session(s)

#### 8.3.1.5 Nmb2_MBSSession_StatusSubscribe operation

The Service Operation is used by MBSF to subscribe to a status(s) of one session, and/or files.

Service operation name: Nmb2_MBSSession_StatusSubscribe

Description: The Subscribe service operation is invoked by MBSF, towards the MBSTF, when it needs to create a subscription to monitor at least one event relevant to the delivery session and/or file(s). The MBSF may subscribe to multiple events in a subscription.

Input, Required: MBS Session ID, Event ID(s), notification target address.

Output, Required: When the subscription is accepted: Subscription Correlation ID.

#### 8.3.1.6 Nmb2_MBSSession_StatusUnsubscribe operation

The Unsubscribe service operation is invoked by MBSF, towards the MBSTF, to remove an existing subscription previously created by itself at the MBSTF.

Service operation name: Nmb2_MBSSession_StatusUnsubscribe

Description: The Unsubscribe service operation is to remove an existing subscription.

Input, Required: Subscription Correlation ID.

Output, Required: Result Indication.

#### 8.3.1.7 Nmb2_MBSSession_StatusNotify operation

The Notify service operation is invoked by the MBSTF, to send a event notification, towards the notification URI, when certain event included in the subscription has taken place.

Service operation name: Nmb2_MBSSession_StatusNotify

Description: This service operation is used by the MBSTF to notify MBSF about the status change of the session or the status of the file.

Input, Required: MBS Session ID, Event ID(s).

Input, Optional: Status information.

Output, Required: Result Indication.

# Annex A (informative):

Deployment and Collaboration Models

# A.1	AF / AS in Trusted DN

Figure A.1-1 depicts a deployment with AF / AS within the Trusted DN. The AF / AS uses the Nmbsf API directly. The MBSTF uses the Nmb9 Reference Point for ingesting data into the MB-UPF.

Figure A.1-1: Deployment with AF / AS in Trusted DN

# A.2	AF / AS in external DN

Figure A.2-1 depicts a deployment with AF / AS within the external DN. The AF / AS uses the Nnef (N33) API for accessing MBSF services. The MBSTF uses the Nmb9 Reference Point for ingesting data into the MB-UPF.

Figure A.2-1: Deployment with AF / AS in External DN

# A.3	MBSF/MBSTF Like functions in external DN

Figure A.3-1 depicts a transport-only deployment. Here, an external AF / AS uses an MBSF / MBSTF -like function for producing delivery method data. The MBSF-like function uses Nnef (N33) for accessing MB-SMF services. The MBSTF-like function uses the N6mb Reference Point for ingesting data into the MB-UPF.

The MBSTF-like function produces a data stream, which is compliant with this specification. The 5MBS Client in the UE follows procedures as defined in this specification.

Figure A.2-1: Deployment with AF / AS in External DN

# A.2	5G Media Streaming

Editor’s Note:	Reference to TS 26.501.

# Annex B. Nmb8 User Plane ingest examples

## B.1 General

This annex provides an overview of the different Nmb8 User Plane protocol stacks for the various delivery method options. The Nmb2 Control Plane is used to select the correct version

## B.2 Object Delivery Method

## B.2.1 Object Delivery Method ingestion with Pull

The AF provides the file URLs to the MBSTF via MBSF and the MBSTF fetches the files using HTTP. The MBSTF is handling all MBS related complexity, e.g. converting the HTTP payload into an IP Multicast suitable protocols, adding AL-FEC, etc. The AF delegates the delivery of MBS of Service Announcement Metadata (i.e. IP Multicast protocol details, etc) to the MBS Client to the MBSF.

Figure B.2-1 illustrates a setup, where the MBSTF pulls files from a File Server. The Nmb2 is used to provide the file URLs to the MBSTF.

Figure B.1-1: Object Delivery Method using Pull Mode (HTTP GET)

The following Session Properties allow the configuration of thisNmb8 mode:

-	Session Type is set by the AF to Files.

-	Ingest Mode (Session Type specific property) is set by the AF to Pull.

-	The File List (Session Type specific property) is updated by the AF with File URLs to be fetched by the MBSTF and then send. The MBSF updates Service Announcement according to the File List information.

## B.2.2 File ingestion with Push

The AF pushes the files using HTTP. The MBSTF is handling all MBMS related complexity, e.g. converting the HTTP payload into an IP Multicast suitable protocols, adding AL-FEC, etc. The AF delegates the delivery of MBMS of Service Announcement Metadata (i.e. DASH MPD, IP Multicast protocol details, etc) to the MBS Client to the MBSTF via MBSF.

Figure B.2-2 illustrates a setup, where a File Server pushes files using HTTP PUT into the MBSTF.

Figure B.3-1: File Delivery using Push Mode (HTTP PUT)

The following Session Properties allow the configuration of this Nmb8 mode:

-	Session Type is set by the AF to Files.

-	Ingest Mode (Session Type specific property) is set by the AF to Push.

-	The MBSTF provides the Push URL (Session Type specific property) to the AF. The value of this property is configured to the File Server.

-	Display Base URL contains the base URL for the files. In the URLs, used in the FLUTE FDT instances and (in some cases) in Service Announcement, the MBSF replaces the Push URL part of the file URL with the value of the Display Base URL.

## B.3 Packets Distribution Method

### B.3.1 RTP Streaming mode

### B.3.2 Proxy mode

This clause illustrates the various Nmb8 options for Packets Delivery Mode. The MBSTF handles the streams in a proxy mode.

Figure B.3-1 illustrates a setup of Packets Delivery Method with Proxy. The Nmb2 is used to provide the necessary information to the MBSTF via MBSF.

Figure B.3-2: Packets Delivery Method with Proxy mode

The following Session Properties allow the configuration of this Nmb9 mode:

-	Session Type is set by the AF to Packets Delivery Method.

-	Delivery Mode Configuration for user plane (Session Type specific property) is set by the AF to Proxy.

-	Session Description Parameters for User Plane (Session Type specific property) is set by the AF and contains the UDP flow mapping descriptions.

### B.3.3 Forward-Only mode

Figure B.3-2 illustrates a setup of Packets Delivery method with Forward-Only. The Nmb2 is used to provide the necessary information to the MBSTF via MBSF.

Figure B.3-3: Packets Delivery Method with Forward-Only

The following Session Properties allow the configuration of this Nmb9 mode:

-	Session Type is set by the AF to Object Delivery Method.

-	Delivery Mode Configuration for user plane (Session Type specific property) is set by the AF to Forward-only.

-	Session Description Parameters for User Plane (Session Type specific property) is set by the AF and contains the UDP flow mapping descriptions.

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2021-07 | Post-SA4#114-e ad hoc | S4aI211206 |  |  |  | Initial skeleton document. | 0.0.1 |
| 2021-09 | SA4#115-e | S4-211270 |  |  |  | Implemented agreements at SA4#115-e | 0.1.0 |