3GPP TSG- S4 Meeting #115e	 S4aI211249

# Electronic Meeting, 18th August – 27th August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.805 | CR | <CR#> | rev | <Rev#> | Current version: | <Version#> |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_NPN4AVProd] Update of SRT and RIST description |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_NPN4AVProd |  |  |  |  |  | Date: |  |  | <Res_date> |
| Category: | <Cat> |  |  |  |  |  | Release: |  |  | <Release> |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The RIST Simple and Main Profile specification were updated. Some additional information is available for SRT. |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system".

[3]	3GPP TS 22.263: "Service requirements for Video, Imaging and Audio for Professional Applications (VIAPA)".

[4]	3GPP TS 22.827: "Study on Audio-Visual Service Production".

[5]	M.P. Sharabayko, M.A. Sharabayko, J. Dube, JS. Kim, JW. Kim: "The SRT Protocol", draft-sharabayko-mops-srt-01

[6]	VSF: "Reliable Internet Stream Transport (RIST) Activity Group",

[7]	VSF TR 06-1:2020: "Reliable Internet Stream Transport (RIST) Protocol Specification – Simple Profile", https://vsf.tv/download/technical_recommendations/VSF_TR-06-1_2020_06_25.pdf

[8]	VSF TR 06-2, "Reliable Internet Stream Transport (RIST) Protocol Specification – Main Profile", https://www.vsf.tv/download/technical_recommendations/VSF_TR-06-2_2020_03_24.pdf

[9]	NewTek: "NDI Encoding/Decoding", https://support.newtek.com/hc/en-us/articles/218109667-NDI-Encoding-Decoding

[10]	NewTek: "NDI Network Bandwidth, https://support.newtek.com/hc/en-us/articles/217662708-NDI-Network-Bandwidth

[11]	David Aleksandersen: "What is NDI® (Network Device Interface)?", https://newsandviews.dataton.com/what-is-ndi-network-device-interface

[12]	Kieran Kunhya and Ciro Noronha: "RIST and SRT: What’s the difference?", https://www.tvbeurope.com/ip-migration/rist-and-srt-whats-the-difference

[13]	Tofik Sonono: "Interoperable Retransmission Protocols with Low Latency and Constrained Delay: A Performance Evaluation of RIST and SRT", Masters Thesis, KTH Stockholm, 2019,

[14]	EBU: "Minimum User Requirements to Build and Manage an IP-Based Media Facility", 15 July 2020, https://tech.ebu.ch/files/live/sites/tech/files/shared/tech/tech3371.pdf.

[15]	AMWA: "NMOS Overview", https://www.amwa.tv/nmos-overview.

[16]	EBU: "The Technology Pyramid For Media Nodes", .

[17]	EBU: "Technology Pyramid Media Node Maturity Checklist", September 2021, https://tech.ebu.ch/publications/technology-pyramid-media-node-maturity-checklist?rec=1.

[18]	AMWA: "NMOS Technical Overview", https://specs.amwa.tv/nmos/branches/main/docs/2.0._Technical_Overview.html.

[19]	AMWA: "Networked Media Systems – the Big Picture",
https://static.amwa.tv/networked-media-systems-big-picture-2021-03-05.pdf.

[20]	AMWA: "NMOS specification repository", https://specs.amwa.tv/nmos.

[21]	SMPTE ST 2110: "Professional Media over Managed IP".

[22]	IEEE 1588-2008: "Precision Time Protocol".

[23]	SMPTE ST 2022-1:2007: "Forward Error Correction for Real-Time Video/Audio Transport Over IP Networks".

[24]	SMPTE ST 2022-6:2012: "Transport of High Bit Rate Media Signals over IP Networks (HBRMT)",

[25]	SMPTE ST 2022-7:2019: "Seamless Protection Switching of RTP Datagrams".

[26]	SMPTE ST 2059-2:2015: "SMPTE Profile for Use of IEEE-1588 Precision Time Protocol in Professional Broadcast Applications".

[27]	SMPTE ST 2110-10:2017: "Professional Media Over Managed IP Networks: System Timing and Definitions".

[28]	SMPTE ST 2110-20:2017: "Professional Media Over Managed IP Networks: Uncompressed Active Video".

[29]	SMPTE ST 2110-22:2019: "Professional Media Over Managed IP Networks: Constant Bit-Rate Compressed Video".

[30]	SMPTE ST 2110-30:2017: "Professional Media Over Managed IP Networks: PCM Digital Audio".

[31]	SMPTE ST 2110-31:2018: "Professional Media Over Managed IP Networks: AES3 Transparent Transport".

[32]	IETF RFC 4585: "Extended RTP Profile for Real-time Transport Control Protocol (RTCP)-Based Feedback (RTP/AVPF)".

[33]	IETF RFC 8086: "GRE-in-UDP Encapsulation".

[34]	Ember+ control protocol,

[35]	SMPTE ST 259:2008: "For Television — SDTV Digital Signal/Data — Serial Digital Interface".

[36]	SMPTE ST 292-1:2012: "1.5 Gb/s Signal/Data Serial Interface".

[37]	3GPP TR 26.925: "Typical traffic characteristics of media services on 3GPP networks".

[38]	Wikipedia: "MADI", last modified 19th April 2021, https://en.wikipedia.org/wiki/MADI

[39]	Wikipedia: "Time-Sensitive Networking", last modified 23rd June 2021, https://en.wikipedia.org/wiki/Time-Sensitive_Networking

[40]	AES67 / SMPTE ST 2110: "COMMONALITIES AND CONSTRAINTS", https://aimsalliance.org/wp-content/uploads/2019/04/AES67-SMPTE-ST-2110-Commonalities-and-Constraints-Updated-April-2019.pdf

[41]	IETF RFC 5104: "Codec Control Messages in the RTP Audio-Visual Profile with Feedback (AVPF)".

[42]	IETF RFC 4585: "Extended RTP Profile for Real-time Transport Control Protocol (RTCP)-Based Feedback (RTP/AVPF)".

**** Next Change ****

### 4.2.3	Secure Reliable Transport (SRT)

Secure Reliable Transport (SRT) [5] is an open-source media transport protocol that uses the UDP transport protocol. The protocol is currently under specofocation in IETF. SRT provides connection and control, reliable transmission similar to TCP at the application layer. It supports packet recovery while maintaining low latency. SRT also supports encryption using AES.

The protocol was derived from the UDT project, designed for fast file transmission. UDT provides its reliability mechanism by using similar methods for connection, sequence numbers, acknowledgements and retransmission of lost packets. UDT uses selective and immediate (NACK-based) retransmission.

SRT has all these features, but also adds several more to support live streaming mode:

1.	Controlled latency, with source time transmission (timestamp-based packet delivery).

2.	Sender bandwidth control.

3.	Conditional “too late” packet dropping (prevents head-of-line blocking caused by a lost packet that wasn’t recovered on time).

4.	Eager packet re-transmission (periodic NACK report).

SRT accepts a wide range of well known and commonly used formats including MPEG2-TS and RTP.

### 4.2.4	Reliable Internet Stream Transport (RIST)

Reliable Internet Stream Transport [6] is an open source, open specification transport protocol designed for reliable transmission of media over lossy networks (including the internet) with low latency and high quality. It is currently being developed and maintained by the Video Services Forum (VSF).

Technically, RIST seeks to provide reliable, high performance media transport by using RTP/UDP at the transport layer to avoid the limitations of TCP. Reliability is achieved by using NACK-based retransmissions to realise an Automatic Repeat Query (ARQ) capability. SMPTE-2022 Forward Error Correction can be combined with RIST but is known to be significantly less effective than ARQ.

RIST Simple Profile [7] was initially published by the VSF in October 2018 and revised in June 2020. It includes the following features:

-	The base stream uses RTP for compatibility with existing equipment.

-	Retransmission requests use RTCP. Two types of retransmission requests are defined:

-	A Bitmask-based NACK, defined in RFC 4585.

-	A Range-based NACK, defined as an application-specific (APP) RTCP packet.

-	Bonding of multiple links for load sharing.

-	Seamless switching using SMTPE-2022-7.

-	Out-of-band transmission of protection data (retransmissions may use a separate link).

-	RTT Echo Request / Response procedure to estimate the roundtrip time.

RIST Main Profile [8] was published in March 2020 and adds the following features to Simple Profile:

-	GRE-in-UDP encapsulation based on RFC 8086, with bidirectional send/receive in the same tunnel.

-	Multiplexing of multiple streams into the same tunnel.

-	In-band data support in the tunnel, useful for remote management.

-	Client/Server architecture.

-	Firewall traversal.

-	DTLS encryption or Pre-Shared Key encryption, with multicast support, access control, and authentication.

-	Advanced authentication options using either public key certificates or TLS-SRP.

-	Bandwidth optimization based on NULL packet deletion.

-	Support for high bit-rate streams by extending the size of the RTP sequence number space.

NOTE: 	RIST Simple Profile does not require or recommendate any RTP payload format. As result, deployments may embed for example HEVC frames into an MPEG2-TS container (RFC 2250) or directly into RTP (RFC7798).

**** Last Change ****