3GPP TSG-SA4 MBS SWG ad hoc meeting	S4aI211251

# Electronic meeting, Telco, Dec 1, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.532 | CR | xxxx | rev | - | Current version: | 0.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of config and report API resource structure for 26.532 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | EVEX |  |  |  |  |  | Date: |  |  | 2021-11-30 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Filling in missing text regarding API. |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 7.2, 7.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### ===================== Start of first change =====================

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[3]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[4]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[5]	3GPP TS 29.517: "5G System; Application Function Event Exposure Service; Stage 3".

[6]	3GPP TS 29.510: "5G System; Network Function Repository Services; Stage 3".

[7]	3GPP TS 26.531: "Data Collection and Reporting; General Description and Architecture".

[8]	OpenAPI: "OpenAPI 3.0.0 Specification", https://github.com/OAI/OpenAPI-Specification/blob/master/versions/3.0.0.md.

[9]	CORS: https://fetch.spec.whatwg.org/#cors-protocol

### ===================== Next change =====================

## 7.2	Data Collection and Reporting Configuration API

### 7.2.1	Overview

This clause specifies the configuration API used by a Direct Data Collection Client, an Indirect Data Collection Client, or an Application Server to obtain a data collection and reporting configuration from the Data Collection AF.

### 7.2.2	Resources

#### 7.2.2.1	Resource Structure

/{sessionId}

/sessions

{apiRoot]/ndcaf-datareporting/v1

Table 7.2.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 7.2.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| Data Collection Sessions | /sessions | POST | Data Collection Client checks-in and provides info about what data it can report and is provided with configuration in response. |
| Individual Data Collection Session | /sessions/ {sessionId} | GET | Reads an Individual Data Collection Session resource. |
|  |  | PUT | Modifies an Individual Data Collection Session |
|  |  | DELETE | Cancels an individual Data Collection Session. |

#### 7.2.2.2	Resource: Data Collection Sessions

##### 7.2.2.2.1	Description

The Data Collection Sessions resource represents all Data Collection sessions at a given Data Collection AF. The resource allows a Data Collection Client to create a new Individual Data Collection Session resource and to receive configuration details for the session.

##### 7.2.2.2.2	Resource definition

Resource URI: {apiRoot}/ndcaf-datareporting/v1/sessions

This resource shall support the resource URI variables defined in table 7.2.2.2.2-1.

Table 7.2.2.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | FQDN of the DC AF |

##### 7.2.2.2.3	Resource Standard Methods

###### 7.2.2.2.3.1	POST

This method shall support the URI query parameters specified in table 7.2.2.2.3.1-1.

Table 7.2.2.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.2.2.2.3.1-2 and the response data structures and response codes specified in table 7.2.2.2.3.1-4.

Table 7.2.2.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| DataCollectionSessionData | M | 1 | Data supplied by the Data Collection Client to enable creation of a new Data Collection Session. |

Table 7.2.2.2.3.1-3: Headers supported for POST requests on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Origin | string | O | 0..1 | Indicates the origin of the requester. |

Table 7.2.2.2.3.1-4: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| DataCollectionSessionConfiguration | M | 1 | 201 Created | The creation of an Individual Data Collection Session is confirmed and configuration data for the Data Collection Client for the session is provided. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [6] also apply. |  |  |  |  |

Table 7.2.2.2.3.1-5: Headers supported by the 201 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/ndcaf-datareporting/v1/sessions/{sessionId} |
| Access-Control-Allow-Origin | string | O | 0..1 | Part of CORS, supplied if the request included the “Origin” header. |
| Access-Control-Allow-Methods | string | O | 0..1 | Part of CORS, supplied if the request included the “Origin” header. Value: “POST, PUT, DELETE” |
| Access-Control-Expose-Headers | string | O | 0..1 | Part of CORS, supplied if the request included the “Origin” header. Value: “Location” |

N.B. standard http re-directing using 3xx response with Location header as well as Alt-Svc are allowed.

#### 7.2.2.3	Resource: Individual Data Collection Session

##### 7.2.2.3.1	Description

The Individual Data Collection Session resource represents a single session to the Ndcaf_DataReporting_Sessions Service at a given Data Collection AF.

##### 7.2.2.3.2	Resource definition

Resource URI: {apiRoot}/nnwdaf-eventssubscription/v1/sessions/{sessionionId}

This resource shall support the resource URI variables defined in table 7.2.2.3.2-1.

Table 7.2.2.3.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See subclause 7.2.2.2.2 |
| sessionId | string | Identifies a session to the Ndcaf_DataReporting_Sessions Service |

##### 7.2.2.3.3	Resource Standard Methods

###### 7.2.2.3.3.1	DELETE

This method shall support the URI query parameters specified in table 7.2.2.3.3.1-1.

Table 7.2.2.3.3.1-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.2.2.3.3.1-2 and the response data structures and response codes specified in table 7.2.2.3.3.1-4.

Table 7.2.2.3.3.1-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.2.2.3.3.1-3: Headers supported for DELETE requests on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Origin | string | O | 0..1 | Indicates the origin of the requester. |

Table 7.2.2.3.3.1-4: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Successful case: The Individual Data Collection Session resource matching the sessionId was deleted. |
| ProblemDetails | O | 0..1 | 307 Temporary Redirect | Temporary redirection, during Individual Data Collection Session deletion. The response shall include a Location header field containing an alternative URI of the resource located in an alternative Data Collection AF (service) instance. Applicable if the feature "ES3XX" is supported. |
| ProblemDetails | O | 0..1 | 308 Permanent Redirect | Permanent redirection, during Individual Data Collection Session deletion. The response shall include a Location header field containing an alternative URI of the resource located in an alternative Data Collection AF (service) instance. Applicable if the feature "ES3XX" is supported. |
| ProblemDetails | O | 0..1 | 404 Not Found | The Individual Data Collection Session resource does not exist. (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the DELETE method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [6] also apply. NOTE 2: Failure cases are described in subclause 7.2.4. |  |  |  |  |

Table 7.2.2.3.3.1-5: Headers supported by the 204 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Access-Control-Allow-Origin | string | O | 0..1 | Part of CORS, supplied if the request included the “Origin” header. |
| Access-Control-Allow-Methods | string | O | 0..1 | Part of CORS, supplied if the request included the “Origin” header. Value: “POST, PUT, DELETE” |
| Access-Control-Expose-Headers | string | O | 0..1 | Part of CORS, supplied if the request included the “Origin” header. Value: “Location” |

Table 7.2.2.3.3.1-6 Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative Data Collection AF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected |
| Access-Control-Allow-Origin | string | O | 0..1 | Part of CORS, supplied if the request included the “Origin” header. |
| Access-Control-Allow-Methods | string | O | 0..1 | Part of CORS, supplied if the request included the “Origin” header. Value: “POST, PUT, DELETE” |
| Access-Control-Expose-Headers | string | O | 0..1 | Part of CORS, supplied if the request included the “Origin” header. Value: “Location” |

Table 7.2.2.3.3.1-7: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative Data Collection AF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected |
| Access-Control-Allow-Origin | string | O | 0..1 | Part of CORS, supplied if the request included the “Origin” header. |
| Access-Control-Allow-Methods | string | O | 0..1 | Part of CORS, supplied if the request included the “Origin” header. Value: “POST, PUT, DELETE” |
| Access-Control-Expose-Headers | string | O | 0..1 | Part of CORS, supplied if the request included the “Origin” header. Value: “Location” |

###### 7.2.2.3.3.2	PUT

This method shall support the URI query parameters specified in table 7.2.2.3.3.2-1.

Table 7.2.2.3.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.2.2.3.3.2-2 and the response data structures and response codes specified in table 7.2.2.3.3.2-4.

Table 7.2.2.3.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| DataCollectionSessionData | M | 1 | Parameters to replace Data Collection Client provided configuration data for an Individual Data Collection Session resource. |

Table 7.2.2.3.3.2-3: Headers supported for PUT requests on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Origin | string | O | 0..1 | Indicates the origin of the requester. |

Table 7.2.2.3.3.2-4: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| DataCollectionSessionConfiguration | M | 1 | 200 OK | The Individual Data Collection Session resource was modified successfully and configuration data for the Data Collection Client for the session is provided. |
| ProblemDetails | O | 0..1 | 307 Temporary Redirect | Temporary redirection, during Individual Data Collection Session modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative Data Collection AF (service) instance. Applicable if the feature "ES3XX" is supported. |
| ProblemDetails | O | 0..1 | 308 Permanent Redirect | Permanent redirection, during Individual Data Collection Session modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative Data Collection AF (service) instance. Applicable if the feature "ES3XX" is supported. |
| ProblemDetails | O | 0..1 | 404 Not Found | The Individual Data Collection Session resource does not exist. (NOTE 2) |
| NOTE 1: The mandatory HTTP error status codes for the PUT method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [6] also apply. NOTE 2: Failure cases are described in subclause 7.2.4. |  |  |  |  |

Table 7.2.2.3.3.2-5: Headers supported by the 200 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Access-Control-Allow-Origin | string | O | 0..1 | Part of CORS, supplied if the request included the “Origin” header. |
| Access-Control-Allow-Methods | string | O | 0..1 | Part of CORS, supplied if the request included the “Origin” header. Value: “POST, PUT, DELETE” |
| Access-Control-Expose-Headers | string | O | 0..1 | Part of CORS, supplied if the request included the “Origin” header. Value: “Location” |

Table 7.2.2.3.3.2-6: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative Data Collection AF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected |
| Access-Control-Allow-Origin | string | O | 0..1 | Part of CORS, supplied if the request included the “Origin” header. |
| Access-Control-Allow-Methods | string | O | 0..1 | Part of CORS, supplied if the request included the “Origin” header. Value: “POST, PUT, DELETE” |
| Access-Control-Expose-Headers | string | O | 0..1 | Part of CORS, supplied if the request included the “Origin” header. Value: “Location” |

Table 7.2.2.3.3.2-7: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located in an alternative Data Collection AF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected |
| Access-Control-Allow-Origin | string | O | 0..1 | Part of CORS, supplied if the request included the “Origin” header. |
| Access-Control-Allow-Methods | string | O | 0..1 | Part of CORS, supplied if the request included the “Origin” header. Value: “POST, PUT, DELETE” |
| Access-Control-Expose-Headers | string | O | 0..1 | Part of CORS, supplied if the request included the “Origin” header. Value: “Location” |

##### 7.2.2.2.4	Resource Custom Operations

None in this release of the specification.

### 7.2.3	Data Model

#### 7.2.3.1	General

This subclause specifies the application data model supported by the API.

Table 7.2.3.1-1 specifies the data types defined for the Ndcaf_DataReporting_Sessions service based interface protocol.

Table 7.2.3.1-1: Ndcaf_DataReporting_Sessions specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| DataCollectionSessionData | 7.2.3.2.2 | Data to be provided by Data Collection Client when offering a session to the DC AF. |  |
| DataCollectionSessionConfiguration | 7.2.3.2.3 | Configuration of the Data Collection Client. |  |

Table 7.2.3.1-2 specifies data types re-used by the Nnwdaf_EventsSubscription service based interface protocol from other specifications, including a reference to their respective specifications.

Table 7.2.3.1-2: Ndcaf_DataReporting_Sessions re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| ApplicationId | 3GPP TS 29.571 | Identifies the application |  |
| DateTime | 3GPP TS 29.571 |  |  |
| DurationSec | 3GPP TS 29.571 |  |  |
| Double | 3GPP TS 29.571 |  |  |
| Float | 3GPP TS 29.571 |  |  |
| Int32 | 3GPP TS 29.571 |  |  |
| Int64 | 3GPP TS 29.571 |  |  |
| Uint16 | 3GPP TS 29.571 |  |  |
| Uint32 | 3GPP TS 29.571 |  |  |
| Uint64 | 3GPP TS 29.571 |  |  |
| Uinteger | 3GPP TS 29.571 |  |  |

#### 7.2.3.2	Structured data types

##### 7.2.3.2.1	Introduction

This subclause defines the structures to be used in resource representations.

##### 7.2.3.2.2	Type DataCollectionSessionData

Table 7.2.3.2.2-1: Definition of type DataCollectionSessionData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| applicationId | ApplicationID | M | 1 | Subscribed events |  |
| authenticationInfo | AuthenticationInformation | M | 1 | 7.2.2.3.2 |  |
| supportedDomain | array(DataDomain) | O | 0..1 | List of domains for which the Data Collection Client can provide data, see 7.2.3.3.2. If this parameter is not supplied, no data can currently be provided. |  |

##### 7.2.3.2.3	Type DataCollectionSessionConfiguration

Table 7.2.3.2.3-1: Definition of type DataCollectionSessionConfiguration

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| sessionId | string | M | 1 | The session identifier |  |
| reportCondition | ReportCondition | M | 1 | 7.2.3.2.4 |  |
| reportForDomains | array(DataDomain) | O | 0..1 | List of domains for which the Data Collection Client is requested to provide data, from the set supported domains, see 7.2.3.2.2 and 7.2.3.3.2. If this parameter is not supplied, no data should currently be provided. |  |
| NOTE 1: xzx. |  |  |  |  |  |

##### 7.2.3.2.4	Type ReportCondition

Table 7.2.3.2.4-1: Definition of type ReportCondition

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| type | ConditionType | M | 1 | Type of condition, see 7.2.3.3.4 |  |
| intervalLength | DurationSec | C | 1 | Only applicable when type is “INTERVAL”. See 7.2.3.3.4 and 7.2.3.1-2 |  |
| threshold | Double, Float, Int32, Int64, Uint16, Uint32, Uint64, or Uinteger | C | 1 | Only applicable when type is “THRESHOLD”. See 7.2.3.3.4 and 7.2.3.1-2 |  |
| parameter | string | C | 1 | Only applicable when type is “THRESHOLD”. See 7.2.3.3.4 and 7.2.3.1-2 |  |
| reportWhenBelow | boolean | C | 1 | Only applicable when type is “THRESHOLD”. See 7.2.3.3.4 and 7.2.3.1-2 |  |
| event | Event | C | 1 | Only applicable when type is “EVENT”. See 7.2.3.3.4 and 7.2.3.3.5. |  |
| NOTE: What attributes to use depends on value of “type”. When type is OFF no other attributes should be provided (and will be ignored by receiver). When type is INTERVAL attribute intervalLenght must be provided, not other should be provided (and will be ignored). When type is THRESHOLD parameter and threshold must be provided. The parameter must be one of the attributes present in the configured report data type, see section 7.3. reportWhenBelow as well as intervalLength may be provided. Other attributes should not be provided (and will be ignored). If intervalLenght is provided, then parameter is averaged over that period (sliding window). When type is EVENT then attribute event must be provided and no other attribute should be provided (and will be ignored) |  |  |  |  |  |

#### 7.2.3.3	Simple data types and enumerations

##### 7.2.3.3.1	Introduction

This subclause defines simple data types and enumerations that can be referenced from data structures defined in the previous subclauses.

##### 7.2.3.3.2	Simple data types

The simple data types defined in table 7.2.3.3.2-1 shall be supported.

Table 7.2.3.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |
| AuthenticationInformation | object | Defined jointly between ASP and CSP, out of scope for standardization. May be e.g. an OAuth2.0 token, e.g. a JWT token. |  |

##### 7.2.3.3.3	Enumeration: DataDomain

Table 7.2.3.3.3-1: Enumeration DataDomain

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| SERVICE_EXPERIENCE | Service Experience related data |  |
| PLANNED_ROUTES | Data related to planned routes |  |
| LOCATION | Location related data |  |
| COMMUNICATION | Data related to communication |  |

##### 7.2.3.3.4	Enumeration: ConditionType

Table 7.2.3.3.4-1: Enumeration ConditionType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| INTERVAL | Report with regular interval |  |
| THRESHOLD | Report when a threshold is passed |  |
| EVENT | Report on event |  |
| OFF | Do not report |  |

##### 7.2.3.3.5	Enumeration: Event

Table 7.2.3.3.5-1: Enumeration Event

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| DESTINATION | A new destination has been recorded |  |

### 7.2.4	Error handling

TBA

### 7.2.4	Mediation by NEF

## 7.3	Data Reporting API

### 7.3.1	Overview

This clause specifies the reporting API used by a Direct Data Collection Client, an Indirect Data Collection Client, or an Application Server to report UE data that has been collected to the Data Collection AF.

### 7.3.2	Resources

#### 7.3.2.1	Resource Structure

/{sessionId}

/report

{apiRoot]/ndcaf-datareporting/v1

Table 7.2.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 7.2.2.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| Data Collection Report | /sessions/{sessionId} | POST | Data Collection Client provides data in the established session |

#### 7.3.2.2	Resource: Data Collection Report

##### 7.3.2.2.1	Description

The Data Collection Report resource allows a Data Collection Client to report data pertaining to an established Individual Data Collection Session. The Data Collection AF can provide an updated configuration in the response.

##### 7.3.2.2.2	Resource definition

Resource URI: {apiRoot}/ndcaf-datareporting/v1/report/{sessionId}

This resource shall support the resource URI variables defined in table 7.3.2.2.2-1.

Table 7.3.2.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | FQDN of the DC AF |

##### 7.3.2.2.3	Resource Standard Methods

###### 7.3.2.2.3.1	POST

This method shall support the URI query parameters specified in table 7.3.2.2.3.1-1.

Table 7.3.2.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.3.2.2.3.1-2 and the response data structures and response codes specified in table 7.3.2.2.3.1-4.

Table 7.3.2.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| DataCollectionReport | M | 1 | Data supplied by the Data Collection Client to enable creation of a new Data Collection Session. |

Table 7.3.2.2.3.1-3: Headers supported for POST requests on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Origin | string | O | 0..1 | Indicates the origin of the requester. |

Table 7.3.2.2.3.1-4: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| DataCollectionSessionConfiguration | O | 0..1 | 200 OK | The report was accepted. A configuration block (updated or unchanged) may optionally be provided in the response. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [6] also apply. |  |  |  |  |

Table 7.3.2.2.3.1-5: Headers supported by the 200 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Access-Control-Allow-Origin | string | O | 0..1 | Part of CORS, supplied if the request included the “Origin” header. |
| Access-Control-Allow-Methods | string | O | 0..1 | Part of CORS, supplied if the request included the “Origin” header. Value: “POST” |

N.B. standard http re-directing using 3xx response with Location header as well as Alt-Svc are allowed.

##### 7.3.2.2.4	Resource Custom Operations

None in this release of the specification.

### 7.3.3	Data Model

#### 7.3.3.1	General

This subclause specifies the application data model supported by the API.

Table 7.3.3.1-1 specifies the data types defined for the Ndcaf_DataReporting_Report service based interface protocol.

Table 7.3.3.1-1: Ndcaf_DataReporting_Report specific Data Types

| Data type | Section defined | Description | Applicability |
| --- | --- | --- | --- |
| DataCollectionReport | 7.3.3.2.2 | Data provided by Data Collection Client when reporting to the DC AF. |  |
| DataCollectionSessionConfiguration | 7.2.3.2.3 | Configuration of the Data Collection Client. |  |

Table 7.3.3.1-2 specifies data types re-used by the Ndcaf_DataReporting_Report service based interface protocol from other specifications, including a reference to their respective specifications.

Table 7.3.3.1-2: Ndcaf_DataReporting_Report re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| ApplicationId | 3GPP TS 29.571 | Identifies the application |  |
| DateTime | 3GPP TS 29.571 |  |  |
| DurationSec | 3GPP TS 29.571 |  |  |
| SvcExperience | 3GPP TS 29.517 |  |  |
| TimeWindow | 3GPP TS 29.122 |  |  |
| Volume | 3GPP TS 29.122 |  |  |
| FlowInfo | 3GPP TS 29.122 |  |  |
| LocationData | 3GPP TS 29.572 |  |  |

#### 7.3.3.2	Structured data types

##### 7.3.3.2.1	Introduction

This subclause defines the structures to be used in resource representations.

##### 7.3.3.2.2	Type DataCollectionReport

Table 7.3.3.2.2-1: Definition of type DataCollectionSessionReport

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| applicationId | ApplicationID | M | 1 | External Application Id |  |
| authenticationInfo | AuthenticationInformation | M | 1 | 7.2.3.3.2 |  |
| communicationReports | array(CommunicationReport) | O | 0..1 | 7.3.3.2.5 |  |
| locationReports | array(LocationReport) | O | 0..1 | 7.3.3.2.7 |  |
| serviceExperienceReports | array(ServiceExperienceReport) | O | 0..1 | 7.3.3.2.3 |  |
| NOTE: It is expected that a report would contain one type of reports. |  |  |  |  |  |

##### 7.3.3.2.3	Type ServiceExperienceReport

Table 7.3.3.2.2-1: Definition of type DataCollectionSessionReport

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| timestamp | DateTime | M | 1 | Time stamp. |  |
| serviceExperience | SvcExperience | M | 1 | TS 29.517 5.6.2.9 |  |
| timeWindow | TimeWindow | M | 1 |  |  |
| componentMetrics | array(ComponentMetrics) | M | 1 | 7.3.3.2.4 |  |

##### 7.3.3.2.4	Type ComponentMetrics

Table 7.3.3.2.2-1: Definition of type DataCollectionSessionReport

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| type | Components | M | 1 |  |  |
| timeWindow | TimeWindow | O | 0..1 | Measurement period |  |
| ipPacketInfo | FlowInfo | O | 0..1 |  |  |
| thrputUl | BitRate | O | 0..1 | Indicates the average uplink throughput over the measurement period. |  |
| thrputDl | BitRate | O | 0..1 | Indicates the average downlink throughput over the measurement period. |  |
| thrputPkUl | BitRate | O | 0..1 | Indicates the peak uplink throughput over the measurement period. |  |
| thrputPkDl | BitRate | O | 0..1 | Indicates the peak downlink throughput over the measurement period. |  |
| ulVol | Volume | O | 0..1 | Uplink traffic Volume |  |
| dlVol | Volume | O | 0..1 | Downlink traffic Volume |  |
| jitter | Double | O | 0..1 | Calculated as defined in section 6.4.1. of [RFC3550]. |  |
| jitterBufferDelay | Double | O | 0..1 | Latency from inject to emit. |  |
| packetsReceived | Double | O | 0..1 | Calculated as defined in section 6.4.1. of [RFC3550]. |  |
| packetsLost | Double | O | 0..1 | Calculated as defined in section 6.4.1. of [RFC3550]. |  |
| bytesReceived | Double | O | 0..1 |  |  |
| codec | Codec | O | 0..1 | Mime type. |  |
| framesPerSecond | Double | O | 0..1 |  |  |
| initialPlayoutDelay | Double | O | 0..1 |  |  |
| stoppedPlayEvents | array(StopPlay) | O | 0..1 |  |  |

##### 7.3.3.2.5	Type CommunicationReport

Table 7.3.3.2.5-1: Definition of type CommunicationReport

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| timeWindow | TimeWindow | M | 1 | Time stamp. |  |
| ulVol | Volume | O | 0..1 |  |  |
| dlvol | Volume | O | 0..1 |  |  |
| NOTE: At least one of ulVol or dlVol shall be provided. |  |  |  |  |  |

##### 7.3.3.2.6	Type StopPlay

Table 7.3.3.2.6-1: Definition of type StopPlay

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| timestamp | DateTime | M | 1 | Time stamp. |  |
| reason | StopReason | M | 1 |  |  |

##### 7.3.3.2.7	Type LocationReport

Table 7.3.3.2.6-1: Definition of type LocationReport

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| timestamp | DateTime | M | 1 | Time stamp. |  |
| location | LocationData | M | 1 |  |  |

#### 7.3.3.3	Simple data types and enumerations

##### 7.3.3.3.1	Introduction

This subclause defines simple data types and enumerations that can be referenced from data structures defined in the previous subclauses.

##### 7.3.3.3.2	Simple data types

The simple data types defined in table 7.3.3.3.2-1 shall be supported.

Table 7.2.3.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

##### 7.3.3.3.1	Enumeration: Components

Table 7.2.3.3.4-1: Enumeration DataSet

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| AUDIO |  |  |
| VIDEO |  |  |
| AUDIO_AND_VIDEO |  |  |

##### 7.3.3.3.2	Enumeration: StopReason

Table 7.2.3.3.4-1: Enumeration DataSet

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| REPRESENTATIONSWITCH |  |  |
| REBUFFERING |  |  |
| ENDOFCONTENT |  |  |

##### 7.3.3.3.3	Enumeration: Event

Table 7.2.3.3.5-1: Enumeration Event

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| LOCATION | A new location has been entered |  |
| DESTINATION | A new destination has been entered |  |