3GPP TSG-S4 Meeting Post #116-e MBS AHG Call	S4aI211260

# Online, 2nd  December 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.531 | CR |  | rev |  | Current version: | 0.1.2 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedures for Access Restriction – Stage 2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | EVEX |  |  |  |  |  | Date: |  |  | 4th November 2021 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Introduces the provisioning for access profiles. |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

## 4.5	Information security model

4.5.1	Transport Security

An encrypted data transfer protocol shall be employed at reference point R2 to protect the secrecy and integrity of collected UE data in transit between the Direct Data Collection Client and the Data Collection AF.

4.5.2	Access Profile Data Model

The data model for provisioning the access profile is specified in Table 4.5.2-1.

Table 4.5.2-1 Definition of AccessProfileConfiguration

| Property name | Type | Cardinality | Usage | Description |
| --- | --- | --- | --- | --- |
| AccessProfileConfiguration | Object | 1..1 | C:RW U:RW | Provides a configuration of the access profiles to limit the level of access to the collected data during event exposure. |
| AccessProfiles | Object | 1..n | C:RW U:RW | One or more access profile defintions, where each access profile defines a level of access. |
| ProfileId | string | 1..1 | C:RW U:RW | A unique identifier of this access profile. |
| AuthorizationURL | string | 0..1 | C:RW U:RW | The URL that should be used to authorize for this level of access. |
| EventConfigurations | Object | 1..n | C:RW U:RW | The access configurations for all event data that is offered by this access profile. |
| EventId | string | 1..1 | C:RW U:RW | An identifier of the event for which access is restricted by this EventConfiguration. |
| TimeAccess | Object | 1..n | C:RW U:RW | Configuration for access restrictions on the time dimension. |
| Period |  | 1..1 | C:RW U:RW | the period of time over which access is to be aggregated. |
| Unit |  | 1..1 | C:RW U:RW | The unit of time over which access is to be aggregated. |
| AggregationFunctions | Array | 1..n | C:RW U:RW | the aggregation functions that must be applied over the event data to be exposed. At least one aggregation function from the list shall be applied. |
| UserAccess | Object | 1..n | C:RW U:RW | Configuration for access restrictions on the user dimension. |
| GroupIds |  | 1..n | C:RW U:RW | Identifier of the UE groups over which access is to be aggregated. |
| UserIds |  | 1..n | C:RW U:RW | Identifier of the UEs that build a group over which access is to be aggregated. |
| LocationAreas | LocationArea5G | 1..n | C:RW U:RW | identifiers of location areas over which access is to be aggregated. Event data is grouped by the location of the UE during the data collection. |
| AggregationFunctions | Array | 1..n | C:RW U:RW | the list of aggregation functions that need to be applied on the exposed data. At least one aggregation function shall be applied. |

The allowed aggregation functions are:

- COUNT: the number of observed events

- AVG: the average of the values of the event

- MAX: the maximal observed value of the event

- MIN: the minimal observed value of the event

- SUM: the sum of the values of the event

- NONE: no aggregation is applied, and all values of the event are returned

The procedure for access authorization is depicted by the following call flow:

The steps are:

- The ASP provisions the data collection and the report exposure functionality using the R1 interface.

- The DCAF sends the reporting configuration to the data collection clients such as on the UE and starts receiving reports using the R2 interface.

- An event consumer sends a subscription request to the DCAF to receive reports on the R6 interface

- The DCAF redirects the event consumer to the authorization Application Server to get access based on the requested access profile.

- The event consumer contacts the authorization AS with its authorization credentials and the requested access profile.

- If access is granted, the authorization AS responds with an access token that is valid for a specific period of time and for the allowed access profile.

- The event consumer resends the subscription request with the access token to the DCAF

- The DCAF may verify the access token with the authorization server, or it may verify it locally

- If verification is successful, the DCAF approves the subscription request for the allowed access profile

- The DCAF sends notifications about reports to the event consumer.