3GPP TSG- S4 Meeting #117e - Post	S4aI221321

# Electronic Meeting, 2nd March – 11th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Psuedo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.805 | CR | <CR#> | rev | <Rev#> | Current version: | 1.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_NPN4AVProd]: Description of KI#4 (Standby and Program Cameras), incl solutions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | S4 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_NPN4AVProd |  |  |  |  |  | Date: |  |  | <Res_date> |
| Category: | <Cat> |  |  |  |  |  | Release: |  |  | <Release> |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

## 6.5	Key Issue #4: Different bit rates for Standby vs Program Cameras

Editor’s Note: This clause should describe implications on protocol usage, when only the program camera(s) send a high quality stream. Standby cameras only send a video stream with preview quality or no data.

### 6.5.1	General

In professional media production use-cases, several cameras are simultaneously capturing the event. Each camera focuses on different parts of the event, like audience, certain actors / players or a panorama view. The video of all cameras are displayed on a display gallery within the local studio, e.g. within the OB van. A program director is deciding, which of the cameras is selected for the outgoing program feed. On request from the program director, the video mixer changes the source from a first video camera to a second video camera.

The camera, which is currently used for the program feed is called Program Camera in the following. In case of fast-fadeover between cameras, typically only a single camera is a Program Camera. However, often some fading effects are applied during the transition from one camera to the next, thus, two or more cameras are contributing to the program feed.

All the cameras, which are not directly contributing to the program feed and just rendered on the disply gallery, are called Standby Cameras.

In principle, the video quality from the standy cameras does not need to be as high as the video quality of the program camera. The assumption in this key issue is, that output quality of a video camera is controlled by the mixer function using a dedicated control flows.

Figure 6.5.1-1:

The setup is depicted in figure 6.5.1-1. Five Cameras UEs are depicted on the left side. Functional wise, the different Camera UEs contain the same functionality. Configuration wise, the Camera UE #1 and Camera UE #4 are configured to operate in a program quality profile, while Camera UEs #2. #3 and #5 operate in stand-by quality profile. The Mixer / Selection function is controlled based on the input from from the program direction.

### 6.5.2 Solutions leveraging QoS

The 3GPP QoS Framework can be used to separate the different flows. Figure 6.5.2-1 illustrates the usage of QoS flows. The video of Camera UE #1 and #3 are mapped to program quality QoS flows. All the other camera output is mapped to Stand-by quality QoS flows.

The traffic for switching the camera profile configuration is handled by yet another QoS flow.

Figure 6.5.2-1:

There are two basic solution alternatives:

A:	Modify the QoS rule within a Camera UE, once the profile is switched between Program and Stand-by

B: Pre-install one QoS rule for each camera profile with different packet filters.

Solution alternative A leverages the procedure to modify a QoS rule. When switching a camera from program to standby, the QoS rule is modified and a new QoS policy (with a different QFI) is installed on the UE and the UPF.

Figure 6.5.2-2: Solution Alternative A

Steps:

For Solution alternative B, two QoS flows (one for Program and one for Standby quality) are established for each camera. The Packet Filters are different, e.g. using different 5-Tuples or different ToS / DSCP values.

Figure 6.5.2-3: Solution Alternative B

**** Next Change ****

**** Last Change ****