3GPP TSG-S4 Ad hoc post Meeting #119-e	S4aI221351

# Online,  , 27th May 2022–9th August 2022	revision of S4aI221342

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 26.512 | CR | – | rev | – | Current version: | 17.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [5GMS_EDGE3] [EVEX] Rel-17 API corrections |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BBC |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS_EDGE_3, EVEX |  |  |  |  |  | Date: |  | 2022-07-25 |
| Category: | F |  |  |  |  |  | Release: |  | Rel-17 |
| Reason for change: |  | Corrections to interface definitions not fully addressed in previous meeting cycle. |  |  |  |  |  |  |  |
| Summary of change: |  | Corrections in normative clauses: - Add array of Edge Resources Configurations to Provisioning Session resource. - Correct camelcasing of Service Access Information resource subobject property names. - Move metrics reporting scheme property from the Dynamic Policy Invocation Configuration to the Client Metrics Reporting Configurations (incorrect implementation of CR0014). - Insert correct cross-references to TS 29.517. OpenAPI corrections: - Bump all OpenAPI version numbers to 2.0.0 (convention to match v2 path element in endpoint URL template). - Fix typo in EdgeProcessingEligibilityCriteria common data type. - Add missing ProvisioningSession.externalApplicationId to ProvisioningSession. - Add edgeResourcesConfigurationIds and eventDataProcessing‌ConfigurationIds to ProvisioningSession. - Add missing createEdgeResourcesConfiguration operation in Edge Resources Provisioning API. - Correct type of M1EASRelocationRequirements.tolerance to reference common EASRelocationTolerance type defintion. - Fix indentation of EASRequirements.easProviderIds array type. - Correct camelcasing of subobject property names in Service‌Access‌Information‌Resource definition. - Fix indentation of eASRelocationRequirements and eASDiscoveryTemplate subobjects of Service‌Access‌Information‌Resource. |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent speification. |  |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.3.1, 11.2.3.1, 18, C.2, C.3, C.4 |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 7.2.3.1	ProvisioningSession resource

The data model for the ProvisioningSession resource is specified in Table 7.2.3.1-1 below. Different properties are present in the resource depending on the type of Provisioning Session indicated in the provisioningSessionType property, and this is specified in the Applicability column.

Table 7.2.3.1-1: Definition of ProvisioningSession resource

| Property name | Type | Cardinality | Usage | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| provisioningSessionId | ResourceId | 1..1 | C: R R: RO | A unique identifier for this Provisioning Session. | All types. |
| provisioningSession‌Type | Provisioning‌Session‌Type | 1..1 | C: RW R: RO U: – | The type of Provisioning Session. | All types. |
| aspId | AspId | 0..1 | C: W R: RO | The identity of the Application Service Provider responsible for this Provisioning Session, as specified in clause 5.6.2.3 of TS 29.514 [34]. | All types. |
| externalApplicationId | ApplicationId | 1..1 | C: RW R: RO U: RO | The external application identifier (see TS 29.571 [12]), nominated by the 5GMS Application Provider, to which this Provisioning Session pertains. | All types. |
| serverCertificateIds | Array(ResourceId) | 0..1 | C: – R: RO | A list of Server Certificate identifiers currently associated with this Provisioning Session. | downlink |
| contentPreparation‌TemplateIds | Array(ResourceId) | 0..1 | C: – R: RO | A list of Content Preparation Template identifiers currently associated with this Provisioning Session. | downlink, uplink |
| metricsReporting‌ConfigurationIds | Array(ResourceId) | 0..1 | C: – R: RO | A list of Metrics Reporting Configuration identifiers currently associated with this Provisioning Session. | downlink, uplink |
| policyTemplateIds | Array(ResourceId) | 0..1 | C: – R: RO | A list of Policy Template identifiers currently associated with this Provisioning Session. | downlink, uplink |
| edgeResources‌ConfigurationIds | Array(ResourceId) | 0..1 | C: – R: RO | A list of Edge Resources Configuration identifiers currently associated with this Provisioning Session. | downlink, uplink |
| eventDataProcessing‌ConfigurationIds | Array(ResourceId) | 0..1 | C: – R: RO | A list of Event Data Processing Configuration identifiers currently associated with this Provisioning Session. | downlink, uplink |

NEXT CHANGE

#### 11.2.3.1	ServiceAccessInformation resource type

The data model for the ServiceAccessInformtion resource is specified in table 11.2.3.1-1 below. Different properties are present in the resource depending on the type of Provisioning Session from which the Service Access Information is derived (as indicated in the provisioningSessionType property) and this is specified in the Applicability column.

Table 11.2.3.1-1: Definition of ServiceAccessInformation resource

| Property name | Type | Cardinality | Usage | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| provisioningSessionId | ResourceId | 1..1 | RO | Unique identification of the M1 Provisioning Session. | All types |
| provisioningSession‌Type | Provisioning‌Session‌Type | 1..1 | RO | The type of Provisioning Session. | All types. |
| streamingAccess | Object | 0..1 | RO |  | downlink |
| mediaPlayerEntry | Url | 0..1 | RO | A document or a pointer to a document that defines a media presentation e.g. MPD for DASH content or URL to a video clip file. |  |
| eMBMS‌Service‌Announcement‌Locator | Url | 0..1 | RO | A pointer to a document that defines a User Service Announcement for eMBMS where the service announcement file is available. | downlink |
| clientConsumptionReporting‌Configuration | Object | 0..1 | RO |  | downlink |
| reportingInterval | DurationSec | 0..1 | RO | The time interval, expressed in seconds, between consumption report messages being sent by the Media Session Handler. The value shall be greater than zero. When this property is omitted, a single final report shall be sent immediately after the media streaming session has ended. |  |
| serverAddresses | Array(Url) | 1..1 | RO | A list of 5GMSd AF addresses (URLs) where the consumption reporting messages are sent by the Media Session Handler. See NOTE. (Opaque URL, following the 5GMS URL format.) |  |
| locationReporting | Boolean | 1..1 | RO | Stipulates whether the Media Session Handler is required to provide location data to the 5GMSd AF in consumption reporting messages (in case of MNO or trusted third parties). |  |
| accessReporting | Boolean | 1..1 | RO | Stipulates whether the Media Session Handler is required to provide consumption reporting messages to the 5GMSd AF when the access network changes during a media streaming session. |  |
| samplePercentage | Percentage | 1..1 | RO | The percentage of media streaming sessions that shall send consumption reports, expressed as a floating point value between 0.0 and 100.0. |  |
| dynamicPolicyInvocation‌Configuration | Object | 0..1 | RO |  | downlink, uplink |
| serverAddresses | Array(Url) | 1..1 | RO | A list of 5GMSd AF addresses (URLs) which offer the APIs for dynamic policy invocation sent by the Media Session Handler. See NOTE. (Opaque URL, following the 5GMS URL format.) |  |
| validPolicyTemplateIds | Array(ResourceId) | 1..1 | RO | A list of Policy Template identifiers which the 5GMS Client is authorized to use. |  |
| sdfMethods | Array(SdfMethod) | 1..1 | RO | A list of recommended service data flow description methods (descriptors), e.g. 5-Tuple, ToS, 2-Tuple, etc., which should be used by the Media Session Handler to describe the service data flows for the traffic to be policed. |  |
| externalReferences | Array(String) | 0..1 | RO | Additional identifier for this Policy Template, unique within the scope of its Provisioning Session, that can be cross-referenced with external metadata about the media streaming session. Example: "HD_Premium". |  |
| clientMetricsReporting‌Configurations | Array(Object) | 0..1 | RO |  | downlink, uplink |
| serverAddresses | Array(Url) | 1..1 | RO | A list of 5GMS AF addresses to which metrics reports shall be sent. See NOTE. (Opaque URL, following the 5GMS URL format.) |  |
| scheme | Uri | 1..1 | RO | The metrics reporting scheme that metrics reports shall use (see clause 4.7.5). |  |
| dataNetworkName | Dnn | 0..1 | RO | The DNN which shall be used when sending metrics reports. If not specified, the name of the default DN shall be used. |  |
| reportingInterval | DurationSec | 0..1 | RO | The time interval, expressed in seconds, between metrics reports being sent by the Media Session Handler. The value shall be greater than zero. When this property is omitted, a single final report shall be sent immediately after the media streaming session has ended. |  |
| samplePercentage | Percentage | 1..1 | RO | The percentage of media streaming sessions that shall report metrics, expressed as a floating point value between 0.0 and 100.0. |  |
| urlFilters | Array(String) | 0..1 | RO | A non-empty list of URL patterns for which metrics reporting shall be done. The format of each pattern shall be a regular expression as specified in [5]. If not specified, reporting shall be done for all sessions. |  |
| metrics | Array(String) | 1..1 | RO | A list of metrics which shall be reported. |  |
| networkAssistanceConfiguration | Object | 0..1 | RO |  | downlink, uplink |
| serverAddress | Url | 1..1 | RO | Address of the 5GMS AF that offers the APIs for 5GMS AF-based Network Assistance, for access by the 5GMSd Media Session Handler. See NOTE. This address shall be an opaque URL, following the 5GMS URL format. |  |
| client‌EdgeResources‌Configuration | Object | 0..1 | RO | Present only for Provisioning Sessions with client-driven edge computing management mode provisioned. | downlink, uplink |
| eligibilityCriteria | Edge‌Processing‌Eligibility‌Criteria | 0..1 | RO | Conditions for activating edge resources for media streaming sessions in the scope of this Service Access Information. (See clause 6.4.3.8.) |  |
| easDiscoveryTemplate | EAS‌Discovery‌Template | 1..1 | RO | A template for the EAS discovery filter that shall be used by the EEC to discover and select a 5GMS EAS instance to serve media streaming sessions in the scope of this Service Access Information. (See clause 11.2.3.2.) |  |
| easRelocation‌Requirements | M5EAS‌Relocation‌Requirements | 0..1 | RO | EAS relocation tolerance and requirements. If absent, the EEC shall assume that relocation is tolerated by all 5GMS EAS instances in the scope of this Service Access Information. (See clause 11.2.3.3.) |  |
| NOTE: In deployments where multiple instances of the 5GMSd AF expose the Media Session Handling APIs at M5, the 5G System may use a suitable mechanism (e.g. HTTP load balancing or DNS resolution) to direct requests to a suitable AF instance. |  |  |  |  |  |

NEXT CHANGE

# 18	Event exposure at R5 and R6

The Naf_EventExposure service specified in TS 29.517 [46] shall be used by event consumer to subscribe to the following types of 5G Media Streaming event notifications, identified by their respective Event IDs, from the Data Collection AF instantiated in the 5GMS AF, and subsequently to receive such notifications:

1.	Media Streaming QoE Event, as specified in clause 5.6.2.23 of [46], comprising

a)	3GPP-defined QoE metrics information, and

b)	non-3GPP-defined QoE metrics information.

2.	Media Streaming Consumption Event, as specified in clause 5.6.2.24 of [46].

3.	Media Streaming Network Assistance Invocation Event, as specified in clause 5.6.2.25 of [46].

4.	Media Streaming Dynamic Policy Invocation Event, as specified in clause 5.6.2.26 of [46].

5.	Media Streaming Access Event, as specified in clause 5.6.2.27 of [46].

In this release, eligible event consumer subscribers to the Naf_EventExposure service as specified in [46] are the following:

-	The NWDAF defined in TS 23.288 [47].

-	The Event Consumer AF defined in TS 26.531 [46] when it is deployed in the Trusted DN.

[-	The NEF defined in TS 23.501 [2] when it is used to expose the Naf_EventExposure service to functions outside the Trusted DN via the Nnef_EventExposure service defined in TS 23.502 [45].]

Implementations of the Data Collection AF instantiated the 5GMS AF shall support negotiation of the optional features relating to 5G Media Streaming specified in table 5.8-1 of TS 29.517 [46]. Feature negotiation by event consumers is achieved as specified in clause 5.8 of [46].

NEXT CHANGE

# C.2	Data Types applicable to several APIs

For the purpose of referencing entities defined in this clause, it shall be assumed that the OpenAPI definitions below are contained in a physical file named "TS26512_CommonData.yaml".

openapi: 3.0.0 info: title: 5GMS Common Data Types version: 2.0.0 description: ¦ 5GMS Common Data Types © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. tags: - name: 5GMS Common Data Types description: '5G Media Streaming: Common Data Types' externalDocs: description: 'TS 26.512 V17.1.0; 5G Media Streaming (5GMS); Protocols' url: 'https://www.3gpp.org/ftp/Specs/archive/26_series/26.512/' paths: {} components: schemas: ################################# # Clause 6.4.2: Simple data types ################################# ResourceId: type: string description: String chosen by the 5GMS AF to serve as an identifier in a resource URI. Percentage: type: number minimum: 0.0 maximum: 100.0 #DurationSec is defined in TS29571_CommonData #DateTime is defined in TS29571_CommonData #Uri is defined in TS29571_CommonData Url: type: string format: uri description: Uniform Resource Locator, comforming with the URI Generic Syntax specified in IETF RFC 3986. ##################################### # Clause 6.4.3: Structured data types ##################################### IpPacketFilterSet: type: object required: - direction properties: srcIp: type: string dstIp: type: string protocol: type: integer srcPort: type: integer dstPort: type: integer toSTc: type: string flowLabel: type: integer spi: type: integer direction: type: string ServiceDataFlowDescription: type: object properties: flowDescription: $ref: '#/components/schemas/IpPacketFilterSet' domainName: type: string M5QoSSpecification: type: object required: - marBwDlBitRate - marBwUlBitRate - mirBwDlBitRate - mirBwUlBitRate properties: marBwDlBitRate: $ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate' marBwUlBitRate: $ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate' minDesBwDlBitRate: $ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate' minDesBwUlBitRate: $ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate' mirBwDlBitRate: $ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate' mirBwUlBitRate: $ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate' desLatency: type: integer minimum: 0 desLoss: type: integer minimum: 0 M1QoSSpecification: type: object properties: qosReference: type: string maxBtrUl: $ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate' maxBtrDl: $ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate' maxAuthBtrUl: $ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate' maxAuthBtrDl: $ref: 'TS29571_CommonData.yaml#/components/schemas/BitRate' defPacketLossRateDl: type: integer minimum: 0 defPacketLossRateUl: type: integer minimum: 0 ChargingSpecification: type: object properties: sponId: type: string sponStatus: $ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/SponsoringStatus' gpsi: type: array items: $ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi' TypedLocation: type: object required: - locationIdentifierType - location properties: locationIdentifierType: $ref: '#/components/schemas/CellIdentifierType' location: type: string OperationSuccessResponse: type: object required: - success properties: success: type: boolean reason: type: string CellIdentifierType: anyOf: - type: string enum: [CGI, ECGI, NCGI] - type: string description: > This string provides forward-compatibility with future extensions to the enumeration but is not used to encode content defined in the present version of this API. SdfMethod: anyOf: - type: string enum: [5_TUPLE, 2_TUPLE, TYPE_OF_SERVICE_MARKING, FLOW_LABEL, DOMAIN_NAME] - type: string description: > This string provides forward-compatibility with future extensions to the enumeration but is not used to encode content defined in the present version of this API. ProvisioningSessionType: anyOf: - type: string enum: [DOWNLINK, UPLINK] - type: string description: > This string provides forward-compatibility with future extensions to the enumeration but is not used to encode content defined in the present version of this API. EndpointAddress: type: object required: - portNumber properties: ipv4Addr: $ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr' ipv6Addr: $ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Addr' portNumber: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uint16' CacheStatus: anyOf: - type: string enum: [HIT, MISS, EXPIRED] - type: string description: > This string provides forward-compatibility with future extensions to the enumeration but is not used to encode content defined in the present version of this API. EdgeProcessingEligibilityCriteria: type: object required: - serviceDataFlowDescriptions - ueLocations - timeWindows - appRequest properties: serviceDataFlowDescriptions: type: array items: $ref: '#/components/schemas/ServiceDataFlowDescription' ueLocations: type: array items: $ref: 'TS29122_CommonData.yaml#/components/schemas/LocationArea5G' timeWindows: type: array items: $ref: 'TS29122_CommonData.yaml#/components/schemas/TimeWindow' appRequest: type: boolean EASRelocationTolerance: anyOf: - type: string enum: [RELOCATION_UNAWARE, RELOCATION_TOLERANT, RELOCATION_INTOLERANT] - type: string description: > This string provides forward-compatibility with future extensions to the enumeration but is not used to encode content defined in the present version of this API.

NEXT CHANGE

# C.3	OpenAPI representation of the M1 APIs

## C.3.1	M1_ProvisioningSessions API

openapi: 3.0.0 info: title: M1_ProvisioningSessions version: 2.0.0 description: ¦ 5GMS AF M1 Provisioning Sessions API © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. tags: - name: M1_ProvisioningSessions description: '5G Media Streaming: Provisioning (M1) APIs: Provisioning Sessions' externalDocs: description: 'TS 26.512 V17.1.0; 5G Media Streaming (5GMS); Protocols' url: 'https://www.3gpp.org/ftp/Specs/archive/26_series/26.512/' servers: - url: '{apiRoot}/3gpp-m1/v2' variables: apiRoot: default: https://example.com description: See 3GPP TS 29.512 clause 6.1. paths: /provisioning-sessions: post: operationId: createProvisioningSession summary: 'Create a new Provisioning Session' responses: '201': description: 'Provisioning Session Created' headers: Location: description: 'URL including the resource identifier of the newly created Provisioning Session.' required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/Url' content: application/json: schema: $ref: '#/components/schemas/ProvisioningSession' /provisioning-sessions/{provisioningSessionId}: parameters: - name: provisioningSessionId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of an existing Provisioning Session.' get: operationId: getProvisioningSessionById summary: 'Retrieve an existing Provisioning Session' responses: '200': description: 'Success' content: application/json: schema: $ref: '#/components/schemas/ProvisioningSession' delete: operationId: destroyProvisioningSession summary: 'Destroy an existing Provisioning Session' responses: '204': description: 'Provisioning Session Destroyed' # No Content components: schemas: ProvisioningSession: type: object description: "A representation of a Provisioning Session." required: - provisioningSessionId - provisioningSessionType - externalApplicationId - contentProtocols properties: provisioningSessionId: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' provisioningSessionType: $ref: 'TS26512_CommonData.yaml#/components/schemas/ProvisioningSessionType' aspId: $ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/AspId' externalApplicationId: $ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationId' serverCertificateIds: type: array items: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' minItems: 1 uniqueItems: true contentPreparationTemplateIds: type: array items: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' minItems: 1 uniqueItems: true metricsReportingConfigurationIds: type: array items: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' minItems: 1 uniqueItems: true policyTemplateIds: type: array items: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' minItems: 1 uniqueItems: true edgeResourcesConfigurationIds: type: array items: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' minItems: 1 uniqueItems: true eventDataProcessingConfigurationIds: type: array items: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' minItems: 1 uniqueItems: true

## C.3.2	M1_ServerCertificatesProvisioning API

openapi: 3.0.0 info: title: M1_ServerCertificatesProvisioning version: 2.0.0 description: ¦ 5GMS AF M1 Server Certificates Provisioning API © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. tags: - name: M1_ServerCertificatesProvisioning description: '5G Media Streaming: Provisioning (M1) APIs: Server Certificates Provisioning' externalDocs: description: 'TS 26.512 V17.1.0; 5G Media Streaming (5GMS); Protocols' url: 'https://www.3gpp.org/ftp/Specs/archive/26_series/26.512/' servers: - url: '{apiRoot}/3gpp-m1/v2' variables: apiRoot: default: https://example.com description: See 3GPP TS 29.512 clause 6.1. paths: /provisioning-sessions/{provisioningSessionId}/certificates: parameters: - name: provisioningSessionId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: The resource identifier of an existing Provisioning Session.' post: operationId: createOrReserveServerCertificate summary: 'Create or reserve a Service Certificate resource' description: 'Without the optional csr query parameter, an X.509 certificate is generated and this is returned. If the csr query parameter is present, a Certificate Signing Request is instead generated and returned, allowing the X.509 certificate to be generated by the invoker and later uploaded.' parameters: - in: query name: csr schema: type: string description: 'When present, return a Certificate Signing Request instead of generating an X.509 certificate' responses: '200': description: 'Server Certificate Created' headers: Location: description: 'URL of the newly created Server Certificate resource' required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/Url' content: 'application/x-pem-file': schema: type: string /provisioning-sessions/{provisioningSessionId}/certificates/{certificateId}: parameters: - name: provisioningSessionId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: The resource identifier of an existing Provisioning Session.' - name: certificateId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of an existing Server Certificate' put: operationId: uploadServerCertificate summary: "Upload the X.509 certificate for a previously reserved Server Certificate resource" requestBody: required: true content: application/x-pem-file: schema: type: string responses: '204': description: 'Server Certificate Updated' get: operationId: retrieveServerCertificate summary: 'Retrieve the X.509 certificate representation of the specified Server Certificate resource' responses: '200': description: 'Success' content: 'application/x-pem-file': schema: type: string '204': description: 'Awaiting Upload' '404': description: 'Not Found' delete: operationId: destroyServerCertificate summary: 'Destroy an existing Server Certificate resource' responses: '204': description: 'Server Certificate Destroyed'

## C.3.3	M1_ContentPreparationTemplatesProvisioning API

openapi: 3.0.0 info: title: M1_ContentPreparationTemplatesProvisioning version: 2.0.0 description: ¦ 5GMS AF M1 Content Preparation Templates Provisioning API © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. tags: - name: M1_ContentPreparationTemplatesProvisioning description: '5G Media Streaming: Provisioning (M1) APIs: Content Preparation Templates Provisioning' externalDocs: description: 'TS 26.512 V17.1.0; 5G Media Streaming (5GMS); Protocols' url: 'https://www.3gpp.org/ftp/Specs/archive/26_series/26.512/' servers: - url: '{apiRoot}/3gpp-m1/v2' variables: apiRoot: default: https://example.com description: See 3GPP TS 29.512 clause 6.1. paths: /provisioning-sessions/{provisioningSessionId}/content-preparation-templates: parameters: - name: provisioningSessionId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of an existing Provisioning Session.' post: operationId: createContentPreparationTemplate summary: 'Create (and optionally upload) a new Content Preparation Template for the specified Provisioning Session' requestBody: description: 'A Content Preparation Template of any type' required: true content: '*/*': schema: type: string responses: '201': description: 'Content Preparation Template Created' headers: Location: description: 'URL of the newly created Content Preparation Template.' required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/Url' /provisioning-sessions/{provisioningSessionId}/content-preparation-templates/{contentPreparationTemplateId}: parameters: - name: provisioningSessionId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of an existing Provisioning Session.' - name: contentPreparationTemplateId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of an existing Content Preparation Template.' get: operationId: retrieveContentPreparationTemplate summary: 'Retrieve the specified Content Preparation Template of the specified Provisioning Session' responses: '200': description: 'Success' content: '*/*': schema: type: string put: operationId: updateContentPreparationTemplate summary: 'Update the specified Content Preparation Template for the specified Provisioning Session' requestBody: description: 'A Content Preparation Template of any type' required: true content: '*/*': schema: type: string responses: '204': description: 'Updated Content Preparation Template' '404': description: 'Not Found' patch: operationId: patchContentPreparationTemplate summary: 'Patch the specified Content Preparation Template for the specified Provisioning Session' requestBody: description: 'A Content Preparation Template patch of any type' required: true content: application/merge-patch+json: schema: type: string application/json-patch+json: schema: type: string responses: '200': description: 'Patched Content Preparation Templates' content: '*/*': schema: type: string '404': description: 'Not Found' delete: operationId: destroyContentPreparationTemplate summary: 'Destroy the specified Content Preparation Template of the specified Provisioning Session' responses: '204': description: 'Destroyed Content Preparation Template' '404': description: 'Not Found'

## C.3.4	M1_ContentProtocolsDiscovery API

openapi: 3.0.0 info: title: M1_ContentProtocolsDiscovery version: 2.0.0 description: ¦ 5GMS AF M1 Content Protocols Discovery API © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. tags: - name: M1_ContentDiscovery description: '5G Media Streaming: Provisioning (M1) APIs: Content Protocols Discovery' externalDocs: description: 'TS 26.512 V17.1.0; 5G Media Streaming (5GMS); Protocols' url: 'https://www.3gpp.org/ftp/Specs/archive/26_series/26.512/' servers: - url: '{apiRoot}/3gpp-m1/v2' variables: apiRoot: default: https://example.com description: See 3GPP TS 29.512 clause 6.1. paths: /provisioning-sessions/{provisioningSessionId}/protocols: parameters: - name: provisioningSessionId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of an existing Provisioning Session.' get: operationId: retrieveContentProtocols summary: 'Retrieve the set of Content Protocols supported by the specified Provisioning Session' responses: '200': description: 'Success' content: application/json: schema: $ref: '#/components/schemas/ContentProtocols' components: schemas: ContentProtocolDescriptor: type: object required: - termIdentifier properties: termIdentifier: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri' descriptionLocator: $ref: 'TS26512_CommonData.yaml#/components/schemas/Url' ContentProtocols: type: object properties: downlinkIngestProtocols: type: array items: $ref: '#/components/schemas/ContentProtocolDescriptor' minItems: 1 uplinkEgestProtocols: type: array items: $ref: '#/components/schemas/ContentProtocolDescriptor' minItems: 1 geoFencingLocatorTypes: type: array items: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri' minItems: 1

## C.3.5	M1_ContentHostingProvisioning API

openapi: 3.0.0 info: title: M1_ContentHostingProvisioning version: 2.0.0 description: ¦ 5GMS AF M1 Content Hosting Provisioning API © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. tags: - name: M1_ContentHostingProvisioning description: '5G Media Streaming: Provisioning (M1) APIs: Content Hosting Provisioning' externalDocs: description: 'TS 26.512 V17.1.0; 5G Media Streaming (5GMS); Protocols' url: 'https://www.3gpp.org/ftp/Specs/archive/26_series/26.512/' servers: - url: '{apiRoot}/3gpp-m1/v2' variables: apiRoot: default: https://example.com description: See 3GPP TS 29.512 clause 6.1. paths: /provisioning-sessions/{provisioningSessionId}/content-hosting-configuration: parameters: - name: provisioningSessionId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of an existing Provisioning Session.' post: operationId: createContentHostingConfiguration summary: 'Create (and optionally upload) the Content Hosting Configuration for the specified Provisioning Session' requestBody: description: 'A JSON representation of a Content Hosting Configuration' required: true content: application/json: schema: $ref: '#/components/schemas/ContentHostingConfiguration' responses: '201': description: 'Content Hosting Configuration Created' headers: Location: description: 'URL of the newly created Content Hosting Configuration (same as request URL).' required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/Url' get: operationId: retrieveContentHostingConfiguration summary: 'Retrieve the Content Hosting Configuration of the specified Provisioning Session' responses: '200': description: 'Success' content: application/json: schema: $ref: '#/components/schemas/ContentHostingConfiguration' '404': description: 'Not Found' put: operationId: updateContentHostingConfiguration summary: 'Update the Content Hosting Configuration for the specified Provisioning Session' requestBody: description: 'A JSON representation of a Content Hosting Configuration' required: true content: application/json: schema: $ref: '#/components/schemas/ContentHostingConfiguration' responses: '204': description: 'Updated Content Hosting Configuration' '404': description: 'Not Found' patch: operationId: patchContentHostingConfiguration summary: 'Patch the Content Hosting Configuration for the specified Provisioning Session' requestBody: description: 'A JSON representation of a Content Hosting Configuration' required: true content: application/merge-patch+json: schema: $ref: '#/components/schemas/ContentHostingConfiguration' application/json-patch+json: schema: $ref: '#/components/schemas/ContentHostingConfiguration' responses: '200': description: 'Patched Content Hosting Configuration' content: application/json: schema: $ref: '#/components/schemas/ContentHostingConfiguration' '404': description: 'Not Found' delete: operationId: destroyContentHostingConfiguration summary: 'Destroy the current Content Hosting Configuration of the specified Provisioning Session' responses: '204': description: 'Destroyed Content Hosting Configuration' '404': description: 'Not Found' /provisioning-sessions/{provisioningSessionId}/content-hosting-configuration/purge: parameters: - name: provisioningSessionId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: A unique identifier of the Provisioning post: operationId: purgeContentHostingCache summary: 'Purge the content of the cache for the Content Hosting Configuration of the specified Provisioning Session' requestBody: description: 'The regular expression pattern for resources to purge from the cache' required: true content: application/x-www-form-urlencoded: schema: properties: pattern: description: 'Keyword' type: string value: description: 'The regular expression' type: string responses: '200': description: 'Content Purged' components: schemas: IngestConfiguration: type: object properties: path: type: string pull: type: boolean protocol: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri' entryPoint: $ref: 'TS26512_CommonData.yaml#/components/schemas/Url' PathRewriteRule: type: object required: - requestPattern - mappedPath properties: requestPattern: type: string mappedPath: type: string CachingConfiguration: type: object properties: urlPatternFilter: type: string CachingDirectives: type: object required: - urlPatternFilter - noCache properties: statusCodeFilters: type: array items: type: integer noCache: type: boolean maxAge: type: integer format: int32 DistributionConfiguration: type: object required: - canonicalDomainName - domainNameAlias properties: contentPreparationTemplateId: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' canonicalDomainName: type: string domainNameAlias: type: string pathRewriteRules: type: array items: $ref: '#/components/schemas/PathRewriteRule' cachingConfigurations: type: array items: $ref: '#/components/schemas/CachingConfiguration' GeoFencing: type: object required: - locatorType - locators properties: locatorType: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri' locators: type: array items: type: string description: 'Format of individual locators depends on the locatorType.' minItems: 1 UrlSignature: type: object required: - urlPattern - tokenName - passphraseName - passphrase - tokenExpiryName - useIPAddress properties: urlPattern: type: string tokenName: type: string passphraseName: type: string passphrase: type: string tokenExpiryName: type: string useIPAddress: type: boolean ipAddressName: type: string certificateId: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' supplementaryDistributionNetworks: type: object properties: default: $ref: '#/components/schemas/DistributionNetworkType' required: - default additionalProperties: $ref: '#/components/schemas/DistributionMode' # Schema for the resource itself ContentHostingConfiguration: type: object required: - name - ingestConfiguration - distributionConfigurations properties: name: type: string ingestConfiguration: $ref: '#/components/schemas/IngestConfiguration' distributionConfigurations: type: array items: $ref: '#/components/schemas/DistributionConfiguration' DistributionNetworkType: anyOf: - type: string enum: [NETWORK_EMBMS] - type: string description: > This string provides forward-compatibility with future extensions to the enumeration but is not used to encode content defined in the present version of this API. DistributionMode: anyOf: - type: string enum: [MODE_EXCLUSIVE, MODE_HYBRID, MODE_DYNAMIC] - type: string description: > This string provides forward-compatibility with future extensions to the enumeration but is not used to encode content defined in the present version of this API.

## C.3.6	M1_ConsumptionReportingProvisioning API

openapi: 3.0.0 info: title: M1_ConsumptionReportingProvisioning version: 2.0.0 description: ¦ 5GMS AF M1 Consumption Reporting Provisioning API © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. tags: - name: M1_ConsumptionReportingProvisioning description: '5G Media Streaming: Provisioning (M1) APIs: Consumption Reporting Provisioning' externalDocs: description: 'TS 26.512 V17.1.0; 5G Media Streaming (5GMS); Protocols' url: 'https://www.3gpp.org/ftp/Specs/archive/26_series/26.512/' servers: - url: '{apiRoot}/3gpp-m1/v2' variables: apiRoot: default: https://example.com description: See 3GPP TS 29.512 clause 6.1. paths: /provisioning-sessions/{provisioningSessionId}/consumption-reporting-configuration: parameters: - name: provisioningSessionId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of an existing Provisioning Session.' post: operationId: activateConsumptionReporting summary: 'Activate the consumption reporting procedure for the specified Provisioning Session by providing the Consumption Reporting Configuration' requestBody: description: 'A JSON representation of a Consumption Reporting Configuration' required: true content: application/json: schema: $ref: '#/components/schemas/ConsumptionReportingConfiguration' responses: '201': description: 'Consumption Reporting Configuration Created' headers: Location: description: 'URL of the newly created Consumption Reporting Configuration (same as request URL).' required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/Url' get: operationId: retrieveConsumptionReportingConfiguration summary: 'Retrieve the Consumption Reporting Configuration of the specified Provisioning Session' responses: '200': description: 'Success' content: application/json: schema: $ref: '#/components/schemas/ConsumptionReportingConfiguration' put: operationId: updateConsumptionReportingConfiguration summary: 'Update the Consumption Reporting Configuration for the specified Provisioning Session' requestBody: description: 'A JSON representation of a Consumption Reporting Configuration' required: true content: application/json: schema: $ref: '#/components/schemas/ConsumptionReportingConfiguration' responses: '204': description: 'Updated Consumption Reporting Configuration' '404': description: 'Not Found' patch: operationId: patchConsumptionReportingConfiguration summary: 'Patch the Consumption Reporting Configuration for the specified Provisioning Session' requestBody: description: 'A JSON representation of a Consumption Reporting Configuration' required: true content: application/merge-patch+json: schema: $ref: '#/components/schemas/ConsumptionReportingConfiguration' application/json-patch+json: schema: $ref: '#/components/schemas/ConsumptionReportingConfiguration' responses: '200': description: 'Patched Consumption Reporting Configuration' content: application/json: schema: $ref: '#/components/schemas/ConsumptionReportingConfiguration' '404': description: 'Not Found' delete: operationId: destroyConsumptionReportingConfiguration summary: 'Destroy the current Consumption Reporting Configuration of the specified Provisioning Session' responses: '204': description: 'Destroyed Consumption Reporting Configuration' '404': description: 'Not Found' components: schemas: ConsumptionReportingConfiguration: type: object properties: reportingInterval: $ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec' samplePercentage: $ref: 'TS26512_CommonData.yaml#/components/schemas/Percentage' locationReporting: type: boolean accessReporting: type: boolean

## C.3.7	M1_MetricsReportingProvisioning API

openapi: 3.0.0 info: title: M1_MetricsReportingProvisioning version: 2.0.0 description: ¦ 5GMS AF M1 Metrics Reporting Provisioning API © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. tags: - name: M1_MetricsReportingProvisioning description: '5G Media Streaming: Provisioning (M1) APIs: Metrics Reporting Provisioning' externalDocs: description: 'TS 26.512 V17.1.0; 5G Media Streaming (5GMS); Protocols' url: 'https://www.3gpp.org/ftp/Specs/archive/26_series/26.512/' servers: - url: '{apiRoot}/3gpp-m1/v2' variables: apiRoot: default: https://example.com description: See 3GPP TS 29.512 clause 6.1. paths: /provisioning-sessions/{provisioningSessionId}/metrics-reporting-configurations: parameters: - name: provisioningSessionId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of an existing Provisioning Session.' post: operationId: activateMetricsReporting summary: 'Activate the Metrics reporting procedure for the specified Provisioning Session by providing the Metrics Reporting Configuration' requestBody: description: 'A JSON representation of a Metrics Reporting Configuration' required: true content: application/json: schema: $ref: '#/components/schemas/MetricsReportingConfiguration' responses: '201': description: 'Metrics Reporting Configuration Created' headers: Location: description: 'URL of the newly created Metrics Reporting Configuration (same as request URL).' required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/Url' /provisioning-sessions/{provisioningSessionId}/metrics-reporting-configurations/{metricsReportingConfigurationId}: parameters: - name: provisioningSessionId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of an existing Provisioning Session.' - name: metricsReportingConfigurationId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of a Metrics Reporting Configuration.' get: operationId: retrieveMetricsReportingConfiguration summary: 'Retrieve the specified Metrics Reporting Configuration of the specified Provisioning Session' responses: '200': description: 'Success' content: application/json: schema: $ref: '#/components/schemas/MetricsReportingConfiguration' put: operationId: updateMetricsReportingConfiguration summary: 'Update the specified Metrics Reporting Configuration for the specified Provisioning Session' requestBody: description: 'A JSON representation of a Metrics Reporting Configuration' required: true content: application/json: schema: $ref: '#/components/schemas/MetricsReportingConfiguration' responses: '204': description: 'Updated Metrics Reporting Configuration' '404': description: 'Not Found' patch: operationId: patchMetricsReportingConfiguration summary: 'Patch the specified Metrics Reporting Configuration for the specified Provisioning Session' requestBody: description: 'A JSON representation of a Metrics Reporting Configuration' required: true content: application/merge-patch+json: schema: $ref: '#/components/schemas/MetricsReportingConfiguration' application/json-patch+json: schema: $ref: '#/components/schemas/MetricsReportingConfiguration' responses: '200': description: 'Patched Metrics Reporting Configuration' content: application/json: schema: $ref: '#/components/schemas/MetricsReportingConfiguration' '404': description: 'Not Found' delete: operationId: destroyMetricsReportingConfiguration summary: 'Destroy the specified Metrics Reporting Configuration of the specified Provisioning Session' responses: '204': description: 'Destroyed Metrics Reporting Configuration' '404': description: 'Not Found' components: schemas: MetricsReportingConfiguration: type: object required: - metricsReportingConfigurationId - scheme properties: metricsReportingConfigurationId: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' scheme: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri' dataNetworkName: $ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn' reportingInterval: $ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec' samplePercentage: $ref: 'TS26512_CommonData.yaml#/components/schemas/Percentage' urlFilters: type: array items: type: string minItems: 1 metrics: type: array items: type: string minItems: 1

## C.3.8	M1_PolicyTemplatesProvisioning API

openapi: 3.0.0 info: title: M1_PolicyTemplatesProvisioning version: 2.0.0 description: ¦ 5GMS AF M1 Policy Templates Provisioning API © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. tags: - name: M1_PolicyTemplatesProvisioning description: '5G Media Streaming: Provisioning (M1) APIs: Policy Templates Provisioning' externalDocs: description: 'TS 26.512 V17.1.0; 5G Media Streaming (5GMS); Protocols' url: 'https://www.3gpp.org/ftp/Specs/archive/26_series/26.512/' servers: - url: '{apiRoot}/3gpp-m1/v2' variables: apiRoot: default: https://example.com description: See 3GPP TS 29.512 clause 6.1. paths: /provisioning-sessions/{provisioningSessionId}/policy-templates: parameters: - name: provisioningSessionId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of an existing Provisioning Session.' post: operationId: createPolicyTemplate summary: 'Create (and optionally upload) a new Policy Template' requestBody: description: 'A JSON representation of a Policy Template' required: true content: application/json: schema: $ref: '#/components/schemas/PolicyTemplate' responses: '201': description: 'Policy Template Created' headers: Location: description: 'URL of the newly created Policy Template resource.' required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/Url' /provisioning-sessions/{provisioningSessionId}/policy-templates/{policyTemplateId}: parameters: - name: provisioningSessionId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'A unique identifier of the Provisioning Session.' - name: policyTemplateId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'A resource identifier of a Policy Template.' get: operationId: retrievePolicyTemplate summary: 'Retrieve a representation of an existing Policy Template in the specified Provisioning Session' responses: '200': description: 'Success' content: application/json: schema: $ref: '#/components/schemas/PolicyTemplate' '404': description: 'Not Found' put: operationId: updatePolicyTemplate summary: 'Update a Policy Template for the specified Provisioning Session' requestBody: description: 'A JSON representation of a Policy Template' required: true content: application/json: schema: $ref: '#/components/schemas/PolicyTemplate' responses: '204': description: 'Updated Policy Template' '404': description: 'Not Found' patch: operationId: patchPolicyTemplate summary: 'Patch the Policy Template for the specified Provisioning Session' requestBody: description: 'A JSON representation of a Policy Template' required: true content: application/merge-patch+json: schema: $ref: '#/components/schemas/PolicyTemplate' application/json-patch+json: schema: $ref: '#/components/schemas/PolicyTemplate' responses: '200': description: 'Patched Content Hosting Configuration' content: application/json: schema: $ref: '#/components/schemas/PolicyTemplate' '404': description: 'Not Found' delete: operationId: destroyPolicyTemplate responses: '204': description: 'Destroyed Policy Template' '404': description: 'Not Found' components: schemas: PolicyTemplate: type: object required: - policyTemplateId - state - apiEndPoint - apiType - externalReference - ApplicationSessionContext properties: policyTemplateId: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' state: anyOf: - type: string enum: [PENDING, INVALID, READY, SUSPENDED] - type: string description: > This string provides forward-compatibility with future extensions to the enumeration but is not used to encode content defined in the present version of this API. apiEndPoint: type: string apiType: anyOf: - type: string enum: [N5, N33] - type: string description: > This string provides forward-compatibility with future extensions to the enumeration but is not used to encode content defined in the present version of this API. externalReference: type: string qoSSpecification: $ref: 'TS26512_CommonData.yaml#/components/schemas/M1QoSSpecification' ApplicationSessionContext: type: object properties: afAppId: $ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/AfAppId' sliceInfo: $ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai' dnn: $ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn' aspId: $ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/AspId' chargingSpecification: $ref: 'TS26512_CommonData.yaml#/components/schemas/ChargingSpecification'

## C.3.9	M1_EdgeResourcesProvisioning API

openapi: 3.0.0 info: title: M1_EdgeResourcesProvisioning version: 2.1.0 description: ¦ 5GMS AF M1 Edge Resources Provisioning API © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. tags: - name: M1_EdgeResourcesProvisioning description: '5G Media Streaming: Provisioning (M1) APIs: Edge Resources Provisioning' externalDocs: description: 'TS 26.512 V17.2.0; 5G Media Streaming (5GMS); Protocols' url: 'https://www.3gpp.org/ftp/Specs/archive/26_series/26.512/' servers: - url: '{apiRoot}/3gpp-m1/v2' variables: apiRoot: default: https://example.com description: See 3GPP TS 29.512 clause 7.10. paths: /provisioning-sessions/{provisioningSessionId}/edge-resources-configurations: parameters: - name: provisioningSessionId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of an existing Provisioning Session.' post: operationId: createEdgeResourcesConfiguration summary: 'Create an Edge Resources Configuration within the scope of the specified Provisioning Session' requestBody: description: 'A JSON representation of an Edge Resources Configuration' required: true content: application/json: schema: $ref: '#/components/schemas/EdgeResourcesConfiguration' responses: '201': description: 'Edge Resources Configuration Created' headers: Location: description: 'URL of the newly created Edge Resources Configuration.' required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/Url' /provisioning-sessions/{provisioningSessionId}/edge-resources-configurations/{edgeResourcesConfigurationId}: parameters: - name: provisioningSessionId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of an existing Provisioning Session.' - name: edgeResourcesConfigurationId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of an existing Edge Resources Configuration.' get: operationId: retrieveEdgeResourcesConfiguration summary: 'Retrieve the Edge Resources Configuration of the specified Provisioning Session' responses: '200': description: 'Success' content: application/json: schema: $ref: '#/components/schemas/EdgeResourcesConfiguration' put: operationId: updateEdgeResourcesConfiguration summary: 'Update an Edge Resources Configuration for the specified Provisioning Session' requestBody: description: 'A JSON representation of an Edge Resources Configuration' required: true content: application/json: schema: $ref: '#/components/schemas/EdgeResourcesConfiguration' responses: '204': description: 'Updated Edge Resources Configuration' '404': description: 'Not Found' patch: operationId: patchEdgeResourcesConfiguration summary: 'Patch the Edge Resources Configuration for the specified Provisioning Session' requestBody: description: 'A JSON representation of a Edge Resources Configuration' required: true content: application/merge-patch+json: schema: $ref: '#/components/schemas/EdgeResourcesConfiguration' application/json-patch+json: schema: $ref: '#/components/schemas/EdgeResourcesConfiguration' responses: '200': description: 'Patched Edge Resources Configuration' content: application/json: schema: $ref: '#/components/schemas/EdgeResourcesConfiguration' '404': description: 'Not Found' delete: operationId: destroyEdgeResourcesConfiguration responses: '204': description: 'Destroyed Edge Resources Configuration' '404': description: 'Not Found' components: schemas: EdgeResourcesConfiguration: type: object required: - edgeResourcesConfigurationId - edgeManagementMode - easRequirements properties: edgeResourcesConfigurationId: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' edgeManagementMode: $ref: '#/components/schemas/EdgeManagementMode' eligibilityCriteria: $ref: 'TS26512_CommonData.yaml#/components/schemas/EdgeProcessingEligibilityCriteria' easRequirements: $ref: '#/components/schemas/EASRequirements' easRelocationRequirements: $ref: '#/components/schemas/M1EASRelocationRequirements' M1EASRelocationRequirements: type: object required: - tolerance properties: tolerance: $ref: 'TS26512_CommonData.yaml#/components/schemas/EASRelocationTolerance' maxInterruptionDuration: $ref: 'TS29571_CommonData.yaml#/components/schemas/UintegerRm' maxResponseTimeDifference: $ref: 'TS29571_CommonData.yaml#/components/schemas/UintegerRm' EASRequirements: type: object required: - easProviderIds - easType - easFeatures - serviceAvailabilitySchedule - serviceContinuityScenarios properties: easProviderIds: type: array items: type: string easType: type: string easFeatures: type: array items: type: string serviceKpi: $ref: 'TS29558_Eees_EASRegistration.yaml#/components/schemas/EASServiceKPI' serviceArea: $ref: 'TS29558_Eecs_EESRegistration.yaml#/components/schemas/GeographicalServiceArea' serviceAvailabilitySchedule: type: array items: $ref: 'TS29122_CpProvisioning.yaml#/components/schemas/ScheduledCommunicationTime' serviceContinuityScenarios: type: array items: $ref: 'TS29558_Eecs_EESRegistration.yaml#/components/schemas/ACRScenario' serviceContinuitySupport: type: array items: $ref: 'TS29558_Eecs_EESRegistration.yaml#/components/schemas/ACRScenario' EdgeManagementMode: anyOf: - type: string enum: [EM_AF_DRIVEN, EM_APP_DRIVEN] - type: string description: > This string provides forward-compatibility with future extensions to the enumeration but is not used to encode content defined in the present version of this API.

## C.3.10	M1_EventDataProcessingProvisioning API

openapi: 3.0.0 info: title: M1_EventDataProcessingProvisioning version: 2.0.0 description: ¦ 5GMS AF M1 Event Data Processing Provisioning API © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. tags: - name: M1_EventDataProcessingProvisioning description: '5G Media Streaming: Provisioning (M1) APIs: Event Data Processing Provisioning' externalDocs: description: 'TS 26.512 V17.1.0; 5G Media Streaming (5GMS); Protocols' url: 'https://www.3gpp.org/ftp/Specs/archive/26_series/26.512/' servers: - url: '{apiRoot}/3gpp-m1/v2' variables: apiRoot: default: https://example.com description: See 3GPP TS 29.512 clause 6.1. paths: /provisioning-sessions/{provisioningSessionId}/event-data-processing-configurations: parameters: - name: provisioningSessionId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of an existing Provisioning Session.' post: operationId: createEventDataProcessingConfiguration summary: 'Supply an Event Data Processing Configuration for the specified Provisioning Session' requestBody: description: 'A JSON representation of a Event Data Processing Configuration' required: true content: application/json: schema: $ref: '#/components/schemas/EventDataProcessingConfiguration' responses: '201': description: 'Event Data Processing Configuration Created' headers: Location: description: 'URL of the newly created Event Data Processing Configuration (same as request URL).' required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/Url' /provisioning-sessions/{provisioningSessionId}/event-data-processing-configurations/{eventDataProcessingConfigurationId}: parameters: - name: provisioningSessionId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of an existing Provisioning Session.' - name: eventDataProcessingConfigurationId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of an Event Data Processing Configuration.' get: operationId: retrieveEventDataProcessingConfiguration summary: 'Retrieve the specified Event Data Processing Configuration of the specified Provisioning Session' responses: '200': description: 'Success' content: application/json: schema: $ref: '#/components/schemas/EventDataProcessingConfiguration' put: operationId: updateEventDataProcessingConfiguration summary: 'Update the specified Event Data Processing Configuration for the specified Provisioning Session' requestBody: description: 'A JSON representation of a Event Data Processing Configuration' required: true content: application/json: schema: $ref: '#/components/schemas/EventDataProcessingConfiguration' responses: '204': description: 'Updated Event Data Processing Configuration' '404': description: 'Not Found' patch: operationId: patchEventDataProcessingConfiguration summary: 'Patch the specified Event Data Processing Configuration for the specified Provisioning Session' requestBody: description: 'A JSON representation of a Event Data Processing Configuration' required: true content: application/merge-patch+json: schema: $ref: '#/components/schemas/EventDataProcessingConfiguration' application/json-patch+json: schema: $ref: '#/components/schemas/EventDataProcessingConfiguration' responses: '200': description: 'Patched Event Data Processing Configuration' content: application/json: schema: $ref: '#/components/schemas/EventDataProcessingConfiguration' '404': description: 'Not Found' delete: operationId: destroyEventDataProcessingConfiguration summary: 'Destroy the specified Event Data Processing Configuration of the specified Provisioning Session' responses: '204': description: 'Destroyed Event Data Processing Configuration' '404': description: 'Not Found' components: schemas: EventDataProcessingConfiguration: type: object required: - eventDataProcessingConfigurationId - eventId - dataAccessProfiles properties: eventDataProcessingConfigurationId: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' eventId: $ref: 'TS29517_Naf_EventExposure.yaml#/components/schemas/AfEvent' authorizationUrl: $ref: 'TS26512_CommonData.yaml#/components/schemas/Url' dataAccessProfiles: type: array items: $ref: 'TS26532_Ndcaf_DataReportingProvisioning.yaml#/components/schemas/DataAccessProfile'

NEXT CHANGE

# C.4	OpenAPI representation of the M5 APIs

## C.4.1	M5_ServiceAccessInformation API

openapi: 3.0.0 info: title: M5_ServiceAccessInformation version: 2.1.0 description: ¦ 5GMS AF M5 Service Access Information API © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. tags: - name: M5_ServiceAccessInformation description: '5G Media Streaming: Media Session Handling (M5) APIs: Service Access Information' externalDocs: description: 'TS 26.512 V17.2.0; 5G Media Streaming (5GMS); Protocols' url: 'https://www.3gpp.org/ftp/Specs/archive/26_series/26.512/' servers: - url: '{apiRoot}/3gpp-m5/v2' variables: apiRoot: default: https://example.com description: See 3GPP TS 29.512 clause 6.1. paths: /service-access-information/{provisioningSessionId}: parameters: - name: provisioningSessionId description: 'The resource identifier of an existing Provisioning Session.' in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' get: operationId: retrieveServiceAccessInformation summary: 'Retrieve the Service Access Information resource' responses: '200': description: 'Success' content: application/json: schema: $ref: '#/components/schemas/ServiceAccessInformationResource' '404': description: 'Not Found' components: schemas: ServerAddresses: type: array items: $ref: 'TS26512_CommonData.yaml#/components/schemas/Url' minItems: 1 ServiceAccessInformationResource: type: object required: - provisioningSessionId - provisioningSessionType properties: provisioningSessionId: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' provisioningSessionType: $ref: 'TS26512_CommonData.yaml#/components/schemas/ProvisioningSessionType' streamingAccess: type: object properties: mediaPlayerEntry: $ref: 'TS26512_CommonData.yaml#/components/schemas/Url' eMBMSServiceAnnouncementLocator: $ref: 'TS26512_CommonData.yaml#/components/schemas/Url' clientConsumptionReportingConfiguration: type: object required: - serverAddresses - locationReporting - samplePercentage properties: reportingInterval: $ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec' serverAddresses: $ref: '#/components/schemas/ServerAddresses' locationReporting: type: boolean accessReporting: type: boolean samplePercentage: $ref: 'TS26512_CommonData.yaml#/components/schemas/Percentage' dynamicPolicyInvocationConfiguration: type: object required: - serverAddresses - validPolicyTemplateIds - sdfMethods properties: serverAddresses: $ref: '#/components/schemas/ServerAddresses' validPolicyTemplateIds: type: array items: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' minItems: 0 sdfMethods: type: array items: $ref: 'TS26512_CommonData.yaml#/components/schemas/SdfMethod' minItems: 0 externalReferences: type: array items: type: string minItems: 1 clientMetricsReportingConfiguration: type: array items: type: object required: - serverAddresses - scheme - samplePercentage - urlFilters - metrics properties: serverAddresses: $ref: '#/components/schemas/ServerAddresses' scheme: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri' dataNetworkName: $ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn' reportingInterval: $ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec' samplePercentage: $ref: 'TS26512_CommonData.yaml#/components/schemas/Percentage' urlFilters: type: array items: type: string minItems: 0 metrics: type: array items: type: string networkAssistanceConfiguration: type: object required: - serverAddress properties: serverAddress: $ref: 'TS26512_CommonData.yaml#/components/schemas/Url' clientEdgeResourcesConfiguration: type: object required: - easDiscoveryTemplate properties: eligibilityCriteria: $ref: 'TS26512_CommonData.yaml#/components/schemas/EdgeProcessingEligibilityCriteria' easDiscoveryTemplate: $ref: '#/components/schemas/EASDiscoveryTemplate' easRelocationRequirements: $ref: '#/components/schemas/M5EASRelocationRequirements' m5EASRelocationRequirements: type: object required: - tolerance properties: tolerance: $ref: 'TS26512_CommonData.yaml#/components/schemas/EASRelocationTolerance' maxInterruptionDuration: $ref: 'TS29571_CommonData.yaml#/components/schemas/UintegerRm' eASDiscoveryTemplate: type: object required: - easType - easProviderIds - serviceFeatures properties: easType: type: string easProviderIds: type: array items: type: string serviceFeatures: type: array items: type: string

## C.4.2	M5_ConsumptionReporting API

openapi: 3.0.0 info: title: M5_ConsumptionReporting version: 2.0.0 description: ¦ 5GMS AF M5 Consumption Reporting API © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. tags: - name: M5_ConsumptionReporting description: '5G Media Streaming: Media Session Handling (M5) APIs: Consumption Reporting' externalDocs: description: 'TS 26.512 V17.1.0; 5G Media Streaming (5GMS); Protocols' url: 'https://www.3gpp.org/ftp/Specs/archive/26_series/26.512/' servers: - url: '{apiRoot}/3gpp-m5/v2' variables: apiRoot: default: https://example.com description: See 3GPP TS 29.512 clause 6.1. paths: /consumption-reporting/{aspId}: parameters: - name: aspId in: path required: true schema: $ref: 'TS29514_Npcf_PolicyAuthorization.yaml#/components/schemas/AspId' description: 'See 3GPP TS 26.512 clause 11.3.2.' post: operationId: submitConsumptionReport summary: 'Submit a Consumption Report' requestBody: description: 'A Consumption Report' required: true content: application/json: schema: $ref: '#/components/schemas/ConsumptionReport' responses: '204': description: 'Consumption Report Accepted' '400': description: 'Bad Request' '415': description: 'Unsupported Media Type' components: schemas: ConsumptionReport: type: object required: - mediaPlayerEntry - reportingClientId - consumptionReportingUnits properties: mediaPlayerEntry: type: string reportingClientId: type: string consumptionReportingUnits: type: array items: $ref: '#/components/schemas/ConsumptionReportingUnit' ConsumptionReportingUnit: type: object required: - mediaConsumed - startTime - duration properties: mediaConsumed: type: string mediaEndpointAddress: $ref: 'TS26512_CommonData.yaml#/components/schemas/EndpointAddress' startTime: $ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime' duration: $ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec' locations: type: array items: $ref: 'TS26512_CommonData.yaml#/components/schemas/TypedLocation' minItems: 1

## C.4.3	M5_MetricsReporting API

openapi: 3.0.0 info: title: M5_MetricsReporting version: 2.0.0 description: ¦ 5GMS AF M5 Metrics Reporting API © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. tags: - name: M5_ConsumptionReporting description: '5G Media Streaming: Media Session Handling (M5) APIs: Metrics Reporting' externalDocs: description: 'TS 26.512 V17.1.0; 5G Media Streaming (5GMS); Protocols' url: 'https://www.3gpp.org/ftp/Specs/archive/26_series/26.512/' servers: - url: '{apiRoot}/3gpp-m5/v2' variables: apiRoot: default: https://example.com description: See 3GPP TS 29.512 clause 6.1. paths: /metrics-reporting/{provisioningSessionId}/{metricsReportingConfigurationId}: parameters: - name: provisioningSessionId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of an existing Provisioning Session.' - name: metricsReportingConfigurationId in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' description: 'The resource identifier of a Metrics Configuration in the specified Provisioning Session.' post: operationId: submitMetricsReport summary: 'Submit a Metrics Report' requestBody: description: 'A Metrics Report' required: true content: application/3gpdash-qoe-report+xml: schema: type: string format: xml application/*: schema: type: string responses: '204': description: 'Metrics Report Accepted' '400': description: 'Bad Request' '415': description: 'Unsupported Media Type'

## C.4.4	M5_DynamicPolicies API

openapi: 3.0.0 info: title: M5_DynamicPolicies version: 2.0.0 description: ¦ 5GMS AF M5 Dynamic Policy API © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. tags: - name: M5_DynamicPolicies description: '5G Media Streaming: Media Session Handling (M5) APIs: Dynamic Policies' externalDocs: description: 'TS 26.512 V17.1.0; 5G Media Streaming (5GMS); Protocols' url: 'https://www.3gpp.org/ftp/Specs/archive/26_series/26.512/' servers: - url: '{apiRoot}/3gpp-m5/v2' variables: apiRoot: default: https://example.com description: See 3GPP TS 29.512 clause 6.1. paths: /dynamic-policies: post: operationId: createDynamicPolicy summary: 'Create (and optionally upload) a new Dynamic Policy resource' requestBody: description: 'An optional JSON representation of a Dynamic Policy resource' content: application/json: schema: $ref: '#/components/schemas/DynamicPolicy' responses: '201': description: 'Created Dynamic Policy Resource' content: application/json: schema: $ref: '#/components/schemas/DynamicPolicy' headers: Location: description: 'The URL of the newly created Dynamic Policy resource' required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/Url' '400': description: 'Bad Request' '401': description: 'Unauthorized' /dynamic-policies/{dynamicPolicyId}: parameters: - name: dynamicPolicyId description: 'The resource identifier of a Dynamic Policy resource' in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' get: operationId: retrieveDynamicPolicy summary: 'Retrieve an existing Dynamic Policy resource' responses: '200': description: 'Success' content: application/json: schema: $ref: '#/components/schemas/DynamicPolicy' '400': description: 'Bad Request' '401': description: 'Unauthorized' '404': description: 'Not Found' put: operationId: updateDynamicPolicy summary: 'Update an existing Dynamic Policy resource' requestBody: description: 'A replacement JSON representation of a Dynamic Policy resource' required: true content: application/json: schema: $ref: '#/components/schemas/DynamicPolicy' responses: '400': description: 'Bad Request' '401': description: 'Unauthorized' '404': description: 'Not found' patch: operationId: patchDynamicPolicy summary: 'Patch an existing Dynamic Policy resource' requestBody: description: 'A JSON patch to a Dynamic Policy resource' required: true content: application/merge-patch+json: schema: $ref: '#/components/schemas/DynamicPolicy' application/json-patch+json: schema: $ref: '#/components/schemas/DynamicPolicy' responses: '200': description: 'Patched Dynamic Policy' content: application/json: schema: $ref: '#/components/schemas/DynamicPolicy' '204': description: 'Patched Dynamic Policy' '400': description: 'Bad Request' '401': description: 'Unauthorized' '404': description: 'Not found' delete: operationId: destroyDynamicPolicy summary: 'Destroy an existing Dynamic Policy resource' responses: '204': description: 'Destroyed Dynamic Policy' '400': description: 'Bad Request' '401': description: 'Unauthorized' '404': description: 'Not Found' components: schemas: DynamicPolicy: type: object required: - dynamicPolicyId - policyTemplateId - serviceDataFlowDescription - provisioningSessionId properties: dynamicPolicyId: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' policyTemplateId: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' serviceDataFlowDescriptions: type: array items: $ref: 'TS26512_CommonData.yaml#/components/schemas/ServiceDataFlowDescription' provisioningSessionId: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' qosSpecification: $ref: 'TS26512_CommonData.yaml#/components/schemas/M5QoSSpecification' enforcementMethod: type: string enforcementBitRate: type: integer

## C.4.5	M5_NetworkAssistance API

openapi: 3.0.0 info: title: M5_NetworkAssistance version: 2.0.0 description: ¦ 5GMS AF M5 Network Assistance API © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. tags: - name: M5_NetworkAssistance description: '5G Media Streaming: Media Session Handling (M5) APIs: Network Assistance' externalDocs: description: 3GPP TS 26.512 V17.1.0; 5G Media Streaming (5GMS); Protocols. url: 'http://www.3gpp.org/ftp/Specs/archive/29_series/26.512/' servers: - url: '{apiRoot}/3gpp-m5/v2' variables: apiRoot: default: https://example.com description: apiRoot as defined in subclause 4.4.1 of 3GPP TS 29.501. paths: /network-assistance/: post: operationId: createNetworkAssistanceSession summary: 'Create a new Network Assistance Session.' responses: '201': description: 'Created Network Assistance Session' content: application/json: schema: $ref: '#/components/schemas/NetworkAssistanceSession' headers: Location: description: 'The URL of the nely created Network Assistance Session resource' required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/Url' '400': description: 'Bad Request' '401': description: 'Unauthorized' /network-assistance/{naSessionId}: parameters: - name: naSessionId description: 'The resource identifier of an existing Network Assistance Session resource' in: path required: true schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' get: operationId: retrieveNetworkAssistanceSession summary: 'Retrieve an existing Network Assistance Session resource' responses: '200': description: 'A JSON representation of a Network Assistance Session resource' content: application/json: schema: $ref: '#/components/schemas/NetworkAssistanceSession' '400': description: 'Bad Request' '401': description: 'Unauthorized' '404': description: 'Not Found' put: operationId: updateNetworkAssistanceSession summary: 'Update an existing Network Assistance Session resource' requestBody: description: 'A replacement JSON representation of a Network Assistance Session resource' required: true content: application/json: schema: $ref: '#/components/schemas/NetworkAssistanceSession' responses: '400': description: 'Bad Request' '401': description: 'Unauthorized' '404': description: 'Not found' patch: operationId: patchNetworkAssistanceSession summary: 'Patch an existing Network Assistance Session resource' requestBody: description: 'A JSON patch to a Network Assistance Session resource' required: true content: application/merge-patch+json: schema: $ref: '#/components/schemas/NetworkAssistanceSession' application/json-patch+json: schema: $ref: '#/components/schemas/NetworkAssistanceSession' responses: '200': description: 'Patched Network Assistance Session' content: application/json: schema: $ref: '#/components/schemas/NetworkAssistanceSession' '204': description: 'Patched Network Assistance Session' '400': description: 'Bad Request' '401': description: 'Unauthorized' '404': description: 'Not Found' delete: operationId: destroyNetworkAssistanceSession summary: 'Destroy an existing Network Assistance Session resource' responses: '204': description: 'Destroyed Network Assistance Session' '400': description: 'Bad Request' '401': description: 'Unauthorized' '404': description: 'Not Found' /network-assistance/{naSessionId}/recommendation: get: operationId: requestBitRateRecommendation summary: 'Obtain a bit rate recommendation for the next recommendation window' parameters: - name: naSessionId description: 'The resource identifier of an existing Network Assistance Session resource' in: path required: true schema: type: string responses: '200': description: 'Success' content: application/json: schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/M5QoSSpecification' '400': description: 'Bad Request' '401': description: 'Unauthorized' '404': description: 'Not Found' /network-assistance/{naSessionId}/boostRequest: post: operationId: requestDeliveryBoost summary: 'Request a delivery boost' parameters: - name: naSessionId description: 'The resource identifier of an existing Network Assistance Session resource' in: path required: true schema: type: string responses: '200': description: 'Delivery Boost Request Processed' content: application/json: schema: $ref: 'TS26512_CommonData.yaml#/components/schemas/OperationSuccessResponse' '400': description: 'Bad Request' '401': description: 'Unauthorized' '404': description: 'Not Found' components: schemas: NetworkAssistanceSession: type: object required: - naSessionId properties: naSessionId: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' serviceDataFlowDescription: type: array items: $ref: 'TS26512_CommonData.yaml#/components/schemas/ServiceDataFlowDescription' minItems: 1 policyTemplateId: $ref: 'TS26512_CommonData.yaml#/components/schemas/ResourceId' requestedQoS: $ref: 'TS26512_CommonData.yaml#/components/schemas/M5QoSSpecification' recommendedQoS: $ref: 'TS26512_CommonData.yaml#/components/schemas/M5QoSSpecification' notficationURL: $ref: 'TS26512_CommonData.yaml#/components/schemas/Url'

END OF CHANGES