3GPP TSG-S4 Ad hoc post Meeting #119-e	S4aI221356

# Online,  27th May 2022–9th August 2022

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 26.532 | CR | – | rev | – | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [EVEX] TS 26.532 PUT/PATCH corrections |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |
| Work item code: | EVEX |  |  |  |  |  | Date: |  | 2022-07-26 |
| Category: | F |  |  |  |  |  | Release: |  | Rel-17 |
| Reason for change: |  | Corrections are necessary to the API specifications of the Data Reporting Provisioning Session and Data Reporting Configuration resources. Regarding the former resource, it is clearly stated in both clauses 4.2.3.2.4 and 6.3.2.1 that the Update operation (by the Provisioning AF of the Application Service Provider) on the DataReportingProvisioningSession resource is not allowed. However, clause 6.2.3.3.2 is inconsistent with those stated rules by permitting the use of either the PUT or PATCH method for modifying this resource. Regarding the latter, clause 4.2.3.3.5 indicates that updating of the DataReportingConfiguration resource is permitted by use of either the PUT (complete replacement) or PATCH (partial replacement) method. However, to be techically correct, a new DataReportingConfigurationPatch resource its usage in the PATCH method would need to be defined in clause 6. Doing so would require additional changes in clause 6 and substantial updating of the OpenAPI specification in Annex B. A simpler solution, as proposed in this document, is to remove the use of the PATCH method and simply using PUT and its associated replacement semantics for any required changes to the resource. As alternative to the “200 OK” response (status) code, the “204 No Content” response code could also be returned by the Data Collection AF to the Provisioning AF as indication of successful update of the Data Reporting Configuration resource. A new, optional property dataAccessProfileId is added to the data model of the DataAccessProfile type in Table 6.3.2.3-1. It is possible for the Authorization AS, when performing OAuth authorization of the client (the event consumer entity) to embed the Data Access Profile along with its identifier as part of the associated claims in the issued access token. The dataAccessProfileId might be necessary in support of the integrity-protection property of the secure channel between the Data Collection AF and the Authorization AS. |  |  |  |  |  |  |  |
| Summary of change: |  | Corrections to the following clauses of TS 26.532: - Clause 4.2.3.3.2: Correction the reference to the definition of the DataAccessProfile data type associated with the dataAccessProfiles property of the Data Reporting Configuration resource, along with a simple editorial change in that clause. - Clause 4.2.3.3.5: Modify the description to restrict the means for updating the Data Reporting Configuration resource to use of the PUT method. - Clause 6.2.1: Delete the operation name UpdateSession and associated HTTP method and description from Table 6.2.1-1. - Clause 6.2.3.3.2: Change the 1st paragraph of this clause to indicate that the update operation is not permitted on the Data Reporting Provisioning Session resource, and delete all subsequent contents of this clause. - Clause 6.2.5.3.2: Change the title of this clause and its contents to solely refer to/describe the use of the PUT method for the update operation on the Data Reporting Configuration resource. This includes change to the description of the ‘200 OK’ response code and addition of the ‘204 No Content’ response codes - Clause 6.3: 1) Add the property dataAccessProfileId currently missing in Table 6.3.2.3 and 2) make a simple typo correction on the title of clause 6.3.3. |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent speification. |  |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.3.3.5, 6.2.1, 6.2.3.3.2, 6.2.5.3.2, 6.3.2.3, 6.3.3 |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

##### 4.2.3.3.2	Data Reporting Configuration entity

A given instance of a Data Reporting Configuration resource is identified by the dataReportingConfigurationId property of the DataReportingConfiguration resource, and applies to one type of data collection client. The properties of this resource, as defined in the following clauses, pertain to UE data collection and reporting by different data collection clients to the Data Collection AF, and control of access by different consumer entities to event data exposed by the Data Collection AF.

The type of a Data Reporting Configuration resource is identified by the dataCollectionClientType property of the DataReportingConfiguration resource as specified in clause 6.3.2.2.

NOTE 1:	The dataCollectionClientType property corresponds to the Data collection client type parameter in table 4.6.2-1 of TS 26.531 [7].

The Data Reporting Configuration resource shall include one or more sets of instructions for data collection clients on the collection and reporting of UE data to the Data Collection AF, and may contain one or more sets of data exposure restrictions, expressed as Data Access Profiles (see clause 6.3.2.3), each one determining the level of access by event consumer entities to the event-related UE data reported to the Data Collection AF. A Data Access Profile defines the granularity of access to a particular subset of collected event data parameters for the Event ID of concern. This granularity is expressed as a set of data aggregation functions along the time, user, and location dimensions. The subset of parameters to be exposed is identified using a controlled vocabulary specific to the Event ID. An authorization procedure is in place to determine which Data Access Profile is granted to a particular event consumer entity.

NOTE 2:	The process of matching event consumers to Data Access Profiles is implementation-specific and therefore beyond the scope of the present document.

NEXT CHANGE

##### 4.2.3.3.5	Update Data Reporting Configuration

The update operation is invoked by the Provisioning AF to initially upload the representation of a Data Reporting Configuration resource created using the procedure in clause 4.2.3.3.3 or, in the case of an existing Data Reporting Configuration resource, to entirely replace all mutable properties of that resource. The HTTP PUT method shall be used for the update operation.

If the procedure is successful, the Data Collection AF shall respond with a 200 (OK) reflecting the successful update operation.

## If the procedure is unsuccessful, the Data Collection AF shall provide a response code as defined in clause 5.3.3.

NEXT CHANGE

## 6.2	Resources

### 6.2.1	Resource structure

Figure 6.2.1-1 depicts the URL path model for the Ndcaf_DataReportingProvisioning service.

Figure 6.2.1-1: URL path model of Ndcaf_DataReportingProvisioning service API

Table 6.2.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.2.1-1: Resources and methods overview

| Service name | Operation name | Resource name | Resource path suffix | HTTP method | Description |
| --- | --- | --- | --- | --- | --- |
| Ndcaf_DataReporting‌Provisioning | CreateSession | Data Reporting Provisioning Sessions collection | /sessions | POST | Provisioning AF establishes a Data Reporting Provisioning Session resource at the Data Collection AF. |
|  | RetrieveSession | Data Reporting Provisioning Session | /sessions/{sessionId} | GET | Retrieves an existing Data Reporting Provisioning Session resource from the Data Collection AF. |
|  | DestroySession |  |  | DELETE | Destroys a Data Reporting Provisioning Session resource. |
|  | CreateConfiguration | Data Reporting Configurations collection | /sessions/‌{sessionId}/‌configurations | POST | Creates a Data Reporting Configuration resource at the Data Collection AF. |
|  | RetrieveConfiguration | Data Reporting Configuration | /sessions/‌{sessionId}/‌configurations/‌{configurationId} | GET | Retrieves an existing Data Reporting Configuration resource from the Data Collection AF. |
|  | UpdateConfiguration |  |  | PUT | Replaces an existing Data Reporting Configuration resource at the Data Collection AF. |
|  | DestroyConfiguration |  |  | DELETE | Destroys a Data Reporting Configuration resource at the Data Collection AF. |

NEXT CHANGE

##### 6.2.3.3.2	Ndcaf_DataReportingProvisioning_UpdateSession operation using PUT or PATCH method

The update operation is not permitted on the Data Reporting Provisioning Session resource.

NEXT CHANGE

##### 6.2.5.3.2	Ndcaf_DataReportingProvisioning_UpdateConfiguration operation using PUT method

This method shall support the URL query parameters specified in table 6.2.5.3.2-1.

Table 6.2.5.3.2-1: URL query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |

This method shall support the request data structures and headers specified in tables 6.2.5.3.2-2 and 6.2.5.3.2-3, respectively, and the response data structures and response codes specified in table 6.2.5.3.2-4.

Table 6.2.5.3.2-2: Data structures supported by the PUT request body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| DataReportingConfiguration | M | 1 | Parameters to replace an existing Data Reporting Configuration resource. |

Table 6.2.5.3.2-3: Headers supported for the PUT request on this resource

| HTTP request header | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Authorization | string | M | 1 | For authentication of the Provisioning AF (see NOTE). |
| Origin | string | O | 0..1 | Indicates the origin of the requester. |
| NOTE : If OAuth 2.0 authorization is used the value is Bearer followed by a string representing the access token, see section 2.1 RFC 6750 [8] |  |  |  |  |

Table 6.2.5.3.2-4: Data structures supported by the PUT response body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| DataReportingConfiguration | M | 1 | 200 OK | Confirmation of successful replacement of a Data Reporting Configuration resource via the “200 OK” status code, along with a representation of the status of the PUT request, carried in the response body, is returned by the Data Collection AF. |
| n/a |  |  | 204 No Content | Confirmation of successful replacement of a Data Reporting Configuration resource via the “204 No Content” status code is returned by the Data Collection AF. |
| ProblemDetails | O | 0..1 | 307 Temporary Redirect | Temporary redirection, during a Data Reporting Configuration replacement procedure. The response shall include a Location header field containing an alternative URL of the resource located in another Data Collection AF (service) instance. Applicable if the feature "ES3XX" (Extended Support of HTTP 307/308 redirection as defined in TS 29.502 [11]) is supported. |
| ProblemDetails | O | 0..1 | 308 Permanent Redirect | Permanent redirection, during a Data Reporting Configuration replacement procedure. The response shall include a Location header field containing an alternative URL of the resource located in another Data Collection AF (service) instance. Applicable if the feature "ES3XX" is supported. |
| ProblemDetails | O | 0..1 | 404 Not Found | This Data Reporting Configuration resource does not exist (see NOTE 2). |
| NOTE 1: The mandatory HTTP error status codes for the PUT method listed in table 5.2.7.1-1 of TS 29.500 [9] also apply. NOTE 2: Failure cases are described in clause 6.4. |  |  |  |  |

Table 6.2.5.3.2-5: Headers supported by the 200 response code on this resource

| HTTP response header | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Access-Control-Allow-Origin | string | O | 0..1 | Part of CORS [10]. Supplied if the request included the Origin header. |
| Access-Control-Allow-Methods | string | O | 0..1 | Part of CORS [10]. Supplied if the request included the Origin header. Valid values: POST, PUT, DELETE. |
| Access-Control-Expose-Headers | string | O | 0..1 | Part of CORS [10]. Supplied if the request included the Origin header. Valid values: Location. |

Table 6.2.5.3.2-6: Headers supported by the 204 response code on this resource

| HTTP response header | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Access-Control-Allow-Origin | string | O | 0..1 | Part of CORS [10]. Supplied if the request included the Origin header. |
| Access-Control-Allow-Methods | string | O | 0..1 | Part of CORS [10]. Supplied if the request included the Origin header. Valid values: POST, PUT, DELETE. |
| Access-Control-Expose-Headers | string | O | 0..1 | Part of CORS [10]. Supplied if the request included the Origin header. Valid values: Location. |

Table 6.2.5.3.2-7: Headers supported by the 307 and 308 response codes on this resource

| HTTP response header | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URL of the resource located in another Data Collection AF (service) instance. |
| 3gpp-Sbi-Target-Nf-Id | string | O | 0..1 | Identifier of the target NF (service) instance towards which the request is redirected |
| Access-Control-Allow-Origin | string | O | 0..1 | Part of CORS [10]. Supplied if the request included the Origin header. |
| Access-Control-Allow-Methods | string | O | 0..1 | Part of CORS [10]. Supplied if the request included the Origin header. Valid values: POST, PUT, PATCH, DELETE |
| Access-Control-Expose-Headers | string | O | 0..1 | Part of CORS [10]. Supplied if the request included the Origin header. Valid values: Location |

NEXT CHANGE

## 6.3	Data model

#### 6.3.2.3	DataAccessProfile type

Table 6.3.2.3-1 Definition of DataAccessProfile type

| Property name | Data type | Cardinality | Usage | Description |
| --- | --- | --- | --- | --- |
| dataAccessProfileId | string | 0..1 | C: RW U: RW | A unique identifier for this Data Access Profile. |
| targetEventConsumer‌Types | Array(Event‌Consumer‌Type) | 1..1 | C:RW U:RW | The set of Event consumer types (see clause 6.3.3.1) to which this Data Access Profile is targeted. If the set is empty, this Data Access Profile applies to all types of Event consumer. |
| parameters | Array(String) | 1..1 | C:RW U:RW | The set of UE data parameters to be collected by the data collection client, and for which the restrictions specified by this Data Access Profile apply. The parameters are uniquely identified by a controlled vocabulary specific to the Event ID indicated by the parent Data Reporting Provisioning Session. If the set is empty, the restrictions apply to all parameters for the Event ID of the parent Data Reporting Provisioning Session. |
| timeAccessRestrictions | Object | 0..1 | C:RW U:RW | Configuration for access restrictions along the time dimension. |
| duration | DurationSec | 1..1 | C:RW U:RW | The period of time over which access is to be aggregated. |
| aggregationFunctions | Array(Data‌Aggregation‌Function‌Type) | 1..1 | C:RW U:RW | An ordered, non-empty list of aggregation functions (see clause 6.3.3.2) applied to the event data prior to exposure to event consumers. |
| userAccessRestrictions | Object | 0..1 | C:RW U:RW | Configuration for access restrictions along the user dimension. |
| groupIds | Array(GroupId) | 1..1 | C:RW U:RW | Identifiers of the UE groups over which access is to be aggregated. |
| userIds | Array(Gpsi) or Array(Supi) | 1..1 | C:RW U:RW | Identifiers of the UEs comprising a group over which access is to be aggregated. |
| aggregationFunctions | Array(Data‌Aggregation‌Function‌Type) | 1..1 | C:RW U:RW | An ordered, non-empty list of aggregation functions (see clause 6.3.3.2) applied to the event data prior to exposure to event consumers. |
| locationAccessRestrictions | Object | 0..1 | C:RW U:RW | Configuration for access restrictions along the location dimension |
| locationAreas | Array(Location‌Area‌5G) | 1..1 | C:RW U:RW | Identifiers of geographical areas over which access is to be aggregated. Event data is grouped by the location of the UE during the data collection. |
| aggregationFunctions | Array(Data‌Aggregation‌Function‌Type) | 1..1 | C:RW U:RW | An ordered, non-empty list of aggregation functions (see clause 6.3.3.2) applied to the event data prior to exposure to event consumers. |
| NOTE: Data types DurationSec, GroupId, Gpsi, Supi and LocationArea5G are defined in TS 29.571 [13]. |  |  |  |  |

### 6.3.3	Simple data types and enumerations

(SNIPPED)

END OF CHANGES