3GPP TSG-S4 Ad hoc post Meeting #119-e	S4aI221359

# Online,  , 27th May 2022–9th August 2022

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 26.531 | CR | – | rev | – | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [EVEX] Corrections to event consumer authorisation procedure |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | BBC |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |
| Work item code: | EVEX |  |  |  |  |  | Date: |  | 2022-07-29 |
| Category: | F |  |  |  |  |  | Release: |  | Rel-17 |
| Reason for change: |  | Discussion of CT3 LS revealed that the procedure is incorrect. |  |  |  |  |  |  |  |
| Summary of change: |  | - Provide cross-references to relevant CT3 specifications for northbound/southbound NEF exposure. - Remove the Data Access Profile parameter from step 2 and subsequent steps because the Event Subscriber does not know this information. |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment with SA2 procedures in TS 23.288. |  |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4.7.1, 5.8 |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

(SNIPPED)

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[3]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[4]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[5]	3GPP TS 29.517: "5G System; Application Function Event Exposure Service; Stage 3".

[6]	3GPP TS 29.510: "Network function repository services; Stage 3".

[7]	3GPP TS 29.532: "Data Collection and Reporting; Protocols and Formats".

[8]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs".

[9]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[10]	3GPP TS 29.522: "5G System; Network Exposure Function Northbound APIs; Stage 3".

[11]	3GPP TS 29.591: "5G System; Network Exposure Function Southbound Services; Stage 3".

NEXT CHANGE

### 4.7.1	Service exposure via Network Exposure Function (NEF)

The following services provided by a Data Collection AF deployed inside the trusted domain shall be exposed northbound by the NEF to an Application Service Provider outside the trusted domain, as depicted in clauses A.3 and A.4:

-	The Ndcaf_DataReportingProvisioning service shall be exposed to Provisioning AF instances deployed outside the trusted domain as Nnef_DataReportingProvisioning. See TS 29.522 [10].

-	The Ndcaf_DataReporting service shall be exposed to Indirect Data Collection Clients and Application Servers deployed outside the trusted domain as Nnef_DataReporting. See TS 29.522 [10].

-	The Naf_EventExposure service shall be exposed to Event Consumer AF instances deployed outside the trusted domain as Nnef_EventExposure. See TS 29.517 [5] and TS 29.522 [10].

The following services provided by an externally deployed Data Collection AF shall be exposed southbound by the NEF to Network Functions deployed inside the trusted domain, as depicted in clauses A.5:

-	The Naf_EventExposure service shall be exposed to Event Consumer AF instances deployed inside the trusted domain as Nnef_EventExposure. See TS 29.517 [5] and TS 29.591 [11].

NEXT CHANGE

## 5.8	Procedures for event consumer authorization

The procedure for authorising access to the events exposed by the Data Collection AF is depicted by the following call flow:

Figure 5.8-1: High-level procedures for event consumer authorization

The steps are:

1.	The Provisioning AF provisions the data collection and the report exposure functionality at reference point R1, per the procedures in clause 5.2, including a set of Data Access Profiles.

1a.	The Data Collection AF may provision the Authorization AS with a Data Access Profile configuration corresponding to step 1, including the Data Access Profile ID. The procedures used in this step are outside the scope of standardisation.

2.	An event consumer sends a subscription request to the Data Collection AF to receive events via reference point R5 or R6, per the procedures in clause 5.3, indicating the Event ID of interest.

3.	In return, the Data Collection AF redirects the event consumer to the Authorization AS in order to obtain access.

4.	The event consumer contacts the Authorization AS (according to the procedures for authorization of NF service access defined in clause 13.4 of TS 33.501 [9]) with a set of valid credentials.

5.	If access is granted, the Authorization AS responds with an access token that is valid for for a specific period of time. The access token may encode a Data Access Profile ID if the authorisation applies narrowly. The response may redirect the event consumer to the Data Collection AF using the initial subscription request URL, enhanced with the access token and optionally with the Data Access Profile ID passed in the clear.

6.	The event consumer resends the subscription request to the Data Collection AF, this time with the access token and, optionally, with the Data Access Profile ID.

7.	The Data Collection AF may verify the access token with the Authorization AS, or it may verify it locally.

8.	If verification is successful, the Data Collection AF approves the subscription request for the requested Access Profile

9.	The Data Collection AF sends event notifications to the event consumer, per the procedures in clause 5.6.

10.	The event consumer cancels its event subscription using the procedures in clause 5.7.

END OF CHANGES