3GPP SA4 MBS SWG	S4aI221391

Online, 06 October 2022

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.806 | CR | pseudo | rev | - | Current version: | 0.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_SmarTAR] Latency Aspects for SmarTAR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_SmarTAR |  |  |  |  |  | Date: |  |  | 06/10/2022 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This pCR assumes that S4-220966 is agreed |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

=====  CHANGE  =====

## 7.1	Key Issue #X: End-to-End QoS for the 5G relay architecture

### 7.1.1	Description

A key challenge for the tethered AR glasses with 5G relay architecture (Figure 4.4.4-1 in clause 4.4.4) is to properly estimate the required QoS allocations for the AR/MR sessions. The QoS allocation must take into account the wireless tethering link from the AR glasses to the 5G device (e.g. phone). This applies to all QoS parameters, namely bitrate, packet loss, delay, and jitter. Figure 7.1.1-1 depicts a breakdown of the end-to-end delay as an example.

Figure 7.1.1-1: End-to-end delay breakdown

To meet the end-to-end latency requirement for the AR/MR session, the phone must estimate the impact of the tethering link on the overall QoS requirements. This corresponds to the Dn,1component in the figure. Also, the delay on the Internet (between the UPF and the edge application server) Dn,2 needs to be considered. The phone (e.g., through Media Access Function) and/or the UPF can perform such estimation by:

-	Running some measurement tests for latency, packet loss, and bitrate

-	Exchanging information with the radio and/or AF on the QoS policy

The 5GC may regularly adjust its QoS allocation for the 5G network based on the observation of the status of the tethering link and the Internet, thus targeting a consistent end-to-end QoS experience.

### 7.1.2	End-to-End Latency

#### 7.1.2.1	General

In an end-to-end connection that includes a tethering link (e.g., Wi-Fi link), a 5G network and the Internet, the Wi-Fi segment and the Internet segment typically cannot guarantee latency. To achieve low end-to-end latency, one approach is to make the latency in the 5G network very conservative such that the end-to-end latency is below a target value. This, however, comes at a cost, because provisioning an uncessarily low latency in the 5G network means excessive resource allocation (e.g., to support a more robust modulation-and-coding scheme (MCS)) or pre-empting many other traffic flows.

An alternative approach is to dynamically adjust the delay in the 5G network in accordance with the total delay incurred elsewhere on the end-to-end path. The delay on a Wi-Fi link may change over time depending on the interference generated by other nearby Wi-Fi networks operating on the same frequency. Similarly, the delay between the UPF and the application server depends on the location of this selected UPF and the network congestion level. Therefore, measurements may be used to estimate these time-varying delays on the non-5G segments.

There are two ways to measure the latency and they fill in the details for step 10 in Figure 5.2-5 in clause 5.2.

#### 7.1.2.2	Segment-by-segment delay measurement

The delay on Wi-Fi link and the delay between the UPF and the application server are measured separately. One simple solution is to use the ICMP ping protocol (ICMP Echo and Echo Reply, IETF RFC792). The 5G phone sends a ping request to the AR glasses, which replies with a ping response. The 5G phone then obtains the RTT over the Wi-Fi link. Similarly, the UPF sends a ping request to the application server, which replies with a ping response, and the UPF obtains the RTT between the UPF and the application server. The respective RTTs can then be halved to get estimates of the one-way delays for the two non-5G segments.

In step 4, the MAF reports the one-way delay estimate Dn,1 to the AF.

In step 7, the UPF reports the one-way delay estimate Dn,2 to the SMF, which forwards the estimate to the AF.

In step 9, the AF determines the desired value for the delay in the 5G network needed to compensate for the variation in the delay in the non-5G segments in order to meet the end-to-end latency requirement for the application, and sends a delay request to the PCF.

Figure 7.1.2.2-1: segment-by-segment delay measurement

#### 7.1.2.3	End-to-end delay measurement

The delay measurement is carried out in an end-to-end fashion. This avoids the potential rejection of a measurement message that originates from the UPF and reaches the application server. The AR glasses sends a ping request message to the application server, which replies with a ping response. The AR glasses then estimate the UL one-way end-to-end delay De2e by halving the RTT. The 5G network estimates the UL one-way delay within the 5G network Dc, e.g., by recording the time when the ping request arrives at the phone and the time when the ping request reaches the UPF and takes the difference. Alternatively, the 5G network can assign a new type of 5QI corresponding to a constant delay as the target delay value, e.g., 8ms. This way, the delay experienced by the measurement packet in the 5G network can be considered as a deterministic known value.

The estimated UL one-way delay on the non-5G segments is then Dn=De2e-Dc.

Figure 7.1.2.3-1: End-to-end delay measurement

#### 7.1.2.4	Time measurement protocol

The ICMP ping protocol uses two timestamps generated at the transmitter to get an estimate of the RTT. The measured delay includes the time gap between the reception of the ping request message and the transmission of the ping response message at the receiver. The time gap contributes to the estimation error, and it depends on the operating system used at the receiver, and may become significant for low latency applications.

Alternatively, ICMP timestamp approach (IETF RFC792) can be used, which, compared toICMP ping, provides the source two timestamps, one for the reception of the timestamp message and the other for the transmission of the timestamp reply message. The two timestamps are carried back to the source, which can use the difference to calculate the time gap and get a more accurate estimate of the RTT.