Source:	Apple, Huawei, Xiaomi, Google

Title:	5GMS Audio Codec Review in 26.511

Agenda Item:	2.7

Document for:	Discussion and agreement

# Introduction

At the last SA4 meeting, a new work item covering Audio Codec in 5GMS was approved [1], with the following objectives:

The purpose of this Work Item is to upgrade relevant 5GMS streaming, TV and VR audio profiles in specifications of 5G-Advanced in Rel-18.

- to specify a new Audio Operation Point in TS 26.117 with support for the xHE-AAC codec to improve efficiency and performance for Audio content of 5G Media streaming services over 5G-Advanced networks and devices.

- to review and, when appropriate, possibly modify requirements (i.e. may, should or shall support) in TS 26.511 applicable to 5GMS Clients for the support of Audio operation Points.

This contribution addresses the second objective.

# Considerations

# General Comments

The mobile industry is very dynamic, largely driven by user demand for media consumption. It is important for the industry to use codec(s) that satisfy the end-user needs and provide best user experience.

The concept of “may”, “should” and “shall” was introduced many years ago to provide some guidance on the relevancy of the codecs. However, the sources believe that this classification is not bringing any real benefit, and even has some disadvantages.

A “may” status does not provide any meaningful guidance for the industry. Instead, considerations about keeping this codec technology in the specification should be seriously considered, eventually leading to removal of this codec requirement.

When a codec is mandated with “shall” it means it will have to be supported even if it is not used. This comes with a cost in maintenance and testing.

A “should” specification gives the best flexibility to the industry to adopt and support newer relevant codec technologies while allowing removing less or non-relevant codec technologies.

Although interoperability could be a concern when several codecs are specified, in practice it is usually not an issue. Typically, there will always be an overlap in time between existing and new coders, to allow the industry to adopt the more beneficial technologies. It is better to let the industry transition between technologies based on market needs rather than trying to force acceptance via specification.

Review and proposal status for the various codecs are listed below

# xHE-AAC®

As mentioned in [1], the xHE-AAC® codec has been widely adopted by the industry and is now ubiquitous in mobile platforms and operating systems and should be added for Media Streaming and TV profiles with “should” status.

# eAAC+

This codec is currently specified with a “should” status for media streaming and TV profiles, and should be kept as is. This coder has been standardized several years ago and its usage will decrease, replaced by xHE-AAC.

# MPEG-H

This codec is currently specified with a “should” in Virtual Reality profiles and should be kept as is.

# AMR_WB+

This codec is currently specified with a “may” status for Media Streaming profile. This codec has been standardized nearly 20 years ago, and the sources believe it to be superseded by the newer coders and recommend it to be removed from Release18 specifications.

# Conclusion and Proposal

Based on the considerations presented in Clause 2, the sources proposed to add the xHE_AAC® codec with a “should” status and keep the status of the other codecs as is in 26.511. Annex A contains the corresponding changes to 26.511 that could be converted into a pCR for presentation and agreement at SA4#122.

# References

- SP-221033 “5G Media Streaming Audio codec phase 2 for 5G-Advanced”, Dolby Laboratories Inc., Fraunhofer IIS, VoiceAge Corporation, Samsung Electronics Co., Ltd, Philips International B.V., ETRI, Qualcomm incorporated, Panasonic Holdings Corporation, Nokia Corporation, Orange, Tencent

# Annex A

====== 1st CHANGE ======

### 5.2.3	Audio decoding

If the 5GMSd Client supports the reception of audio:

- 	xHE-AAC stereo decoding capability should be supported as defined in 3GPP TS 26.117 [4] clause 5.3.

NOTE:	xHE-AAC® is a registered trademark of Fraunhofer in Germany and other countries and is used with Fraunhofer’s permission.

-	eAAC+ decoding capability should be supported as defined in 3GPP TS 26.117 [4] clause 5.3.

-	AMR-WB+ decoding capability may be supported as defined in 3GPP TS 26.117 [4] clause 5.3.

====== 2nd CHANGE ======

#### 5.2.7.4	Audio media profiles

If the 5GMSd Client supports the reception of audio, then the following should be supported:

-	the xHE-AAC stereo playback requirements as defined in 3GPP TS 26.117 [4] clause 7.7.2.4.

-	the eAAC+ stereo playback requirements as defined in 3GPP TS 26.117 [4] clause 7.5.2.4.

If the 5GMSd Client supports the reception of audio, then the following may be supported:

-	the AMR-WB+ playback requirements as defined in 3GPP TS 26.117 [4] clause 7.6.2.4.

====== 3rd CHANGE ======

#### 5.2.8.3	Audio media profiles

For AMR:

-	the <profiles> parameter is defined in TS 26.117, clause 7.2.2.4 as audio/mp4 profiles='camr'

-	the <codecss> parameter is defined in TS 26.117, clause 7.2.2.4 as 'samr'

For AMR-WB:

-	the <profiles> parameter is defined in TS 26.117, clause 7.3.2.4 as audio/mp4 profiles='camw'

-	the <codecs> parameter is defined in TS 26.117, clause 7.3.2.4 as 'samw'

For EVS:

-	the <profiles> parameter is defined in TS 26.117, clause 7.4.2.4 as audio/mp4 profiles='cevs'

-	the <codecs> parameter is defined in TS 26.117, clause 7.4.2.4 as 'sevs'

For xHE-AAC stereo:

-	the <profiles> parameter is defined in TS 26.117, clause 7.7.2.4 as audio/mp4 profiles=''

-	the <codecs> parameter is defined in TS 26.117, clause 7.7.2.4 as 'mp4a'

For eAAC+ stereo:

-	the <profiles> parameter is defined in TS 26.117, clause 7.6.2.4 as audio/mp4 profiles='ceac'

-	the <codecs> parameter is defined in TS 26.117, clause 7.6.2.4 as 'mp4a'

For AMR-WB+:

-	the <profiles> parameter is defined in TS:26.117, clause 7.7.2.4 as audio/mp4 profiles='camp'

-	the <codecs> parameter is defined in TS:26.117, clause 7.7.2.4 as 'sawp'

====== 4th CHANGE ======

### 5.3.3	Audio encoding

If the 5GMS UE supports the transmission of audio:

- 	the sender requirements for the xHE-AAC stereo Operation Point as defined in 3GPP TS 26.117 [4] clause 6.3.2.3 should be supported.

-	the sender requirements for the eAAC+ stereo Operation Point as defined in 3GPP TS 26.117 [4] clause 6.3.2.3 should be supported.

====== 5th CHANGE ======

#### 5.3.5.4	Audio media profile

If the 5GMSu Client supports the transmission of audio, then the following should be supported:

-	the xHE-AAC stereo content generation requirements as defined in 3GPP TS 26.117 [4] clause 7.6.2.5.

-	the eAAC+ stereo content generation requirements as defined in 3GPP TS 26.117 [4] clause 7.6.2.5.

====== 6th CHANGE =====

### 5.4.2	Audio decoding

If the 5GMSd Client supports the Television (TV) profile, it should support xHE-AAC stereo Operation point as defined in 3GPP TS 26. 117 [4], clause 6.3.3.

If the 5GMSd Client supports the Television (TV) profile, it /should support eAAC+ stereo Operation point as defined in 3GPP TS 26. 117 [4], clause 6.3.3.

====== 7th CHANGE ======

#### 5.4.4.3	Audio media profiles

If the 5GMSd Client supports the Television (TV) profile, then the following should be supported:

-	the xHE-AAC stereo playback requirements as defined in 3GPP TS 26.117 [4] clause 7.5.2.4.

-	the eAAC+ stereo playback requirements as defined in 3GPP TS 26.117 [4] clause 7.5.2.4.