3GPP SA4 MBS SWG	S4aI230044

Telco 9th February 2023

Source:	Apple

Title:	Pseudo-CR Correction for XR framework reference and privacy for TS 26.565

Spec:	3GPP TS 26.565

Agenda item:	2.3

Document for:	Discussion

1. Introduction

This CR focuses on addressing two issues:

- The split rendering architecture exchanges sensitive user information outside of the UE (e.g. between Split rendering EAS). It is pivotal that privacy and security of this sensitive data is analysed and ensured. A place holder note is proposed for now.

- It has been agreed via S4-221624 that XR related specifications will not be restricted to a specific runtime framework like OpenXR. Specific text in draft v0.2.0 for TS 26.565 in S4-221507 creates ambiguity in this regard and is proposed to be updated to avoid this.

2. Proposal

It is proposed to integrate the marked up changes in updated draft TS 26.565.

* * * First Change * * * *

# 5	Reference Architecture and Procedures

Editor’s Note: The terminology and reference-point naming convention need to be aligned in this document.

Editor’s Note: Privacy and security aspects of sensitive user data exchanged between UE and UE-external functions need to be considered.

* * * Next Change * * * *

### 5.2.2 	Call flow for Split Rendering session setup

The split rendering operation can be described by the call flow as depicted in the Figure 5.2.2.X:

Figure 5.2.2.X: High-level call flow for split rendering session setup.

The steps are:

- The Presentation Engine discovers the split rendering EAS and sets up a connection to it. It provides information about its rendering capabilities and runtime configuration, e.g. that the OpenXR configuration may be used for this purpose.

- In response, the split rendering EAS creates a description of the split rendering output and the input it expects to receive from the UE.

- The Presentation Engine requests the buffer streams from the MAF, which in turn establishes a connection to the split rendering EAS to stream pose and retrieve split rendering buffers.

- The Source Manager retrieves pose and user input from the XR runtime.

- The Source Manager shares the pose predictions and user input actions with the split rendering EAS.

- The split rendering EAS uses that information to render the frame.

- The rendered frame is encoded and streamed down to the MAF.

* * * End of Changes * * * *