3GPP TSG-SA4 MBS Post # 122	S4aI230052

# Online, 9th Mar 2023 - 30th Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Draft CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.502 | CR | 0015 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [5MBUSA] Correction of Nmb5 / Nmb10 notification events |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBUSA |  |  |  |  |  | Date: |  |  | 11.2.2023 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | During the SA4/CT4/CT3 Joint Rapporteur's call on 5MBS, a couple of issues have been discussed. One issue is the list of events and notifications from the 5G System to the MBS Application Provider. Clause 5 (Procedures) contains more indications for notification events than listed in Clause 4.6.2. Because of this, CT3 has included more events into TS 29.580 (Table 6.2.6.3.4-1), than listed in Clause 4.6.2 of TS 26.502. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The procedure description in Clause 5 is clarified, explicitly naming the event of Table 4.6.2-1. Table 4.6.2-1 is extended, adding the missing events from Clause 5. The used terminology in clause 5 is aligned to the rest of the specification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent Specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.2, 5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

### 4.6.2	Notification events

During the lifecycle of a MBS User Service Session, a number of notification messages may be sent by different Network Functions. The MBSF may modify event information when passing on notifications from other Network Functions to the MBS Application Provider (AF/AS) at reference point Nmb10 or (via the NEF) at reference point Nmb5.

The events may originate from different sources:

-	If the event originates in the MB-SMF at reference point Nmb1 or in the PCF at reference point Nmb12, it is propagated to the MBS Application Provider (AF/AS) at reference point Nmb10 (or via the NEF at reference point Nmb5). The MBSF conditions the event payload for external usage.

-	If the event originates in the MBSTF at reference point Nmb2, it is propagated to the MBS Application Provider (AF/AS) at reference point Nmb10 (or via the NEF at reference point Nmb5). The MBSF conditions the event payload for external usage.

-	If the event originates in the MBSF itself, it is propagated directly to the MBS Application Provider (AF/AS) at reference point Nmb10 (or via the NEF at reference point Nmb5).

The notification events listed in table 4.6.2-1 below are used by the MBSF to notify the MBS Application Provider (AF/AS) or NEF. The relevant stimulating reference point is indicated in brackets in the final column of the table.

Table 4.6.2-1: Notification events

| Event | Description | Applicable reference point(s) |
| --- | --- | --- |
| Session started | The MBS Session is started. | (Nmb1), Nmb10/Nmb5 |
| Session released | The MBS Session is released. | (Nmb1), Nmb10/Nmb5 |
| Distribution Session starting | The MBSF is starting to establish the MBS Distribution session. | Nmb10/Nmb5 |
| Distribution Session established | The MBS Distribution Session is established. | Nmb10/Nmb5 |
| Distribution Session service management failure | The MBS Distribution Session could not be started, for example because the necessary resources could not be allocated by the MBS System. | (Nmb1), (Nmb2), Nmb10/Nmb5 |
| Distribution Session policy control failure | The MBS Distribution Session could not be started because of a policy authorization/control failure or rejection. | (Nmb12), Nmb10/Nmb5 |
| Distribution Session activated | The MBS Distribution Session is activated successfully. | (Nmb2), Nmb10/Nmb5 |
| Distribution Session deactivated | The MBS Distribution Session is deactivated. | (Nmb2), Nmb10/Nmb5 |
| User Data Ingest Session starting | The MBSTF is starting to establish the data ingest session | Nmb10/Nmb5 |
| User Data Ingest Session established | The MBSTF has successfully established the User Data Ingest Session. In the case of pull-based content ingest for the Object Distribution Method, the MBSTF has successfully connected at least once to the origin server at reference point Nmb8. In the case of the Packet Distribution Method or push-based content ingest for the Object Distribution Method, the MBSTF has successfully received some data at reference point Nmb8. | (Nmb2), Nmb10/Nmb5 |
| User Data Ingest failure | The MBSTF is expecting ingest data for an active MBS Distribution session, but is not receiving data. | (Nmb2), Nmb10/Nmb5 |
| User Data Ingest Session terminated | The MBSTF has (temporarily) stopped the User Data Ingest Session. In the case of pull-based content ingest for the Object Distribution Method, the MBSTF has stopped pulling data. In the case of the Packet Distribution Method or push-based cotnent ingest for the Object Distribution Method, the MBSTF is ignoring data. | (Nmb2), Nmb10/Nmb5 |
| User Service Announcement | The MBSF passes the User Service Announcement information to the MBS Application Provider. This may be a URL (when the Service announcement modes parameter indicates unicast) and/or the full User Service Announcement information (when the Service announcement modes parameter indicates external delivery). | Nmb10/Nmb5 |

**** Next Change ****

# 5	Procedures for 5G Multicast–Broadcast User Services

## 5.1	General

This clause defines the high-level procedures for 5G Multicast–Broadcast User Services. All events referred to in call flows are listed in table 4.6.2-1.

## 5.2	High-level baseline procedures

The high-level baseline procedures for MBS User Services are shown in figure 5.2-1.

NOTE:	In the interests of brevity, the prefix MBS is omitted from the numbered steps in the figure.

Figure 5.2-1: MBS User Service high-level baseline procedures

The basic procedures are as follows:

1.	The MBS Application Provider provisions an  MBS User Service by invoking the Nmbsf service on the MBSF.

2.	The MBSF provisions one or more MBS Distribution Session by invoking the Nmbstf on the MBSTF.

3.	The MBSF provides a User Service announcement document that may be accessed by the MBS Client of interested UEs.

4.	The MBS Application Provider informs the MBS-Aware Application via MBS-8 that the specific Application Service can be accessed via an MBS User Service by means of an Application Service Announcement.

[Alternatively, the MBSF Client synthesises the Application Service Announcement from the User Service Announcement received in step 3 and informs the MBS-Aware Application via MBS-6 that the Application Service can be accessed via an MBS User Service.]

5.	The MBS Application Provider creates an MBS User Data Ingest Session with the MBSTF (via the MBSF) and the latter ingests the user data via Nmb8.

6.	The MBS-Aware Application requests the MBS Client to access a specific MBS User Service by invoking procedures at MBS-6.

7.	Optional: If it has not already been received in step 3 above, the MBSF Client may discover additional access information about this MBS User Service by querying the MBSF via MBS-5.

8.	The MBSF Client provides the MBS Distribution Session information acquired as part of the User Service Announcement information to the MBSTF Client via MBS-6′.

9.	The MBSTF Client activates reception of the MBS Distribution Session.

10.	The MBSF Client continuously handles the MBS Distribution Session data based on updates received via MBS-5.

11.	The MBSTF Client receives MBS Distribution Session data via MBS-4-MC as part of the MBS Distribution Session.

12. The MBSTF provides the MBS Distribution Session data to the MBS-Aware Application via MBS-7 in an MBS Application Data Session.

13.	The MBS-Aware Application controls the MBS User Service by invoking MBS Application Service Control procedures on the MBSF Client via MBS-6.

## 5.3	Procedures for User Service provisioning

The procedure begins with the MBS Application Provider provisioning an MBS User Service and, within its scope, a set of MBS User Data Ingest Sessions, as shown in figures 5.3-1 and 5.3-2 below.

Figure 5.3-1: Call flow for MBS User Service provisioning by MBS Application Provider

First, the MBS Application Provider provisions a new MBS User Service Session in the MBS System:

1.	To support Use Cases (e.g. Group Communication) where there is a requirement for TMGI allocation to be managed outside the MBS System, the MBS Application Provider may pre-allocate a TMGI for some or all of the MBS Distribution Sessions declared in step 3 below by invoking the Nmbsmf_TMGI_Allocate service operation on the MB-SMF at reference point Nmb13 (or N33+N29mb, if invoked via the NEF), as specified in clause 9.1.2.2 of TS 23.247 [5].

2.	The MBS Application Provider invokes the Nmbsf_MBSUserService_Create service operation at reference point Nmb10 (or N33+Nmb5 if invoking via the NEF) to create a new MBS User Service, as defined in clause 4.5.3.

Immediately, or at some later time, the MBS Application Provider creates at least one MBS User Data Ingest Session (as defined in clause 4.5.5) within the scope of the MBS User Service created in step 2 above:

3.	The MBS Application Provider creates an MBS User Data Ingest Session by invoking the Nmbsf_MBSUserDataIngestSession_Create service operation at reference point Nmb10 (or N33+Nmb5, if invoked via the NEF).

The MBS User Data Ingest Session optionally includes a schedule of start and end times referred to as active periods.

The MBS User Data Ingest Session comprises the details of one or more MBS Distribution Session(s), as defined in clause 4.5.6. Each such MBS Distribution Session fully specifies one of the distribution methods defined in clause 6 and may optionally nominate a TMGI to be used if one was pre-allocated in step 1 above.

4.	The MBS Application Provider subscribes to status events from the MBSF relating to the MBS User Data Ingest Session just created by invoking Nmbsf_MBSUserDataIngestSession_StatusSubscribe service operation at reference point Nmb10 (or N33+Nmb5, if invoked via the NEF).

5.	The MBSF may notify the status of each created MBS User Data Ingest Session to the MBS Application Provider by invoking the Nmbsf_MBSUserDataIngestSession_StatusNotify callback service operation at reference point Nmb10 (or N33+Nmb5, if invoked via the NEF) using the User Data Ingest Session starting event.

Shortly before a provisioned MBS User Data Ingest session is scheduled to become active (see clause 4.5.5), or immediately if no schedule of active periods is provisioned, the MBSF establishes in the MBSTF all MBS Distribution Sessions comprising that MBS User Data Ingest Session as shown in figure 5.3-2 below.

Figure 5.3-2: Call flow for MBS User Service internal provisioning

For each such MBS Distribution Session of one User Data Ingest session:

6.	If a TMGI was not nominated by the MBS Application Provider in step 1 above, the MBSF allocates one at this point for the MBS Distribution Session by invoking the Nmbsmf_TMGI_Allocate service operation on the MB-SMF at reference point Nmb1, as specified in clause 9.1.2.2 of TS 23.247 [5].

7.	The MBSF creates an MBS Session to reserve resources in the MBS System for the MBS Distribution Session by invoking the Nmbsmf_MBSSession_Create service operation on the MB-SMF at reference point Nmb1, as specified in clause 9.1.3.6 of TS 23.247 [5]). The TMGI reserved for the MBS Distribution Session in step 1 or step 6 above is provided as an input parameter. The MBSF determines the other input parameters as specified in clause 4.5.9.

8.	The MBSF creates the MBS Distribution Session in the MBSTF by invoking the Nmbstf_‌MBS‌Distribution‌Session_‌Create service operation at reference point Nmb2. This is a mirror of the entity in the MBSF (see clause 4.5.6).

-	In the case of the Object Distribution Method with push-based Object acquisition method, the response includes additional content ingest parameters chosen by the MBSTF for this MBS Distribtuion Session (see Object ingest base URL in table 4.5.6-2).

-	In the case of the Packet Distribution Method, the response includes additional content ingest parameters chosen by the MBSTF for this MBS Distribution Session (see MBSTF ingest endpoint addresses in table 4.5.6-3).

9.	The MBSF invokes the Nmbsf_MBSUserDataIngestSession_‌StatusNotify callback service operation at reference point Nmb10 (or Nmb5+N33, if invoked via the NEF) to inform the MBS Application Provider of the content ingest parameters that have been chosen for this MBS Distribution Session using the Distribution Session starting event (see MBSTF ingest endpoint addresses in table 4.5.6-3).

10.	The MBSF subscribes to status events from the MBSTF relating to the MBS Distribution Session just created by invoking the Nmbstf_MBSDistributionSessionStatusSubscribe service operation at reference point Nmb2.

11.	In the case of the Packet Distribution Method and pull-based content ingest using the Object Distribution Method, the MBSTF attempts to establish content ingest from the MBS Application Provider at reference point Nmb8 according to the ingest parameters and distribution method provisioned for the MBS Distribution Session in question (see table 4.5.6-1).

On success, the state of the MBS Distribution Session in the MBSTF becomes ESTABLISHED; on failure, it remains INACTIVE (see step 2 in clause 4.6.1).

NOTE:	Success of this step varies according to the provisioned distribution method and its configuration. Success may, for example, be defined as establishing a network association with the MBS Application Provider (using the additional parameters defined in table 4.5.6-3), or it may require successful ingest of an initial object from the MBS Application Provider (using the additional parameters defined in table 4.5.6-2).

12.	The MBSTF invokes the Nmbstf_MBSDistributionSession_StatusNotify callback service operation at reference point Nmb2 to inform the MBSF of the (un)successful establishment of content ingest using the Distribution Session established event.

On success, the state of the MBS Distribution Session in the MBSF becomes ESTABLISHED; on failure, it remains INACTIVE (see step 2 in clause 4.6.1).

13.	The MBSF invokes the Nmbsf_MBSUserDataIngestSession_StatusNotify callback service operation at reference point Nmb10 (or Nmb5+N33, if invoked via the NEF) to inform the MBS Application Provider of the (un)successful establishment of content ingest for the MBS Distribution Session in the context of its parent MBS User Data Ingest Session using either the Distribution Session established event or Distribution failure event, as appropriate.

14.	If content ingest was established successfully in step 11 above, the MBSF compiles the metadata relating to this MBS Distribution Session into an MBS Distribution Session Announcement, as defined in clause 4.5.8.

13.	The MBSF invokes the Nmbsf_MBSUserDataIngestSession_StatusNotify callback service operation at reference point Nmb10 (or Nmb5+N33, if invoked via the NEF) to inform the MBS Application Provider of the (un)successful establishment of content ingest for the MBS Distribution Session in the context of its parent MBS User Data Ingest Session using either the User Data Ingest Session established event or User data ingest failure event, as appropriate.

## 5.4	Procedures for User Service advertisement/discovery

At this point, the MBS User Service Session is advertised to the MBSF Client, as shown in figure 5.4-1 below.

Figure 5.4-1: Call flow for MBS User Service advertisement/discovery

The steps are as follows:

1.	The MBSF compiles a composite MBS User Service Announcement from the set of individual MBS Distribution Session Announcements compiled in step 14 of clause 5.3. The compiled MBS User Service Announcement describes the current set of MBS Distribution Sessions that comprise the active MBS User Data Ingest Session. The advertised start date–time is the next start time indicated in the MBS User Data Ingest Session schedule of active periods, or the current date–time if no schedule is provisioned.

2.	The MBS User Service Announcement is distributed using one or more of the following mechanisms:

a.	The MBS User Service Announcement is made available for unicast retrieval by the MBSF Client at reference point MBS-5. An URL to the MBS User Service Announcement is passed back to the MBS Application Provider by invoking the Nmbsf_MBSUserDataIngestSession_StatusNotify callback service operation at reference point Nmb10 (or Nmb5+N33, if invoked via the NEF) using the User Service Announcement event.

b.	The MBS User Service Announcement is made available via a suitable multicast/broadcast Session Announcement Channel at reference point MBS-4-MC. The MBS Client uses one of the External service identifiers (see Table 4.5.7 1) for MBS User Service identification.

c.	The MBS User Service Announcement is passed back to the MBS Application Provider by invoking the Nmbsf_MBSUserDataIngestSession_StatusNotify callback service operation at reference point Nmb10 (or Nmb5+N33, if invoked via the NEF) using the User Service Announcement event.

As a result, the MBS Application Provider advertises the MBS User Service Announcement to the MBS-Aware Application by private means at reference point MBS-8.

The MBSF may rescind an MBS User Service Announcement at any time for operational reasons.

## 5.5	Procedures for User Service data transfer

At the next start time indicated in the MBS User Data Ingest Session schedule of active periods, or immediately if no schedule is provisioned, the MBSF activates all MBS Distribution Sessions comprising that MBS User Data Ingest Session, as shown in figures 5.5-1 and 5.5-2 below.

Figure 5.5-1: Call flow for MBS Distribution Session activation by MBSF

For each such MBS Distribution Session:

1.	The MBSTF activates the MBS Session in the MB-SMF corresponding to the MBS Distribution Session:

a.	For MBS Distribution Sessions of Service type Multicast, the MBSF activates the corresponding MBS Session, allowing UEs to join the MBS Session(s). In this case, the MBSTF should hold back the data for some time, to allow establishment of the MBS Session.

b.	For MBS Distribution Sessions of Service type Broadcast, the MB-SMF notifies the MBSF upon successful establishment of the corresponding MBS Session(s).

2.	The MBSF notifies the MBS Application Provider about the successful establishment of the MBS Session(s) corresponding to the MBS Distribution Session using one or more Session started events.

3.	The MBSF invokes the Nmbstf_MBSDistributionSession_Update service operation on the MBSTF at reference point Nmb2, updating the current state of the MBS Distribution Session to ACTIVE (see step 3 in clause 4.6.1).

4.	The MBSTF notifies the MBSF about the successful activation of the MBS Distribution Session by sending the Distribution Session activated event.

5.	The MBSF notifies the MBS Application Provider about the successful activation of the MBS Distribution Session by sending the Distribution Session activated event.

6.	As a direct result of the previous step, the MBSTF begins to ingest content from the MBS Application Provider (if the pull-based Object acquisition method is provisioned) or starts accepting content published by the MBS Application Provider (if the Packet Distribution Method or the push-based Object acquisition method is provisioned).

7.	The MBSTF processes the ingested content according to the provisioned distribution method, as defined in clause 4.3.3. This may optionally include the computation of Application Level FEC (AL-FEC) information.

8.	The MBSTF distributes the resulting MBS data at reference point MBS-4-MC. This is achieved by passing the MBS data to the MB-UPF via reference point Nmb9 and/or to the MBMS-GW via reference point SGi-mb, according to the protocol stacks defined in clause 8.2 of TS 23.247 [5].

9.	On successful content ingest and MBS data distribution, the state of the MBS Distribution Session in the MBSTF becomes and remains ACTIVE (see step 3 in clause 4.6.1); on failure, it transitions through DEACTIVATING to INACTIVE (see step 4 in clause 4.6.1).

The MBSTF invokes the Nmbstf_MBSDistributionSession_StatusNotify callback service operation at reference point Nmb2 to inform the MBSF of any changes to the state of the MBS Distribution Session.

10.	The MBSF invokes the Nmbsf_MBSUserDataIngestSession_StatusNotify callback service operation at reference point Nmb10 (or Nmb5+N33, if invoked via the NEF) to inform the MBS Application Provider of any changes to the state of the MBS Distribution Session in the context of its parent MBS User Data Ingest Session.

At the next end time indicated in the MBS User Data Ingest Session schedule of active periods, the MBSF deactivates all MBS Distribution Sessions comprising that MBS User Data Ingest Session:

11.	The MBSF invokes the Nmbstf_MBSDistributionSession_Update service operation on the MBSTF at reference point Nmb2, updating the current state of the MBS Distribution Session to DEACTIVATING (see step 5 in clause 4.6.1).

12.	The MBSF tears down the MBS Session(s) corresponding to the MBS Distribution Session.

13.	 The MBSF notifies the MBS Application Provider about the termination of data ingest by sending the the User Data Ingest Session terminated event.

4.	The MBSTF notifies the MBSF about the successful deactivation of the MBS Distribution Session by sending the Distribution Session deactivated event.

15.	Once it has completed sending the data it had already ingested, the MBSF notifies the MBS Application Provider about the deactivation of the MBS Distribution Session by sending the Distribution Session deactivated event.

The MBS Client in the UE activates reception of an MBS User Service by establishing an MBS User Service Session between the MBSF Client and the MBSF, and consequently activating reception of one or more MBS Distribution Sessions by the MBSTF Client that are currently being distributed by the MBSTF. This call flow is shown in figure 5.5-2 below:

Figure 5.5-2: Call flow for MBS User Service activation by MBS Client

The steps are as follows:

16.	The MBS-Aware Application invokes a client API exposed by the MBSF Client at reference point MBS-6 to activate the MBS User Service Session.

If the MBS User Service Announcement was received by the MBS-Aware Application in step 2c in clause 5.4, this is passed as one of the parameters in the API call.

Otherwise, the target service is identified by one of the service identifiers in the MBS User Service entity (see clause 4.5.3) and this identifier is used by the MBSF Client to locate an MBS User Service Announcement obtained according to step 2a or step 2b in clause 5.4.

For each MBS Distribution Session listed in the composite MBS User Service Announcement:

17.	The MBSF Client invokes a client API exposed by the MBSTF Client at reference point MBS-6′ to activate reception of the MBS Distribution Session in question. The Session Description parameters needed to receive the MBS Distribution Session are taken from the relevant MBS Distribution Session Announcement which, in turn, is extracted from the composite MBS User Service Announcement.

18.	MBS data from the MBSTF is received by the MBSTF Client at reference point MBS-4-MC.

## 5.6	Procedure for User Service data repair

In the case of the Object Distribution Method (as defined in clause 6.1), the MBSTF Client may collaborate with the MBS AS at reference point MBS-4-UC to recover lost portions of content corresponding to MBS data that was not successfully received by the MBSTF Client at reference point MBS-4-MC (see step 9 in clause 5.5).

The procedure for data repair is illustrated in figure 5.6-1 below:

Figure 5.6-1: Call flow for MBS User Service data repair

**** Last Change ****