3GPPSA4-e (AH) MBS SWG post 124	S4aI230144

online, 29 Jun – 10 Aug 2023

Agenda item: 	2.8

Source: 	Qualcomm Incorporated

Title: 	[ARCH] Alignment of Media Functions and Protocols – Next Steps

Document for	Agreement

# Introduction

In document S4aI230132, Qualcomm provided input to the discussion on media service architectures and specifications in SA4. The details notes and agreements are provided below.

| S4aI230132 | [ARCH] Alignment of Media Functions and Protocols | Qualcomm CDMA Technologies | Thomas Stockhammer |
| --- | --- | --- | --- |

Presenter: Thomas Stockhammer

Online Discussion:

- Stephane: How do you see communication services in 26.114 in relation to this? I have some questions about the naming. Do we have a new spec in parallel to 26.114 or replace?

  - Thomas: 26.114 is out of scope for this. It would proceed separately.

- Yoshihiro: How will the differences between 5GMS and RTC be covered? For example, under Figure 2, Media AS is described as dedicated to media delivery. How about RTC aspects? It needs to support WebRTC signaling aspects at least.

  - Thomas: These are details for starting point. We could extend the definition to include all the RTC aspects during this work.

- Spencer: Question on bullet point 5: if we are adding everything to 26.114, do you feel that we will be adding everything to 114?

  - Thomas: I believe it would be a large unstructured document if we add to 114. Preference is not to aggregate everything in 114. It would be better if we start with a blank page.

  - Spencer: I would support starting new

  - Richard: I think we are going to have the problem of mingling different aspects and domains anyway even if we start a new spec. Do we envisage the two new specs are abstractions of M1 and M5. Is that the idea?

  - Thomas: I believe we will have a hard time to separate media streaming and real time communication anyway. We have to look at each feature separately

  - Richard: It may not be much different to either update the current spec, or end up having new specs. Starting with a new spec might be appealing.

- Spencer: Table 1 is helpful. It will help us understand and make it clear as to the features that are supported.

- Iraj: About “add updated functionalities directly to TS 2651x and TS 26.51y, add reference from 26.512 as needed” - when we are adding new text, how do we make sure we are not conflicting 512 text. Do we need to be checking? There might be issues

  - Thomas: Yes, we need to careful. Make sure we are doing it carefully everytime we add some new text to new spec.

- Richard: WIth this proposal , where would be OpenAPI live for Rel-18, and what would happen with Open API for Rel17?

  - Thomas: It would stay as is right? We need a bit of thinking. We can address this

- Yoshihiro: Title of 51y has to be modified.

- Yoshihiro: In conclusion #4, we have to add an objective for creating specifications about protocols.

  - Thomas: It is included in item 3, as it is in 26.506, and we are bringing everything from 26.506.

Decision: agreed. Implementation of agreements needed

S4aI230132 is agreed.

This document summarizes the decision and provides a first implementation of the agreements.

# Background

In S4-230818, commonalities and differences between the across the different work items and specifications were identified.

Commonalities across different work items are summarized in the following. For Media Delivery, extensions to the 5G System architecture were developed to address media delivery. This includes different delivery systems including download, streaming, real-time communication, etc. Key in the media delivery is that the media is time-continuous. Streaming points to the fact that the media is predominantly sent only in a single direction and consumed as it is received. Real-time communication refers more to bi-directional traffic for which media is delivered in both directions.

Generalized Media Support within the 5G System is shown in Figure 1 and Figure 2 below.

Figure 1 – Generalized Media Support within the 5G System

Figure 2 Generalized 5G Media Delivery Architecture

Functional definitions may be generalized as follows:

-	Media AF: An Application Function similar to that defined in TS 23.501, clause 6.2.10, dedicated to 5G Media Delivery.

-	Media AS: An Application Server dedicated to 5G Media Delivery.

-	Media Client: A UE internal function dedicated to 5G Media Delivery.

-	Media Session Handler: A function on the UE that communicates with the Media AF in order to establish, control and support the delivery of a media session.

-	Media Access Function: A UE internal function A function on the UE that communicates with the Media AS in order to access and deliver media content. The media access function for example may be further sub-divided into content delivery protocols, codecs, media types and metadata representation.

The following interfaces and APIs may be defined for 5G Media Delivery:

-	M1 (Provisioning API): External API, exposed by the Media AF which enables the Media Application Provider to provision the usage of the 5G Media Delivery and to obtain feedback.

-	M2 (User Plane interface): External interface provided by the Media AS and used when the Media AS in the trusted DN to exchange data media data with the application service provider.

-	M3: (Server Configuration API): API used to exchange information between Media AF and Media AS for configuration purposes.

-	M4 (Media Delivery Interface): Interface and reference point between media access function and Media AS in order to exchange media content.

-	M5 (Session Handling API): APIs exposed by a Media AF to the Media Session Handler for media session handling, control, reporting and assistance that also include appropriate security mechanisms, e.g. authorization and authentication.

-	M6 (Client Configuration APIs): APIs exposed by a Media Session Handler to the Application and media access function for client-internal communication.

-	M7 (Media Access APIs): APIs exposed by a Media Access function to configure and communicate with the Media access function.

-	M8 (Application reference point): application interface used for information exchange between the Media Application and the Media Application Provider.

While on architecture and interface level, commonalities are pretty obvious, questions may arise what is common on the next level. For this, the core functions are further divided into

-	Media Delivery Functions including codecs, content delivery protocol, encapsulation

-	Media Session Handling functions

What has been of lower priority until now, but is getting more importance and never found a real home, are aspects around HTTP versions, HTTP headers in use, the usage and configuration for HTTP and QUIC for media user plane services, secure media delivery, and so on. While some of those aspects may be dealt specifically for a stack, some layering is needed, and also components, extensions and profiling of these stacks is needed.

A summary what may be useful for either of the frameworks is provided in Table 1.

Table 1 Overview of media and media handling functions in different service scenarios

| Feature | Downlink media streaming | Uplink media streaming | Real-time communication |
| --- | --- | --- | --- |
| Media Delivery Functions and Protocols |  |  |  |
| CMAF | Yes | yes | No |
| DASH/HLS | Yes | No | No |
| MP4 | Yes | No | No |
| RTP/AVP | Not until now | Not until now | No |
| webRTC | Not until now | Not until now | yes |
| HTTP/1.1 and TCP/IP | Yes | Yes | No |
| HTTP/2 | Yes | Yes | No |
| HTTP/3 and QUIC | Yes | Yes | No |
| UDP/IP | Yes | Yes | yes |
| Media Handling Functions |  |  |  |
| Service Access | Yes | Yes | Yes In form of configuration information |
| Content hosting | Yes | No | ??? |
| Content publishing | No | Yes | ??? |
| Content preparation | Yes | No | ??? |
| Network assistance | Yes | Yes | Yes |
| Dynamic policies | Yes | Yes | Yes |
| Remote control | No | Yes | ??? |
| Consumption reporting | Yes | No | Yes |
| QoE metrics reporting | Yes | ??? | ??? |
| Service URL Handling | Yes | Yes | Yes |
| Edge Computing | Yes | Yes | Yes |
| eMBMS delivery | Yes | No | ??? |
| Data Collection | Yes | ??? | ??? |
| MBS delivery | Yes | No | ??? |

At the end, many of the features collected above may be considered a toolbox which are applicable to different service scenarios. Differences for the different service scenarios in the content delivery protocols as well as in the applicable tools.

# Agreed Actions and Proposals from S4-230132

Based on the SA4 agreements, the status check and the background, the following concrete actions and proposals are need to be taken

- Reconfirm that a separation on stage-3 in user plane (AS and Media Client) and control plane (AF and Media Session Handler) independent of the Media Service is a principle way forward. (see agreement 5 in Table 2-1)

- Minimize in the remaining Rel-18 timeframe the work on stage-2 in TS 26.501 and TS 26.506, but address this principle agreement by adding the proposal 2 in Table 2-1 to TS 26.506 and TS 26.501 in CRs at SA4#125.

- At the end of Rel-18, have (addressing agreement 6 in Table 2-1)

  - two new stage-3 specifications

    - TS 26.51x: "5G Media Session Handling" addressing stage-3 Media Handling features and interfaces M1, M5 and M6 for all media services.

    - TS 26.51y: "5G Media Content Delivery" addressing stage-3 Media Content delivery features and interfaces M2, M3, M4 and M7 for all media services taking into account different content and media delivery protocols.

  - TS 26.512 and possibly other specifications referring to specific features in these new specifications, and gradually deprecate this.

- In order to get to this, (implementing the second option of 6 in Table 2-1)

  - create a new stage-3 work item as follows

    - 5G Media Delivery – General Features, Real-time Communication and Alignment with Streaming

    - Main objectives:

      - Create a generalized media session handling framework consolidating the stage-3 specification for interfaces M1, M5 and M6

      - Create a generalized media delivery framework consolidating the stage-3 specification for interfaces M2, M3, M4 and M7

      - Address the stage 3 functionalities of TS 26.506 in alignment with functionalities already defined in TS 26.512.

    - Impacted specifications: TS 26.51x, TS 26.51y

  - Update the stage-3 work item 5GMS_Pro_Ph2 as follows

    - Add to the objectives

      - Support a generalized media session handling framework consolidating the stage-3 specification for interfaces M1, M5 and M6

      - Support a generalized media delivery framework consolidating the stage-3 specification for interfaces M2, M3, M4 and M7

    - Add new functionalities directly to TS 26.51x and TS 26.51y, and reference from TS 26.512 as needed.

    - Add updated functionalities directly to TS 26.51x and TS 26.51y, and reference from TS 26.512.

    - Updated impacted specifications and add TS 26.51x, TS 26.51y

  - Run a permanent document on the side to investigate the commonality and differences between 26.501 and 26.506 especially for the functionalities, interfaces, procedures, and resources associated and document them in TS 26.506 using the progress and findings on a common stage-3 specification. (addressing proposal 4 in Table 2-1)

  - Run these two work items in a single SA4 SWG or in a joint session of two SWGs.

- If proposal 3 is not acceptable for Rel-18 then consider

  - not creating any new specification, but add everything to TS 26.512 and use a subset of the proposal 4 (implementing the first option of 6 in Table 2-1)

  - re-using existing specifications such TS 26.113

If proposals are agreed, we will support the procedures by:

- drafting relevant work items and work item updates

- supporting being rapporteur of work items, specifications and permanent documents

- support execution of the above plans

# Proposed Implementation of Agreements

| # | Agreed proposal in S4aI0132 | Proposed Implementation |
| --- | --- | --- |
| 1 | Reconfirm that a separation on stage-3 in user plane (AS and Media Client) and control plane (AF and Media Session Handler) independent of the Media Service is a principle way forward. | No actions needed |
| 2 | Minimize in the remaining Rel-18 timeframe the work on stage-2 in TS 26.501 and TS 26.506, but address this principle agreement by adding the proposal 2 in Table 2-1 (see below) to TS 26.506 and TS 26.501 in CRs at SA4#125. Add a diagram to TS 26.506 (or at least at a bullet point to the outstanding issues) to address that RTC AF and 5GMS AF functionalities are included in a common Media AF on the network and also the Media Session Handler. - A conceptual example is illustrated as follows; (Note: Dotted line in RTC AF means it may need further clarification along with its stage-3 work) Encourage that the same action is done in TS 26.501 to add a not | Create a CR for TS 26.501 (not yet provided, but will be ready for SA4#125) Create a CR for TS 26.506 (not yet provided, but will be ready for SA4#125) |
| 3 | At the end of Rel-18, have (addressing agreement 6 in Table 2-1) - two new stage-3 specifications - TS 26.51x: "5G Media Session Handling" addressing stage-3 Media Handling features and interfaces M1, M5 and M6 for all media services. - TS 26.51y: "5G Media Content Delivery" addressing stage-3 Media Content delivery features and interfaces M2, M3, M4 and M7 for all media services taking into account different content and media delivery protocols. - TS 26.512 and possibly other specifications referring to specific features in these new specifications, and gradually deprecate this. | Provide Draft TS 26.51x (not yet provided, but will be ready for SA4#125) Provide Draft TS 26.51y (not yet provided, but will be ready for SA4#125) |
| 4a | create a new stage-3 work item as follows - 5G Media Delivery – General Features, Real-time Communication and Alignment with Streaming - Main objectives: - Create a generalized media session handling framework consolidating the stage-3 specification for interfaces M1, M5 and M6 - Create a generalized media delivery framework consolidating the stage-3 specification for interfaces M2, M3, M4 and M7 - Address the stage 3 functionalities of TS 26.506 in alignment with functionalities already defined in TS 26.512. Impacted specifications: TS 26.51x, TS 26.51y | Attached is a draft for GeneReAl work item |
| 4b | Update the stage-3 work item 5GMS_Pro_Ph2 as follows - Add to the objectives - Support a generalized media session handling framework consolidating the stage-3 specification for interfaces M1, M5 and M6 - Support a generalized media delivery framework consolidating the stage-3 specification for interfaces M2, M3, M4 and M7 - Add new functionalities directly to TS 26.51x and TS 26.51y, and reference from TS 26.512 as needed. - Add updated functionalities directly to TS 26.51x and TS 26.51y, and reference from TS 26.512. Updated impacted specifications and add TS 26.51x, TS 26.51y | Attached is a draft update for 5GMS_Pro_Ph2 work item |
| 4c | Run a permanent document on the side to investigate the commonality and differences between 26.501 and 26.506 especially for the functionalities, interfaces, procedures, and resources associated and document them in TS 26.506 using the progress and findings on a common stage-3 specification. | Provide Draft PD (not yet provided, but will be ready for SA4#125) |
| 4d | Run these two work items in a single SA4 SWG or in a joint session of two SWGs. | Attached is a draft work plan for GeneReAl work item |

# Proposal

It is proposed to

- Agree the proposed implementations in clause 4

- Collect initial feedback for the attached documents

- Collect co-signers for the different documents