3GPP TSG-SA4 Meeting # -e (AH) MBS SWG post 125                               S4aI230173

# Online,  , 2nd November 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.565 | CR |  | rev | - | Current version: | 0.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [SR_MSE] Split Rendering in SR_MSE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | SR_MSE |  |  |  |  |  | Date: |  |  | 02-11-2023 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Split rendering, as currently defined in the draft TS, does not fully leverage rendering resources of the UE. In addition to perfoming pose correction, rendering resources of the UE can be used to render some parts/objects of the scene to improve the QoE of a rendering session. This is especially relevant when network conditions are sub optimal, e.g. there is high motion to render to photon latency. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Addition of a new rendering flag and a renderingSplit object in the Split rendering configuration. - Addition of a renderingSplit adaptation message format. |  |  |  |  |  |  |  |  |
| tr |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE rendering resources will be under-utilized in split rendering and QoE may fluctuate substantially with link quality. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.2.2, 8.2.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | N/A |  |  |  |  |  |  |  |  |

- Introduction and Motivation

Split Rendering Media Service Enabler Work Item collects 5G media functions to build a media service enabler for the realization of split rendered applications. Split Rendering, by definition, splits the rendering operation between a powerful edge server and a relatively resource constrained “client”. In the current version of TS 26.565, only the simplest case of split rendering is considered. In this simple case, the server does most of the rendering operations and transmits the rendered frame to the UE which displays the rendered frame after pose correction. We believe this simple case, while useful for low end devices, does not utilize the resources available at a typical UE. Currently, available UEs have sizeable rendering resources which can be leveraged to substantially improve the QoE of a split rendering session, especially in sub-optimal network conditions. To leverage these resources, the split rendering client should do rendering operations in addition to pose correction. A simple approach towards that is rendering some objects of a scene at the split rendering client. This will allow the QoE of a split rendering session to be maintained in sub optimal conditions, for example, when motion-to-render-to-photon latency is high. This approach of adapting the split rendering session can be used in synergy with codec level rate control of the rendered media.

We propose to use the format conformant data channel messages for SRS and SRC adapt where rendering of objects in a scene should be done

1st Change

#### 8.4.2.2	 Split Rendering Configuration Format

The session configuration information shall be in JSON format. It shall have the following format:

| Name |  | Type | Cardinality |  | Description |  |
| --- | --- | --- | --- | --- | --- | --- |
| renderingFlags | Array(SR_CONFIG_FLAGS) |  |  | 0..1 |  | Provides a set of flags to activate/deactivate selected rendering functions. The defined SR_CONFIG_FLAGS are: - FLAG_ALPHA_BLENDING - FLAG_DEPTH_COMPOSITION - FLAG_EYE_GAZE_TRACKING - FLAG_ADAPTIVE_SPLIT_RENDERING |
| spaceConfiguration |  | Object | 0..1 |  | The space configuration is typically sent by the split rendering server to the split rendering client. Upon reception of this information, the SR client uses this information to create the reference and action spaces as well as to agree on common identifiers for the XR spaces. |  |
| referenceSpaces |  | Array | 0..1 |  | An array of reference spaces and their identifiers. |  |
| id |  | number | 1..1 |  | A unique identifier of the XR space in the context of the split rendering session. |  |
| refSpace |  | enum | 1..1 |  | One of the defined reference spaces in OpenXR. These may be: XR_REFERENCE_SPACE_TYPE_VIEW, XR_REFERENCE_SPACE_TYPE_LOCAL, or XR_REFERENCE_SPACE_TYPE_STAGE |  |
| actionSpaces |  | Array | 0..1 |  | An array of action spaces that need to be defined by the split rendering client in the XR session. |  |
| id |  | number | 1..1 |  | A unique identifier of the XR space in the context of the split rendering session. |  |
| actionId |  | number | 1..1 |  | Provides the unique identifier of the action. |  |
| subactionPath |  | string | 1..1 |  | The subaction path identifies the action, which can then be mapped by the XR runtime to user input modalities. |  |
| initialPose |  | Pose | 0..1 |  | Provides the initial pose of the new XR space’s origin. |  |
| viewConfiguration |  | Object | 0..1 |  | Conveys the view configuration that is configured for the XR session. |  |
| type |  | Enum | 1..1 |  | The type indicates the view configuration. Defined values are MONO and STEREO. Other values may be added. |  |
| width |  | number | 1..1 |  | The recommended width of the swapchain image. |  |
| height |  | number | 1..1 |  | The recommended height of the swapchain image. |  |
| compositionLayer |  | string | 1..1 |  | An identifier of the selected composition layer. |  |
| environmentBlendMode |  | enum | 1..1 |  | The type indicates the environment blend mode configuration. Defined values are OPAQUE, ADDITIVE and ALPHA_BLEND. Other values may be added. |  |
| actionConfiguration |  | Array | 0..1 |  | This contains a list of the actions that are to be defined by the SR client. |  |
| action |  | Object | 1..n |  | A definition of a single action object. |  |
| id |  | number | 1..1 |  | A unique identifier of the action. |  |
| actionType |  | enum | 1..1 |  | The type of the action state. This can be a Boolean, float, vector2, pose, vibration output, etc. |  |
| subactionPaths |  | string | 1..n |  | An array of subaction paths associated with this action. The split rendering client will provide the state of all defined sub-action paths. |  |
| renderingSplit |  | Object | 1,.1 |  | An JSON object identifying objects to be rendered and where they are to be rendered (SRS or SRC), for example, as a dictionary with keys “SRS” and “SRC” and lists of object indices from a scene description or a scene graph. |  |
| extraConfigurations |  | Object | 0..1 |  | A placeholder for addition configuration information. |  |

End of 1st Change

2nd Change

#### 8.2.2.4	Split Adaptation Message Format

#### During a split rendering session, the operating environment of the split rendering server, the split rendering client or the network conditions may change. Consequently, the rendering split may need to be adapted to deliver a consistent QoE. When true adaptive split rendering is enabled , the SRS or SRC shall request a new rendering split by sending a message of the type “urn:3gpp:split-rendering:v1:sr-split” . The same message type shall be used to acknowledge, accept or reject the request by the receiver.

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| id | string | 1..1 | An unique identifier of the message in the scope of the data channel session. |
| type | string | 1..1 |  |
| message | Object | 1..1 | Message content |
| subtype | string | 1..n | An identifier of the subtype of the message, it may be a request or new split or acknowledgement, acceptance or rejection of a request |
| renderingSplit | Object | 1..1 | An JSON object identifying objects to be rendered and where they are to be rendered (SRS or SRC), for example, as a dictionary with keys “SRS” and “SRC” and lists of object indices from a scene description or a scene graph |

End of 2nd Change