3GPP TSG-SA WG4 ad hoc Meeting post-#126	S4-230176

Online, 30th November 2023–18th January 2024

Title:	Further discussion on OpenAPI design

Source:	BBC

Agenda Item:	8.8 (5GMS_Pro_Ph2)

Document for:	Discussion and agreement

Contact:	Richard Bradbury <richard dot bradbury at bbc dot co dot uk>

# Abstract

As part of the work to port the 5GMS AF provisioning and media session handling APIs from TS 26.512 [1] to TS 26.510 [3] in Release 18, this discussion paper analyses which of the provisioning and media session handling APIs are common between 5GMS and RTC.

# 1.	Background

It was agreed in SA4#126 to follow the general OpenAPI design outlined in Discussion Paper S4-231641. This gives new names to the provisioning API at reference point M1 and the media session handling API at reference point M5. In summary:

| API purpose | Applicable reference point(s) | Agreed Rel-18 API name | Agreed Rel-18 base URL |
| --- | --- | --- | --- |
| Provisioning | M1 | Maf_Provisioning | {apiRoot}/3gpp-maf-provisioning/‌{apiVersion}/ |
| Media Session Handling | M5, M3 | Maf_SessionHandling | {apiRoot}/3gpp-maf-session-handing/‌{apiVersion}/ |

This Discussion Paper provides some analysis of the applicability of the individual 5GMS APIs currently specified in TS 26.512 [1] to RTC, to be specified in TS 26.113 [2].

A work-in-progress prototype implementation of the two top-level APIs that allows mixing and matching of sub-APIs by TS 26.512 [1] and TS 26.113 [2] can be found here in 3GPP Forge:

- https://forge.3gpp.org/rep/bradburyr/5gms_pro_ph2/-/blob/main/TS26512_Maf_Provisioning.yaml

- https://forge.3gpp.org/rep/bradburyr/5gms_pro_ph2/-/blob/main/TS26512_Maf_SessionHandling.yaml

# 2.	Applicability of 5GMS APIs to RTC

## 2.1	Media Delivery provisioning APIs

It is proposed to divide the APIs for provisioning the 5GMS AF at reference point M1 between TS 26.512 and TS 26.510 as follows:

| Agreed Rel-18 API name | Agreed Rel-18 base URL | 5GMS | RTC | Rel-18 target |
| --- | --- | --- | --- | --- |
| Maf_Provisioning_‌ProvisioningSessions | {apiRoot}/3gpp-maf-provisioning/{apiVersion}/‌provisioning-sessions/ |  |  | TS 26.510 |
| Maf_Provisioning_‌ContentProtocols | {apiRoot}/3gpp-maf-provisioning/{apiVersion}/‌content-protocols |  | ? | TS 26.510? |
| Maf_Provisioning_‌ServerCertificates | {apiRoot}/3gpp-maf-provisioning/{apiVersion}/‌server-certificates/ |  | ? | TS 26.510? |
| Maf_Provisioning_‌PolicyTemplates | {apiRoot}/3gpp-maf-provisioning/{apiVersion}/‌policy-templates/ |  |  | TS 26.510 |
| Maf_Provisioning_‌ContentPreparationTemplates | {apiRoot}/3gpp-maf-provisioning/{apiVersion}/‌content-preparation-templates/ |  |  |  |
| Maf_Provisioning_‌ContentHosting | {apiRoot}/3gpp-maf-provisioning/{apiVersion}/‌content-hosting-configuration |  |  |  |
| Maf_Provisioning_‌ContentPublishing | {apiRoot}/3gpp-maf-provisioning/{apiVersion}/‌content-publishing-configuration |  |  |  |
| Maf_Provisioning_‌ConsumptionReporting | {apiRoot}/3gpp-maf-provisioning/{apiVersion}/‌consumption-reporting-configuration |  | ? |  |
| Maf_Provisioning_‌MetricsReporting | {apiRoot}/3gpp-maf-provisioning/{apiVersion}/‌metrics-reporting-configurations/ |  |  | TS 26.510 |
| Maf_Provisioning_‌EdgeResources | {apiRoot}/3gpp-maf-provisioning/{apiVersion}/‌edge-resources-configurations/ |  |  | TS 26.510 |
| Maf_Provisioning_‌EventDataProcessing | {apiRoot}/3gpp-maf-provisioning/{apiVersion}/‌event-data-processing-configurations/ |  |  |  |

## 2.2	Network Media Session Handling APIs

It is proposed to divide the APIs for media session handling at reference point M5 (and M3) between TS 26.512 and TS 26.510 as follows:

| Agreed API name | Proposed base URL | 5GMS | RTC | Rel-18 target |
| --- | --- | --- | --- | --- |
| Maf_SessionHandling_‌ServiceAccessInformation | {apiRoot}/3gpp-maf-session-handing/‌{apiVersion}/‌service-access-informations/‌{externalServiceId} or {apiRoot}/3gpp-maf-session-handing/‌{apiVersion}/‌sessions/‌{sessionId} |  |  | TS 26.510 |
| Maf_SessionHandling_‌DynamicPolicy | {apiRoot}/3gpp-maf-session-handing/‌{apiVersion}/‌dynamic-policies/ or {apiRoot}/3gpp-maf-session-handing/‌{apiVersion}/‌sessions/‌{sessionId}/‌dynamic-policies/ |  |  | TS 26.510 |
| Maf_SessionHandling_‌NetworkAssistance | {apiRoot}/3gpp-maf-session-handing/‌{apiVersion}/‌network-assistance-sessions/ or {apiRoot}/3gpp-maf-session-handing/‌{apiVersion}/‌sessions/‌{sessionId}/‌network-assistance-sessions/ |  |  | TS 26.510 |
| Maf_SessionHandling_‌MetricsReporting | {apiRoot}/3gpp-maf-session-handing/‌{apiVersion}/‌metrics-reporting or {apiRoot}/3gpp-maf-session-handing/‌{apiVersion}/‌sessions/‌{sessionId}/‌metrics-reporting |  |  | TS 26.510 |
| Maf_SessionHandling_‌ConsumptionReporting | {apiRoot}/3gpp-maf-session-handing/‌{apiVersion}/‌consumption-reporting or {apiRoot}/3gpp-maf-session-handing/‌{apiVersion}/‌sessions/‌{sessionId}/‌consumption-reporting |  | ? |  |

# 3.	Proposal

It is proposed that SA4 agrees:

- To complete the tables in section 2 based on discussion.

- Once complete, to pursue the division of the Media AF provisioning (M1) and media session handling (M5) APIs outlined in section 2 when porting from TS 26.512 [1] to TS 26.510 [3].

- To record this agreement by incorporating the text in section 2 of this Discussion Paper into the Permanent Document associated with this Work Item.

# References

- [1]	3GPP TS 26.512: "5G Media Streaming; Protocols and APIs", Release 17.

- [2]	3GPP TS 26.113: "Real-Time Media Communication; Protocols and APIs".

- [3]	3GPP TS 26.510: "Media delivery; interactions and APIs for provisioning and media session handling", Release 18.