| 3GPP SA4-e (AH) MBS SWG post 126 S4aI230183 # Online, Earth, 30th November 2023 CR-Form-v12.2. PSEUDO CHANGE REQUEST. 26.143; CR; <CR#>; rev; <Rev#>; Current version:; 0.2.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; Core Network. Title:; pCR 26143 v0.2.0 on updates to Messaging Media Profiles (Rel-18). Source to WG:; Dolby France SAS. Source to TSG:; S4. Work item code:; PROMISE; Date:; 11-29-2023. Category:; B; Release:; Rel-18. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19). Reason for change:; WID objectives. Summary of change:; Updating TS, especially audio capabiliies, but also references and other clean-up. Consequences if not approved:. Clauses affected:; 8, 10.2.4, 10.3.4, 10.4.4, 10.5.4. Y; N. Other specs; Other core specifications; TS/TR ... CR .... affected:; Test specifications; TS/TR ... CR .... (show related CRs); O&M Specifications; TS/TR ... CR .... Other comments:. This CR's revision history:. 3GPP TS 26.143 V0.2.0 (2023-11) |
| --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Messaging Media Profiles; (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	6

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	8

4	Examples for styles	8

4.1	Heading styles	8

4.2	Other common styles	8

"TSG <Name>" on the front page	9

Page setup parameters	9

Proforma copyright release text block	11

X.1	The right to copy	11

Abstract Test Suite (ATS) text block	12

Y	Abstract Test Suite (ATS)	12

Y.1	Introduction	12

Y.2	The TTCN Graphical form (TTCN.GR)	12

Y.3	The TTCN Machine Processable form (TTCN.MP)	12

Annex <A> (normative): <Normative annex for a Technical Specification>	13

Annex <B> (informative): <Informative annex for a Technical Specification>	14

B.1	Heading levels in an annex	14

Annex <C>: <Informative annex title for a Technical Report>	15

Annex <D> (informative): Bibliography	16

Annex <E> (informative): Index	17

Annex <X> (informative): Change history	18

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document specifies the media types, formats, codecs capabilities and profiles for the messaging applications used over the 5G System. The scope of the present document extends to codecs for speech, audio, video, still images, bitmap graphics, 3D scenes and assets, and other media in general, as well as scene description.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x13]	Khronos glTF 2.0, glTF™ 2.0 Specification (khronos.org)

[x14] 	ISO/IEC 23090-14 AMD 2, Information technology — Coded representation of immersive media — Part 14: Scene description — Amendment 2: Support for haptics, augmented reality, avatars, Interactivity, MPEG-I audio, and lighting

[x15]	3GPP TS 26.511: "5G Media Streaming (5GMS); Profiles, Codecs and Formats".

[x16]	3GPP TS 26.117: "5G Media Streaming (5GMS); Speech and audio profiles".

[x17]	3GPP TS 26.250: "Codec for Immersive Voice and Audio Services - General overview"

[x18]	3GPP TS 26.252: "Codec for Immersive Voice and Audio Services - Test sequences"

[x19]	3GPP TS 26.253: "Codec for Immersive Voice and Audio Services - Detailed Algorithmic Description incl. RTP payload format and SDP parameter definitions"

[x20]	3GPP TS 26.254: "Codec for Immersive Voice and Audio Services - Rendering"

[x21]	3GPP TS 26.255: "Codec for Immersive Voice and Audio Services - Error concealment of lost packets"

[x22]	3GPP TS 26.256: "Codec for Immersive Voice and Audio Services - Jitter Buffer Management"

[x23]	3GPP TS 26.251: "Codec for Immersive Voice and Audio Services - C code (fixed-point)"

[x24]	3GPP TS 26.258: "Codec for Immersive Voice and Audio Services; C code (floating-point)"

[x25]	IETF Draft draft-ietf-mimi-content-01: "More Instant Messaging Interoperability (MIMI) message content", 	Rohan Mahy

[x26]	3GPP TS 22.140: "Multimedia Messaging Service (MMS); Stage 1".

[x27]	Open Mobile alliance, "MMS Architecture" OMA-AD-MMS-V1_3-20110913-A.

[x28]	Open Mobile alliance, "Multimedia Messaging Service Encapsulation Protocol" OMA-TS-MMS_ENC-V1_3-20110913-A.

[x29]	GSMA "RCS Universal Profile Service Definition Document", Version 2.6, 19 December 2022

[x30]	GSMA PRD RCC.07 version 13.0 - "Rich Communication Suite - Advanced Communications Services and Client Specification" 19 December 2022

[x31]	IETF RFC 2046: "Multipurpose Internet Mail Extensions (MIME) Part Two: Media Types"

[x32]	ISO/IEC 14496-12: "Information technology - Coding of audio-visual objects -Part 12: ISO base media file format".

[x33]	ISO/IEC 23000-XX:2023 Preliminary Draft of: Information technology — Multimedia application format (MPEG-A) — Part XX: Messaging Media Application Format (MeMAF) ".

Editor’s Note: This is ongoing work and may not be available as a reference before completion of the work in Rel-18

[x34]	3GPP 23.140: "Multimedia Messaging Service (MMS); Functional Description; Stage 2".

[x35]	ITU-T Recommendation T.81: "Information technology; Digital compression and coding of continuous-tone still images: Requirements and guidelines".

[x36]	"JPEG File Interchange Format", Version 1.02, September 1, 1992.

[x37]	"Exchangeable image file format for digital still cameras: EXIF 2.2", Specification by the Japan Electronics and Information Technology Industries Association (JEITA), April 2002, URL: http://www.exif.org/

[x38]	CompuServe Incorporated: "GIF Graphics Interchange Format: A Standard defining a mechanism for the storage and transmission of raster-based graphics information", Columbus, OH, USA, 1987.

[x39]	Compuserve Incorporated, Columbus, Ohio (1990): "Graphics Interchange Format (Version 89a)".

[x40]	IETF RFC 2083: "PNG (Portable Networks Graphics) Specification version 1.0 ", T. Boutell, et. al., March 1997.

[x41]	ISO/IEC 23000-22:2019 Information technology — Multimedia application format (MPEG-A) — Part 22: Multi-image application format (MIAF)

“”

# 3	Definitions of terms, symbols and abbreviations

“”

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Messaging Media Profile: a set of UE capability requirements associated to a media-centric messaging service scenario.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

glTF 	Graphics Library Transmission Format

IANA	Internet Assigned Numbers Authority

MIMI	More Instant Messaging Interoperability

MMBP	multimedia messaging body part

MMS	Multimedia Messaging Service

RCS	Rich Communication Services

glTF 	Graphics Library Transmission Format

# 4	Overview and Context

## 4.1	Background and Assumptions

Messaging services typically define a message container. Such a container typically carries one or more body parts with the actual message content (for example, an emoji used in a reaction, a plain text or rich text message or reply, a link, or an inline image, or richer media types).

An important feature of messages are body parts that include media content. Different media content exists, such as simple and rich text, still images, graphics, speech, audio, video, 3D scenes and many other media types.

This specification is not defining a container format, but it addresses the usability of 3GPP defined media types and formats into messages as part of a message body within message containers. Examples for message containers are MMS [x26][x27][x28][x34], IETF MIMI [x25] or GSMA RCS [x29][x30]. However, this specification is not restricted to be used with fully specified Messaging Service, it may as well be used as part of third-party messaging services as a well specified component. It may also serve to support content interoperability across different messaging services.

The term media type is used as short to refer to the IANA media type, subtype, and parameters as defined in [x31] and provides a well defined property of a content.

In order to use this specification as part of a container format, it is expected that the container format supports the following functionalities:

1)	It can carry an octet string representing the content

2)	It can signal the media type of the content.

3) 	The content and media type of the content is not restricted, but allows to send to send and receive formats that are not defined in the core container format.

The above is referred to as a single media type content.

In addition, a container format may support one or more of the following functionalities:

-	the body can have multiple, possibly nested parts with one of the following properties and structures

-	mixed: there are multiple media types associated with the same message which need to be rendered together, for example a rich-text message with an inline image. The receiver is expected to process as many of the nested parts at this level as possible.

-	alternate: there are multiple media types associated with the same message and the receiver can choose an appropriate one based on its own policies using the media type or possibly other parameters (e.g. a language) of each part

-	related: there are multiple media types associated with the same message and all the nested body parts at this level are part of a single entity that are processed jointly, possibly by providing a root object for initial processing. If the receiver does not understand even one of the nested parts at this level, the receiver is not expected to process any of them.

-	nested: there are multiple media types associated with the same message, and one or several of the media types are representing a single, mixed, alternate or related structure.

-	it may have body parts that reference external content via URI that will be processed automatically. It includes a media type and may optionally include the size of the data, an expiration timestamp and other parameters. The content may be rendered with the other parts of the message, or a be downloaded or rendered separately. The content may also be encrypted.

The focus of this specification is the definition of body parts that carry multimedia content, referred to as multimedia messaging body part (MMBP). This specification does not generally define how the body party is encoded, functionalities such as multi-part MIME or MIMI [x25] may be used. However, this specification defines the definition of a body-part using the ISO Base Media File format to provide features for mixing multiple assets into a single body part.

## 4.2	System Description

Based on the background and assumptions in clause 4.1, Figure 4.2-1 provides an example system for a messaging services and highlights scope of this specification, namely the definition of a multimedia messaging body part (MMBP) and the associated metadata.

Figure 4.2-1 Example system for Messaging multimedia message exchange

A Messaging Service Sender instructs a MMBP generator to generate an MMBP, for example using an API. This for example allows to define configurations on codecs, size, experiences or other attributes of the MMBP. The details of such an API are outside the scope of this specification. The sender adds the MMBP to a Container Message (either includes or by reference), together with MMBP metadata parameters that provide information about the MMBP. Metadata includes, but is not limited to:

-	The media type of the MMBP, including subtypes and parameters for codecs, etc.

-	The size of the MMBP

-	Accessibility or language information about the MMBP

-	processing requirements of recommendations of the MMBP

The client of the messaging service receives the container message that includes the above information. The client communicates with a MMBP player its capabilities whether the MMBP can be played back, and if multiple alternatives are present, which of those are to be selected. Then the messaging service client instructs the MMBP player to playback the MMBP as part of the messaging service, based on the processing requirements and instructions. Playback may be combined with additional instructions for a player, including play, pause, seek, etc.

## 4.3	MMBP Player Model

The design of the formats defined in this document is based on the player model as shown in Figure 4.3-2. The figure illustrates the logical components of a conceptual MMBP Player model. In this figure, the MMBP parser receives the MMBP, and playback instructions. The Messaging Service Client may use metadata provided in a container message for playback selection. Such metadata may for example include codec capability information, language codes, accessibility information and other information for the selection of alternative parts in the MMBP.

The client then provides the tracks for playback to media decoders, and controls those for playback. The rendering output may be handed back to the Messaging Service client for inband rendering or may be rendered directly.

Figure 4.3-1 MMBP Player Model

Beyond the MMBP formats, this specification also defines capabilities of 3GPP-based MMBP players.

Editor’s Note: Do we need profiles?

## 4.4	Generic MMBP Data Model

Editor’s Note: formalize the data model – stage 2 level

## 4.5	Media Capabilities and Profiles

Editor’s Note: Provide a summary of media capabilities and profiles. Formulate what the profiles are about

# 5	Messaging Media Profiles

## 5.1	Introduction

In order to guarantee a minimum support and compatibility between messaging capable terminals, MMBP Generators and MMBP Players in Ues supporting specific media types shall comply with the following selection of media formats.

Media Types supported in this specification are provided in Table 5.1-1.

Table 5.1-1 Media Types in TS 26.143

| Media Type in the present TS | Corresponding media type in RFC 2046 [x31] | Supported Media subtypes | Definition |
| --- | --- | --- | --- |
| Text | text | text/plain |  |
| Speech | audio | audio/mp4 |  |
| Audio | audio | audio/mp4 |  |
| Image | Image | image/jpg image/gif image/png [MIAF subtype TBA] |  |
| Video | video | video/mp4 |  |
| 3d scenes and assets | model | model/gltf-binary model/gltf+json |  |
| Media synchronization and presentation format | application | ??? |  |

## 5.2	Media formats

Editor’s note: from S4-231213

[Entry Point formats may be

MP4 media file with an appropriate brand to be defined, e.g. ISO8

An HTML-5 page

A glTF document (see more details in second contribution)

Packaging of media assets may be done in one of the following ways

All assets included in MP4 container

Using Multipart MIME

Recommendations:

Define 3 different entry points

Define packaging in MP4 container and as Multipart MIME

Timed Media Assets need to be encapsulated into a proper container format

MP4 container is used by default

Any other File formats or Media Synchronization and presentation formats are to be ignored.]

## 5.3	Text

Plain text. Any character encoding (charset) that contains a subset of the logical characters in Unicode [2] shall be used (e.g. US-ASCII [3], ISO-8859-1 [4], UTF-8 [5], Shift_JIS, etc.).

Unrecognized subtypes of “text” shall be treated as subtype “plain” as long as the MIME implementation knows how to handle the charset. Any other unrecognized subtype and unrecognized charset shall be treated as “application/octet – stream”.

Interoperability with SMS/MMS text type is according to [x34].

Editor’s note: add TTML or webVTT [TBD].

## 5.4	Image

If still images are supported, ISO/IEC JPEG [8] together with JFIF [x35] shall be supported. The support for ISO/IEC JPEG only apply to the following two modes:

-	mandatory: baseline DCT, non-differential, Huffman coding, as defined in table B.1, symbol ‘SOF0’ in [x36];

-	optional: progressive DCT, non-differential, Huffman coding, as defined in table B.1, symbol ‘SOF2’ [x36].

For JPEG baseline DCT, EXIF compressed image file format should also be supported, as defined in [x37].

If still images are supported, MIAF [HEVC 8 or 10 bits (TBD) Basic Profile or AVC Basic Profile (TBD)] [x41] should be supported.

If bitmap graphics is supported, the following bitmap graphics formats should be supported:

-	GIF87a [x38];

-	GIF89a, [x39];

-	PNG, [x40].

Editor’s note: above is aligned with MMS. In addition from S4-231213:

[It is recommended to support a common JPEG format, likely SOF0 and SOF2 (done).

It is recommended to recommend MIAF Basic Profile (added). HEVC Basic Profile or AVC Basic Profile is TBD

Additional HEIC profiles should be study as part for FS_HEVC_Profiles.]

## 5.5	Speech and Audio

Editor’s note: proposed playback requirements in alignment with TS 26.511 (from S4-231213):

If the Messaging Client supports the reception of Audio or Speech, then the following shall be supported:

-	the EVS playback requirements as defined in 3GPP TS 26.117 [x16] clause 7.4.2.4.

If the Messaging Client supports the reception of Speech, then the following shall be supported:

-	the AMR-WB playback requirements as defined in 3GPP TS 26.117 [x16] clause 7.3.2.4.

If the Messaging Client supports the reception of Speech, then the following shall be supported:

-	the AMR playback requirements as defined in 3GPP TS 26.117 [x16] clause 7.2.2.4.

If the Messaging Client supports the reception of Audio, then the following should be supported:

-	the xHE-AAC stereo playback requirements as defined in 3GPP TS 26.117 [4] clause 7.8.4.

If the Messaging Client supports the reception of Audio, then the following shall be supported:

-	the eAAC+ stereo playback requirements as defined in 3GPP TS 26.117 [4] clause 7.6.2.4.

Editor’s note: proposed encoding requirements:

If the Messaging Client supports the generation of Audio or Speech messages, then the following shall be supported:

-	the EVS Content Generation Requirements as defined in 3GPP TS 26.117 [x16] clause 7.4.2.5.

Editor’s note: except CMAF aspects

If the Messaging Client supports the generation of Speech messages, then the following [may] be supported:

-	the AMR-WB Content Generation Requirements as defined in 3GPP TS 26.117 [x16] clause 7.3.2.5.

Editor’s note: except CMAF aspects.

Editor’s note: The specification is not required to ensure backwards compatibility with MMS.

If the Messaging Client supports the generation of Speech messages, then the following [may] be supported:

-	the AMR Content Generation Requirements as defined in 3GPP TS 26.117 [x16] clause 7.2.2.5.

Editor’s note: except CMAF aspects.

Editor’s note: The specification is not required to ensure backwards compatibility with MMS.

If the Messaging Client supports the generation of Audio messages, then the following should be supported:

-	the xHE-AAC stereo Content Generation Requirements as defined in 3GPP TS 26.117 [x16] clause 7.6.2.5.

Editor’s note: except CMAF aspects

If the Messaging Client supports the generation of Audio messages, then the following shall be supported:

-	the eAAC+ stereo Content Generation Requirements as defined in 3GPP TS 26.117 [x16] clause 7.8.5.

Editor’s note: except CMAF aspects

[When generating a Speech message, the Messaging Client [MMBP Generator] shall at least include an EVS Content together with potential alternate(s).

When generating an Audio message, the Messaging Client [MMBP Generator] shall at least include an eAAC+ stereo Content together with potential alternate(s).]

## 5.6	Video

Editor’s note:

16:9 and 9:16 image formats are needed. Resolutions should be aligned with the considerations above, also in terms for video signal characteristics.

[If the Messaging Client supports the reception of video, then the following applies:

-	the AVC-HD playback requirements as defined in clause 4.2.1.3.1.4 of [x15] shall be supported.

-	the HEVC-HD playback requirements as defined in clause 4.2.2.3.1.4 of [x15] should be supported.

If the Messaging Client supports the reception of video and HD-HDR Capabilities, then the following applies:

-	the AVC-FullHD playback requirements as defined in clause 4.2.1.3.2.4 of [x15] shall be supported.

-	the HEVC-FullHD playback requirements as defined in clause 4.2.2.3.2.4 of [x15] shall be supported.

-	the HEVC-UHD playback requirements as defined in clause 4.2.2.3.3.4 of [x15] may be supported.]

Editor’s note: do we want to include stereoscopic as currently supported in MMS ?

Editor’s note: Based on the above considerations, video is preferably aligned with TS 26.511.

## 5.7	Subtitle

If timed text is supported, Messaging clients shall support [35] with 3GP files using Basic profile [33].

Editor’s note: proposal to support IMSC1.1. Text Track in reference to TS 26.511.

[If the Messaging Client supports the reception of timed text and subtitles, then the following should be supported:

-	the IMSC1.1 text track playback requirements as defined in clause 4.5.1.5. of TS 26.511 []]

## 5.8	3D scenes and assets

If 3D scenes and assets are supported in multimedia messaging services, the Messaging clients shall support the following:

- glTF 2.0 scenes as specified in [x13].

- The GLB binary format of the glTF 2.0 scene.

It is recommended that all components of the 3D scene be encapsulated in a GLB file or present as MIME parts of the multi-part MIME message. If the 3D asset is the main component of the message, then the first part of the message should be the glTF document or GLB file.

For support of Augmented Reality (AR), the MPEG_anchor extension, EXT_lights_image_based and MPEG_lights_texture_based extensions as defined in [x14] shall be supported.

## 5.7	Media synchronization and presentation format

Editor’s note: HTML5 (3GPP TS 26.307 Presentation layer for 3GPP services).

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 08/23 | SA4#125 | S4-231444 |  |  |  | Initial draft | 0.1.0 |
| 11/23 | SA4#126 | S4-231670 |  |  |  | New draft: S4-231993 (alignment from 26.140), S4-231958, S4-231871 | 0.2.0 |