-3GPP TSG-SA WG4 Meeting #127	S4-231781

# Meeting, 13 – 17 November 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.565 | CR | pCR | rev |  | Current version: | 0.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [SR_MSE] Split-rendering session relocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | SR_MSE |  |  |  |  |  | Date: |  |  | 07/11/2023 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Split rendering relocation was proposed in S4-231781 during SA4#126 in order to address scenarios where a new split-rendering server is provisioned with the split-rendering session being transferred to it. One of the WID objectives states as below: - define edge requirements, such as the EAS profiles, as well as edge discovery and relocation configurations appropriate for split rendering In this context, we have proposed a split rendering relocation procedure which addresses the above objective. The proposed procedure also falls under “clause 6.4.4.4 EASRelocationTolerance enumeration” of TS 26.512, which satisfies “The EASERelocationTolerance enumeration” as specified in table 6.4.4.4-1. Furthermore, as suggested during S4-231781 presentation, the proposed relocation procedure is moved to a dedicated Annex. The proposed mechanism is agnostic to the provisioning and context relocation mechanism used and is providing call-flow to ensure the transition is done in a seamless manner for the user. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Changes in clause 7.2.2.4.2 - easRelocationRequirements can now support relocation for services requiring it. The default configuration being to not support relocation. - Addition of a new Annex defining relocation procedure for split-rendering - Monitoring and relocation triggering - Call flows and procedures to set up the new SRS, to transfer the split-rendering configuration and perform a seamless transition - Split-rendering use-plane |  |  |  |  |  |  |  |  |
| tr |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Reduced QoE for services leveraging split-rendering and service interruption. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.2.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | N/A |  |  |  |  |  |  |  |  |

===== CHANGE #1 =====

##### 7.2.2.4.2	Guidelines on Edge Resource Configuration

A split rendering application may use the procedures defined in TS26.512 clause 7.10 to define an edge resource configuration to be used for split-rendering  session. In this case:

-	The eligibilityCriteria shall be present and shall have appRequest set to true.

-	The easRequirements shall indicate “SR” as the easType and shall include “3gpp-sr” among the easFeatures. The serviceKpi shall be present and indicate the SRS processing and networking capabilities and requirements.

-	If the application does not tolerate relocation, no relocation procedure is expected for the service, and the easRelocationRequirements shall indicate “RELOCATION_INTOLERANT” in the tolerance field as per clause 6.4.4.4 of TS 26.512 [7] .

- If the application  may tolerate relocation, then the easRelocationRequirements shall be set to “RELOCATION_TOLERANT” as per clause 6.4.4.4 of TS 26.512 [7] and the procedures described in Annex D should be applied.

===== CHANGE #2 =====

Annex D (normative):
Split Rendering Relocation Profile

## D.1 	Introduction

This Annex defines split rendering profile that should be used when implementing relocation procedure.

- Clause D.2 defines call flow and procedures to monitor and trigger a relocation procedure.

- Clause D.3 defines call flow and procedures to set up a new SRS, to which the session will be relocated.

- Clause D.4 defines call flow and procedures to seamlessly transition from the current to the new SRS.

- Clause D.5 defines signalling protocol extension assisting the relocation procedure.

## D.2 	Session monitoring and relocation triggering

Figure D.2-1 describes a call-flow for monitoring of existing split-rendering session(s).

Figure D.2-1: High-level call flow for split-rendering monitoring

This call flow applies once a session has already been created, as described in step 0 (clause 5.2.1.1). The following steps will aim at monitoring, detect, and relocate the session when needed.

The steps are:

- The RTC-AF constantly (or regularly) monitors split-rendering media session degradations. When a recurrent degradation is observed, over a time threshold, the RTC-AF tells the AP that the current session is threatened, and that the session needs to be relocated.

Then, the step 1 of clause 5.2.1.1 is re-used to provision and configure a new split-rendering session, the same, or higher configuration is expected to be used, but the final configuration is left to the AP:

- In this optional step, the AP requests and sets up the edge server(s) used for the split-rendering as described in TS 26.506 clauses 6.1 or 6.2. The Application provider may use any other method to provision edge servers configurations or leave it to the MNO to set up appropriate edge servers to run the split-rendering process.

- The Application Provider provisions the split-rendering session using RTC-1 and RTC-5 and sets up the split-rendering session as defined in clause 5.2.3.2. If the edge servers were provisioned in step 2, the edge servers ids are provided in this session to employ them for split-rendering.

- The rendering session relocation is achieved, as described in clause 5.2.3.3.

- Once the relocation is properly done, the RTC-AF tells the AP that the SRS can be un-provisioned.

## D.3 	Setting up a new SRS

Figure D.3-1 demonstrates a call flow for setting up the new split rendering by the client.

Figure D.3-1: High-level call flow for setting up split-rendering relocation

Steps:

- Based on relocation trigger achieved by the current SRS monitoring, the AF tells the Application provider that a relocation to a new server shall be done. If not already done, a new SRS server is provisioned. An update of the different split rendering session configurations is done to changed them to “relocation” mode and populate relocation parameters.

- The provisioning of a new split rendering server is announced to the Application as part of the Service Access Information.

- The Application requests the client media functions from the SRC to setup the new split-rendering session.

- The SRC requests the current SRS to transfer its configuration to the newly provisioned SRS.

Following steps are similar as what was documented in clause 5.2.1.2, except that the rendered frames shall not be sent back to the SRC, this transition will be addressed during the relocation.

- The SRS starts the split rendering process.

- The SRC establishes the WebRTC session.

8. The SRC informs the application that the split-rendering on edge is running.

9. The SRC sends uplink metadata, such as pose and action information.

## D.4 	Session transitioning from the current to the new SRS

The split rendering relocation operation can be described by the as depicted in the call flow in the Figure D.4-1.

Figure D.4-1 High-level call flow for split rendering relocation

The first steps 1-3 consists in setting and establish communication with the new SRS, similarly to what is described in 5.2.2 :

- The Presentation Engine discovers the split rendering server and sets up a connection to it. It provides information about its rendering capabilities and the XR runtime configuration, e.g the OpenXR configuration may be used for this purpose. This is an optional step as an exact same SRS may have been provisioned.

- In response, the split rendering server creates a description of the split rendering output and the input it expects to receive from the UE. This is an optional step as an exact same SRS may have been provisioned.

- The Presentation Engine requests the buffer streams from the MAF, which in turn establishes a connection to the split rendering server to stream pose and retrieve split rendering buffers.

Then, the relocation process is starting:

- The RTC-AF negotiates the session relocation with the two SRS, the result of this negotiation is a timestamp from which the MAF will start receiving buffer frame(s) from the new SRS and from which the current SRS will stop sending buffer frame(s).

- The RTC-AF tells the MAF and the SRS to start the relocation. The AF propagates time from which the relocation will be triggered to the MAF and the SRS.

- The rendering will then temporarily switch from the Rendering Loop defined in clause 5.2.2 to the Rendering Relocation Loop, the steps are:The Source Manager retrieves pose and user input from the XR runtime.

- The Source Manager shares the pose predictions and user input actions with the two split rendering server.

- The split rendering server uses that information to render the frame.

- The rendered frame is encoded and streamed down to the MAF, the source of the frame changes from the existing SRS to the new SRS when the transition timestamp is reached.

- The raw buffer frames are passed to the XR runtime to be displayed

- The XR runtime achieves the final composition and rendering

- Once the transition has been done in the rendering loop, the rendering goes back to the loop defined in clause 5.2.2 with the new SRS as principal rendering server. The relocation ends by closing the established connections, the final step is:The scene manager terminates the connection with the existing SRS.

## D.4 	Split-Rendering User Plane

### D.4.1	Split Rendering Relocation Signalling Protocols

For Split Rendering transfer, the following application-specific messages shall be supported:

- The rendering transfer description message carries the description of the session transfer to be done from the current SRS to a new one. It shall be identified by the type “urn:3gpp:sr-mse:sr-transfer” and the object shall be formatted according to clause 8.4.3. The rendering description message provides the semantics of the media that is delivered over WebRTC from the SRS to SRC.

The SWAP message exchange for the negotiation of a split rendering session relocation is depicted by the following call flow diagram:

Pre-requisites:

-	The SRC has discovered the identifier of the new SRS that it will use for the transfer of the split rendering session.

-	The SRC has retrieved the address of the SWAP server as part of the configuration.

The steps are as follows:

- The SRC sends the configuration transfer message as an application-specific SWAP message to the SWAP server. It provides the identifier of both the current SRS and the new SRS.

- The SWAP server uses the provided matching criteria to locate the two SRS.

- The SWAP server forwards the configuration transfer message to the current SRS.

- The SWAP server confirms the successful forwarding of the message to the SRC.

- The current SRS processes the SR transfer configuration message. It may prepare the configuration message and runtime information to be sent to the new SRS.

- The SRS sends the configuration message to the SWAP server. It provides the identifier of the target relocation SRS.

- The SWAP server forwards the configuration message to the new SRS.

- The SWAP server confirms the successful forwarding of the message to the new SRS.

- The new SRS processes the SR configuration message. It may for instance verify application and resource availablity, launch the application, configure its rendering, and create a rendering description.

- The new SRS sends the rendering description message as an application-specific SWAP message to the SWAP server.

- The SWAP server forwards the message to the SRC.

- The SWAP server acknowledges the successful forwarding of the message to the new SRS.

- The SRC processes the rendering description and identifies the required data channel and media sessions.

- SRC sends a connect message with the SDP offer to the new SRS. The offer reflects the negotiated media and data channel streams.

- The SWAP server acknowledges the forwarding of the message to the new SRS

- The new SRS replies with an accept message that includes the SDP answer. The SDP answer reflects the information that was provided in the split rendering description.

- The SWAP server acknowledges the forwarding of the message to the SRC

### D.4.2	Split rendering relocation format

The session transfer configuration information shall be in JSON format. It shall have the following format:

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| sourceId | Object | 1..1 | The identifiers of the currently running SRS from which the transfer will be done. |
| destId | Object | 1..1 | The identifiers of the newly provisioned SRS, the target of the relocation. |
| transferConfiguration | Object | 1..1 | The expected transfer configuration, including time window for relocation. |

===== END OF CHANGES =====