| 3GPP TS 26.517 V17.4.0 (2023-09) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; 5G Multicast-Broadcast User Services; Protocols and Formats (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	8

3.1	Terms	8

3.2	Symbols	8

3.3	Abbreviations	8

4	System overview	8

5	MBS User Service Announcement	8

5.0	Overview	8

5.1	MBS User Service Description data model	9

5.1.1	General	9

5.2	Semantics	11

5.2.1	General	11

5.2.2	MBS User Service Bundle Description metadata unit	11

5.2.3	MBS User Service Description metadata unit	11

5.2.4	MBS Distribution Session Description metadata unit	11

5.2.5	Session Description metadata unit	12

5.2.6	MBS Application Service Description metadata unit	12

5.2.7	MBS Schedule Description metadata unit	13

5.2.8	MBS Object Repair Parameters metadata unit	15

5.3	Delivery of User Service Description	16

5.3.1	General	16

5.3.2	Delivery of User Service Description in object carousel	16

5.3.3	Delivery of User Service Description via unicast PDU Session	16

6	Object Distribution Method	16

6.1	General	16

6.1.1	Overview	16

6.1.2	Object manifest	17

6.2	Usage of FLUTE for Object Distribution Method	17

6.2.1	General	17

6.2.2	Session Description metadata unit	18

6.2.2.1	General	18

6.2.2.2	MBS service type of MBS Session	18

6.2.2.3	SDP examples for FLUTE Session	19

6.2.3	Operating modes for Object Distribution Method	20

6.2.3.1	Introduction	20

6.2.3.2	Single object operating mode	20

6.2.3.3	Object collection operating mode	20

6.2.3.4	Object carousel operating mode	20

6.2.3.5	Segment streaming operating mode	20

7	Packet Distribution Method	21

7.1	General	21

7.2	Re-using MBMS Delivery Method as Packet Distribution Method	22

7.2.1	General	22

7.2.3	Session Description	22

7.2.3.1	General	22

7.2.3.2	SDP examples for Packet Distribution Method	23

8	General aspects of APIs for MBS User Services	24

8.1	HTTP resource URIs and paths	24

8.2	Usage of HTTP	24

8.2.1	HTTP protocol version	24

8.2.1.1	General	24

8.2.1.2	MBSF	24

8.2.1.3	MBSTF	25

8.2.1.4	MBS AF	25

8.2.1.5	MBS AS	25

8.2.2	HTTP message bodies for API resources	25

8.2.3	Usage of HTTP headers	26

8.2.3.1	General	26

8.2.3.2	User Agent identification	26

8.2.3.2.1	General	26

8.2.3.2.2	MBSF identification	26

8.2.3.2.3	MBSTF identification	26

8.2.3.2.4	MBSF Client identification	26

8.2.3.2.5	MBSTF Client identification	26

8.2.3.3	Server identification	27

8.2.3.3.1	General	27

8.2.3.3.2	MBSF identification	27

8.2.3.3.3	MBSTF identification	27

8.2.3.3.4	MBS AF identification	27

8.2.3.3.5	MBS AS identification	27

8.2.3.4	Support for conditional HTTP GET requests	27

8.2.3.5	Support for conditional HTTP POST, PUT, PATCH and DELETE requests	28

8.3	HTTP response codes	28

9	MBS AF APIs	28

9.1	General	28

9.2	User Service Description retrieval API	28

9.2.1	General	28

9.2.2	Resource structure	29

9.2.3	Data model	29

9.2.3.1	UserServiceDescriptions resource type	29

9.2.3.2	UserServiceDescription resource type	29

Annex A (normative): Syntax for Service Announcement	30

A.1	XML-based representation	30

A.1.1	MBS User Service Description schema	30

A.1.2	Object Repair Parameters schema	32

A.1.3	Schedule Description schema	33

A.2	JSON-based representation	34

A.2.1	MBS User Service Announcement schema	34

Annex B (informative): Service Announcement examples	39

B.1	XML-based representation	39

B.2	JSON-based representation	39

Annex C (normative): Controlled vocabulary of conformance profiles	41

Annex D (normative): Syntax for object manifest	42

D.1	Object manifest schema	42

Annex E (normative): IANA registration	43

E.1	General	43

E.2	Registration of MIME media type "application/mbs-user-service-description+json"	43

E.2.1	General	43

E.2.2	Profiles parameter	43

Annex F (informative): Change history	45

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document defines protocols and formats for User Services as defined in TS 26.502 [6] and conveyed using the 5G multicast–broadcast capabilities of the 5G System defined in TS 23.501 [2], TS 23.502 [3] and TS 23.247 [5].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[3]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[4]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[5]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services; Stage 2".

[6]	3GPP TS 26.502: "5G multicast–broadcast services; User Service architecture".

[7]	3GPP TS 26.346: “MBMS; Protocols and Codecs".

[8]	IETF RFC 8866: "Session Description Protocol".

[9]	W3C: "XML Schema Part 2: Datatypes".

[10]	3GPP TS 23.003: "Numbering, addressing and identification".

[11]	3GPP TS 24.008: "Mobile radio interface Layer 3 specification; Core network protocols; Stage 3".

[12]	IETF RFC 3926: "FLUTE - File Delivery over Unidirectional Transport".

[13]	IETF RFC 2616: "Hypertext Transfer Protocol -- HTTP/1.1".

[14]	OpenAPI: "OpenAPI 3.0.0 Specification", https://github.com/OAI/OpenAPI-Specification/blob/master/versions/3.0.0.md.

[15]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[16]	3GPP TS 29.501: "5G System: Principles and Guidelines for Services Definition; Stage 3".

[17]	3GPP TS 29.580: "5G System; Multicast/Broadcast Service Function services; Stage 3".

[18]	3GPP TS 29.581: "5G System; Multicast/Broadcast Service transport services; Stage 3".

[19]	IETF RFC 9110: "HTTP Semantics", June 2022.

[20]	IETF RFC 9111: "HTTP Caching", June 2022.

[21]	IETF RFC 9112: "HTTP/1.1", June 2022.

[22]	IETF RFC 9113: "HTTP/2", June 2022.

[23]	Reserved for future use.

[24]	IETF RFC 8446: "The Transport Layer Security (TLS) Protocol Version 1.3", August 2018.

[25]	Open Mobile Alliance: "OMNA BCAST Service Class Registry", https://technical.openmobilealliance.org/OMNA/bcast/bcast-service-class-registry.html.

[26]	IETF RFC 3629: "UTF-8, a transformation format of ISO 10646".

[27]	IETF RFC 8141: "Uniform Resource Names (URNs)".

[28]	ISO 639-2: "Codes for the representation of names of languages – Part 2: Alpha-3 code".

[29]	IETF RFC 6381: "The 'Codecs' and 'Profiles' Parameters for "Bucket" Media Types".

[30]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[31]	3GPP TS 26.512: "5G Media Streaming (5GMS); Protocols".

[32]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[33]	3GPP TS 33.246: "3G Security; Security of Multimedia Broadcast/Multicast Service (MBMS)".

[34]	IETF RFC 3986: "Uniform Resource Identifier (URI): Generic Syntax".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1], TS 23.501 [2], TS 23.502 [3], TS 23.247 [5], TS 26.502 [6] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Add new definitions

MBS User Service Description document

## 3.2	Symbols

Void.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1], TS 23.501 [2], TS 23.502 [3], TS 23.247 [4] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

CMAF	Common Media Application Format

FLUTE	File Delivery over Unidirectional Transport

MBS	Multicast–Broadcast Services

MB-SMF	Multicast–Broadcast Session Management Function

MB-UPF	Multicast–Broadcast User Plane Function

MBSF	Multicast–Broadcast Service Function

MBSTF	Multicast–Broadcast Service Transport Function

PCF	Policy and Charging Function

NEF	Network Exposure Function

SDP	Session Description Protocol

TMGI	Temporary Mobile Group Identity

UE	User Equipment

UML	Unified Markup Language

XML	eXtensible Markup Language

## 3.4	Syntax documentation conventions

The conventions from TS 29.501 [16] apply to this document with the following extensions for structured data type definitions as defined in clause 5.2.4.1 of TS 29.501 [16]:

1.	Data types are annotated in bold face.

2.	Properties of data types are noted in codestyle.

# 4	System overview

The present document defines protocols and formats for MBS User Services as defined in TS 26.502 [6] and conveyed using the 5G Multicast–Broadcast capabilities of the 5G System defined in TS 23.501 [2], TS 23.502 [3] and TS 23.247 [5].

MBS User Services enable high-level applications to make use of the low-level features of the MBS System. An MBS User Service is provided by the MBSF and MBSTF working in combination to support configuration option 2 and configuration option 3 defined in annex A of TS 23.247 [5]. The MBS User Services architecture is defined in clause 4.2.2 of TS 26.502 [6] and shows the MBS-related entities involved in providing MBS User Services delivery and control.

The MBSF and MBSTF offer service layer functionality for sending MBS data via MBS Sessions. The MBSF offers control plane functionality while the MBSTF offers user plane functionality. The MBSTF acts as a User Plane anchor when it sources IP multicast traffic. Clause 4.3.1 of TS 26.502 [6] defines the set of functional entities involved in supporting MBS User Services, including client functions in the UE.

# 5	Announcement of MBS User Services

## 5.0	Overview

MBS User Service Announcement is needed in order to advertise MBS User Services in advance of, and potentially during, the MBS User Service Sessions described. MBS User Service Announcement (as defined in clauses 4.5.7 and 4.5.8 of TS 26.502 [3]) is provided by means of an MBS User Service Description, the syntax of which is defined in this clause.

The MBS User Service Announcement plays an equivalent role to the MBMS User Service announcement as defined in TS 26.346 [7], but it is simplified and extended for the needs of Multicast–Broadcast Services.

## 5.1	MBS User Service Description data model

### 5.1.1	General

An MBS User Service Description is described by a set of metadata documents that are delivered as described in clause 4.3.2 of TS 26.502 [3]. The data model defined in this clause subdivides the parameters defined in [3] and groups them into a set of metadata documents.

The metadata consists of:

-	An MBS User Service Bundle Description document (see clause 5.2.2) describing a bundle of one or more MBS User Services, and containing one or more:

-	MBS User Service Description object (see clause 5.2.3) describing an MBS User Service Session that is associated with:

-	One or more MBS Distribution Session Description objects (see clause 5.2.4), each of which references a Session Description document [8] that may be packaged with the MBS User Service Bundle Description, and each of which may optionally reference an Object Repair Parameters document (see clause 5.2.7) describing the object repair parameters for the MBS Distribution Session.

-	Zero or more MBS Application Service Description objects (see clause 5.2.5), each of which references an Application Service Entry Point document that may be packaged with the MBS User Service Bundle Description. Additional resources referenced by the entry point document may also be packaged with the MBS User Service Bundle Description.

-	Zero or one MBS Schedule Description objects (see clause 5.2.6) advertising the delivery schedule for the MBS User Service Session.

Figure 5.1-1 illustrates the relationships between these metadata entities using UML for a single MBS User Service Bundle.

NOTE:	“N” means any number in each instance.

Figure 5.1-1: User Service Data Model simple description

An MBS User Service Bundle Description document shall contain one or more MBS User Service Description objects, each of which describes a single MBS User Service Session within the MBS User Service Bundle.

Each MBS User Service Description object shall include at least one MBS Distribution Service Description object describing the set of MBS Distribution Sessions currently associated with the MBS User Service Session.

-	The MBS Distribution Session Description object shall refer to one Session Description document.

-	Each MBS Distribution Session Description object may include an Object Repair Parameters object.

Each MBS User Service Description object may include zero or more MBS Application Service Description objects, each one referencing an Application Service Entry Point document (e.g. a DASH MPD, HLS Master Playlist or HTML document) which describes the root of the Application Service associated with this MBS User Service. When multiple Application Service Entry Point documents are referenced, an MBS Client shall select only one on the basis of a distinct MIME content type indicated in the Application Service Description object.

Each MBS User Service Description object may include an MBS Schedule Description object. If included, the UE can expect to receive MBS User Service data during the time periods described in the Schedule Description object.

In the case of the Object Distribution Method, the Schedule Description object may include an object transmission schedule for individual transmission objects associated with the MBS User Service Session. The UE may select which objects to receive based on the object transmission schedule information published in the Schedule Description.

Editor's Note:	MULTIPART MIME NEEDS TO BE DEFINED

## 5.1A	Encoding

The MBS User Service Description shall be a JSON instance document formatted according to the schema in clause A.2 with the following constraints:

-	The MIME type of the MBS User Service Description document is specified in clause D.2.

-	The encoding of the MBS User Service Description document shall be UTF-8 as defined in IETF RFC 3629 [26].

-	The extension of the schemas specified in clause A.2, in particular the addition of information elements, is reserved to 3GPP.

-	Additional rules for schema extension are for future study.

## 5.2	Semantics

### 5.2.1	General

The following description in this clause presumes a JSON encoding of the information comprising the MBS User Service Announcement as specified in clause 5.1A.

The data types in table 5.2.1-1 from other 3GPP specifications are reused in the remainder of the present document.

Table 5.2.1-1: Externally defined data types
used by MBS User Service Announcement

| Data type | Comments | Reference |
| --- | --- | --- |
| Uri | A Uniform Resource Locator | TS 29.571 [30] |
| DateTime | A date–time value. |  |
| MbsServiceArea | An MBS Service Area. |  |
| MbsFsaId | An MBS Frequency Selection Area identifier. |  |
| DurationSec | A time duration expressed in seconds. |  |
| AbsoluteUrl | An absolute URL | TS 29 512 [31] |

The data types in table 5.2.1-2 are defined in the present document.

Table 5.2.1-2: MBS User Service Description data types
 defined in the present document

| Data type | Clause |
| --- | --- |
| UserServiceDescription | 5.2.3 |
| Availability‌Information | 5.2.3 |
| Distribution‌Session‌Description | 5.2.4 |
| App‌Service‌Description | 5.2.6 |
| Schedule‌Description | 5.2.7 |

### 5.2.2	User Service Description document

The MBS User Service Description document conveys one or more MBS User Service Description objects. Table 5.2.2-1 provides the detailed semantics for the UserServiceDescriptions data type that conveys these objects.

Table 5.2.2-1: Semantics of UserServiceDescriptions data type

| Property name | Data Type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| version | integer | O | 0..1 | Provides the version number of the MBS User Service Description document. Any two documents that are different shall have different version numbers. A higher number overwrites a lower version number. Minimum value: 1. If not present, the document version number is unknown. |
| userServiceDescriptions | array(User‌Service‌Description) | M | 1..N | A set of one or more MBS User Service Description objects (see clause 5.2.3). |

### 5.2.3	User Service Description data type

The UserServiceDescription data type carries information about a single MBS User Service corresponding to an MBS User Service Announcement as defined in clause 4.5.7 of TS 26.502 [6]. Table 5.2.3-1 provides the detailed semantics for this data type.

Table 5.2.3-1: Semantics of UserServiceDescription data type

| Property name |  | Data Type | P | Cardinality | Description |
| --- | --- | --- | --- | --- | --- |
| serviceIds |  | array(Uri) | M | 1..N | The set of External service identifiers for the MBS User Service as defined in table 4.5.7-1 of TS 26.502 [6]. Each value is expressed as a URI. |
| class |  | Uri | M | 1 | The Service class of the MBS User Service as defined in table 4.5.7-1 of TS 26.502 [6]. The value is expressed as a fully-qualified term identifier URI. |
| names |  | array(object) | O | 1..N | The Service names of the MBS User Service as defined in table 4.5.7-1 of TS 26.502 [6]. Each service name is differentiated by a language as defined in the lang attribute. |
|  | name | string | M | 1 | One of the Service names. |
|  | lang | string | M | 1 | Language of the service name encoded using an ISO "alpha-3" language code according to ISO 639-2 [28]. |
| descriptions |  | array(object) | O | 1..N | The Service descriptions of the MBS User Service as defined in table 4.5.7-1 of TS 26.502 [6]. |
|  | description | string | M | 1 | One of the Service descriptions. |
|  | lang | string | M | 1 | Language of the service description encoded using an ISO "alpha-3" language code according to ISO 639-2 [28]. |
| serviceLanguage |  | string | O | 0..1 | The Main service language of the MBS User Service as defined in table 4.5.7-1 of TS 26.502 [6], encoded using an ISO "alpha-3" language code according to ISO 639-2 [28]. |
| sessionStart |  | DateTime | O | 0..1 | The current Start date-time of the MBS User Service as defined in table 4.5.7-1 of TS 26.502 [6]. |
| sessionEnd |  | DateTime | O | 0..1 | The current End date-time of the MBS User Service as defined in table 4.5.7-1 of TS 26.502 [6]. |
| distribution‌Session‌Descriptions |  | array(Distribution‌Session‌Description) | M | 1..N | One or more MBS Distribution Session Descriptions for the MBS User Service (see clause 5.2.4). |
| schedule‌Description |  | array(Service‌Schedule) | O | 1..N | An array of service schedules for the MBS User Service (see clause 5.2.7). |
| appService‌Description |  | App‌Service‌Description | O | 0..1 | An Application Service Description for the MBS User Service (see clause 5.2.6). |

### 5.2.4	Distribution Session Description data type

---

The userServiceDescription element may include an availabilityInfo child element providing additional information pertaining to the availability of the MBS Distribution Session within the 5G Network. If present, the availabilityInfo element shall include one or more infoBinding child elements. The infoBinding element shall contain the child elements serviceArea, mbsFSAId and radiofrequency:

-	The serviceArea element declares the one or more service areas in which the MBS Session corresponding to this MBS Distribution Session is currently available.

-	In the case of a broadcast MBS Session corresponding to this MBS Distribution Session, the mbsFSAId element identifies a preconfigured area within which, and in proximity to, the cell(s) announce the MBS FSA ID and its associated frequency.

NOTE:	This is used to guide frequency selection by the UE for a broadcast MBS Session.

-	The radioFrequency element indicates the one or more radio frequencies in the NG-RAN downlink which transmit the MBS Session corresponding to this MBS Distribution Session in the service area(s) identified by the serviceArea element.

The DistributionSessionDescription data type describes one MBS Distribution Session associated with an MBS User Service and carries the MBS Distribution Session Parameters as defined in clause 4.5.8 of TS 26.502 [6]. Table 5.2.4-1 provides the detailed semantics of this data type.

Table 5.2.4-1: Semantics of DistributionSessionDescription data type

| Property name | Data Type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| distribution‌Method | Distribution‌Method | M | 1 | The distribution method used for this MBS Distribution Session. For details, refer to table 5.2.4-2. |
| conformance‌Profiles | array(Uri) | O | 1..N | A list of profiles indicating the set of features that this MBS Distribution Session conforms to and which the MBS Client needs to support in order to fully decode the MBS Distribution Session. Each member of the array shall be a fully-qualified term identifier URI from the controlled vocabulary defined in annex C. The term identifiers in the controlled vocabulary shall conform to either the pro-simple or pro-fancy productions specified in section 4.5 of IETF RFC 6381 [29], without the enclosing DQUOTE characters, i.e. only the unencodedv or encodedv productions respectively. Profile identifiers defined in the present document are URNs and shall conform to IETF RFC 8141 [27]. Externally defined profiles may use profile identifiers that are URNs or URLs. When a URL is used, it should also contain a month-date in the form YYYY-MM; the assignment of the URL must have been authorized by the owner of the domain name in that URL on or very close to that date, to avoid problems when domain names change ownership. |
| session‌Description‌Locator | AbsoluteUrl | M | 1 | URL to a Session Description document carrying the Session Description parameters for this MBS Distribution Session as defined in table 4.5.8-1 of TS 26.502 [6]. For details, refer to clause 5.2.5. |
| post‌Session‌Object‌Repair‌Parameters | ObjectRepair‌Parameters | O | 0..1 | Parameters to be used by the MBSTF Client at reference point MBS-4-UC for post-session unicast object repair of this MBS Distribution Session, as defined in table 4.5.8-2 of TS 26.502 [6]. For details, refer to clause 5.2.8. |
| mbs‌App‌Service | array(Application‌Service) | O | 1..N | Equivalent Application Services for this MBS Distribution Session(see Rel-18 of 5GMS Multiple service entry points). For details, refer to clause 5.2.6. |
| unicast‌App‌Services | array(Application‌Service) | O | 1..N | <needs reference> |
| availability‌Infos | array(Availability‌Information) | O | 1..N | Additional information pertaining to the availability of this MBS Distribution Session within the MBS System. For details, refer to clause 5.2.V. |
| security‌Description | Security‌Description | O | 0..1 | The security parameters for this MBS Distribution Session, as defined in table 4.5.8-1 of TS 26.502 [6]. For details, refer to clause 5.2.X. |

Table 5.2.4-2: Semantics of DistributionMethod enumeration

| Enumerated value | Description |
| --- | --- |
| OBJECT | The Object Distribution Method. |
| PACKET | The Packet Distribution Method. |

### 5.2.5	Session Description document

Each Session Description document shall describe one MBS Distribution Session and shall be formatted according to RFC 8866 [8]. The Session Description document may be packaged in the same MBS User Service Bundle as the MBS User Service Description document.

-	The session description for the MBS Object Distribution Method is specified in clause 6.2.3

-	The session description for the MBS Packet Distribution Method is specified in clause 7.2.3.

### 5.2.6	Application Service Description data type

Table 5.2.6-1 provides the detailed semantics for the AppServiceDescription data type.

Table 5.2.6-1: Semantics of AppServiceDescription data type

| Property name | Data Type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| entry‌Point‌Locator | Uri | M | 1 | Reference to an Application Service Entry Point document, for example to a DASH MPD. |
| contentType | string | M | 1 | Format of the Application Service Entry Point document, for example application/dash+xml. |

If the MBS User Service Description contains a reference to an Application Service Entry Point document, then:

1)	At least one MBS Distribution Session Description of type Object Distribution Method shall be present, i.e. the MBS User Service Description shall include at least one DistributionSessionDescription object in the distributionSessionDescriptions property referencing a Session Description Document that describes an Object Distribution Method as defined in clause 7.

2)	When multiple MBS Distribution Session Descriptions of type Object Distribution Method are present, the appServiceDescription property shall define a mapping between the Application Service Entry Point document and the associated MBS Distribution Session.

3)	The MBS Distribution Session described by the Session Description document shall deliver objects that are directly or indirectly referenced by the Application Service Entry Point document.

4)	When the Application Service Entry Point document is a DASH MPD, then all of the following shall hold:

a)	The MBS Distribution Session shall deliver the objects such that the last packet of the delivered object is available to the MBS Client by no later than its availability time as announced in the DASH MPD.

b)	The Content-Location element in the FLUTE File Delivery Table for the delivered object shall match the URL in the DASH MPD.

Editor’s Note: Bullets 4 and 5 should be moved to Clause 6.

5)	If an update to the Application Service Entry Point document is delivered as a FLUTE transmission object then the Content-Location element in the FLUTE File Delivery Table for the delivered object shall match the URL of the referenced Application Service Entry Point document.

In the case of 3GP-DASH formatted content, the appServiceDescription property may refer to a unified media manifest document which describes Representations available for both MBS reception and unicast retrieval, and this shall be used by MBS Clients compliant with this specification. In practical deployments, different subsets of the Representations described by the unified manifest document and referenced by such appServiceDescription may be specified for:

-	Availability via MBS delivery only,

-	Availability via both unicast and MBS delivery,

-	Availability via unicast only, and the Representation is redundant in MBS area coverage, i.e. the usage of these resources does not provide an improved user experience. As an example, this may be a lower bitrate Representation of a media component for which a higher bitrate is available over MBS distribution, and

-	Availability always via unicast, and the Representation is supplementary in MBS area coverage, i.e. even in MBS area coverage these resources provide an improved user experience. As an example, this may be a secondary language that is only accessible over unicast.

All resources that are directly or indirectly referenced in the Application Service Entry Point document of this metadata unit that are expected to be retrieved by HTTP GET shall be delivered by at least one of the MBS Distribution Sessions associated with the MBS User Service Description.

### 5.2.7	Schedule Description data type

Table 5.2.7-1 provides the detailed semantics for the ScheduleDescription data type.

Table 5.2.7-1: Semantics of ServiceSchedule data type

| Property name | Type | P | Cardinlity | Description |
| --- | --- | --- | --- | --- |
| session‌Schedule | Session‌Schedule |  | 1 | Specifies a time after which MBS Client shall seek to update its schedule information by acquiring the latest available Schedule Description instance document. For details, refer to table 5.2.7-3. |
| session‌Override‌Schedule | Session‌Schedule‌Override |  | 0..1 | Indicates either the cancellation of the session occurrence, or schedule override. For details, refer to table 5.2.7-5. |
| objectSchedule | Object‌Schedule |  | 0..1 | The Object distribution schedule of the MBS Distribution as defined in table 4.5.8-2 of TS 26.502 [6]. For details, refer to table 5.2.7-4. |
| serviceId | string |  | 1 | the value defines the service identifier serviceID of the MBS User Service as defined in table 5.2.3.1 to which this schedule applies |
| serviceClass | Uri |  | 1 | the value defines the service class class of the MBS User Service to which this schedule applies, as defined in table 5.2.3.1. |

A Schedule Description instance document describes the distribution schedule of the MBS User Service and the availability of content via unicast delivery in terms of:

-	Start/stop lists,

-	Recurrence information,

-	The service ID or service class to which the schedule may apply,

An MBS User Service containing multiple content components may be carried on a single MBS Distribution Session, or on multiple MBS Distribution Sessions. The MBS Client can expect to receive MBS data during the described time period(s) when at least one of the MBS Distribution Sessions for the MBS User Service is active.

A Schedule Description instance document may also include a schedule of when individual objects are intended to be transmitted as part of an MBS Distribution Session using the Object Distribution Method. The object schedule information is defined in terms of:

The Schedule Description metadata unit is delivered to the MBS Client

prior to the MBS Distribution Session as part of the MBS User Service Announcement along with the Session Description metadata unit and may be updated subsequently along with the parent MBS User Service Description metadata unit.

.

Table 5.2.7-3 provides the detailed semantics for the SessionSchedule object.

Table 5.2.7-3: Semantics of SessionSchedule object

| Property name | Type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| start | DateTime |  | 1 | The start time of this Session schedule. |
| stop | DateTime |  | 1 | The stop time of this Session schedule. |
| reoccurencePattern | string |  | 0..1 |  |
| numberOfTimes | integer |  | 0..1 |  |
| index | integer |  | 0..1 | Starting index of the first session occurrence with the index value increased by one for each session reoccurrence. |
| reoccurenceStopTime | string |  | 0..1 |  |
| fDTInstanceURI | Uri |  | 0..1 |  |

Table 5.2.7-4 provides the detailed semantics for the ObjectSchedule object.

Table 5.2.7-4: Semantics of ObjectSchedule object

| Property name |  |  | Use | Type | Description |
| --- | --- | --- | --- | --- | --- |
| ObjectSchedule |  |  | 0..1 | array | Object schedule. |
|  | objectURI |  | 0..1 | Uri |  |
|  | sessionId |  | 0..1 | string | Identifies the MBS Distribution Session for each object. If not present, an MBS Client shall instead determine the MBS Distribution Session by examining the Session Description metadata unit for the MBS Distribution Session. |
|  | objectEtag |  | 0..1 | string | Represents the object version identifier. If present, the purpose of this entity tag is to enable an MBS Client to determine if an object has changed since a prior reception without having to download the object. |
|  | unicastOnly |  | 0..1 | boolean |  |
|  | deliveryInfo |  | 0..N | object |  |
|  |  | start | 0..1 | DateTime | The start time of this Object schedule. |
|  |  | stop | 0..1 | DateTime | The stop time of this Object schedule. |

The session schedule and object transmission schedule are described in the Schedule Description instance document respectively by the sessionSchedule and objectSchedule objects.

-	The start and stop time of a single sessionSchedule is specified by the start and stop properties.

-	The start and stop time of a single objectSchedule is specified by the start and stop properties as included in the deliveryInfo object.

In both cases the time is specified as the absolute date and UTC time. The duration may be determined by subtracting the start time from the stop time.

The MBS Distribution Session shall be available to the MBS Client during the time interval(s) announced by the session schedule (i.e. scheduleDescription/serviceSchedule/sessionSchedule element of the Schedule Description instance document), for either unicast or MBS reception. In particular, for unicast reception, the Schedule Description is indicative of the time availability for unicast access of an MBS User Service while the TMGI for the MBS Distribution Session is not activated, as well as for unicast fallback reception when the MBS Client is not located in the MBS coverage area for the service.

The MBS Client may activate reception of that MBS Distribution Session only within the sessionSchedule (and the objectSchedule if present) time window.

When an objectSchedule object is present in a serviceSchedule object, then:

-	The MBS Client should not expect that an object described by an objectSchedule will be updated during a time window instance, defined by start and stop properties, within a deliveryInfo object of that objectSchedule.

-	There shall be only one object version (as defined in the File-ETag attribute in the FLUTE File Delivery Table) transmitted in a time window defined by the start and stop keys within a deliveryInfo element for a given objectSchedule element.

-	If objectETag property is not present, the objects transmitted in the time windows from different deliveryInfo elements in an objectSchedule should not be expected to be the same object version.

-	If the objectETag property is present, there shall be only one object version transmitted in all of the time windows delimited by the start and end attributes of each of the one or more deliveryInfo elements.

-	In-band Schedule Description instance document updates can be used to provide a dynamic schedule update to override the existing delivery schedule, such as using the cancellation mechanism specified in this clause.

-	A sessionSchedule element in the same serviceSchedule element shall be present, and its start and stop properties shall specify a time window that completely overlaps the time windows specified in each of the objectSchedule elements of the same serviceSchedule.

When a sessionSchedule property is present and there are no objectSchedule objects in a serviceSchedule object, then the MBS Client should download each new object, independently of whether the MBS Distribution Session uses the MBS Object Distribution Method or the MBS Packet Distribution Method.

The objectSchedule property specifies details about the objects to be delivered during an MBS Distribution Session. The sessionId property, if present, identifies the MBS Distribution Session for each object. If not present, an MBS Client shall instead determine the MBS Distribution Session by examining the Session Description metadata unit for the MBS Distribution Session. The objectETag property of the objectSchedule object is the version identifier of the object. If present, the purpose of this entity tag is to enable an MBS Client to determine if an object has changed since a prior reception without having to download the object.

The scheduleUpdate object specifies a time after which MBS Client shall seek to update its schedule information by acquiring the latest available Schedule Description instance document.

An index property is included in the sessionSchedule object. If the sessionSchedule does not describe any session reoccurrence, then the index corresponds to the single session occurrence. If the sessionSchedule describes one or more reoccurrences the index is the starting index of the first session occurrence with the index value increased by one for each session reoccurrence.

A cancelled key is defined as a child of the objectSchedule/objectURI element.

-	If the cancelled property is set to "true" or "1", then the transmission of the object identified by the objectURI element is cancelled, and the MBS Client shall cancel any applicable repair and/or reception reporting procedures for that object.

If this object schedule-level cancellation indication in the updated schedule description is received after the associated object has already been delivered, then any related repair or reception reporting for that object (associated with its parent service), either in progress or yet to occur, shall be aborted.

-	If the cancelled key is set to "false" or "0" or is absent, then normal object transmission and associated delivery procedures, if applicable, shall occur.

Table 5.2.7-5 provides the detailed semantics for the SessionScheduleOverride object.

Table 5.2.7-5: Semantics of SessionScheduleOverride object

| Property name |  | Use | Type | Description |
| --- | --- | --- | --- | --- |
| SessionScheduleOverride |  |  | object |  |
|  | start | 1 | DateTime |  |
|  | stop | 1 | DateTime |  |
|  | index | 0..1 | integer |  |
|  | cancelled | 0..1 | boolean | The default value if omitted is false. |
|  | sessionDescriptionURI | 0..1 | Uri |  |

A sessionScheduleOverride object is defined as part of the serviceSchedule object. If present, the sessionScheduleOverride object indicates either the cancellation of the session occurrence, or schedule override, as follows:

-	If the cancelled property (a child of sessionScheduleOverride object) is set to "true" or "1", then the transmission of the MBS Distribution Session identified by the index attribute (a child of sessionScheduleOverride object) is cancelled, and the MBS Client shall cancel any applicable repair and/or reception reporting for all objects belonging to that MBS Distribution Session.

If this session schedule-level cancellation indication in the updated schedule description is received after any of the associated objects have already been delivered, then any related repair or reception reporting for those objects (associated with their parent service(s)), either in progress or yet to occur, shall be aborted.

-	If the cancelled property (a child of sessionScheduleOverride object) is set to "false" or "0" or is absent, then the start and stop time elements (children of sessionScheduleOverride object) shall override the nominal start and stop time of the transmission schedule of the session as identified by the index property (a child of sessionScheduleOverride object).

The value of the index property in the sessionScheduleOverride object corresponds to any of the value of the index key in the reoccurenceStartStopType in the sessionSchedule object.

Schedule information received in the Schedule Description metadata unit shall take precedence over timing information that may have been received in the Session Description metadata unit (t and/or r lines in the SDP).

### 5.2.8	Object Repair Parameters data type

Object Repair Parameters configure the Object Repair as defined in clause 6.2.4.

Table 5.2.8-1 provides the detailed semantics for the ObjectRepairParameters data type.

Table 5.2.8-1: Semantics of ObjectRepairParameters data type

| Property name | Data Type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| offsetTime | DurationSec | O | 0..1 | See TS 26.346 [7]. If not present the value is assumed to be zero. |
| randomTimePeriod | DurationSec | O | 0..1 | See TS 26.346 [7]. If not present the value is assumed to be zero. |
| object‌Distribution‌BaseLocator | Uri | O | 0..1 | The Object distribution base URL of the MBS Distribution as defined in table 4.5.6-2 of TS 26.502 [6]. |
| object‌Repair‌BaseLocator | array(AbsoluteUrl) | M | 0..1 | The Object repair base URL of the MBS Distribution as defined in table 4.5.6-2 of TS 26.502 [6]. |

### 5.2.V	Availability Information data type

The userServiceDescription element may include an availabilityInfo child element providing additional information pertaining to the availability of the MBS Distribution Session within the 5G Network. If present, the availabilityInfo element shall include one or more infoBinding child elements. The infoBinding element shall contain the child elements serviceArea, mbsFSAId and radiofrequency:

-	The serviceArea element declares the one or more service areas in which the MBS Session corresponding to this MBS Distribution Session is currently available.

-	In the case of a broadcast MBS Session corresponding to this MBS Distribution Session, the mbsFSAId element identifies a preconfigured area within which, and in proximity to, the cell(s) announce the MBS FSA ID and its associated frequency.

NOTE:	This is used to guide frequency selection by the UE for a broadcast MBS Session.

-	The radioFrequency element indicates the one or more radio frequencies in the NG-RAN downlink which transmit the MBS Session corresponding to this MBS Distribution Session in the service area(s) identified by the serviceArea element.

Table 5.2.V-1: Semantics of AvailabilityInformation data type

| Property name | Type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| mbsService‌Area | array(Mbs‌Service‌Area) |  | 0..N | The Target service areas of this MBS Distribution Session, as defined in table 4.5.8-1 of TS 26.502 [6]. |
| mbs‌FSA‌Id | MbsFsaId |  | 0..1 | The MBS Frequency Selection Area (FSA) Identifier of the MBS Distribution Session in the parent service area, as defined in table 4.5.8-1 of TS 26.502 [6]. |
| radio‌Frequency | array(integer) |  | 0..N | The transmission frequency associated with the MBS Frequency Selection Area (FSA) Identifier in the parent service area. |

### 5.2.X	Security Description data type

In order to support the transport security for MBS, the DistributionSessionDescription object may contain an object of type SecurityDescription indicating the User Plane security parameters for the MBS Distribution Session as defined in table 4.5.8-1 of TS 26.502 [6]. The User Plane security mechanisms between the MBS client and the MBSSF are defined in clause W.4.1.3 of TS 33.501 [32] and clause 6 of TS 33.246 [33].

Table 5.2.X-1: Semantics of SecurityDescription object

| Property name |  | Type | P | Cardinality | Description |
| --- | --- | --- | --- | --- | --- |
| mBSSFAddresses |  | array(AbsoluteURL) |  | 1..1 | A list of key management servers (i.e., MBSSF instances). At least one endpoint address shall be listed. |
| mBSServiceKeyInfo |  | object |  | 1..1 | Information about the MBS Service Key (MSK). |
|  | mSKId | string |  | 1..1 | Identifies the MSK, as defined in clause 6.3.2.1 of TS 33.246 [33]. |
|  | mSKDomainId | string |  | 1..1 | Identifies the MSK domain, as defined in clause 6.3.2.1 of TS 33.246 [33]. |
| uICCKeyManagement |  | boolean |  | 0..1 | Indicates whether UICC-based key management is required for the parent MBS Distribution Session. If false or omitted, this is not required. |
| 2GGBAallowed |  | boolean |  | 0..1 | Indicates whether the 2G GBA is permitted for the parent MBS Distribution Session. If false or omitted, this is not permitted. |
| backOffMode |  | object |  | 0..1 | The back-off behaviour of the MBS Client when requesting MBS Service Keys from the MBSSF. If omitted, no back-off mode is used in MSK requests. |
|  | offsetTime | DurationSec |  | 0..1 | Indicates the time that a MBS Client shall wait before sending the MSK requests. |
|  | randomTimePeriod | DurationSec |  | 0..1 | Indicates the time window length over which an MBS Client shall calculate a random time for the initiation of the MSK requests. |

## 5.3	Delivery of User Service Description

### 5.3.1	General

An MBS User Service Description may be delivered to the MBS Client via MBS Distribution Sessions at reference point MBS-4-MC (see clause 5.3.2) and/or via a regular unicast PDU Session at reference point MBS-5 (see clause 5.3.3) and/or via application-private means at reference point MBS-8.

The syntax of the User Service Description is specified in clause A.2.

The MIME content type of the MBS User Service Description bundle is specified in clause E.2.

### 5.3.2	Delivery of User Service Description in object carousel

In this case, a bundle of MBS User Service Descriptions is delivered repeatedly by the MBSTF to the MBSTF Client via a suitable MBS Distribution Session at reference point MBS-4-MC using the Object Distribution Method, as defined in clause 4.2.4 of TS 26.502 [6]. The operating mode of this MBS Distribution Session shall be set to OBJECT_CAROUSEL and relies on an object manifest to characterize the repetition and the update pattern of the MBS User Service Announcement information. The object manifest is specified in clause 6.1.2.

As defined in clause 5.4 of TS 26.502 [6], the MBSTF Client announces the arrival of an MBS User Service Description to the MBSF Client using a suitable notification at internal reference point MBS-7′, and the MBSF Client may subsequently retrieve the MBS User Service Description from the MBSTF Client via internal reference point MBS-6′. It may do so using procedures equivalent to those specified for unicast retrieval at reference point MBS-5 (see clause 5.3.3). In this case, the MBSTF Client acts as a proxy for the MBS AF.

### 5.3.3	Delivery of User Service Description via unicast PDU Session

In this case, a bundle of one or more MBS User Service Descriptions is retrieved by the MBSF Client from the MBS AF at reference point MBS-5 via a regular unicast PDU Session.

The API at this reference point is specified in clause 9.2. The OpenAPI [14] specification of the User Service Description retrieval API can be found in clause A.2.

# 6	Object Distribution Method

## 6.1	General

### 6.1.1	Overview

The Object Distribution Method supports the transmission of media segments, e.g. CMAF media segments [7] and also non-real-time objects.

The MBS Distribution Session shall be provisioned to accommodate the bit rate of the aggregated object flow, accounting for in-band carriage of metadata units, protocol header overheads, and FEC redundancy (if configured).

### 6.1.2	Object manifest

An object manifest document describes a list of objects to be distributed in an MBS Distribution Session. The syntax of the object manifest is specified in clause D.1. The semantics of the document are specified in table 6.1.2-1 below.

For each object to be delivered in the MBS Distribution session, the attributes under the objects property in table 6.1.2-1 shall be maintained by the MBSTF.

Table 6.1.2-1: Properties of object manifest

| Property name | Cardinality | Description |
| --- | --- | --- |
| updateInterval | Optional | The time interval, expressed in seconds, according to which the MBSTF attempts to re-acquire the object manifest when pull-based object acquisition is provisioned. Ignored by the MBSTF for push-based object acquisition. |
| objects | Mandatory | The list of objects to be carouselled from the MBSTF to the MBSTF Client. |
| locator | Mandatory | The URL from which the object is to be ingested by the MBSTF. |
| repetitionInterval | Optional | The time interval, expressed in milliseconds, according to which the MBSTF will periodically send the object to the MBSTF Client. If omitted, the MBSTF determines the repetition interval for the object. Ignored by the MBSTF in Object Collection operating mode. |
| keepUpdatedInterval | Optional | The time interval, expressed in seconds, according to which the MBSTF is expected to check for updates made to the object at its origin (as indicated by locator or a redirect from there to another location). In case of conflicting information, the MBSTF should give precedence in determining the update interval to the HTTP cache control metadata included in the HTTP response from the object origin. Any changes to the origin object that are detected by the MBSTF shall be reflected in the MBS Distribution Session at the earliest opportunity and the replacement of one object with another shall be signalled to the MBSTF Client by means of the object transport protocol provisioned at reference point MBS-4-MC. If omitted, the MBSTF shall not attempt to check for updates to the object. Ignored by the MBSTF in Object Collection operating mode. |
| earliestFetchTime | Optional | The MBSTF shall fetch the object no sooner than this UTC timestamp. If absent, then the object shall be present at its origin (as indicated by locator or a redirect from there to another location) and the MBSTF may fetch it at a time of its choosing. |
| latestFetchTime | Optional | The MBSTF shall fetch the object no later than this UTC timestamp. If absent, then the object shall be present at its origin (as indicated by locator or a redirect from there to another location) and the MBSTF may fetch it at a time of its choosing. |

## 6.2	Usage of FLUTE for Object Distribution Method

### 6.2.1	General

If FLUTE [12] is used to realise the Object Distribution Method, the MBS Distribution Session shall conform to the MBMS Download Profile as defined in clause L.4 of TS 26.346 [7] with the additional requirements in clause 6.2 of the present document.

The usage of this distribution method is identified in the MBS Session Description metadata unit as defined in clause 6.2.3, in particular by the indication of the protocol FLUTE/UDP in combination with the MBS service type.

The MBSTF shall use the Profiled FDT Schema according to clause L.6 of TS 26.346 [7] to describe the object list currently being transmitted in the MBS Distribution Session.

Generally, the end of transmission of an object is the expiry time for the latest FDT instance describing the object. Objects shall be described in an FDT Instance with the Expires attribute. Depending on the operating mode (clause 6.2.4), different settings of the expiry time and different numbers of objects per FDT Instance are recommended.

Inclusion of the @Content-MD5 and @File-ETag FDT Instance attributes is optional.

The @File-ETag represents the value of the HTTP entity tag as defined in section 2.3 of  RFC 7232 [13]which may also serve as the version identifier of the File object described by the FDT Instance.

In order to fetch missing portions of an object, the MBS Client may use the Object Repair service provided by the MBS AS at reference point MBS-4-UC, using the MBS User Service Announcement parameters specified in clause 5.2.8 to identify its endpoint address. The Object Repair service is realized as a Byte-Range based File Repair, as specified in clause 9.3.6.2 of TS 26.346 [7].

NOTE:	The use of Alternate-Content-Location-1 and Alternate-Content-Location-2 to advertise the Object Repair parameters in the FLUTE FDT instance as defined in TS 26.346 [7] is not supported in MBS User Services.

### 6.2.2	Session Description metadata unit

#### 6.2.2.1	General

The Session Description metadata unit contains the needed information to activate the reception of an Object Distribution Method. The Session Description metadata unit is formatted according to the Session Description Protocol [8]. The Session Description metadata unit for the Object Distribution Method is based on the Session Description parameters as defined in clause 7.3 of TS 26.346 [7] with the following restrictions and extensions.

Restrictions:

-	The Mode of MBMS bearer per media parameter (clause 7.3.2.7 of [7]) shall not be used.

-	The QoE Metrics (as defined in clauses 7.3.2.0 of [7]) shall not be used

-	The Service-language(s) per media (clause 7.3.2.9 of [7]) shall not be used. It is assumed that the service languages are described within an application manifest.

-	The Alternative TMGI (clause 7.3.2.12 of [7]) shall not be used.

-	The Start time and End time of the session (SDP t-line) shall indicate a superset of the active times specified in the MBS Schedule Description metadata unit, if present. If there is no schedule specified, both values should be set to zero indicating undefined times.

Extensions:

-	When an MBS Session is of MBS Service Type Broadcast or when the Multicast MBS Session Type uses a TMGI as MBS Session ID, the MBS service type of MBS Session declaration attribute as defined in clause 6.2.2.2 shall be present in the Session Description.

#### 6.2.2.2	MBS service type of MBS Session

A new MBS service type declaration attribute mbs-servicetype is defined which results in, e.g.:

-	a=mbs-servicetype:broadcast 123869108302929

or:

-	a=mbs-servicetype:multicast 123869108302929

The MBS service type declaration attribute shall be used in Session Description metadata to indicate the type of the corresponding MBS Distribution Session as defined in table 6.2.2.2-1.

Table 6.2.2.2-1: Assignment of mbs-servicetype attribute value

| Attribute value | Meaning |
| --- | --- |
| multicast | The MBS Distribution Session is delivered using a Multicast MBS Session. |
| broadcast | The MBS Distribution Session is delivered using a Broadcast MBS Session. |

The MBS service type attribute shall be declared at session level in the Session Description metadata unit. The session level attribute applies to all media entries without a media-level occurrence of the mbs-servicetype attribute. The Session Description metadata unit shall include only a single instance of MBS service type declaration attribute.

Definition:

-	mbs-service-type-declaration-line = "a=mbs-servicetype:" ("broadcast"/"multicast" SP tmgi) CRLF

-	tmgi = 1*15DIGIT

EXAMPLE:

UK MCC = 234 (MCC Digit 1 = 2; MCC Digit 2 = 3 and MCC Digit 3 = 4)

Vodafone UK MNC = 15

and, with padding, Vodafone UK MNC = 15F (MNC Digit 1 = 1; MNC Digit 2 = 5 and MNC Digit 3 = F)

MBS Service ID = 70A886

Therefore, TMGI = 70A886 32F451 (Hex) or 123869108302929 (Decimal)

The Temporary Mobile Group Identity (tmgi) information element is defined in TS 24.008 [11] including the coding of the fields. Octets 3 to 8 (MBS Service ID, MCC and MNC) shall be placed in the tmgi attribute of the MBS service type declaration line, and are encoded as a decimal number. Octet 3 is the most significant octet. Because this is encoded as a decimal number, leading zeros of the MBS Service ID field may be omitted.

#### 6.2.2.3	SDP examples for FLUTE Session

Listing 6.2.2.3-1 provides a full example of an SDP description describing a FLUTE-based MBS Distribution Session using the Object Distribution Method with a TMGI as MBS Session Id:

Listing 6.2.2.3-1: Session Description metadata unit for
FLUTE-based MBS Distribution Session with TMGI

v=0 o=user123 2890844526 2890842807 IN IP6 2201:056D::112E:144A:1E24 s=Object Distribution session example i=More information t=2873397496 2873404696 a=mbs-servicetype:broadcast 123869108302929 a=FEC-declaration:0 encoding-id=1 a=source-filter: incl IN IP6 * 2001:210:1:2:240:96FF:FE25:8EC9 a=flute-tsi:3 m=application 12345 FLUTE/UDP 0 c=IN IP6 FF1E:03AD::7F2E:172A:1E24/1 b=1000 a=lang:EN a=FEC:0

Listing 6.2.2.3-2 provides a second example of an SDP description describing a FLUTE-based MBS Distribution Session using the Object Distribution Method and which indicates that 25% redundant FEC protection is applied to the FEC encoding of the video Segments of the associated DASH-formatted content:

Listing 6.2.2.3-2: Session Description metadata unit for
FLUTE-based MBS Distribution Session with TMGI and 25% FEC redundancy

v=0 o=user123 2890844526 2890842807 IN IP6 2201:056D::112E:144A:1E24 s=Object Distribution session carrying 2-hour DASH-packaged programme i=More information t=3615124600 3615131800 a=mbs-servicetype:broadcast 123869108302929 a=FEC-declaration:0 encoding-id=1 a=FEC-redundancy-level:0 redundancy-level=25 a=source-filter: incl IN IP6 * 2001:210:1:2:240:96FF:FE25:8EC9 a=flute-tsi:5 m=video 10111 FLUTE/UDP 0 c=IN IP6 FF1E:03AD::7F2E:172A:1E24/1 b=2048 a=lang:EN

### 6.2.3	Operating modes for Object Distribution Method

#### 6.2.3.1	Introduction

The operating modes for the Object Distribution Method are defined in clause 6.1 of TS 26.502 [6]. Operating modes primarily describe the operation of the MBSTF to convert ingest data into an MBS Distribution Session. The following clauses specify how FLUTE is used for each operating mode.

The Object Repair mechanism for FLUTE specified in clause 6.2.4 may be used with any of the following operating modes.

#### 6.2.3.2	Single object operating mode

Single object operating mode (OBJECT_SINGLE) refers to the case in which a single object is distributed via the Object Distribution Method.

No specific aspects beyond the general provisions in clauses 6.1, 6.2.1 and 6.2.2 apply to this operating mode.

#### 6.2.3.3	Object collection operating mode

Object collection operating mode (OBJECT_COLLECTION) refers to the case in which multiple objects are distributed via the Object Distribution Method. The list of objects to be distributed is described by an object manifest document as specified in clause 6.1.2. The objects listed in the manifest are distributed only once. Each object listed in the manifest is pulled by the MBSTF from the location indicated prior to inclusion in the FLUTE Session corresponding to the MBS Distribution Session.

In this operating mode, each FDT Instance delivered in the session should describe all objects that are part of the current collection.

#### 6.2.3.4	Object carousel operating mode

Object carousel operating mode (OBJECT_CAROUSEL) refers to the case in which one or multiple objects are distributed via the Object Distribution Method in a repeated fashion. The list of objects to be distributed is described by an object manifest document as specified in clause 6.1.2. Each object listed in the manifest is pulled by the MBSTF from the location indicated prior to inclusion in the FLUTE Session corresponding to the MBS Distribution Session.

The list of objects described in the manifest may be updated over time by providing a replacement object manifest.

In this operating mode, the FDT Instance should describe all objects that are currently available in the FLUTE Session, considering the potential object update interval.

#### 6.2.3.5	Segment streaming operating mode

Segment streaming operating mode (OBJECT_STREAMING) refers to the case for which a sequence of objects, typically representing timed segments from a timed presentation, are distributed using the Object Distribution Method. The sequence of objects is referred to as an object flow. This operating mode is recommended for streaming DASH or HLS content to a Media Player in the UE using MBS User Services.

NOTE:	This operating mode may also be used for non-media object flows, e.g. in the absence of an Application Service Description.

For each object associated with the object flow to be delivered in the MBS distribution session the following information shall be maintained by the MBSTF in an object list:

-	The URL used by the MBS-Aware Application to request the object, derived from the object ingest URL.

-	The object’s latest availability start time at the MBS Client. After this time, the MBS-Aware Application may request the full object from the MBSTF Client by using the URL of the object.

This value is determined for each object based on an availability start time at the point of ingest (i.e. reception of first byte of the object) combined with a configured distribution offset.

-	The object’s availability end time from the MBSTF Client. After this time, the object may no longer be requested by the MBS-Aware Application.

This value is determined for each object based on an availability start time at the point of ingest (i.e. reception of first byte of the object) combined with a configured clean-up time.

The object list is typically extended over time, for example as new objects (e.g. media segments) become available.

The object list may, for example, be provided by an explicit object distribution manifest.

NOTE: An object distribution manifest format is not defined in the present document.

The object list may also be defined by a presentation manifest (e.g. DASH MPD), for example in the case of an Application Service, for which the manifest is provided as part of the User Service Description.

When the Application Service Entry Point document is a DASH MPD, this document is used by the MBSTF to update the object list. The DASH MPD may itself be included in the object list, and hence be delivered in band with the media segment objects it describes on the same MBS Distribution Session. If the content of the Application Service Entry Point document changes during an MBS User Data Ingest Session, the updated document shall be reflected in the MBS Distribution Session at the soonest opportunity.

For the segment streaming operating mode, the MBSTF acts as follows based on the object list:

-	The MBSTF shall transmit each object in the object list such that the last packet of the delivered FLUTE transmission object (including any FEC recovery packets, when configured) is available at the MBSTF Client latest at its latest availability start time.

-	An FDT Instance object should be sent frequently by the MBSTF, describing all objects of the object list that are not yet fully transmitted.

-	The Content-Location element in the FDT Instance shall match the URL of the corresponding object in the object list. The URL may be rewritten by the MBSTF using the Object distribution base URL property of the MBS Distribution Session.

-	The File@Expires attribute for each object shall be set such that it is equal to or earlier than its latest availability start time.

-	The Cache-Control@Expires attribute shall be used to indicate the availability end time of the object.

-	Content-MD5 and File-ETag may optionally be used.

### 6.2.4	Object Repair mechanism for FLUTE

#### 6.2.4.1	General

In order to fetch missing portions of an object, the MBS Client may use the Object Repair service provided by the MBS AS at reference point MBS-4-UC.

Profiling of MBMS.

Protocols – refer to clause 10

Combining

MBS clients shall support Byte-Range based File Repair as specified in clause 6.3.2.

Byte-Range based File Repair is realized as specified in clause 9.3.6.2 of TS 26.346 [7] using the MBS User Service Announcement parameters specified in clause 5.2.8 to identify its endpoint address.

In particular

Availability time

NOTE:	The use of Alternate-Content-Location-1 and Alternate-Content-Location-2 to advertise the Object Repair parameters in the FLUTE FDT instance as defined in TS 26.346 [7] is not supported in MBS User Services.

No in session repair as there too many questions – possibly update stage-2 (during session in clause 4.2.6 of TS 26.502 is converted to a note that it is not supported in the present release – make it a Rel-19 topic, and how does it work for low-latency is even more a question – also for Rel-19).

Questions are for example:

When to start unicast requests

We need a clause 10 for MBS APIs and this clause references

#### 6.2.4.2	Post-session object repair procedure

The post-session object repair procedure is aligned with the File Repair procedures defined in clause 9.3 of TS 26.346 [7].

Editor's Note:	ASSUMPTION: Is there also a case for File repair without FDT? (see clause 9.3.9)

It is assumed that a File Delivery Table (FDT) Instance as defined in clause 7.2.9 of TS 26.346 [7] is available in the MBS Client that contains at least the following information:

-	@Expires attribute indicating the expiry date of the FDT Instance.

-	For one or multiple transmission objects in the FLUTE session, a File element with:

-	@Content-Location attribute indicating the URI of the transmission object.

-	@TOI attribute indicating the ALC Transport Object Identifier of the transmission object.

Then, based on the introduction in clause 9.3.1 of TS 26.346 [7], the MBS Client generally acts as follows.

1.	The MBS Client identifies the end of MBS Distribution Session. The latest time for this is the value of the @Expires attribute indicated in the root element of the FDT Instance. However, according to clause 9.3.2 of TS 26.346 [7], an MBS Client may determine an earlier end of transmission of files,

a)	if an end-of-session signal (A-flag) is received in an ALC/FLUTE header in the ongoing Object Distribution Session before the FDT instance expires, or

b)	if the end of file transmission time is reached in the Schedule object (add more details once schedule data is defined)

2.	The MBS Client identifies that data is missing from an MBS Object Distribution Session for one or multiple transmission objects delivered in the Object Distribution Session following the principles in clause 9.3.3 of TS 26.346 [7].

3.	The MBS Client shall calculate a random back-off time after the end of transmission of files as determined in step 1 according to clause 6.4.2.4 using the offsetTime and randomTimePeriod parameters indicated in the ObjectRepair object as defined in table 5.2.8-1. The MBS Client shall start making repair requests as described the remaining steps at the computed back-off time. The MBS Client shall not start sending any repair request messages before this calculated time back-off time has elapsed after the initial transmission ends.

4.	The MBS Client shall select an MBS AS instance repair URL randomly out of a list of object‌Repair‌BaseURL (if present) as provided in the ObjectRepair object as defined in table 5.2.8-1.

5.	For each incomplete file defined by a File element in the FDT Instance and as identified in step 2:

a)	The MBS Client shall form the network location location (URL) of the repair object as defined in clause 6.4.2.5 using the value of the File@Content-Location attribute, the repair URL selected in step 4 (if present) and the value of objectDistributionBaseURL (if present) as provided in the ObjectRepair object as defined in table 5.2.8-1 as distribution URL.

b)	The MBS Client shall define an appropriate list of byte ranges from the repair object using the list of received symbols, the ESI, source block length and so on as defined in clause 6.4.2.6.

6.	Once the calculated time back-off time has elapsed after the initial transmission ends the MBSTF Client requests a list of byte ranges using the protocol defined in clause 10. In the procedures of requesting the information, MBSTF client uses the FDT with all File entries, the network location location for each file entry, the list of byte ranges for each file entry.

7.	The MBS Client uses the received list of byte ranges and the received data in the MBS Object Distribution Session to recover the missing object as defined in clause 6.4.2.7.

#### 6.2.4.3	In-session object repair procedure

This procedure is for future study.

#### 6.2.4.4	Back-off timing computation

The computation of the back-off time uses the offsetTime parameter and the randomTimePeriod parameter as provided in the ObjectRepair object as defined in Table 5.2.8-1. In particular, the MBS Client shall implement the back-off timing computation specified in clause 9.3.4 of TS 26.346 [7] as follows:

-	The value of the offsetTime parameter is used as the OffsetTime as defined in clause 9.3.4.1 of TS 26.346 [7].

-	The value of the randomTimePeriod parameter is used as the Random Time Period as defined in clause 9.3.4.2 of TS 26.346 [7] to determine a RandomTime.

-	The back-off time is the sum of the OffsetTime and the RandomTime.

#### 6.2.4.5	Network location of repair object

The determination of the network location location of the repair object uses the @Content-Location of the File element in the FDT Instance, a repair URL (if present) and a distribution URL (if present) as follows:

-	If the repair URL is not present, then the network location of the repair object is the value of File@Content-Location.

-	If the repair URL is present, but not the distribution URL, then the network location of the repair object is the value of File@Content-Location entry with the Retrieval URI according to RFC 3986 [34] replaced with the repair URL.

-	If both the repair URL and the distribution URL are present, then the network location of the repair object is the value of File@Content-Location with the Base URI according to RFC 3986 [34] being the distribution URL replaced with the repair URL.

#### 6.2.4.6	Byte Range Selection of Repair Object

When the MBMS FEC scheme is used, the MBMS client shall consider already received repair symbols when making the determination of the further symbols required. In this case, the client should either:

Editor's Note:	To be completed.

#### 6.2.4.7	Object Recovery

Editor's Note:	To be completed.

# 7	Packet Distribution Method

## 7.1	General

The Packet Distribution Method reuses different delivery concepts from TS 26.346 [7]. Additional distribution methods may be defined in future.

## 7.2	Re-using MBMS Delivery Method as Packet Distribution Method

### 7.2.1	General

The Packet Distribution Method combines three different delivery methods of TS 26.346 [7] (namely the MBMS Streaming Delivery Method, Group Communication Delivery Method and Transparent Delivery Method) into a single distribution method, with a set of modifications.

For the Packet Distribution Method, the MBSTF may handle the ingested content on two different protocol layers according to the operating mode provisioned for the MBS Distribution Session:

-	Proxy mode: The MBSTF handles UDP packet payloads and forwards UDP packet payloads from ingest into the MBS Distribution Session. The MBSTF may use different UDP ports for the MBS Distribution Session. The MBSTF re-uses the Proxy Mode of the Transparent Delivery Method as defined in clause 8B of [7].

-	Forward-only mode: The MBS receives complete IP packets and forwards the ingested packets as MBS PDUs. The MBSTF re-uses the Group Communication Delivery Method as defined in clause 8A of [7] and the Forward-Only Mode of the Transparent Delivery Method as defined in clause 8B of [7].

NOTE:	A specific treatment of RTP sessions, for example as provided by the MBMS Streaming Delivery Method, is not provided by MBS User Services. However, RTP sessions may be delivered in proxy or forward-only mode.

### 7.2.3	Session Description

#### 7.2.3.1	General

The Session Description metadata unit contains the needed information to activate the reception of a Packet Distribution Method. The Session Description metadata unit is formatted according to the Session Description Protocol [8]. The Session Description metadata unit for the Packet Distribution Method is based on the Session Description parameters as defined in clauses 8.3, 8A.3 and 8B.3 of TS 26.346 [7] with the following restrictions and extensions.

Restrictions:

-	The Mode of MBMS bearer per media parameter (clauses 8.3.1.5 and 8B.3.2 of [7]) shall not be used.

-	The QoE Metrics (as defined in clauses 8.3.2.1 and 8.4 of [7]) shall not be used.

-	ROHC header compression (as defined in clauses 8A.4 and 8B.4 of [7]) shall not be used.

NOTE:	ROHC is handled by RAN in 5MBS.

-	The Alternative TMGI (clause 7.3.2.12 of [7]) shall not be used.

-	The Start time and End time of the session (SDP t line) shall indicate a superset of the active times specified in the MBS Schedule Description metadata unit, if present. If there is no schedule specified, both values should be set to zero indicating undefined times.

Extensions:

-	When the MBS User Service is of MBS Service Type Broadcast or when an MBS User Service of type Multicast uses a TMGI as its MBS Session ID, the MBS service type of MBS Session declaration attribute as defined in clause 6.2.2.2 shall be present in the Session Description.

#### 7.2.3.2	SDP examples for Packet Distribution Method

Below is a full example of SDP description describing the media streams part of an MBS Packet Distribution session for RTP streaming:

Listing 7.2.3.2-1: Session description for RTP streaming

v=0 o=ghost 2890844526 2890842807 IN IP4 192.168.10.10 s=3GPP MBS Packet Distribution SDP Example i=Example of MBS Packet Distribution SDP file u=http://www.infoserver.example.com/ae600 e=ghost@mailserver.example.com c=IN IP6 FF1E:03AD::7F2E:172A:1E24 t=0 0 b=AS:77 a=mbs-mode:broadcast 123869108302929 a=source-filter: incl IN IP6 * 2001:210:1:2:240:96FF:FE25:8EC9 m=video 4002 RTP/AVP 96 b=TIAS:62000 b=RR:0 b=RS:600 a=maxprate:17 a=rtpmap:96 H264/90000 a=fmtp:96 profile-level-id=42A01E; packetization-mode=1; sprop-parameter-sets=Z0IACpZTBYmI,aMljiA==

The following is a full example of SDP description for transparent streaming with two MPEG-2 Transport Streams:

Listing 7.2.3.2-2: Session description for MPEG-2 Transport Stream

v=0 o=ghost 2890844526 2890842807 IN IP4 192.168.10.10 s=3GPP MBS Transport-only SDP Example i=Example of MBS transport-only SDP file u=http://www.infoserver.example.com/ae600 e=ghost@mailserver.example.com c=IN IP6 FF1E:03AD::7F2E:172A:1E24 t=3034423619 3042462419 b=AS:8000000 a=mbs-mode:broadcast 123869108302929 a=source-filter: incl IN IP6 * 2001:210:1:2:240:96FF:FE25:8EC9 m=video 4002 UDP/RTP/AVP 96 b=TIAS:4000000 a=mms-framing-header:0 2 a=rtpmap:100 MP2T/90000 m=video 4002 RTP/AVP 98 b=TIAS:4000000 a=rtpmap:100 MP2T/90000 a=MBS-framing-trailer:0 2

# 8	General aspects of APIs for MBS User Services

## 8.1	HTTP resource URIs and paths

The resource URI used in each HTTP request to the API provider shall have the structure defined in subclause 4.4.1 of TS 29.501 [16], i.e.:

{apiRoot}/{apiName}/{apiVersion}/{apiSpecificResourceUriPart}

with the following components:

-	{apiRoot} shall be set as described in TS 29.501 [16].

-	{apiName} shall be set as defined by the following clauses.

-	{apiVersion} shall be set to "v1" in this release.

-	{apiSpecificResourceUriPart} shall be set as described in the following clauses.

## 8.2	Usage of HTTP

### 8.2.1	HTTP protocol version

#### 8.2.1.1	General

Content interfaces at reference points specified in the present document shall expose an HTTP/1.1 [21] endpoint to API clients. They may additionally expose an HTTP/2 [22] endpoint, including support for the HTTP/2 starting mechanisms specified in section 3 of [22]. The API client may choose any supported HTTP protocol version. TLS [24] shall be supported on these interfaces and, where the option to use cleartext HTTP is available in the version of HTTP selected by the API client, it should opt for HTTPS interactions in preference.

#### 8.2.1.2	MBSF

The HTTP protocol version used to invoke Nmbsf service operations on the MBSF at reference point Nbm10 is specified in clauses 6.1.2.1 and 6.2.2.1 of TS 29.580 [17].

#### 8.2.1.3	MBSTF

The HTTP protocol version used to invoke Nmbstf service operations on the MBSTF at reference point Nmb2 is specified in clause 6.1.2.1 of TS 29.581 [18].

The endpoint exposed to the MBSF at reference point Nmb2 for the purpose of pushing object manifests into the MBSTF shall comply with the general provisions specified in clause 8.2.1.1.

The endpoint exposed to the MBS Application Provider (AF/AS) at reference point Nmb8 for the purpose of pushing objects into the MBSTF shall comply with the general provisions specified in clause 8.2.1.1.

#### 8.2.1.4	MBS AF

The endpoint exposed to the MBSF Client at reference point MBS-5 for the purpose of retrieving User Service Descriptions using the API specified in clause 9.2 shall comply with the general provisions specified in clause 8.2.1.1.

Editor's Note: Possibly add MBS-10 here to specify the use of HTTP in the User Plane security procedure, or else specify the use of this reference point in a new clause 8.2.1.6 specifically for the MBSSF.

The endpoint exposed to the MBSTF at reference point MBS-11 for the purpose of retrieving object manifests and User Service Descriptions shall comply with the general provisions specified in clause 8.2.1.1.

All responses from the MBS AF that carry a message body shall include a strong entity tag in the form of an ETag response header field and a modification timestamp in the form of a Last-Modified response header per section 8.8 of RFC 9110 [19].

All endpoints exposed by the MBS AF shall support conditional HTTP requests using the header fields If-none-Match and If-Modified-Since per section 13 of RFC 9110 [19].

#### 8.2.1.5	MBS AS

The endpoint exposed to the MBSTF Client at reference point MBS-4-UC for the purpose of unicast object repair shall comply with the general provisions specified in clause 8.2.1.1.

Byte range requests per section 14 of RFC 9110 [19] shall be supported by the MBS AS at reference point MBS-4-UC for the purpose of efficient unicast object repair by the MBSTF Client.

### 8.2.2	HTTP message bodies for API resources

Individual APIs in the present document specify the syntax and encoding of HTTP request and response message bodies. MIME content types for a subset of these are registered in annex E.

### 8.2.3	Usage of HTTP headers

#### 8.2.3.1	General

Standard HTTP headers shall be used in accordance with clause 5.2.2 of TS 29.500 [15], encoded appropriately for the version of HTTP in use.

#### 8.2.3.2	User Agent identification

##### 8.2.3.2.1	General

When one of the MBS User Services functions defined in TS 26.502 [6] makes requests to an HTTP endpoint specified in the present document, it shall identify itself to the HTTP server using a User-Agent request header field (see section 10.1.5 of RFC 9110 [19]) that includes a product identifier indicating the type of client function making the request in its token element.

The optional product-version suffix shall be present and should indicate the version number of the present document (without the leading "V") with which the client implementation complies and shall, at minimum, indicate the 3GPP release number with which the implementation complies.

The User-Agent request header field may also include comment elements (see section 5.6.5 of RFC 9110 [19]) following the above specified product identifier, as well as additional vendor-specific product identifiers and comment elements compliant with the syntax and guidance provided in section 10.1.5 of RFC 9110 [19].

EXAMPLE 1:	MBSTF/17.4.0 (build2114) libhttp/1.23.2

EXAMPLE 2:	MBSFClient/17

##### 8.2.3.2.2	MBSF identification

When invoking the Nmbstf service at reference point Nmb2, the MBSF identifies itself to the MBSTF using a User-Agent request header as specified in clauses 6.1.2.2.1 and 6.2.2.2.1 of TS 29.580 [17].

##### 8.2.3.2.3	MBSTF identification

When ingesting content using the pull-based object acquisition method (see table 4.5.6-2 of TS 26.502 [6]), the MBSTF shall identify itself to the MBS Application Provider (AF/AS) at reference point Nmb8 and to the MBS AF at reference point MBS-11 using a User-Agent request header field that complies with the general provisions specified in clause 8.2.3.2.1. The product identifier token shall be set to the value MBSTF.

##### 8.2.3.2.4	MBSF Client identification

The MBSF Client shall identify itself to the MBS AF at reference point MBS-5 and to the MBSSF at reference point MBS-10 using a User-Agent request header field that complies with the general provisions specified in clause 8.2.3.2.1. The product identifier token shall be set to the value MBSF‌Client.

##### 8.2.3.2.5	MBSTF Client identification

The MBSTF Client shall identify itself to the MBS AS at reference point MBS-4-UC using a User-Agent request header field that complies with the general provisions specified in clause 8.2.3.2.1. The product identifier token shall be set to the value MBSTF‌Client.

#### 8.2.3.3	Server identification

##### 8.2.3.3.1	General

When one of the MBS User Services functions defined in TS 26.502 [6] responds to an HTTP request, it shall identify itself to the requesting client using a Server response header (see section 10.2.4 of RFC 9110 [19]) that includes a product identifier indicating the type and host name of the responding server in its token element. The server type and host name shall be separated by a single hyphen ('-') character.

The optional product-version suffix shall be present and should indicate the version number of the present document (without the leading "V") with which the server implementation complies and shall, at minimum, indicate the 3GPP release number with which the implementation complies.

The Server response header field may also include comment elements (see section 5.6.5 of RFC 9110 [19]) following the above specified product identifier, as well as additional vendor-specific product identifiers and comment elements compliant with the syntax and guidance provided in section 10.2.4 of RFC 9110 [19].

EXAMPLE 1:	MBSTF-vm10665.mno.net/17.4.0 (api=1.0.0) libsbi/2.1 libnf/1.2

EXAMPLE 2:	MBSAF-vm10240.mno.net/17 (api=1.0.0) libsbi/2.1 libnf/1.2

##### 8.2.3.3.2	MBSF identification

When responding to Nmbsf service operations made by the MBS Application Provider (AF/AS) at reference point Nmb10, the MBSF's Server response header is set as specified in clauses 6.1.2.2.1 and 6.2.2.2.1 of TS 29.580 [17].

##### 8.2.3.3.3	MBSTF identification

When responding to Nmbstf service operations made by the MBSF at reference point Nmb2, the MBSTF's Server response header is set as specified in clause 6.1.2.2.1 of TS 29.581 [18].

When acknowledging objects published using the push-based object acquisition method by the MBSF at reference point Nmb2 or by the MBS Application Provider (AF/AS) at reference point Nmb10, the MBSTF shall identify itself using a Server response header field that complies with the general provisions specified in clause 8.2.3.3.1. The product identifier token shall be set to the value MBSTF.

##### 8.2.3.3.4	MBS AF identification

The MBS AF shall identify itself to the MBSF Client at reference point MBS-5 and to the MBSTF at reference point MBS-11 using a Server response header field that complies with the general provisions specified in clause 8.2.3.3.1. The product identifier token shall be set to the value MBSAF.

##### 8.2.3.3.5	MBS AS identification

The MBS AS shall identify itself to the MBSTF Client at reference points MBS-4-UC using a Server response header field that complies with the general provisions specified in clause 8.2.3.3.1. The product identifier token shall be set to the value MBSAS.

#### 8.2.3.4	Support for conditional HTTP GET requests

The provisions in clause 5.2.2 of TS 29.500 [15] relating to conditional GET requests using the If-None-Match and If-Modified-Since request headers apply to all Network Functions in the MBS System. In particular:

-	This is specified for invocations of the Nmbsf service at reference point Nmb10 in clauses 6.1.2.2.1 and 6.2.2.2.1 of TS 29.580 [17].

-	This is specified for invocations of the Nmbstf service at reference point Nmb2 in clause 6.1.2.2.1 of TS 29.581 [18].

All responses from the MBS AF at reference points MBS-5 and MBS-11 that carry a resource message body shall include:

-	a strong entity tag for the resource, conveyed in an ETag response header per section 8.8.3 of RFC 9110 [19],

-	a resource modification timestamp, conveyed in a Last-Modified response header per section 8.8.2 of RFC 9110 [19], and

-	a predicted time-to-live period for the resource, conveyed in a Cache-Control: max-age response header per section 5.2 of RFC 9111 [20].

All API endpoints on the MBS AF that expose the HTTP GET method shall support conditional requests using the If-None-Match and If-Modified-Since request headers per section 13.1.2 and 13.1.3 respectively of RFC 9110 [19]. API clients should not attempt to revalidate their cached copy of a resource using a conditional GET request before the indicated time-to-live period has elapsed.

#### 8.2.3.5	Support for conditional HTTP POST, PUT, PATCH and DELETE requests

The provisions in clause 5.2.2 of TS 29.500 [15] relating to conditional POST, PUT, PATCH and DELETE requests using the If-Match request header apply to all Network Functions in the MBS System. In particular:

-	This is specified for invocations of the Nmbsf service at reference point Nmb10 in clauses 6.1.2.2.1 and 6.2.2.2.1 of TS 29.580 [17].

-	This is specified for invocations of the Nmbstf service at reference point Nmb2 in clause 6.1.2.2.1 of TS 29.581 [18].

## 8.3	HTTP response codes

Guidelines for error responses to the invocation of APIs of NF services are specified in clause 4.8 of TS 29.501 [16]. API-specific error responses are specified in the respective technical specifications.

# 9	MBS AF APIs (MBS-5)

## 9.1	General

This clause specifies the network APIs exposed by the MBS AF with reference to the general provisions of clause 8 as they apply to the reference point in question.

## 9.2	User Service Description retrieval API

### 9.2.1	General

In the case where Service announcement modes (see table 4.5.3-1 of TS 26.502 [6]) indicates that the MBS User Service Announcement for an MBS User Service is advertised at reference point MBS-5, the User Service Description retrieval API is used by the MBSF Client to retrieve a User Service Description document (or a set of User Service Description documents) from the MBS AF that enable reception of the MBS User Service(s) to be initiated by the MBSF Client.

In the case where Service announcement modes (see table 4.5.3-1 of TS 26.502 [6]) indicates that the MBS User Service Announcement for an MBS User Service is advertised via the User Service Announcement Channel at reference point MBS-4-MC, the User Service Description retrieval API is used by the MBSF Client at reference point MBS-7′ to retrieve a User Service Description document (or a set of User Service Description documents) from the MBSTF Client that enable reception of the MBS User Service(s) to be initiated by the MBSF Client.

In the absence of prior knowledge about which Service announcement mode(s) are configured for currently provisioned MBS User Services, an MBSF Client may use either or both of the above procedures to proactively discover MBS User Service Announcements.

### 9.2.2	Resource structure

The User Service Description retrieval API is accessible from the MBS AF at reference point MBS-5 and from the MBSTF Client at reference point MBS-7′ through the following URL base path:

{apiRoot}/3gpp-mbs-user-service-discovery/{apiVersion}/

The operations and the corresponding HTTP methods in table 9.2.2-1 are supported through the above API base path. In each case, the sub-resource path specified in the second column shall be appended to the URL base path.

Table 9.2.2-1: Operations supported by the User Service Description retrieval API

| Operation | Sub-resource path | Allowed HTTP method(s) | Description |
| --- | --- | --- | --- |
| Discover User Service Descriptions | user-service-descriptions?‌{queryParameters} | GET | Used to discover a set of User Service Descriptions that match a set of filtering criteria corresponding to at least one of the query parameters specified in table 9.2.2-2. Multiple query parameters may be concatenated using the ampersand ('&') character as a separator with the resulting semantics of logical conjunction (i.e., Boolean AND). It is an error to invoke this operation with no query parameters. A UserServiceDescriptions bundle resource (see clause 9.2.3.1) is returned containing User Service Descriptions matching all of the specified filtering criteria, which may be empty if none match all of the criteria. |
| Retrieve User Service Descritption | user-service-descriptions/‌{externalServiceId} | GET | The {externalServiceId} uniquely identifies a single User Service Description resource in the MBS AF. If the requested User Service is known to the MBS AF, a User‌Service‌Description resource (see clause 9.2.3.2) is returned. Otherwise, a suitable HTTP error response code is returned. |

Table 9.2.2-2 specifies the query parameters that may be combined with the operations specified in table 9.2.2-1.

Table 9.2.2-2: Query parameters supported by the User Service Description retrieval API

| Applicable operation | Filter name | Query parameter | Description |
| --- | --- | --- | --- |
| Discover User Service Descritptions | Service class | service-class=‌{serviceClassTermId} | Used to select User Service Descriptions that are tagged with the supplied service class term identifier (see table 4.5.3-1 of TS 26.502 [6]), which is expressed as a fully-qualified URI string from a controlled vocabulary (e.g., OMNA BCAST Service Class [25]) with appropriate URL encoding applied. |

### 9.2.3	Data model

#### 9.2.3.1	UserServiceDescriptions resource type

The data model for the UserServiceDescriptions bundle resource is specified in clause 5.2.2 and clause A.2.

#### 9.2.3.2	UserServiceDescription resource type

The data model for the UserServiceDescription resource is specified in clause 5.2.3 and clause A.2.

# 10	MBS Unicast Protocol (MBS-4-UC)

## 10.1	General

This clause specifies the protocol between the MBSTF client and the MBS AS with reference to the general provisions of clause 8 as they apply to the reference point in question.

Ths clause defines the following protocols:

-	Object repair retrieval protocol in clause 10.2 between the MBS Client and the MBS AF when the delivery of one of several files in a within an MBS download delivery session was not successful

## 10.2	Object repair retrieval protocol

### 10.2.1	Overview

This clause defines the object repair retrieval protocol between the MBSTF Client and the MBS AS when the delivery of one of several files in a within an MBS download delivery session was not successful.

For this purpose the following is specified:

-	the parameters provided to the MBS unicast client to generate the appropriate requests to the MBS AS as defined in clause 10.2.2,

-	the MBS unicast client request procedures in order to request missing data to initiate the repair of data as defined in clause 10.2.3,

-	the MBS AS procedures as a response to the unicast requests initiated by an MBSTF client as defined in clause 10.2.4.

### An MBS client shall implement the procedures defined in clause 10.2.3 based on the parameters defined in clause 10.2.2. An MBS AS shall implement the procedures defined in clause 10.2.4 based on the requests defined in clause 10.2.3.

### 10.2.2	Parameters

The following parameters are assumed to be available to the MBSTF client for unicast repair

-	The File Delivery Table (FDT) associated to the download delivery session

-	for each file with missing data and identified by a TOI in the FDT

-	the network location location referring to an object hosted on MBS AS

-	the list of byte ranges for each file that are required to permit the repair,

### 10.2.3	MBSTF Client unicast request procedures

The MBSTF client sends one or more requests to an MBS AS requesting transmission of data that allows recovery of missing file data. All file repair requests and repair responses for a particular MBMS transmission shall take place in a single TCP session using the HTTP protocol (RFC 9110  [XX]). The repair request is routed to the MBS AS IP address resolved from the selected network location.

The MBSTF client shall start the initial request once iniated after the backoff time (see clause 6.2.4). If there is more than one repair request to be made these are sent immediately after the first.

If the MBMS UE only requests transmission of a subset of the source symbols or sub-symbols the UE uses the HTTP partial GET request with the Range request header  as defined in 14.35.2 of RFC 2616 [18].  The MBMS UE shall indicate the specific source symbols or sub-symbols as a byte-range-spec as defined in 14.35.1 of RFC 2616 [18].

For messaging efficiency, the HTTP GET method allows the UE to include multiple byte range requests within a single partial GET request.  If the UE includes multiple byte ranges in a single request the HTTP GET request should not exceed 2048 bytes in length to avoid truncation by the HTTP server.

If the MBMS UE determines that it can select among multiple subsets of the source symbols or sub-symbols, the MBMS UE should request the subset with the lowest ESI values, i.e., choose the missing source symbols or sub-symbols from the beginning of the source block or source sub-block, respectively.  This improves the caching efficiency of the HTTP file repair servers.

If more than one file were downloaded in a particular MBMS download session, and, if the MBMS client needs repair data for more than one file received in that session, the MBMS client shall send separate HTTP GET requests for each file.

If "File-ETag" is present in the FDT Instance, its value shall be used as the entity-tag in the "If-Match" or "If-Range" header of a conditional byte-range file request.

If "File-ETag" is not present in the FDT Instance, but "Content-MD5" is, the latter may be used as the entity-tag in the "If-Match" or "If-Range" header of a conditional byte-range file request, or the UE may choose to send an HTTP GET request containing the "Range" header for the requested byte range(s), without the "If-Match" or "If-Range" header.

For the UE, the nominal objective of using the "If-Match" header is to receive the requested range(s) of the file associated with the entity-tag, or no repair data if the request cannot be satisfied by the repair server.  The nominal objective of using the "If-Range" header is to receive the latest version of the entire file in case the version associated with the entity-tag is no longer available on the repair server. To reduce the impact to capacity, the UE should not use the "If-Range" header if it can request the range(s) from other repair servers.

If the "Content-Encoding" element is included in the FDT Instance for the file and is set to "gzip", then the MBMS UE shall make the request to a modified URL, that is, the original file URL with the ".gz" extension added to the full path name but prior to the query part of the URL, if any. The MBMS UE shall only use this request if a) the "File-ETag" attribute is present in the FDT Instance of that file, for use as the entity-tag in the request, or b) the "Content-MD5" attribute is present in the FDT Instance for that file, for use as the entity-tag in the request. Otherwise, the MBMS UE should rather request the complete file instead of using byte range requests.

As an example, a FLUTE receiver partially receives the transport object with URL "http://www.example.com/service1/document.pdf",Content-Encoding set to "gzip", and with the Content-MD5 set to "B2B359591E961C6B0F468FE536BCD920=" while the "File-ETag" attribute is absent in the FDT Instance. It issues a repair request to the host server to fetch the missing bytes. The request is as follows:

GET	/service1/document.pdf.gz  HTTP/1.1
If-Match: "B2B359591E961C6B0F468FE536BCD920="
Range: bytes=5018640-5042399

Host: www.example.com

The conditional request is used by the repair server to ensure that the byte range it will serve to the client is from the exact same compressed file. The conditional repair procedure is described earlier in this section.

As a second example, assume that the "Alternate-Content-Location-1" element in the FDT Instance of the file indicates that byte range repair requests are supported by the HTTP server at URI www.example.com/service1/news_service/latest_news.mp4. The UE determines that it requires the byte ranges 5018640-5042399 and 19037040-19050239.  The "Content-MD5" attribute provided in the FDT Instance of the file is Base64 encoded as B2B359591E961C6B0F468FE536BCD920, and the "File-ETag" attribute is absent in the FDT Instance.  The HTTP GET request may look as follows:

GET	/service1/news_service /latest_news.mp4  HTTP/1.1
If-Match: "B2B359591E961C6B0F468FE536BCD920="
Range: bytes=5018640-5042399,19037040-19050239

Host: www.example.com

In case the version identifier, indicated by the "Content-MD5" value as the entity-tag in the ‘If-Match’ header cannot be matched, the server will reply with a 412 "Precondition Failed" reply. Otherwise, the server will satisfy the request and reply with a 206 "Partial Content" if the request would be successful without the ‘If-Match’ header.

The following is an example of a response from the repair server:

HTTP/1.1 412 Precondition Failed
Content-Range: bytes=5018640-5042399,19037040-19050239
ETag:  "B2B359591E961C6B0F468FE536BCD920="
Content-Length: 0

As a third example, assume that the "Alternate-Content-Location-1" element in the FDT Instance of the file indicates that byte range repair requests are supported by the HTTP server at URI www.example.com/service2/magazine_service/article_xyz.pdf. The UE determines that it requires the byte ranges 5000-7999 and 25500-40500. The "File-ETag" attribute is present in the FDT Instance and its value is "10690a1-4f2-40d45ae1". The HTTP GET request may look as follows:

GET	/service2/magazine_service /article_xyz.pdf  HTTP/1.1
If-Match: "10690a1-4f2-40d45ae1"
Range: bytes=5000-7999, 25500-40500

Host: www.example.com

In this example, the version identifier of the file, represented by the value of the FDT Instance’s "File-ETag" and used as the entity-tag in the ‘If-Match’ header, matches the file version at the byte-range repair server. The server will send a 206 "Partial Content" response, providing the requested byte ranges in the payload:

HTTP/1.1 206 Partial Content

Date: Wed, 15 Nov 2015 06:25:24 GMT

ETag: "10690a1-4f2-40d45ae1"

Content-Length: 18001

Content-Type: multipart/byteranges; boundary=SEPARATION_STRING

--SEPARATION_STRING

Content-Type: application/pdf

Content-Range: bytes 5000-7999

...<the first range>...

-- SEPARATION_STRING

Content-type: application/pdf

Content-range: bytes 25500-40500

...<the second range>…

-- SEPARATION_STRING

### 10.2.4	MBS AS Response messages

### URLs of the files need to be resolved to the AS

Once the MBMS file repair server has assembled a set of encoding symbols that contain sufficient data to allow the UE to reconstruct the file data from a particular file repair request, the MBMS file repair server sends one message to the UE. Each file repair response occurs in the same TCP and HTTP session as the repair request that initiated it.

An MBMS client shall be prepared for any of these 5 response scenarios:

-	The server returns a repair response message where a set of encoding symbols forms an HTTP payload as specified below (see 9.3.7.2 for details).

-	The server returns a repair response message where a byte range or set of byte ranges forms an HTTP payload as specified below (see 9.3.7.2a for details).

-	The server returns the requested file or file groups (see 9.3.7.5 for details).

-	The server redirects the client to a broadcast/multicast delivery (an MBMS download session).

-	The server redirects the client to another file repair server (if a server is functioning correctly but is temporarily overloaded).

### -	An HTTP error code is returned (note that sub-clause 9.3.8 describes the case of no server response).

The response message to the byte-range-based repair request message in 9.3.6.2 follows the format and procedures in RFC 2616 [18] for responding to byte range requests.

When the HTTP message includes the content of a single byte range the repair server can provide the HTTP response with a "206 Partial content" status, include the Content-Range header, and use the content-range-spec to indicate the byte range of the repair data as specified in 14.16 of RFC 2616 [18].

When the repair server receives a request for multiple byte ranges it should attempt to transmit all the requested ranges in a single HTTP response.  When an HTTP message includes multiple byte ranges, these are transmitted as a multipart message using the "multipart/byteranges" media type as defined in appendix 19.2 of RFC 2616 [18].

The file repair response message consists of HTTP header and one or more complete files.

The HTTP header shall provide:

-	HTTP status code, set to 200 OK for the case of a successful response.

-	Content type shall be set to multipart/related

NOTE:	Other HTTP headers (RFC 2616 [18]) may also be used but are not mandated by this mechanism.

The server shall encapsulate the requested files into a multipart mime container. Each part of the multipart mime shall contain at least the Content-Location of the embedded files.

In the error case where a UE determines that the its selected file repair server is not responding it shall return to the serverURI list of repair servers and uniformly randomly select another server from the list, excluding any servers it has determined are not responding. All the repair requests message(s) from that UE shall then be immediately sent to the newly selected file repair server.

If all of the repair servers from the serverURI list are determined to be not responding, the UE may attempt an HTTP GET to retrieve a, potentially new, instance of the session's Associated Procedure Description; otherwise UE behaviour in this case is unspecified.

A UE determines that a file repair server is not responding if any of these conditions apply:

1.	The UE is unable to establish a TCP connection to the server.

2.	The server does not respond to any of the HTTP repair requests that have been sent by the UE (it is possible that second and subsequent repair requests are sent before the first repair request is determined to be not-responded-to).

3.	The server returns an unrecognized message (not a recognizable HTTP response).

4.	The server returns an HTTP server error status code (in the range 500 to 505).

Annex A (normative):
Syntax for Service Announcement

# A.1	Void

-

-

# A.2	JSON-based representation

## A.2.1	MBS User Service Announcement schema

Below is the schema specifying the format of MBS User Service Description instance documents using an XML-based representation. Documents following this schema shall be identified with the MIME type application/mbs-user-service-description+json as registered in clause D.5. The schema filename is TS26517_MBSUserServiceAnnouncement.yaml.

openapi: 3.0.0 info: title: 'MBS User Service Announcement' version: 1.2.0 description: ¦ MBS User Service Announcement Element units. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. externalDocs: description: 3GPP TS 26.517 V17.5.0; 5G Multicast-Broadcast User Services; Protocols and Formats url: http://www.3gpp.org/ftp/Specs/archive/26_series/26.517/ paths: /user-service-descriptions: get: operationId: discoverUserServiceDescriptions summary: 'Discover User Service Descriptions' description: 'Discover User Service Descriptions that match the supplied query filter(s). At least one filter query parameter must be included in the request URL.' parameters: - in: query name: service-class schema: type: string required: true description: 'Filter for User Service Descriptions tagged with the supplied service class term identifier expressed as a fully-qualified URI string from a controlled vocabulary' responses: '200': # OK description: "Success" content: application/json: schema: $ref: '#/components/schemas/UserServiceDescriptions' '204': # No Content (no matching User Service Descriptions) description: "No Matches Found" '500': # Internal Server Error $ref: 'TS29571_CommonData.yaml#/components/responses/500' '503': # Service Unavailable $ref: 'TS29571_CommonData.yaml#/components/responses/503' default: $ref: 'TS29571_CommonData.yaml#/components/responses/default' /user-service-descriptions/{externalServiceId}: get: operationId: retrieveUserServiceDescription summary: 'Retrieve User Service Description' description: 'Retrieve the User Service Description of a single service by supplying its external service identifier.' parameters: - name: externalServiceId in: path required: true schema: type: string description: 'The external service identifier of a User Service provisioned in the MBSF.' responses: '200': # OK description: "Success" content: application/json: schema: $ref: '#/components/schemas/UserServiceDescription' '404': # Not Found $ref: 'TS29571_CommonData.yaml#/components/responses/404' '500': # Internal Server Error $ref: 'TS29571_CommonData.yaml#/components/responses/500' '503': # Service Unavailable $ref: 'TS29571_CommonData.yaml#/components/responses/503' default: $ref: 'TS29571_CommonData.yaml#/components/responses/default' components: schemas: UserServiceDescriptions: description: 'A document announcing one or more MBS User Services.' type: object properties: version: type: integer minimum: 1 userServiceDescriptions: type: array items: $ref: '#/components/schemas/UserServiceDescription' minItems: 1 required: - userServiceDescriptions UserServiceDescription: description: 'A description of a single MBS User Service.' type: object properties: serviceIds: type: array items: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri' minItems: 1 class: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri' names: type: array items: type: object properties: name: type: string lang: type: string pattern: '^[a-zA-Z]{3}$' example: 'eng' required: - name - lang minItems: 1 descriptions: type: array items: type: object properties: description: type: string lang: type: string pattern: '^[a-zA-Z]{3}$' example: 'eng' required: - description - lang minItems: 1 serviceLanguage: type: string pattern: '^[a-zA-Z]{3}$' example: 'eng' sessionStart: $ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime' sessionEnd: $ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime' distributionSessionDescriptions: type: array items: $ref: '#/components/schemas/DistributionSessionDescription' scheduleDescription: $ref: '#/components/schemas/ScheduleDescription' appServiceDescription: $ref: '#/components/schemas/AppServiceDescription' required: - serviceIds - class - distributionSessionDescriptions DistributionSessionDescription: type: object properties: distributionMethod: $ref: '#/components/schemas/DistributionMethod' conformanceProfiles: type: array items: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri' minItems: 1 sessionDescriptionLocator: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri' postSessionObjectRepairParameters: $ref: '#/components/schemas/ObjectRepairParameters' mbsAppService: type: array items: $ref: '#/components/schemas/ApplicationService' minItems: 1 unicastAppServices: type: array items: $ref: '#/components/schemas/ApplicationService' minItems: 1 availabilityInfos: type: array items: $ref: '#/components/schemas/AvailabilityInformation' minItems: 1 securityDescription: $ref: '#/components/schemas/SecurityDescription' required: - distributionMethod - sessionDescriptionLocator DistributionMethod: anyOf: - type: string enum: [OBJECT, PACKET] - type: string description: > This string provides forward-compatibility with future extensions to the enumeration but is not used to encode content defined in the present version of this API. AppServiceDescription: type: object properties: entryPointLocator: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri' contentType: type: string pattern: '^[a-zA-Z]+\/[a-zA-Z]+$' example: 'application/dash+xml' ApplicationService: type: object properties: basePattern: type: string required: - basePattern AvailabilityInformation: type: object properties: mbsServiceArea: type: array items: $ref: 'TS29571_CommonData.yaml#/components/schemas/MbsServiceArea' mbsFSAId: $ref: 'TS29571_CommonData.yaml#/components/schemas/MbsFsaId' radioFrequency: type: array items: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger' ObjectRepairParameters: type: object required: - offsetTime properties: serviceLocators: type: array items: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri' offsetTime: $ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec' randomTimePeriod: $ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec' objectDistributionBaseLocator: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri' objectRepairBaseLocator: $ref: 'TS26512_CommonData.yaml#/components/schemas/AbsoluteUrl' ScheduleDescription: type: array items: $ref: '#/components/schemas/ServiceSchedule' ServiceSchedule: type: object properties: sessionSchedule: $ref: '#/components/schemas/SessionSchedule' sessionScheduleOverride: $ref: '#/components/schemas/SessionScheduleOverride' objectSchedule: $ref: '#/components/schemas/ObjectSchedule' serviceId: type: string serviceClass: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri' required: - sessionSchedule - serviceId - serviceClass SessionSchedule: type: array items: type: object properties: start: $ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime' stop: $ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime' reoccurencePattern: type: string numberOfTimes: type: integer minimum: 1 reoccurenceStopTime: type: string index: type: integer fDTInstanceLocator: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri' required: - start - stop SessionScheduleOverride: type: array items: type: object properties: start: $ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime' stop: $ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime' index: type: integer cancelled: type: boolean sessionDescriptionLocator: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri' ObjectSchedule: type: array items: type: object properties: objectLocator: $ref: 'TS29571_CommonData.yaml#/components/schemas/Uri' sessionId: type: string objectEtag: type: string unicastOnly: type: boolean deliveryInfo: type: array items: type: object properties: start: $ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime' stop: $ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime' SecurityDescription: type: object properties: mBSSFAddresses: type: array items: $ref: 'TS26512_CommonData.yaml#/components/AbsoluteUrl' minItems: 1 mBSServiceKeyInfo: type: object properties: mBSId: type: string mBSDomainId: type: string required: - mBSId - mBSDomainId uICCKeyManagement: type: boolean 2GGBAallowed: type: boolean backOffParameters: type: object properties: offsetTime: $ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec' randomTimePeriod: $ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec' required: - mBSSFAddresses - mBSSessionKeyInfo

Annex B (informative):
Service Announcement examples

# B.1	Void

# B.2	JSON-based representation

{ "bundleDescription":[ { "userServiceDescription":{ "name":[ "test1" ], "serviceLanguage":[ "en-us" ], "serviceId":"urn:test:test:D4-Service:D4-SB:D4-US", "distributionSessionDescription":{ "conformanceProfile":"urn:3gpp:...", "sessionDescriptionURI":"http://www.test.com/D4-Service/D4-SB/D4-US.sdp", "dataNetworkName":"media-dnn", "mbsAppService":[ {"basePattern":"http://www.test.com/D4-Service/D4-SB/D4-US/video/2048/"}, {"basePattern":"http://www.test.com/D4-Service/D4-SB/D4-US/audio/1/"} ], "unicastAppServices":[ {"unicastAppService":[ {"basePattern":"http://www.test.com/D4-Service/D4-SB/D4-US/video/1024/"}, {"basePattern":"http://www.test.com/D4-Service/D4-SB/D4-US/audio/1/"}] }, {"unicastAppService":[ {"basePattern":"http://www.test.com/D4-Service/D4-SB/D4-US/video/2048/"}, {"basePattern":"http://www.test.com/D4-Service/D4-SB/D4-US/audio/1/"}] } ] }, "mbsAppService":{ "MediaManifestDescriptionURI":"http://www.test.com/D4-Service/D4-SB/D4-US/adpd.xml", "mimeType":"application/dash+xml;profiles=urn:3GPP:PSS:profile:DASH10", "identicalContents":[ { "identicalContent":[ {"basePattern":"http://www.test.com/D4-Service/D4-SB/D4-US/video/1024/"}, {"basePattern":"http://www.test.com/D4-Service/D4-SB/D4-US/video/2048/"} ], },{ "identicalContent":[ {"basePattern":"http://www.test.com/D4-Service/D4-SB/D4-US/audio/1/"} ] }] }, "availabilityInfo":[ { "infoBinding":{ "mbsServiceArea":[ { "ncgiList":[ { "NcgiTai":{ "tai":{ "plmnId":{ "mcc":"860", "mnc":"15" }, "tac":"0fa0" }, "cellList":[ { "Ncgi":{ "plmnId":{ "mcc":"860", "mnc":"15" }, "nrCellId":"999999999" } }, { "Ncgi":{ "plmnId":{ "mcc":"860", "mnc":"15" }, "nrCellId":"999999998" } } ] } } ], "taiList":[ { "tai":{ "plmnId":{ "mcc":"860", "mnc":"15" }, "tac":"0fa0" } }, { "tai":{ "plmnId":{ "mcc":"860", "mnc":"15" }, "tac":"0fa0" } } ] } ], "mbsFSAId":[ "25532" ], "radioFrequency":[ "9410" ] } } ] } } ] }

Annex C (normative):
Controlled vocabulary of conformance profiles

The controlled vocabulary is for future study.

Annex D (normative):
Syntax for object manifest

# D.1	Object manifest schema

Below is the formal syntax of the object manifest for use with the Object Collection or Object Carousel operating mode. The schema shall have the filename "TS26517_MBSObjectManifest.yaml".

openapi: 3.0.0 info: title: MBS User Services Object Manifest version: 1.0.0 description: ¦ MBS User Services Object Manifest syntax © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. tags: - name: MBS User Services Object Manifest description: '5G Media Streaming: Common Data Types' externalDocs: description: 'TS 26.517 V17.3.0; 5G Multicast-Broadcast User Services; Protocols and Formats' url: 'https://www.3gpp.org/ftp/Specs/archive/26_series/26.517/' paths: {} components: schemas: ObjectManifest: type: object description: A manifest describing a set of binary objects to be transmitted by the MBSTF as part of the MBS Distribution Session. required: - objects properties: updateInterval: type: integer format: int32 description: The time period (in seconds) after which the MBSTF attempts to re-acquire the object manfiest when pull-based object acquisition is provisioned. objects: type: array description: The list of binary objects to be carouselled from the MBSTF to the MBSTF Client. items: $ref: '#/components/schemas/Object' Object: type: object description: A binary object to be transmitted by the MBSTF as part of the MBS Distribution Session. required: - locator properties: locator: $ref: 'TS26512_CommonData.yaml#/components/schemas/AbsoluteUrl' repetitionInterval: type: integer format: int32 description: The MBSTF sends the object repeatedly to the MBSTF Client with the given interval (in milliseconds). This parameter is ignored in the case of Object Collection operating mode. keepUpdatedInterval: type: integer format: int32 description: The MBSTF checks for changes to the object with the given interval (in seconds). This parameter is ignored in the case of Object Collection operating mode. earliestFetchTime: type: string format: date-time description: The MBSTF shall pull each object from its origin location no sooner than this time or, if this parameter is omitted, at a time of its choosing. latestFetchTime: type: string format: date-time description: The MBSTF shall pull each object from its origin location no later than this time, or, if this parameter is omitted, at a time of its choosing.

Annex E (normative):
IANA registration

# E.1	General

This annex provides the formal registrations of MIME media types for different resources specified in the present document. It is referenced from the IANA registry at http://www.iana.org/.

# E.2	Registration of MIME media type "application/mbs-user-service-description+json"

## E.2.1	General

The MIME media type application/mbs-user-service-description+json denotes that the message body is an MBS User Service Description instance document compliant with the YAML schema specified in clause A.2.1.

Table E.2.1-1 provides the MIME media type registration for application/mbs-user-service-description+json.

Table E.2.1-1: MIME media type registration for application/mbs-user-service-description+json

| Parameter | Value |
| --- | --- |
| MIME media type name | application |
| MIME subtype name | mbs-user-service-description+json |
| Required parameters | None |
| Optional parameters | The 'profiles' parameter as specified in clause E.2.2. |
| Encoding considerations | This is a JSON document, and the encoding considerations are the same as for media type application/json defined in IETF RFC 8259. |
| Security considerations | This media format is used to configure the receiver on how to participate in a service. This format is highly susceptible to manipulation or spoofing for attacks desiring to mislead a receiver about a session. Both integrity protection and source authentication are recommended to prevent misleading of the receiver. |
| Interoperability considerations | The specification defines a platform-independent expression of an entry point document, and it is intended that wide interoperability can be achieved. |
| Published specification | 3GPP TS 26.517 |
| Applications which use this media type | 3GPP MBS-based applications and services |
| Additional information | File extension(s): json Intended usage: COMMON |
| Other information/general comment | None |
| Person & email address to contact for further information | Thomas Stockhammer (tsto@qti.qualcomm.com) 3GPP TSG SA WG4 |
| Restrictions on usage | None |
| Author/Change controller | 3GPP TSG SA WG4 |

## E.2.2	Profiles parameter

Table D.2.2-1 provides the definition of the profiles parameter to be used with the MBS User Service Description instance document as defined in clause D.5.1.

Table E.2.2-1: Definition of profiles parameter

| Parameter | Value |
| --- | --- |
| Parameter name | profiles |
| Parameter value | Optional attribute indicating one or more profiles to which the resource representation claims conformance. The contents of this attribute shall conform to either the pro-simple or pro-fancy productions specified in section 4.5 of IETF RFC 6381. The set of profile identifiers indicated in this parameter should match the set indicated in the profiles attribute of the corresponding User Service Description. |

EXAMPLE:

application/mbs-user-service-description+json;profiles="1,2"

Annex F (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-02 | SA4#117-e | S4-200141 |  |  |  | Initial skeleton document. | 0.0.1 |
|  |  | S4-220285 |  |  |  | Revised skeleton document | 0.1.0 |
| 2022-03 | SA4#117-e | SP-220249 |  |  |  | Presentation for information at SA#95-e | 1.0.0 |
| 2022-04 | SA4#118-e | S4-220521 |  |  |  | S4-220570: Service Announcement specification and schemas. S4-220470: Packet Distribution Method initial specification. S4-220471: Object Distribution Method initial specification | 1.1.0 |
| 2022-05 | SA4#119-e | S4-220867 |  |  |  | S4-220864: Service Announcement corrections. S4-220865: Object Distribution Method updates. S4-220866: Packet Distribution Method updates. | 1.2.0 |
| 2022-06 | SA#96 | SP-220605 |  |  |  | For presentation to Plenary | 2.0.0 |
| 2022-06 | SA#96 | SP-220605 |  |  |  | Under Change Control | 17.0.0 |
| 2022-12 | SA#98-e | SP-221059 | 0003 | 3 | F | [5MBP3] Alignment of User Service Announcement with Stage 2 | 17.1.0 |
| 2023-03 | SA#99 | SP-230254 | 0006 | 1 | F | [5MBP3] Corrections on Headings and Terms | 17.2.0 |
| 2023-06 | SA#100 | SP-230744 | 0007 | 7 | F | [5MBP3] Manifest format for Object Collection and Carousel | 17.3.0 |
| 2023-09 | SA#101 | SP-230916 | 0010 | 3 | F | [5MBP3] API for unicast retrieval of MBS User Service Announcement | 17.4.0 |