3GPP TSG-SA4 Meeting #0-e (AH) MBS SWG post 128	S4aI240068

# Online, , 6th Jun 2024 - 11th Jul 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.802 | CR | 0002 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_AMD] MBS User Service and Delivery Protocols for eMBMS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Germany |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_AMD |  |  |  |  |  | Date: |  |  | 2024-06-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The MBS User Service architecture and protocol follows the modern design philosophies of the 5G System with separation of user services from transport, a service-based architecture and RESTful APIs. At the same time, eMBMS and enTV as used for LTE-based 5G Broadcast support a transparent delivery mode. While interworking in between MBMS and MBS is addressed in TS 23.247, interworking between these two systems at the User Service level is not addressed. In order for MBMS and LTE-based 5G broadcast to leverage MBS User Service technologies, a study is warranted to identify the gaps to fully support this functionality. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addresses the work item objectives for this key issue - Documents the key issue in more detail, in particular how they relate to the 3GPP Media Delivery architecture and/or the MBS User Service architecture - Studies collaboration scenarios between the Application Service Provider and the 5G System and for each of the key topics. - Based on existing architectures, provides one or more deployment architectures that address the key topics and the collaboration models. - Maps the key topics to basic functions and develop high-level call flows. - Identifies the issues that need to be solved. - Provides candidate solutions including call flows, protocols and APIs for each of the identified issues. Identifies gaps and recommend potential normative work for stage-2 and stage-3, including which existing specifications would be impacted and/or if any new specifications would preferably be developed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.10 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TR 26.802 CR 0001 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Updated scope, references and abbreviations are in CR 0001 References [X] Abbrevations: XXX XXXX |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## =====  CHANGE =====

## 5.10	Key Issue #9: MBS User Service and Delivery Protocols for eMBMS

### 5.10.1	Description

The MBS User Service architecture and protocol follows the modern design philosophies of the 5G System with separation of user services from transport, a service-based architecture and RESTful APIs. At the same time, eMBMS and enTV as used for LTE-based 5G Broadcast support a transparent delivery mode. While interworking in between MBMS and MBS is addressed in TS 23.247, interworking between these two systems at the User Service level is not addressed. In order for MBMS and LTE-based 5G broadcast to leverage MBS User Service technologies, a study is warranted to identify the gaps to fully support this functionality.

Editor’s Note: Background – Interworking of MBS and MBMS in TS 23.247, clause 5.2

Open Questions

-	Extensions to MBS User Service to support interworking

-	Operation of MBS delivery protocols on top of MBMS an 5G Broadcast

-	Extension of MBS User service protocols to address common issues and having a common delivery protocol specification for multicast and broadcast.

-	Specific needs from 5G Broadcast?

### 5.10.2	Collaboration Scenarios

Editor’s Note: Study collaboration scenarios between the 5G System and Application Provider for each of the key topics.

### 5.10.3	Architecture Mapping

Editor’s Note: Based on existing architectures, develop one or more deployment architectures that address the key topics and the collaboration models.

### 5.10.4	High-level Call Flow

Editor’s Note: Map the key topics to basic functions and develop high-level call flows.

### 5.10.5	Gap Analysis and Requirements

Editor’s Note: Identify the issues that need to be solved.

### 5.10.6	Candidate Solutions

Editor’s Note: Provide candidate solutions including call flows, protocols and APIs for each of the identified issues.

### 5.10.7	Summary and Conclusions