## 3GPP TSG-SA4 Meeting #128																       S4-241273

# Jeju, Korea, 20-24 May 2024															(revision of S4-241230)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.804 | CR | 0006 | rev | 5 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_AMD] Multi-CDN and Multi-Access Media Delivery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Dolby France SAS, Huawei Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_AMD |  |  |  |  |  | Date: |  |  | 2024-05-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Document the FS_AMD key topic “c) Multi-Access and Multi-CDN Delivery” description and collaboration scenarios. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of new clause 5.x Multi-CDN and Multi-Access Media Delivery including sub-clause structure and headings. Addition of prose for Description and Collaboration Scenarios. Dynamic content publishing from distributed sources, presented for discussion in S4al240063 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | FS_AMD objectives not achieved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.x (NEW) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S4-240591: SA4#127-bis-e, description and collaboration scenarios S4-240844: updates with comments received at SA4#127-e-bis S4aI240052: updated to match proposed template and address pending comments. S4al240894: S4al240052 endorsed at SA4 post 127-bis-e and comments/changes accepted for ease of reading. Updates with comments received at SA4 post 127-bis-e and introduction of candidate solutions. More background on candidate solution is contained in S4-240895. Multi-access media delivery overview and collaboration scenarios have been moved to S4-241082. S4-241230: updates to address pending comments. S4-241273: updates to address pending comments. |  |  |  |  |  |  |  |  |

## FIRST CHANGE

## 2	References

[DASH9]	Draft Text of ISO/IEC FDIS 23009-9 Information technology - Dynamic adaptive streaming over HTTP (DASH) – Part 9: Redundant Encoding and Packaging for segmented live media (REaP), ISO/IEC JTC 1/SC 29/WG 3 NO 1165, Jan. 26, 2024. [Online]. Available: https://www.mpeg.org/standards/MPEG-DASH/9/

[UNPKG24]	Emma Roth, "A popular open-source content delivery network went down for hours", The Verge, Apr. 12, 2024. [Online]. Available: https://www.theverge.com/2024/4/12/24128276/open-source-unpkg-cdn-down (accessed May 9, 2024).

[NET23]	Sebastian Moss, "Cloudflare recovers from service outage after power failure at core North American data center", Data Center Dynamics, Nov. 3, 2023. [Online]. Available: https://www.datacenterdynamics.com/en/news/cloudflare-recovers-from-service-outage-after-power-failure-at-core-north-american-data-center/ (accessed May 9, 2024).

[FSLY21]	Brian Barrett, "How an Obscure Company Took Down Big Chunks of the Internet", Wired, Jun. 8, 2021. [Online]. Available: https://www.wired.com/story/fastly-cdn-internet-outages-2021/ (accessed May 9, 2024).

[AKAM21]	Josh Fomon, "CDN Provider Akamai Takes Down Popular Internet Services During Outage", Ookla, Jul. 22, 2021. [Online]. Available: https://www.ookla.com/articles/akamai-outage-july-22-2021 (accessed May 9, 2024).

[NET22]	Charlotte Trueman, "Cloudflare outage brings hundreds of sites, services temporarily offline", Computer World, Jun. 21, 2022. [Online]. Available: https://www.computerworld.com/article/1627967/cloudflare-outage-brings-hundreds-of-sites-services-temporarily-offline.html (accessed May 9, 2024).

[VZ19]	Jim Salter, "The Internet broke today: Facebook, Verizon, and more see major outages", Ars Technica, Jul. 3, 2019. [Online]. Available: https://arstechnica.com/information-technology/2019/07/facebook-cloudflare-microsoft-and-twitter-suffer-outages/ (accessed May 9, 2024).

[DEMX01]	Marc Hoppner, "A content owner, a CDN and a player walk into a bar", (Jan. 6, 2023). Accessed: May 9, 2024. [Online Video]. Available: https://www.youtube.com/watch?v=S9EdoQFOQ9I&list
=PLkyaYNWEKcOf98lZxnCcL6y7ZIVU3oSYO&index=12

[DEMX02]	Guillaume du Pantavice, "Improving streaming experience with Bayesian optimization, from AB to AZ test", (Dec. 25, 2021). Accessed: May 9, 2024. [Online Video]. Available: https://www.youtube.com/
watch?v=t4nRrLygVwo&list=PLkyaYNWEKcOfD1GYFxFbZXDP03XM-cZPg&index=19

[IEEE01]	E. Ghabashneh and S. Rao, "Exploring the interplay between CDN caching and video streaming performance", IEEE INFOCOM 2020 – IEEE Conference on Computer Communications, Toronto, ON, Canada, 2020, pp. 516-525.

[ACM01]	K. Vermeulen, L. Salamatian, S. H. Kim, M. Calder, and E. Katz-Bassett, "The central problem with distributed content: common CDN deployments centralize traffic in a risky way", In Proceedings of the 22nd ACM Workshop on Hot Topics in Networks (HotNets ’23). Association for Computing Machinery, New York, NY, USA, 70-78.

[MHV01]	A. Bentaleb, R. Farahani, F. Tashtarian, C. Timmerer, H. Hellwagner, and R. Zimmermann, "Which CDN to Download From? A Client and Server Strategies", (Jan. 6, 2024). Accessed: May 9, 2024. [Online Video]. Available: https://www.youtube.com/watch?v=xCZmCnWgQRE

[VAS01]	Will Law, "Content steering with MPEG DASH", (May 4, 2023). Accessed: May 9, 2024. [Online Video]. Available: https://www.youtube.com/watch?v=o9Pa5y-Usxw

[MWS23]	W. Law and Y. Reznik, "DASH content steering at scale", Media Web Symposium (MWS’23), June 2023.

[DIFCS]	ETSI TS 103 998: "DASH-IF: Content steering for DASH".

[CMMF]	ETSI TS 103 973: "Coded multisource media format (CMMF) for content distribution and delivery".

[TS26510]	3GPP TS 26.510: "Interactions and APIs for provisioning and media session handling (Release 18)".

[TS26512]	3GPP TS 26.512: "5G Media streaming (5GMS) protocols (Release 18)".

[MHV02]	Roberto Ramos-Chavez, Espen Braastad, Jamie Fletcher, and Arjen Wagenaar. 2024. Standards based OTT A/B Watermarking at Scale. In Proceedings of the 3rd Mile-High Video Conference (MHV '24). Association for Computing Machinery, New York, NY, USA, 94. https://doi.org/10.1145/3638036.3640251

## SECOND CHANGE

## 5.x	Multi-CDN media delivery

### 5.x.1	Description

#### 5.x.1.1	Introduction

Media streaming applications traditionally obtain content from a single source over a single path within a network. This imposes several limitations:

1.	Performance is constrained to that of the source and path chosen. Whatever the limits on network bandwidth and latency between the client and that source are directly translated to the client’s achievable Quality of Service (QoS) and Quality of Experience (QoE).

2	Disruptions or degraded performance caused by the source in use or on any of the network links between the client and source can lead to poor user experience, often in the form of lower playback quality, rebuffering, or complete playback failure.

This study considers integration of different technologies into the 5G Media Streaming System that addresses these, and similar, issues by allowing media streaming applications to efficiently access content across multiple Content Delivery Networks (CDNs) and/or multiple access networks. Different client implementations may then beneficially use the content on these multiple sources or networks concurrently, potentially guided by service or network provider. In addition, formats and techniques for generating content for multiple CDN or multiple access network delivery such as MPEG-DASH Part 9 (ReAP) [DASH9] may be considered. Further extensions include the ability for a client to use multiple access networks at the same time to support media delivery. Study of integration of different technologies into the 5G Media Streaming System is of relevance to address content provisioning, content hosting, impacts on user plane reference points M2 and M4, and on media session handling at reference point M5 as well as potential benefits in terms of quality and resource usage.

#### 5.x.1.2	Challenges Multi-CDN deployments aim to address

CDNs are often used by content distributors to globally scale delivery of their content to end-users. These networks consist of a number of Points of Presence (PoPs) located at various locations around the networks’ edge. These PoPs help load-balance delivery of content as well as improve Quality of Service (QoS) by reducing the distance/latency between every client and the content they are accessing. In many cases, content distributors employ multiple CDNs to leverage the strengths of one CDN over another in every location those CDNs have a PoP. For example, a client experiencing degraded performance while using one CDN may switch to another that is offering better performance at that time and location. As another example, a content distributor may prefer one CDN over another at a given time to reduce delivery costs and/or meet monthly contractual commitments. These Multi-CDN deployments aim to solve content delivery issues that exist when only one CDN is used; but the benefits they provide may not be fully realized because of the various challenges experienced and underlying methods used to stream content to every client.

Challenges Multi-CDN deployments and architectures aim to address may include:

1.	Sustained CDN-/network-wide service disruptions where network access, connectivity or QoS is severely degraded. Examples may include cases where an entire CDN’s network is degraded because of a network-wide misconfiguration or power failure. The duration of these events may last minutes to hours and affect a majority of the client population. Examples of recorded instances can be found in [UNPKG24], [NET23], [FSLY21], [AKAM21], [NET22], and [VZ19].

2.	Intermittent or short-term disruptions affecting QoS for an individual or small group of clients. Examples include short periods of congestion within the network, isolated HTTP request/response failures or delays caused by application server congestion, etc. The following discusses these in greater detail: [DEMX01], [DEMX02], [IEEE01], [ACM01], [MHV01], [VAS01], and [MWS23]

3.	Augmentation of one CDN's performance with that of another to achieve a level of performance that neither can provide on its own. An example is a peer-to-peer CDN where each peer has limited uplink capacity and is unable to satisfactorily service client demand on its own.

-

### 5.x.2	Collaboration scenarios

#### 5.x.2.1	Multi-CDN media delivery

In this scenario, the 5GMSd Client requests adaptive media streaming content from two or more 5GMSd Application Servers. The Client may choose one 5GMSd AS or use multiple simultaneously. This allows the client to distribute network load across Application Servers and M4 downlink transports, optimize costs, as well as improve QoS.

The client’s Media Session Handler discovers the URLs of these Application Servers from the 5GMSd Application Function (AF), either through a Media Entry Point or from a separate piece of metadata. QoE metrics from the client may be used by the AF to determine the best Application Server(s) for each client to use when streaming media.

Figure 5.x.2.1-1 shows the client communicating with multiple Application Servers. Each AS has no direct communication with its peers; rather it communicates (minimally) with the Application Provider and with the 5GMSd AF (not depicted) via reference point M3d.

Figure 5.x.2.1-1: Multi-CDN media delivery within 5G system

#### 5.x.2.2	Multi-access media delivery

These scenarios address integration of different technologies into the 5G Media Streaming System that allow media streaming applications to efficiently access content across multiple access networks. These scenarios are expanded upon in clause 5.x.

Editor’s Note:	These collaboration scenarios are proposed as a separate CR to TR 26.804 labelled Multi-access media delivery. References to the appropriate clauses should be updated accordingly once clause numbers have been finalized.

-

#### 5.x.2.3	Joint multi-CDN and multi-access media delivery

In this scenario, the 5GMSd Client is directly connected to multiple data, or access, networks (e.g., an unmanaged Wi-Fi network and the 5G network), as described in clause 5.x.2.2. The client requests adaptive media streaming content from two or more 5GMSd Application Servers. The Client may choose one or use multiple simultaneously. This allows the client to distribute network load across access networks and Application Servers, optimize costs, as well as improve QoS.

The client’s Media Session Handler discovers the URLs of these Application Servers from the 5GMSd Application Function (AF), either through a Media Entry Point or from a separate piece of metadata. QoE metrics from the client may be used by the AF to determine the best Application Server(s) for each client to use when streaming media.

Figure 5.x.2.3-1 shows the client communicating with multiple Application Servers through different data networks. Neither data network nor AS has direct communication with its peers. Rather each 5GMSd AS communicates (minimally) with the Application Provider at reference point M2 and with the 5GMSd AF (not depicted) via reference point M3d.

Figure 5.x.2.3-1: Multi-access media delivery within 5G system

### 5.x.3	Architecture mapping

### 5.x.4	High-level call flow

### 5.x.5	Gap analysis and requirements

#### 5.x.5.1       Dynamic Content Publishing from multiple sources (CDN)

When streaming media presentations are originated from different sources, such as in the multi-CDN case, consistency of the media presentations is important.

This is trivial to achieve for the case of static non-changing content.

However, live media streaming presentation get updated frequently over time, resulting in different update media presentation descriptions (MPD), updated media playlists (for HTTP Live Streaming case), and different segments becoming  available or no longer being available.

The support for such live streaming cases is important for multi-CDN scenarios, as it is desirable that both media segments and media presentation descriptions are interchangeable even when they originate from different sources.

In practice, AS/CDN source A may have a slightly delayed version of a presentation compared to AS/CDN source B resulting in different segment availability, and potentially conflicting media presentation descriptions and media playlists at between the different AS/CDN sources.

Such timing differentiations may happen upstream in the workflow due to different delays or differently configured upstream components. Even if the delays are relatively modest, still discrepancies between sources may occur that are disruptive to the user experience.

Practice has shown, that players of streaming media presentations are sensitive to such issues. For example, retroactive changes to the media timeline may cause playback failure.

In Figure YY we show the example case of a retroactive timeline changes caused by switching to an AS 2 that has a delayed version of the dynamic media presentation.

In step 1 the media presentation description is received with given MPD@publishTime, earliest media presentation time EPT, last segment presentation time LPT in the DVR window. The playback starts and after an MPD update request the DVR window is updated without problems as the update is consistent as it comes from the same sources. In a subsequent request the player switches to an alternative AS 2 and this has delayed input, now an updated MPD is received with a later publish time, but the earliest presentation time of the newest segment is decreased and the timeline is changed retro-actively.

As a consequence the playback is interrupted as players can usually not handle such cases.

Other related issues that may occur due to different source versions may be different timeline gaps in the media presentation and overall differences in the segment availability between subsequent updates.

Figure YY Sample case of playback failure in multi-CDN delivery

#### 5.x.5.2       Examples of inconsistent Content Publishing from multiple sources (CDN)

In this clause we provide some examples how inconsistent timelines can occur in dynamic content publishing, even if the segments are interchangeable and both source contents have an identical timeline. The cause can be that content publishing origin B gets a delayed input compared to content publishing origin A.

Each dynamic content publishing origin generates the media playlist description based on its input and sets the publishTime based on the system clock time that is synchronized to global timing systems.

In this case the first MPD is retrieved from origin A that is ahead, in this case the newest segment time is

1031634228096 + 1152 x (312 + 1) = 1031634588672

Then the player receives the MPD from content publishing origin B in a delayed data center it gets an MPD with a later publish time, but the start time of the newest segment is:

1031634226944 + 1152 x  (312 + 1) = 1031634587520

Which is earlier, this implies that the newest segment from the MPD from origin A is not the MPD, but the publishtime is later, leading to a retro-active timeline change interpreted by the player.

Table 1  Snapshot of publishTime and segmentTimeline in MPD retrieved from origin A

<?xml version="1.0" encoding="utf-8"?> <MPD xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xmlns="urn:mpeg:dash:schema:mpd:2011" xsi:schemaLocation="urn:mpeg:dash:schema:mpd:2011 http://standards.iso.org/ittf/PubliclyAvailableStandards/MPEG-DASH_schema_files/DASH-MPD.xsd" type="dynamic" availabilityStartTime="1970-01-01T00:00:00Z" publishTime="2024-06-26T08:36:24.332711Z" minimumUpdatePeriod="PT2S" timeShiftBufferDepth="PT10M" maxSegmentDuration="PT2S" minBufferTime="PT10S" profiles="urn:mpeg:dash:profile:isoff-live:2011,urn:com:dashif:dash264"> <Period id="1" start="PT0S"> <AdaptationSet …… <SegmentTemplate timescale="600" initialization="live-$RepresentationID$.dash" media="live-$RepresentationID$-$Time$.dash"> <!-- 2024-06-26T08:26:20.160000Z / 1719390380 - 2024-06-26T08:36:21.120000Z --> <SegmentTimeline> <S t="1031634228096" d="1152" r="312" /> </SegmentTimeline> …… </MPD>

Table 2 Snapshot of publishTime and segmentTimeline in MPD retrieved from origin B

<MPD xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xmlns="urn:mpeg:dash:schema:mpd:2011" xsi:schemaLocation="urn:mpeg:dash:schema:mpd:2011 http://standards.iso.org/ittf/PubliclyAvailableStandards/MPEG-DASH_schema_files/DASH-MPD.xsd" type="dynamic" availabilityStartTime="1970-01-01T00:00:00Z" publishTime="2024-06-26T08:36:25.378162Z" minimumUpdatePeriod="PT2S" timeShiftBufferDepth="PT10M" maxSegmentDuration="PT2S" minBufferTime="PT10S" profiles="urn:mpeg:dash:profile:isoff-live:2011,urn:com:dashif:dash264"> <Period id="1" start="PT0S"> <AdaptationSet …. <SegmentTemplate timescale="600" initialization="live-$RepresentationID$.dash" media="live-$RepresentationID$-$Time$.dash"> <!-- 2024-06-26T08:26:18.240000Z / 1719390378 - 2024-06-26T08:36:19.200000Z --> <SegmentTimeline> <S t="1031634226944" d="1152" r="312" /> </SegmentTimeline> …. </MPD>

### 5.x.6	Candidate solutions

#### 5.x.6.1	Server-side CDN switching

These candidate solutions include approaches where a media streaming client or population of clients changes or switches between two or more CDNs based on recommendations from a remote server. An example includes the DASH Industry Forum’s content steering architecture [DIFCS].

Editor’s Note:	Inclusion and expansion on these sets of solutions is dependent on interest from working group.

#### 5.x.6.2	Client-side CDN switching

These candidate solutions include approaches where a media streaming client changes or switches between two or more CDNs based on decisions made locally.

Editor’s Note:	Inclusion and expansion on these sets of solutions is dependent on interest from working group.

#### 5.x.6.3	Concurrent CDN access using CMMF

This candidate solution includes approaches where a 5GMSd Client accesses and downloads, via reference point M4, CMMF-encoded media objects [CMMF], and possibly original source media (e.g., MPEG-DASH or HLS media segments), from two or more 5GMSd Application Servers simultaneously. Additionally, the 5GMSd Client may access these 5GMSd Application Servers over different access networks (such as 3GPP and non-3GPP access networks).

In this solution, support for multi-CDN media delivery can be realized by the following procedures:

1.	CMMF-encoded media objects, and possibly original source media (e.g., MPEG-DASH or HLS media segments), are striped across multiple 5GMSd Application Servers. The 5GMSd Application Provider may make the CMMF-encoded media objects, and possibly original source media (e.g., MPEG-DASH or HLS media segments), available at reference point M2 or they may be created by the 5GMSd Application Server performing content preparation on regular media objects (e.g., MPEG-DASH or HLS media segments) that have been ingested from the 5GMSd Application Provider at reference point M2.

2.	Upon initialization of a playback session, the 5GMSd Media Client’s Media Session Handler obtains relevant Service Access Information from the 5GMSd Application Function at reference point M5. At a minimum, this includes details concerning the location of each 5GMSd Application Server where the CMMF-encoded and possibly original media (e.g., MPEG-DASH or HLS media segments) may be obtained, as well as appropriate signalling to indicate whether the media at each location is CMMF-encoded.

3.	The 5GMSd Media Client connects to and downloads CMMF-encoded media objects, and possibly the original source media (e.g., MPEG-DASH or HLS media segments), from each 5GMSd Application Server simultaneously via reference point M4, terminating the download from each 5GMSd AS early upon obtaining enough CMMF-encoded objects to recover the source media (e.g., MPEG-DASH or HLS media segment). Once decoded, the source media is delivered to the Media Player in the 5GMSd Media Client for presentation.

#### 5.x.6.4	Content Formatting at multiple AS sources following MPEG-DASH part-9 REaP

This solution proposed to solve the issue of dynamic content generation from multiple sources 5.x.5.y is comprised of using additional format constraints that can be applied to DASH and HLS streaming media presentations (both media segments and media playlist or media presentation description).

This solution is based on Redundant Encoding and Packaging as defined in [DASH9]. This solution defines formats for usage in the streaming head-end that typically resides in the application server enabling generation interchangeable media presentations at different sources.

In MPEG-DASH part 9 REaP this output is defined as the Delivery Media presentation Description (D-MPD), it adds constraints on the formatting and output generation of the media presentation description to make sure that outputs form different distributed origins application servers are interchangeable by a player (i.e. no retroactive timeline changes). These constraints apply to the representations, the setting of MPD@publishTime and some other attributes/elements in the MPD. A corresponding version for HTTP Live Streaming playlists is defined as well.

The solution starts from defining a reference/example head-end architecture, that in the case of 5G Media streaming may be  the AS or even components before the AS.  The key assumption is that a common contribution source (encoder) with a common timeline is used that can be converted back to a timeline relative to Unix Epoch, that is 00:00 1-1-1970 UTC.  This assumption can hold in many cases where a common contribution signal is used in the streaming head-end. This way all tracks and streams have the same timeline origin. Secondly a regular media segmentation strategy is proposed resulting in aligned media segment boundaries of the different segments.

This is used in the output of 1 or more distribution encoders, that produce ISO BMFF tracks with aligned segment boundaries and a common shared timeline relative to Unix Epoch. In addition REaP defines the I-MPD, a constrained version of the media presentation description to announce the streams that is for example used for live ingest or distribution encoder egress.

The critical part are the distributed packagers in combination with the origin as this corresponds to the output generated at the AS or multiple AS’s in the multi-source dynamic content generation case.

For this case REaP defines the Delivery MPD (D-MPD) that constrains the formatting of the output MPD by linking some of the fields in DASH MPD that can cause retroactive changes to the media timeline. For example, by linking the MPD@publishTime explicitly to the earliest presentation time of the newest segment, and constraining the way the set of available segments are updated, it is possible to generate consistent media presentation description or HTTP live streaming playlists.

The Approach in REaP can work in modestly well synchronized AS’s up to 100 ms out of sync. A key aspect for a REaP setup is to configure the expected maximum delay incurred in the workflow/AS to account for this setting MPD@publishTime as a function of the earliest media presentation time of the latest segment at the live edge.

The approach has been shown using open source based implementation demo at popular conference in [MHV02] to implement a pseudo watermarking workflow, in pseudo watermarking the 2 AS sources create interchangeable content that only differs by an embedded watermark. In this demo the watermark was shown visually on the screen, showing how the player is able to seamlessly play content generated dynamically from different sources.

The advantage of using REaP as a solution is that changes will be limited, and the output format from the AS can be easily checked for conformance using existing conformance tooling. In addition, very limited or up to no signalling is needed between AS’s making the solution very easy to adopt and maintain in practice.

In REaP this problem case shown in 5.X.2 is overcome by coupling the publish time to the media timeline in the output to enable consistent dynamic content publishing between sources. If the publish time was coupled to the media time this specific case of retro-active timeline change could have been avoided, but it requires some further configuration.

Therefore some configurations from the REaP may be considered in 5GMS, such as segment duration, the epoch used, the synchronization time stamp for linking the publishtime and media timeline. Details will be provided in a later stage.

Figure ZZ MPEG-DASH part 9 reap architecture for redundant encoding and packaging

### 5.x.7	Summary and Conclusions

## END OF CHANGES