## 3GPP TSG-SA4-e MBS SWG post #128								              								     S4al240095

# Electronic, 6th June-26th July 2024															(revision of S4-241273)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.804 | CR | 0006 | rev | 7 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_AMD] Multi-CDN and Multi-Access Media Delivery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Dolby France SAS |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_AMD |  |  |  |  |  | Date: |  |  | 2024-05-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Document the FS_AMD key topic “c) Multi-Access and Multi-CDN Delivery” description and collaboration scenarios. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of new clause 5.x Multi-CDN and Multi-Access Media Delivery including sub-clause structure and headings. Addition of prose for Description and Collaboration Scenarios. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | FS_AMD objectives not achieved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.x (NEW) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S4-240591: SA4#127-bis-e, description and collaboration scenarios S4-240844: updates with comments received at SA4#127-e-bis S4aI240052: updated to match proposed template and address pending comments. S4al240894: S4al240052 endorsed at SA4 post 127-bis-e and comments/changes accepted for ease of reading. Updates with comments received at SA4 post 127-bis-e and introduction of candidate solutions. More background on candidate solution is contained in S4-240895. Multi-access media delivery overview and collaboration scenarios have been moved to S4-241082. S4-241230: updates to address pending comments. S4-241273: updates to address pending comments. S4al240095: S4-241273 endorsed at SA4#128. Comments/changes accepted for ease of reading. CMMF candidate solution architecture and reference point descriptions added. |  |  |  |  |  |  |  |  |

## FIRST CHANGE

## 2	References

[DASH9]	Draft Text of ISO/IEC FDIS 23009-9 Information technology - Dynamic adaptive streaming over HTTP (DASH) – Part 9: Redundant Encoding and Packaging for segmented live media (REaP), ISO/IEC JTC 1/SC 29/WG 3 NO 1165, Jan. 26, 2024. [Online]. Available: https://www.mpeg.org/standards/MPEG-DASH/9/

[UNPKG24]	Emma Roth, "A popular open-source content delivery network went down for hours", The Verge, Apr. 12, 2024. [Online]. Available: https://www.theverge.com/2024/4/12/24128276/open-source-unpkg-cdn-down (accessed May 9, 2024).

[NET23]	Sebastian Moss, "Cloudflare recovers from service outage after power failure at core North American data center", Data Center Dynamics, Nov. 3, 2023. [Online]. Available: https://www.datacenterdynamics.com/en/news/cloudflare-recovers-from-service-outage-after-power-failure-at-core-north-american-data-center/ (accessed May 9, 2024).

[FSLY21]	Brian Barrett, "How an Obscure Company Took Down Big Chunks of the Internet", Wired, Jun. 8, 2021. [Online]. Available: https://www.wired.com/story/fastly-cdn-internet-outages-2021/ (accessed May 9, 2024).

[AKAM21]	Josh Fomon, "CDN Provider Akamai Takes Down Popular Internet Services During Outage", Ookla, Jul. 22, 2021. [Online]. Available: https://www.ookla.com/articles/akamai-outage-july-22-2021 (accessed May 9, 2024).

[NET22]	Charlotte Trueman, "Cloudflare outage brings hundreds of sites, services temporarily offline", Computer World, Jun. 21, 2022. [Online]. Available: https://www.computerworld.com/article/1627967/cloudflare-outage-brings-hundreds-of-sites-services-temporarily-offline.html (accessed May 9, 2024).

[VZ19]	Jim Salter, "The Internet broke today: Facebook, Verizon, and more see major outages", Ars Technica, Jul. 3, 2019. [Online]. Available: https://arstechnica.com/information-technology/2019/07/facebook-cloudflare-microsoft-and-twitter-suffer-outages/ (accessed May 9, 2024).

[DEMX01]	Marc Hoppner, "A content owner, a CDN and a player walk into a bar", (Jan. 6, 2023). Accessed: May 9, 2024. [Online Video]. Available: https://www.youtube.com/watch?v=S9EdoQFOQ9I&list
=PLkyaYNWEKcOf98lZxnCcL6y7ZIVU3oSYO&index=12

[DEMX02]	Guillaume du Pantavice, "Improving streaming experience with Bayesian optimization, from AB to AZ test", (Dec. 25, 2021). Accessed: May 9, 2024. [Online Video]. Available: https://www.youtube.com/
watch?v=t4nRrLygVwo&list=PLkyaYNWEKcOfD1GYFxFbZXDP03XM-cZPg&index=19

[IEEE01]	E. Ghabashneh and S. Rao, "Exploring the interplay between CDN caching and video streaming performance", IEEE INFOCOM 2020 – IEEE Conference on Computer Communications, Toronto, ON, Canada, 2020, pp. 516-525.

[ACM01]	K. Vermeulen, L. Salamatian, S. H. Kim, M. Calder, and E. Katz-Bassett, "The central problem with distributed content: common CDN deployments centralize traffic in a risky way", In Proceedings of the 22nd ACM Workshop on Hot Topics in Networks (HotNets ’23). Association for Computing Machinery, New York, NY, USA, 70-78.

[MHV01]	A. Bentaleb, R. Farahani, F. Tashtarian, C. Timmerer, H. Hellwagner, and R. Zimmermann, "Which CDN to Download From? A Client and Server Strategies", (Jan. 6, 2024). Accessed: May 9, 2024. [Online Video]. Available: https://www.youtube.com/watch?v=xCZmCnWgQRE

[VAS01]	Will Law, "Content steering with MPEG DASH", (May 4, 2023). Accessed: May 9, 2024. [Online Video]. Available: https://www.youtube.com/watch?v=o9Pa5y-Usxw

[MWS23]	W. Law and Y. Reznik, "DASH content steering at scale", Media Web Symposium (MWS’23), June 2023.

[DIFCS]	ETSI TS 103 998: "DASH-IF: Content steering for DASH".

[CMMF]	ETSI TS 103 973: "Coded multisource media format (CMMF) for content distribution and delivery".

[TS26501]	3GPP TS 26.501: “5G Media Streaming (5GMS) General description and architecture (Release 18)”.

[TS26510]	3GPP TS 26.510: "Interactions and APIs for provisioning and media session handling (Release 18)".

[TS26512]	3GPP TS 26.512: "5G Media streaming (5GMS) protocols (Release 18)".

## SECOND CHANGE

## 5.x	Multi-CDN media delivery

### 5.x.1	Description

#### 5.x.1.1	Introduction

Media streaming applications traditionally obtain content from a single source over a single path within a network. This imposes several limitations:

1.	Performance is constrained to that of the source and path chosen. Whatever the limits on network bandwidth and latency between the client and that source are directly translated to the client’s achievable Quality of Service (QoS) and Quality of Experience (QoE).

2	Disruptions or degraded performance caused by the source in use or on any of the network links between the client and source can lead to poor user experience, often in the form of lower playback quality, rebuffering, or complete playback failure.

This study considers integration of different technologies into the 5G Media Streaming System that addresses these, and similar, issues by allowing media streaming applications to efficiently access content across multiple Content Delivery Networks (CDNs) and/or multiple access networks. Different client implementations may then beneficially use the content on these multiple sources or networks concurrently, potentially guided by service or network provider. In addition, formats and techniques for generating content for multiple CDN or multiple access network delivery such as MPEG-DASH Part 9 (ReAP) [DASH9] may be considered. Further extensions include the ability for a client to use multiple access networks at the same time to support media delivery. Study of integration of different technologies into the 5G Media Streaming System is of relevance to address content provisioning, content hosting, impacts on user plane reference points M2 and M4, and on media session handling at reference point M5 as well as potential benefits in terms of quality and resource usage.

#### 5.x.1.2	Challenges Multi-CDN deployments aim to address

CDNs are often used by content distributors to globally scale delivery of their content to end-users. These networks consist of a number of Points of Presence (PoPs) located at various locations around the networks’ edge. These PoPs help load-balance delivery of content as well as improve Quality of Service (QoS) by reducing the distance/latency between every client and the content they are accessing. In many cases, content distributors employ multiple CDNs to leverage the strengths of one CDN over another in every location those CDNs have a PoP. For example, a client experiencing degraded performance while using one CDN may switch to another that is offering better performance at that time and location. As another example, a content distributor may prefer one CDN over another at a given time to reduce delivery costs and/or meet monthly contractual commitments. These Multi-CDN deployments aim to solve content delivery issues that exist when only one CDN is used; but the benefits they provide may not be fully realized because of the various challenges experienced and underlying methods used to stream content to every client.

Challenges Multi-CDN deployments and architectures aim to address may include:

1.	Sustained CDN-/network-wide service disruptions where network access, connectivity or QoS is severely degraded. Examples may include cases where an entire CDN’s network is degraded because of a network-wide misconfiguration or power failure. The duration of these events may last minutes to hours and affect a majority of the client population. Examples of recorded instances can be found in [UNPKG24], [NET23], [FSLY21], [AKAM21], [NET22], and [VZ19].

2.	Intermittent or short-term disruptions affecting QoS for an individual or small group of clients. Examples include short periods of congestion within the network, isolated HTTP request/response failures or delays caused by application server congestion, etc. The following discusses these in greater detail: [DEMX01], [DEMX02], [IEEE01], [ACM01], [MHV01], [VAS01], and [MWS23]

3.	Augmentation of one CDN's performance with that of another to achieve a level of performance that neither can provide on its own. An example is a peer-to-peer CDN where each peer has limited uplink capacity and is unable to satisfactorily service client demand on its own.

### 5.x.2	Collaboration scenarios

#### 5.x.2.1	Multi-CDN media delivery

In this scenario, the 5GMSd Client requests adaptive media streaming content from two or more 5GMSd Application Servers. The Client may choose one 5GMSd AS or use multiple simultaneously. This allows the client to distribute network load across Application Servers and M4 downlink transports, optimize costs, as well as improve QoS.

The client’s Media Session Handler discovers the URLs of these Application Servers from the 5GMSd Application Function (AF), either through a Media Entry Point or from a separate piece of metadata. QoE metrics from the client may be used by the AF to determine the best Application Server(s) for each client to use when streaming media.

Figure 5.x.2.1-1 shows the client communicating with multiple Application Servers. Each AS has no direct communication with its peers; rather it communicates (minimally) with the Application Provider and with the 5GMSd AF (not depicted) via reference point M3d.

Figure 5.x.2.1-1: Multi-CDN media delivery within 5G system

#### 5.x.2.2	Multi-access media delivery

These scenarios address integration of different technologies into the 5G Media Streaming System that allow media streaming applications to efficiently access content across multiple access networks. These scenarios are expanded upon in clause 5.x.

Editor’s Note:	These collaboration scenarios are proposed as a separate CR to TR 26.804 labelled Multi-access media delivery. References to the appropriate clauses should be updated accordingly once clause numbers have been finalized.

#### 5.x.2.3	Joint multi-CDN and multi-access media delivery

In this scenario, the 5GMSd Client is directly connected to multiple data, or access, networks (e.g., an unmanaged Wi-Fi network and the 5G network), as described in clause 5.x.2.2. The client requests adaptive media streaming content from two or more 5GMSd Application Servers. The Client may choose one or use multiple simultaneously. This allows the client to distribute network load across access networks and Application Servers, optimize costs, as well as improve QoS.

The client’s Media Session Handler discovers the URLs of these Application Servers from the 5GMSd Application Function (AF), either through a Media Entry Point or from a separate piece of metadata. QoE metrics from the client may be used by the AF to determine the best Application Server(s) for each client to use when streaming media.

Figure 5.x.2.3-1 shows the client communicating with multiple Application Servers through different data networks. Neither data network nor AS has direct communication with its peers. Rather each 5GMSd AS communicates (minimally) with the Application Provider at reference point M2 and with the 5GMSd AF (not depicted) via reference point M3d.

Figure 5.x.2.3-1: Multi-access media delivery within 5G system

### 5.x.3	Architecture mapping

### 5.x.4	High-level call flow

### 5.x.5	Gap analysis and requirements

### 5.x.6	Candidate solutions

#### 5.x.6.1	Server-side CDN switching

These candidate solutions include approaches where a media streaming client or population of clients changes or switches between two or more CDNs based on recommendations from a remote server. An example includes the DASH Industry Forum’s content steering architecture [DIFCS].

Editor’s Note:	Inclusion and expansion on these sets of solutions is dependent on interest from working group.

#### 5.x.6.2	Client-side CDN switching

These candidate solutions include approaches where a media streaming client changes or switches between two or more CDNs based on decisions made locally.

Editor’s Note:	Inclusion and expansion on these sets of solutions is dependent on interest from working group.

#### 5.x.6.3	Concurrent CDN access using CMMF

This candidate solution includes approaches where a 5GMSd Client accesses and downloads, via reference point M4, CMMF-encoded media objects [CMMF], and possibly original source media (e.g., MPEG-DASH or HLS media segments), from two or more 5GMSd Application Servers simultaneously. Additionally, the 5GMSd Client may access these 5GMSd Application Servers over different access networks (such as 3GPP and non-3GPP access networks).

CMMF is an extensible container format designed to facilitate the management and interchange of audio-visual media and metadata in one or more coded representations (e.g., encoded with application-layer, linear, network, or channel codes). The coded media representations supported by CMMF enable the efficient use of multisource, multipath, and multiaccess connectivity for network-delivered media applications. The use of CMMF does not replace the basic media streaming architectures and procedures already defined. Rather, it is intended to supplement them to provide additional capabilities.

Generally,

support for multi-CDN and/or multi-access media delivery using CMMF can be realized by the following:

1.	CMMF-encoded media objects, and possibly original source media (e.g., MPEG-DASH or HLS media segments), are striped across multiple 5GMSd Application Servers. The 5GMSd Application Provider may make the CMMF-encoded media objects, and possibly original source media (e.g., MPEG-DASH or HLS media segments), available at reference point M2 or they may be created by the 5GMSd Application Server performing content preparation on regular media objects (e.g., MPEG-DASH or HLS media segments) that have been ingested from the 5GMSd Application Provider at reference point M2. Detailed examples for preparing original source media for delivery from multiple Application Servers using CMMF are provided in [CMMF].

2.	Upon initialization of a playback session, the 5GMSd Media Client’s Media Session Handler obtains relevant Service Access Information from the 5GMSd Application Function at reference point M5. At a minimum, this includes details concerning the location of each 5GMSd Application Server where the CMMF-encoded and possibly original media (e.g., MPEG-DASH or HLS media segments) may be obtained, as well as appropriate signalling to indicate whether the media at each location is CMMF-encoded.

3.	The 5GMSd Media Client connects to and downloads CMMF-encoded media objects, and possibly the original source media (e.g., MPEG-DASH or HLS media segments), from each 5GMSd Application Server simultaneously via reference point M4, terminating the download from each 5GMSd AS early upon obtaining enough CMMF-encoded objects to recover the source media (e.g., MPEG-DASH or HLS media segment). Once decoded, the source media is delivered to the Media Player in the 5GMSd Media Client for presentation.

A general architecture and procedure incorporating these steps are provided within the next sub-clauses.

##### 5.x.6.3.1	Architecture for 5G Downlink Media Streaming using CMMF

##### Figure 5.x.6.3.1-1 below depicts an architecture for downlink 5G Media Streaming using CMMF that combines the functions and reference points of the 5GMS System with those of CMMF.

Figure 5.x.6.3.1-1: Architecture for downlink 5G Media Streaming using CMMF

This arrangement allows CMMF to be integrated within an existing 5GMS-based downlink media streaming system. Operation consists of the following:

- The 5GMSd AF configures the delivery of 5GMSd content to a CMMF Client in the UE via multiple 5GMSd Application Servers. Additional to clause 4.1.2 of TS 26.501 [TS26501], the 5GMSd Application Function may also be used to configure a CMMF Bitstream Generator/Source as defined in [CMMF] via the xCMMF-C reference point.

- The CMMF Bitstream Generator/Source receives source media via the M2d reference point, creates multiple CMMF bitstreams/objects (each intended for a different 5GMSd AS) as defined in [CMMF], and delivers these CMMF bitstreams/objects via multiple xCMMF-U reference points. In situations where original source media is intended to be streamed to the 5GMSd Client, the CMMF Bitstream Generator/Source may pass this media through to the 5GMSd AS unmodified.

- The 5GMSd Client acts as a CMMF-Aware Application for the CMMF Client. Thus, the CMMF Client is controlled by the 5GMSd Client via the CMMF-API-C reference point in Figure 5.x.6.3.1-1 above.

- The CMMF Client receives media from one or more 5GMSd Application Servers via multiple M4d reference points. In the case the media is encoded within CMMF bitstreams/objects as defined in [CMMF], the original source media is recovered once enough CMMF-encoded content is obtained. Upon successful recovery, any in-process CMMF Bitstream transfers are terminated. In the case where the content obtained from the 5GMSd AS is the original source media, the CMMF Client passes this content directly through to the 5GMSd Client.

- The Media Player sends requests according to the signalled object availability within the manifest to the Media Server via reference point CMMF-API-U. Once the CMMF Client obtains the original source content, the Media Server exposes this content to Media Player via the same reference point.

Figure 5.x.6.3.1-1 shows the CMMF functions as separate entities within 5GMS. Alternatively, these CMMF functions can be integrated directly within existing functions within 5GMS.

- Two alternate options exist for the CMMF Bitstream Generator/Source. The CMMF Bitstream Generator/Source may be implemented as a function within the 5GMSd Application Provider (e.g., as part of the content preparation process) or distributed within each of the 5GMSd Application Servers (i.e., the CMMF Bitstream Generator/Source within a 5GMSd AS only creates the CMMF Bitstream of the source media intended to be delivered by that 5GMSd AS).

- The CMMF Client and Media Server can be directly integrated within the 5GMSd Client with appropriate changes to both the Media Session Handler and Media Player so that CMMF is natively supported within both functions.

##### 5.x.6.3.2	Usage of 5GMS reference points for CMMF-based delivery

###### 5.x.6.3.2.1	Usage of M1d

Reference point M1d is used as defined in clauses 4.1 to 4.4 of TS 26.501 [TS26501].

In addition, the 5GMSd Application Provider may authorize via M1d that 5GMS content may be distributed via CMMF. This may also include the provisioning of the 5GMSd AF with configuration information necessary for the proper operation of CMMF (see [CMMF]).

###### 5.x.6.3.2.2	Usage of M2d

###### Reference point M2d is used as defined in clauses 4.1 to 4.4 of TS 26.501 [TS26501].

###### 5.x.6.3.2.3	Usage of M3d

Reference point M3d is used as defined in clauses 4.1 to 4.4 of TS 26.501 [TS26501].

In some implementations where the CMMF Bitstream Generator/Source is integrated within the 5GMSd AS, reference point M3d may also include additional information necessary to properly configure each 5GMSd AS to encode and deliver CMMF Bitstreams/Objects.

###### 5.x.6.3.2.4	Usage of M4d

Reference point M4d is used as defined in clauses 4.1 to 4.4 of TS 26.501 [TS26501].

###### 5.x.6.3.2.5	Usage of M5d

Reference point M5d is used as defined in clauses 4.1 to 4.4 of TS 26.501 [TS26501].

In addition, reference point M5d includes relevant information necessary for the CMMF Client to connect with and access CMMF encoded content from each of the available 5GMSd Application Servers. In some implementations, this additional information may include a list of host URLs to each of the 5GMSd Application Servers that is used by the CMMF Client to access each of the CMMF Bitstreams/Objects on each respective 5GMSd AS.

###### 5.x.6.3.2.6	Usage of M6d

Reference point M6d is used as defined in clauses 4.1 to 4.4 of TS 26.501 [TS26501].

###### 5.x.6.3.2.7	Usage of M7d

Reference point M7d is used as defined in clauses 4.1 to 4.4 of TS 26.501 [TS26501].

###### 5.x.6.3.2.8	Usage of M8

Reference point M8d is used as defined in clauses 4.1 to 4.4 of TS 26.501 [TS26501].

##### 5.x.6.3.3	Usage of CMMF reference points and interfaces

###### 5.x.6.3.3.1	Usage of xCMMF-C

The 5GMSd AF provisions the CMMF Bitstream Generator/Source with configuration information as needed. This may include information such as the CMMF code type to use or how to distribute CMMF encoded content among the available 5GMSd Application Servers. Examples are provided in [CMMF].

###### 5.x.6.3.3.2	Usage of xCMMF-U

Reference point xCMMF-U is used to transfer CMMF Bitstreams/Objects from the CMMF Bitstream Generator/Source to each of the 5GMSd Application Servers.

In some implementations, the reference point may also include information from the 5GMSd AS to the CMMF Bitstream Generator/Source concerning which CMMF encoded representation is expected to be delivered to the 5GMSd AS from the CMMF Bitstream Generator/Source.

###### 5.x.6.3.3.3	Usage of CMMF-API-C

The Media Session Handler may expose information to the CMMF Client to manage connections with each available 5GMSd AS that is used to retrieve media. An example may include the host URL of each 5GMSd AS that was provided to the 5GMSd Client via reference point M5d.

The CMMF Client may provide consumption, QoS, and QoE metrics reports to the Media Session Handler via reference point CMMF-API-C.

###### 5.x.6.3.3.4	Usage of CMMF-API-U

The Media Client provides relative URLs (obtained from the manifest) to the media objects the CMMF Client is required to obtain. The CMMF Client uses these relative URLs in combination with the host URLs provided via reference point CMMF-API-C to connect with and download the media from the set of available 5GMSd Application Servers.

Upon successful decoding of the obtained CMMF-encoded media, the CMMF Client provides fully- and partially-received media objects to the Media Player in the 5GMSd Client.

##### 5.x.6.3.4	Procedures for 5G Downlink Media Streaming using CMMF

### 5.x.7	Summary and Conclusions

## END OF CHANGES