3GPP TSG-SA4 Meeting #0-e (AH) MBS SWG post 128	S4aI240096

# Online, , 6th Jun 2024 - 11th Jul 2024										revision of S4aI240064

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.804 | CR | 0014 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_AMD] Specification Structure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Germany |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_AMD |  |  |  |  |  | Date: |  |  | 2024-07-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | One of the open issues identified in the Rel-18 feasibility study 5GMS_Pro_Ph2 is the need for a specification that addresses interoperability considerations around content delivery protocol features and general technologies for segmented media streaming and the IP/PDU 5G System Layer. This points to the further study media plane issues to support additional functionalities, but also identifies what needs to be ported from legacy TS 26.512 to a generalised media plane technical specification. The relation to media session handling (as specified in TS 26.510) is identified in TR 26.804, but enhancements to media session handling are not the primary focus of this study. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adds discussion and conclusions on a Media Delivery specification |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Study item objectives are not complete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 1, 2, 3, 4, 5.15 (new), 6.15 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S4aI240064; [FS_AMD] Specification Structure; Qualcomm Germany; Thomas Stockhammer. Revisions: none Presenter: Thomas Stockhammer Online Discussion: - Fred: Do you want to use that for new references? - Thomas : - Rufail: I think this is just bringing 5GMS architecture - Thomas: The idea is to discuss how the idea of new spec will look like when with all of the content delivery aspects. - Rufail: you mean is this a KI ? - Thomas: no, this is not KI, but it could be part of the KI and then you could structure. - Ffed: can we endorse it - Rufail: I like more time to review. I thought this was just aligning. I would like a revision. Decision: we can go for endorsement while making the changes. S4aI240064 is endorsed. ## Clause 1 (scope) Note that the original list from Rel-17 was already extended once in Rel-18 by the following topics, only the first of which is currently listed in this clause: - 3GPP Service Handler and URLs - 5GMS Application Server configuration and management The new (Rel-19) topics are therefore a third set of topics. ## Clause 5.15 (Generalised Media Delivery architecture) Note that there was an update to the media delivery architecture figure 4.1.2.2-1 in TS 26.506 agreed at SA4#128 as CR0005r2 [S4-241363]. This includes an additional media plane reference point M12 which, although not immediately relevant to FS_AMD (since it is only instantiated in the RTC System) is worth reminding readers of here in TR 26.804. We should be trying to think more holistically about media delivery in general, and looking for solutions that are applicable to both 5GMS and RTC wherever possible. This would be the logical requirement for a generalised media plane technical specification, as proposed in your text. P.S. The new generalised media delivery architecture figure can also be found in TS 26.501 CR0093 [S4-240976] which was agreed to maintain alignment with TS 26.506 CR0005. That might be the more appropriate variant to include in this TR 26.804 CR0014. This document addresses the comments marked in green |

## =====  CHANGE =====

# 1	Scope

This Technical Report identifies and evaluates a set of potential improvements and extensions, referred to as key topics. The initial set of key topics were:

-	Content Preparation

-	Traffic Identification

-	Additional / New transport protocols

-	Uplink media streaming

-	Background traffic

-	Content Aware Streaming

-	Network Event usage

-	Per-application-authorization

-	Support for encrypted and high-value content

-	Scalable distribution of unicast Live Services

-	Network Slicing Extensions for 5G Media Streaming

-	3GPP Service Handler and URLs

In an extension, a second set of key topics were collected as follows:

-	Media Delivery Specification.

-	Common Client Metadata.

-	Common Server-and Network-Assisted Streaming.

-	Multi-CDN and Multi-Access Media Delivery.

-	Multi-Access with ATSSS.

-	Modem Usage Optimized Media Streaming.

-	DASH/HLS Interoperability.

-	Further harmonization of RTC and Streaming for Advanced Media Delivery.

-	Improved QoS support.

-	5GMS Application Server configuration and management.

For each of the above key topics, the following objectives are identified:

1.	Document the above key topics in more detail, in particular how they relate to the 5GMS Architecture and protocols.

2.	Study collaboration scenarios between the 5G System and Application Provider for each of the key topics.

3.	Based on the 5GMS Architecture, develop one or more deployment architectures that address the key topics and the collaboration models.

4.	Map the key topics to basic functions and develop high-level call flows.

5.	Identify the issues that need to be solved.

6.	Provide candidate solutions (including call flows) for each of the identified issues.

7.	Coordinate work with other 3GPP groups e.g. SA2, SA3, SA5, and others as needed.

8.	Coordinate work with external organizations such as DASH-IF, CTA WAVE, ISO/IEC JTC29 WG3 (MPEG Systems), or IETF, as needed.

9.	Identify gaps and recommend potential normative work for stage-2 call flows and possibly stage-3.

## =====  CHANGE =====

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	Akamai Blog, "A QUICk Introduction to HTTP/3", April 2020, https://developer.akamai.com/blog/2020/04/14/quick-introduction-http3

[3]	Fielding, R., Nottingham, M., and J. Reschke, "HTTP/1.1", Work in Progress, Internet-Draft, draft-ietf-httpbis-messaging-13, 14 December 2020, http://www.ietf.org/internet-drafts/draft-ietf-httpbis-messaging-13.txt

[4]	Belshe, M., Peon, R., and M. Thomson, Ed., "Hypertext Transfer Protocol Version 2 (HTTP/2)", RFC 7540, May 2015, https://www.rfc-editor.org/info/rfc7540

[5]	draft-ietf-quic-http-34, "Hypertext Transfer Protocol Version 3 (HTTP/3)", February 2021

[6]	D. Bhat, A. Rizk, and M. Zink, "Not so QUIC: A Performance Study of DASH over QUIC," NOSSDAV'17: Proceedings of the 27th Workshop on Network and Operating Systems Support for Digital Audio and VideoJune 2017 Pages 13–18 https://doi.org/10.1145/3083165.3083175

[7]	AWS: "Achieving Great Video Quality Without Breaking the Bank", Streaming Media June 2019, https://pages.awscloud.com/rs/112-TZM-766/images/GEN elemental-wp-achieving-great-video-quality-without-breaking-the-bank.pdf

[8]	Netflix, "Optimized shot-based encodes: Now Streaming!", Netflix Blog, May 2018, https://netflixtechblog.com/optimized-shot-based-encodes-now-streaming-4b9464204830

[9]	DASH-IF/DVB: "Report on Low-Latency Live Service with DASH", July 2017, available here: https://dash-industry-forum.github.io/docs/Report%20on%20Low%20Latency%20DASH.pdf

[10]	DASH-IF: "IOP Guidelines v5, Low-latency Modes for DASH", available here: https://dash-industry-forum.github.io/docs/CR-Low-Latency-Live-r8.pdf

[11]	ISO/IEC 23009-1: "Information technology — Dynamic adaptive streaming over HTTP (DASH) — Part 1: Media presentation description and segment formats".

[12]	IETF RFC 8673: "HTTP Random Access and Live Content".

[13]	3GPP TR 26.939: "Guidelines on the Framework for Live Uplink Streaming (FLUS)".

[14]	3GPP TS 26.238: "Uplink Streaming".

[15]	3GPP TS 26.501: "5G Media Streaming (5GMS); General description and architecture".

[16]	3GPP TS 26.512: "5G Media Streaming (5GMS); Protocols".

[17]		ISO/IEC 13818-1:2019: "Information technology — Generic coding of moving pictures and associated audio information — Part 1: Systems".

[18]	SCTE 35 2020: "Digital Program Insertion Cueing Message", https://www.scte.org/pdf-redirect/?url=https://scte-cms-resource-storage.s3.amazonaws.com/SCTE-35-2020_notice-1609861286512.pdf

[19]	ISO/IEC 23000-19:2020: "Information technology — Multimedia application format (MPEG-A) —Part 19: Common media application format (CMAF) for segmented media".

[20]	ISO/IEC 23009-1:2019/DAMD1: "Information technology — Dynamic adaptive streaming over HTTP (DASH) — Part 1: Media presentation description and segment formats — Amendment 1: CMAF support, events processing model and other extensions".

[21]	VSF TR-06-01:2020, "RIST Simple Profile", https://vsf.tv/download/technical_recommendations/VSF_TR-06-1_2020_06_25.pdf

[22]	VSF TR-06-02: "RIST Main Profile", https://www.videoservicesforum.org/download/technical_recommendations/VSF_TR-06-2_2020_03_24.pdf

[23]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[24]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[25]	3GPP TS 29.517: "5G System; Application Function Event Exposure Service; Stage 3".

[26]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane nodes; Stage 3".

[27]	IETF RFC 6733: "Diameter Base Protocol".

[28]	3GPP TS 29.514: "5G System; Policy and Charging Control over Rx reference point; Stage 3".

[29]		IETF RFC 7657: "Differentiated Services (Diffserv) and Real-Time Communication", November 1995.

[30]	IETF RFC 3168: "The Addition of Explicit Congestion Notification (ECN) to IP", September 2001.

[31]	C. Krasic, M. Bishop, and A. Frindell, Ed., draft-ietf-quic-qpack-21, "QPACK: Header Compression for HTTP/3", Work in Progress, Internet-Draft, 2 February 2021.

[32]	IETF RFC 9000: "QUIC: A UDP-Based Multiplexed and Secure Transport", May 2021.

[33]	IETF RFC 9001: "Using TLS to Secure QUIC", May 2021.

[34]	IETF, RFC 9002: "QUIC Loss Detection and Congestion Control", May 2021.

[35]	IETF RFC 5681: "TCP Congestion Control".

[36]	M. Kuehlewind and B. Trammell, draft-ietf-quic-manageability-11, "Manageability of the QUIC Transport Protocol", Work in Progress, Internet-Draft, 30 June 2021.

[37]	N. Cardwell et. al. "BBR Updates: Internal Deployment, Code, Draft Plans", 9 March 2021, https://datatracker.ietf.org/meeting/110/materials/slides-110-iccrg-bbr-updates-00.pdf

[38] 	ETSI TS 103 799: "Publicly Available Specification (PAS); DASH-IF Content Protection Information Exchange Format".

[39]	ISO/IEC JTC1/SC29/WG11/N19062 23090-8 FDIS: "MPEG-I: Network-based Media Processing — Network-Based Media Processing Specification".

[40]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".[41]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[42]	3GPP TS 29.514: "5G System; Policy Authorization Service; Stage 3".

[43]	3GPP TS 29.522: "5G System; Network Exposure Function Northbound APIs; Stage 3".

[44]	3GPP TS 29.122: "T8 reference point for Northbound APIs".

[45]	3GPP TS 29.512: "5G System; Session Management Policy Control Service; Stage 3".

[46]	3GPP TS 26.803: "5G Media Streaming (5GMS); Architecture extensions".

[47]	3GPP TS 23.558: "Architecture for enabling Edge Applications (EA)".

[48]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[49]	Tdoc S4-210723: "Generic architecture for data collection and reporting", submission from BBC, Dolby Laboratories Inc., LM Ericsson and Qualcomm Incorporated to SA4#114-e, May 19-28, 2021.

[50]	Tdoc S2-2103267: "Extension of Naf_EventExposure for observed service experience data collection from UEs", CR from InterDigital to SA2#144e, Apr 12-16, 2021.

[51]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia telephony; Media handling and interaction".

[52]	Tdoc S2-2104496: "Extension of Naf_EventExposure for observed service experience data collection from UEs", CR from Qualcomm Incorporated to SA2#145e, May 17-28, 2021.

[53]	3GPP TS 26.118: "Virtual Reality (VR) profiles for streaming applications".

[54]	3GPP TS 26.346: "Multimedia Broadcast/Multicast Service (MBMS); Protocols and codecs".

[55]	3GPP TS 29.554: "Background Data Transfer Policy Control Service; Stage 3".

[56]	3GPP TS 28.530: "Management and orchestration; Concepts, use cases and requirements".

[57]	3GPP TS 28.531: "Management and orchestration; Provisioning".

[58]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[59]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[60]	3GPP TS 28.540: "Management and orchestration; 5G Network Resource Model (NRM); Stage 1".

[61]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[62]	3GPP TS 28.542: "Management and orchestration of networks and network slicing; 5G Core Network (5GC) Network Resource Model (NRM); Stage 1".

[63]	3GPP TS 28.543: "Management and orchestration of networks and network slicing; 5G Core Network (5GC) Network Resource Model (NRM); Stage 2 and stage 3".

[64]	3GPP TS 28.545: "Management and orchestration; Fault Supervision (FS)".

[65]	3GPP TS 28.546: "Management and orchestration of networks and network slicing; Fault Supervision (FS); Stage 2 and stage 3".

[66]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[67]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[68]	3GPP TS 23.434: " Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows ".

[69]	3GPP TS 23.700-99: " Study in Network slice capability exposure for application layer enablement (NSCALE)".

[70]	3GPP TS 29.520: " 5G System; Network Data Analytics Services; Stage 3".

[71]	3GPP TR 23.700-40: "Study on enhancement of network slicing; Phase 2".

[72]	3GPP TS 26.531: “Data Collection and Reporting; General Description and Architecture”.

[73]	3GPP TR 26.802: "Multicast Architecture Enhancement for 5G Media Streaming".

[74] 	IETF RFC 822: "STANDARD FOR THE FORMAT OF ARPA INTERNET TEXT MESSAGES", August 13, 1982.

[75]	IETF RFC 1521: "MIME (Multipurpose Internet Mail Extensions)", September 1993.

[76]	IETF RFC 2474: "Definition of the Differentiated Services Field (DS Field) in the IPv4 and IPv6 Headers".

[77]	IETF RFC 2475: "An Architecture for Differentiated Services".

[78]	IETF RFC 3246: "An Expedited Forwarding PHB (Per-Hop Behavior)".

[79]	IETF RFC 2597: "Assured Forwarding PHB Group".

[80]	S. Hurst, draft-hurst-quic-rtp-tunnelling: "QRT: QUIC RTP Tunnelling", Internet-Draft, Work in Progress.

[81]	J. Ott and M. Engelbart, draft-engelbart-rtp-over-quic: "RTP over QUIC", Internet-Draft, Work in Progress.

[82]	SRT Alliance, “Secure Reliable Transport (SRT) Protocol”, https://github.com/Haivision/srt

[83]	M.P. Sharabayko and M.A. Sharabayko, draft-sharabayko-srt-over-quic-00 ,“Tunnelling SRT over QUIC”, Internet-Draft, Work in Progress, 28 July 2021.

[84]	Robin Marx, Luca Niccolini, Marten Seemann, draft-ietf-quic-qlog-main-schema-01, "Main logging schema for qlog", Internet-Draft, Work in Progress, 25 October 2021.

[85]	Robin Marx, Luca Niccolini, Marten Seemann, draft-ietf-quic-qlog-h3-events-00, "HTTP/3 and QPACK event definitions for qlog", Internet-Draft, Work in Progress, 10 June 2021.

[86]	Robin Marx, Luca Niccolini, Marten Seemann, draft-ietf-quic-qlog-quic-events-00, "QUIC event definitions for qlog", Internet-Draft, Work in Progress, 10 June 2021.

[87]	Roger Pantos and William May, Jr., "HTTP Live Streaming", RFC 8216, August 2017.

[88]	3GPP TR 26.925: "Typical traffic characteristics of media services on 3GPP networks".

[89]	3GPP TR 26.917: "Multimedia Broadcast Multicast Services (MBMS) and Packet-switchedStreaming Service (PSS) enhancements to support television services".

[90]		"DASH-IF WebRTC-based Streaming", https://dashif.org/news/webrtc/

[91]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[92]	IETF RFC 6750: "The OAuth 2.0 Authorization Framework: Bearer Token Usage".

[93]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[94]	3GPP TS 26.531: "Data Collection and Reporting; General Description and Architecture".

[95]	3GPP TS 26.532: "Data Collection and Reporting; Protocols and Formats".

[96]	3GPP TS 26.511: "5G Media Streaming (5GMS); Profiles, codecs and formats".

[97]	ETSI TS 103 770: "Digital Video Broadcasting (DVB); Service Discovery and Programme Metadata for DVB-I".

[98]	Android Developer Documentation: "Handling Android App Links",
https://developer.android.com/training/app-links

[99]	3GPP TS 26.347: "Multimedia Broadcast/Multicast Service (MBMS); Application Programming Interface and URL".

[100]	ETSI TS 103 769: "Digital Video Broadcasting (DVB); Adaptive media streaming over IP multicast".

[101]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services".

[102]	3GPP TS 29.558: "Enabling Edge Applications; Application Programming Interface (API) specification; Stage 3".

[103]	IETF RFC 2045: "Multipurpose Internet Mail Extensions Part One: Format of Internet Message Bodies".

[104]	IETF RFC 3986: "Uniform Resource Identifier (URI): Generic Syntax".

[26510]	3GPP TS 26.510: "Media delivery; interactions and APIs for provisioning and media session handling".

[RFC9000]	IETF RFC 9000: "QUIC: A UDP-Based Multiplexed and Secure Transport", May 2021.

[RFC9001]	IETF RFC 9001: "Using TLS to Secure QUIC", May 2021.

[RFC9114]	IETF RFC 9114: "HTTP/3", June 2022.

[RFC9112]	IETF RFC 9112: "HTTP/1.1", June 2022.

[RFC9110]	IETF RFC 9110: "HTTP Semantics", June 2022.

[RFC9111]	IETF RFC 9111: "HTTP Caching", June 2022.

[RFC8446]	IETF RFC 8446: "The Transport Layer Security (TLS) Protocol Version 1.3", August 2018.

[RFC9113]	IETF RFC 9113: "HTTP/2", June 2022.

## =====  CHANGE =====

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

API	Application Programming Interface

CDN	Content Delivery Network

DS	Differentiated Service

EAS	Edge Application Server

EES	Edge Enabler Server

FAR	Forward Action Rule

MAR	Multi-Access Rule

NRF	Network Repository Function

PDR	Packet Detection Rule

PFCP	Packet Forwarding Control Protocol

QER	QoS Enforcement Rule

QLOG	QUIC Logging

PHB	Per-Hop Behaviour

PFD	Packet Flow Description

SDF	Service Data Flow

URL	Uniform Resource Locator

URR	Usage Reporting Rule

## =====  CHANGE =====

# 4	5G Media Streaming

The 5G Media Streaming architecture is defined in TS 26.501 [15].

Protocols and APIs are specified in TS 26.512 [16], with reference to the generalized Media Session Handling to TS 26.510 [26510].

Profiles, codecs and formats are provided in TS 26.511 [96].

## =====  CHANGE =====

## 5.15	Media Delivery Specification

### 5.15.1	Description

The primary focus of a new specification in 3GPP is addressing the of segmented media objects in the media plane, i.e. at reference points M2, M3, M4, M7, M11 and M12 of the Media Delivery architecture as shown in Figure 5.15.1-1.

Figure 5.15.1-1 Media Delivery Architecture as defined in TS 26.501 [15] with emphasis for protocol specification (M2, M3, M4, M7, M11 and M12) to be developed.

Such a specification is expected to address interoperability considerations around content delivery protocol features and general technologies for segmented media streaming and the IP/PDU 5G System Layer. This points to the further study media plane issues to support additional functionalities, but also identifies what needs to be ported from legacy TS 26.512 [16] to a generalised media plane technical specification. The relation to media session handling (as specified in TS 26.510 [26510]) is also relevant, but not the focus of a new specification.

M12 is not in scope for this Technical Report and the expected new specification, the focus is on Media AS from/to UE.

Key aspects of such a specification should include common protocols on M2and M4, as well as common APIs on M7 and M11. In addition, consistent extensions to such protocols need to be reviewed, for example custom HTTP headers, query parameters, etc.

M12 is not in scope for this Technical Report and the expected new specification.

### 5.15.2	General Outline of Specification

The following outline is considered for a new specification addressing the media plane.

1	Scope

2	References

3	Definitions

4	Overview and Assumptions

4.1	General Assumptions and Protocol Stack for M2 and M4: IPv4 or IPv6 and HTTP according to RFC 9110

-	HTTP/1.1, TLS (optional), TCP, IP – parallel requests, RFC 9112

-	HTTP/2, TLS, TCP, IP – one TCP connection, RFC 9113

-	HTTP/3, QUIC (+TLS), UDP, IP – one QUIC connection, RFC 9114

-	HTTP Methods

-	HTTP Headers

4.2	General Assumptions for M7 and M11

-	Existence of a reference API in Media Access function

4.3	General Assumptions for M3

-	Existence of a reference API in Media AS

4.4	Features

-	What are features?

-	Configurable UE and Media AS functionalities.

-	Features may be mandatory or optional, but are typically optional

-	Features are fully specified and normative

-	How can the features be configured?

-	What are the requirements for each feature?

-	Overview of features and mapping to reference points

5	General Procedures for Functions and Reference Points

Editor’s Note: do we want to differentiate uplink and downlink streaming?

-	Media AS

-	Overview

-	Reference Point M2

-	Reference Point M3

-	Reference Point M4

-	Media Access Client

-	Overview

-	Reference Point M4

-	Reference Point M7

-	Reference Point M11

6	Media Delivery Features

Editor’s Note: as an alternative, reference points are the top headings and then we add to each reference point what the feature needs to support.

-	For each feature

-	Overview

-	Procedures (if not in stage-2, possibly referenced)

-	Requirements for each function and reference point

-	Implementation Guidelines

### 5.15.3	Existing Features

The following features are already documented in other 3GPP specifications, in particular in TS 26.512 [26512].

Editor’s Note: Details are for further study

### 5.15.4	Importing from other specifications

The following information already exists in other specifications

Editor’s Note: Details are for further study

## =====  CHANGE =====

## 6.15	Media Delivery Specification

Editor’s Note: Details are for further study