3GPP TSG-SA4 Meeting #0-e (AH) MBS SWG post 128	S4aI240098

# Online, , 6th Jun 2024 - 11th Jul 2024										revision of S4aI240066

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.804 | CR | 0015 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_AMD] Common Client Metadata |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Germany |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_AMD |  |  |  |  |  | Date: |  |  | 2024-07-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | While 3GPP and MPEG in DASH support DASH metrics, the reporting is not common to any player, for example all DASH players as well as HLS players. As an example, CTA WAVE has developed: CTA-5004: Web Application Video Ecosystem Common-Media-Client-Data (CMCD) with an excellent overview here: https://ottverse.com/common-media-client-data-cmcd/. It is worthwhile to study the benefits of integrating commonly supported metrics and client data reporting in 5GMS workflows. The focus is the integration of already defined metrics rather than developing new metrics. Examples of study include support of specific metric keys, player APIs, sending options from client to server (user plane, M5 reference point, EVEX), M3 reference point impact, as well as usage of the data in operations. A study of creating a common harmonized reporting framework and studying the interaction of different frameworks may be included. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addresses the work item objectives for this key issue - Documents the key issue in more detail, in particular how they relate to the 3GPP Media Delivery architecture and/or the MBS User Service architecture - Studies collaboration scenarios between the Application Service Provider and the 5G System and for each of the key topics. - Based on existing architectures, provides one or more deployment architectures that address the key topics and the collaboration models. - Maps the key topics to basic functions and develop high-level call flows. - Identifies the issues that need to be solved. - Provides candidate solutions including call flows, protocols and APIs for each of the identified issues. - Identifies gaps and recommend potential normative work for stage-2 and stage-3, including which existing specifications would be impacted and/or if any new specifications would preferably be developed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.16 (new), 6.16 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TR 26.804 CR 0014 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Updated scope, references and abbreviations are in CR 0014 References [CTA-5004] CTA-5004: Web Application Video Ecosystem Common-Media-Client-Data (CMCD) [26531] 3GPP TS 26.531, Data Collection and Reporting; General Description and Architecture [26532] 3GPP TS 26.532, Data Collection and Reporting; Data Collection and Reporting; Protocols and Formats Abbreviations: CDN Content Delivery Network CMCD Common-Media-Client-Data |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S4aI240066; [FS_AMD] Common Client Metadata; Qualcomm Germany; Thomas Stockhammer. Revisions: none Presenter: Thomas Stockhammer Online Discussion: - Fred: Reference to EVEX? - Thomas: That's correct. I need a proper reference - Rufail: : if we are going to Common Media Metadata, this was used for CDN right? I think we should use AS instead of CDN. - Thomas: They don’t know what AS is. It is our duty to do this. - Rufail: maybe we can add a sentence saying this, the description of CDN is more generic. - Fred: this is architecture mapping we are asking for - Thomas: that's part of work - Frd: That's coming then. - Rufail: CMCD is a very generic technology. I am just making this comment that the description needs to be specific. - Fred: Could you clarify how you will address the reference. - Thomas: The idea is to basically put it as comments. Decision: endorsed. S4aI240066 is endorsed. Thomas – Thanks also for this contribution. S4aI240066; CR; [FS_AMD] Common Client Metadata; Qualcomm Germany; Spec: 26.804 CR0015. What would be most useful in this one is a feature comparison matrix (probably in the gap analysis clause) that compares our QoE metrics report for DASH (TS 26.247) plus our consumption report for 5GMS (TS 26.510) with the key–value pairs for CMCD. This revision addresses updates based on the above comments and the editor’s Note. |

## =====  CHANGE =====

## 5.16	Common Client Metadata

### 5.16.1	Description

While 3GPP and MPEG in DASH support DASH metrics, the reporting is not common to any player, for example all DASH players as well as HLS players. As an example, CTA WAVE has developed: CTA-5004: Web Application Video Ecosystem Common-Media-Client-Data (CMCD) [CTA-5004]. It is worthwhile to study the benefits of integrating commonly supported metrics and client data reporting in 5GMS workflows. The focus is the integration of already defined metrics rather than developing new metrics. Examples of study include support of specific metric keys, player APIs, sending options from client to server (user plane, M5 reference point [26510], Data collection and reporting [26531], [26532]), M3 reference point impact, as well as usage of the data in operations. A study of creating a common harmonized reporting framework and studying the interaction of different frameworks may be included.

CMCD defines set of structured key/value pairs that allow communicating mutually beneficial media-related information from a player to a CDN via either (i) a set of custom headers, (ii) a query arg, or (iii) a JSON object. Common is used because the same data structure can be used across all players and all CDNs. CTA WAVE is currently extending the specification and version is expected to be available by the end of 2024. A player may be instructed through a configuration API, for example defined in dash.js, on how to exactly report. In addition, in the 6th edition of MPEG-DASH in ISO/IEC 23009-1 [11], an API based configuration and an MPD-based signaling is provided in order for the DASH client to be instructed on CMCD reporting. A basic overview of CMCD is provided in Figure 5.16.1-1. With every HTTP request, typically and primarily for the request of a Media Segment, some custom data is sent either as HTTP header or as query appended to the segment request. In a third mode that is not widely implemented, the media client may also provide the information in a JSON object to the application. The use-cases which CMCD enables are broad – including robust prefetching, analytics solutions, forensic debugging, CDN delivery optimization, alerting and monitoring systems, low latency optimizations, server-side switching, research analytics and content steering decisioning.

Figure 5.16.1-1 Basic Idea of CMCD – Media client sends continuous key value pairs to CDN.

In the context of 5G Media Streaming, the CMCD information may be used for several purposes that are elaborated further in the remainder of this clause:

1)	Operational optimization of the AS: In the case the Application Server uses CMCD information to optimize its operation.

2)	Operational optimization of the AF and 5G Media Streaming: In this case, the AF uses the information available from CMCD in order to communicate with the 5G System for optimized delivery.

3)	Data collection: In this case, the AF uses the data collection and reporting framework to provide CMCD collected information to the 5G System or external service providers.

### 5.16.2	Collaboration Scenarios

Assuming that query-based or header-based reporting is used in CMCD, a couple of collaboration scenarios are discussed, in context of the collaboration scenarios in 3GPP TS 26.501 [26501], Annex A.

1)	Media AS deployed in an external Data Network (OTT) as documented in TS 26.501 [26501], Annex A.1. In this case, a third-party service provider logs data from clients about operational performance. The application provider may use the data in order to optimize its overall service including clients served through a 5G system.

2)	Both AF and AS deployed in the trusted Data Network as documented in TS 26.501 [26501], Annex A.2. In this case, the AS can provide the collected data to the AF via M3, which can further process the information for delivery optimizations. Many use cases may be considered, for example network assistance, policy updates, etc. The value of each metric key needs to be analyzed to what extent they can be used in operational context. In a similar fashion, when AS and AF are external (see TS 26.501 [26501], Annex A.2), such considerations may be taken into account, and in case AS is internal and AF is external, as documented in TS 26.501 [26501], Annex A.6.

3) 	AF deployed in the trusted Data Network and AS deployed in an external Data Network as documented in TS 26.501 [26501], Annex A.4. In this case, the provisioning updates may be done by the Application service provider based on CMCD metrics. Similar aspects apply for TS 26.501 [26501], Annex A.4.

The core scenarios for collaboration are summarized in bullet 2), namely the case that the AS collects information which is shared with the AF for further actions. The remaining discussion in this clause is considering that Media AS and Media AF are in a trusted domain. However, it is generally also possible for a third-party provided in a non-trusted environment to use CMCD data according to the following scenarios.

In the context of collaboration scenarios, also the three different use cases elaborated above may be considered:

1)	Operational optimization of the AS: In the case the Application Server uses CMCD information to optimize its operation. If the AS is external, the CMCD information may be considered as information describing the media delivery session.

2)	Operational optimization of the AF and 5G Media Streaming: In this case, the AF uses the information available from CMCD in order to communicate with the 5G System for optimized delivery. If the AF is external, NEF-based access to 5G System functionalities may be applied to optimize the delivery based on CMCD data, for example applying network assistance or dynamic policy updates.

3)	Data collection: In this case, the AF uses the data collection and reporting framework to provide CMCD collected information to the 5G System or external service providers. The Data reporting server may be external and the AF exposes CMCD based information as collected in the operational service.

### 5.16.3	Architecture Mapping

No architectural updates are needed to the one presented in TS 26.501 [26501], clause 4.1.1, and the generalized media delivery architecture in TS 26.501 [26501], clause 4.1.2.1.

However, the function of the Media AS or 5GMSd AS are extended to collect CMCD data and share with Media AF via M3.

Furthermore, in case of data reporting, the functional architecture in clause 4.2 of TS 26.531 [26531] applies. However, what is important that no explicit data collection client is run in the UE, but the data collection client is operated in the AS, and the AS then provides the information to the Data Collection AF as shown in Figure 5.16.3-1.

Figure 5.16.3-1: Reference architecture for data collection and reporting when AS is delegated for collecting the data

### 5.16.4	High-level Call Flow

The focus of the call flow is on scenario 2 above, for which the CMCD data is initially sent to the Media AS, and then provided to the Media AF. The call flow is aligned with Metrics collection and reporting as documented in clause 5.5 of TS 26.501 [26501], but addresses the user plane aspects.

Figure 5.16.4-1 illustrates a scenario where CMCD collection and reporting is configured by the 5GMSd AF. In this example, it is assumed that the CMCD collection information is provided by the 5GMSd AF comprises instructions/rules regarding CMCD reporting. It is further assumed that the 5GMSd AF is required to deliver CMCD reports to separate destination entities, upon optionally having performed post-processing the collected report information. The 5GMSd AF and 5GMSd AS functions can be either trusted or untrusted.

Figure 5.16.3-1: CMCD-based data collection in 5GMSd

The message sequence steps are described below:

1:	The 5GMSd AF is provisioned with CMCD reporting configuration information, each pertaining to CMCD collection by the Media Player and retrieval of those collected CMCD from the Media Player through the 5GMSd/Media AS. The 5GMSd AF is provisioned with different CMCD collecting and processing information regarding required post-processing functionality and subsequent and separate delivery of processed CMCD to the 5GMSd Application Provider and to the OAM Server. The AF may also use such information for operational benefits. The AF also instructs the AS to collect CMCD data and provides relevant information to the AF.

2:	The 5GMSd-Aware Application triggers the Service Announcement and Content Discovery procedure. The Service Announcement includes the whole Service Access Information that contains CMCD configuration, with their associated CMCD reporting configurations to be used by the Media Player. The information typically includes a session identifier.

3:	Time passes until the 5GMSd UE initiates session establishment and media playback.

4: Streaming Session and media playback is established.

4a:	The 5GMSd-Aware Application informs the Media Player of impending media playback.

4b:	The Media Player requests the establishment of a streaming session with the Media Session Handler which acknowledges the request.

4c:	The Media Session Handler requests the establishment of a streaming session with the 5GMSd AF which confirms the streaming session establishment.

4d:	The Media Session Handler informs the Media Player the successful set-up of the streaming session.

4e:	Media playback pipeline is set up between the Media Player, the 5GMSd AS and the 5GMSd Application Provider.

5:	The Media Session Handler queries the Media Player on its capability to perform CMCD collection (measurement and reporting function) and instructs the player for CMCD collection, including a session identifier.

6:	The Media Player acknowledges its support for the collection of the required CMCD data.

During the course of media playback, steps 7a to 7f below may be repeated, depending on the duration of the playback.

7a:	The Media Player requests media content from the 5GMSd AS and includes configured CMCD information.

7b:	The Media AS extracts and processes CMCD information from this request.

7c:	Assuming a live streaming service, media content may be delivered via push ingest from the 5GMSd Application Provider to the 5GMSd AS. However, in this case it is considered that the AS pulls the information from the 5GMSd Application Provider to the 5GMSd AS. The AS checks if the requested data is already cached and if not, requests the data from the upstream 5GMSd Application Provider. The AS may also use information in the CMCD data to already request future segments. Note that current CMCD data does not include a timestamp, when the future data would be available.

7d:	The 5GMSd AS delivers the requested media data to the Media Player.

7e:	The Media Player starts playback.

7f:	The Media AS provides the requested CMCD information to the Media AF.

7g:	The Media AF extracts the relevant CMCD information and processes the data.

7h:	Based on certain information, for example buffer levels, etc., the Media AF may decide to communicate with the 5G System to initiate Media handling functions such as network assistance.

8:	In accordance with its CMCD reporting configuration as provisioned in step 1, the 5GMSd AF performs separate post-processing in accordance of the received types of CMCD reports.

9:	The 5GMSd AF sends a processed CMCD report in accordance with some configuration to the 5GMSd Application Provider.

10:	The 5GMSd AF sends a processed CMCD report in accordance with some configuration to the OAM Server.

NOTE:	Although not explicitly shown or described in Figure 5.16.3-1, should the 5GMSd AF represent an untrusted network entity and the OAM Server represent a trusted network entity, the delivery of processed metrics reports from the 5GMSd AF to the OAM Server, as depicted in step 10, is mediated by the NEF (Network Exposure Function).

### 5.16.5	Gap Analysis and Requirements

In order to understand the gaps and requirements from the above call flow, relevant gaps are highlighted in bold. In summary, the following gaps are identified:

1)	Lack of provisioning information to configure CMCD reporting, including delivery to the 5GMSd Application Provider and to the OAM Server on reference point M1d in step 1 in clause 5.16.4.

2)	Lack of configuration signaling for the 5GMSd/Media AS to collect CMCD information for specific sessions or all clients on reference point M3d in step 1a and 1b in clause 5.16.4.

3)	Lack of Service Access information that contains CMCD configuration information and a session identifier on M5d in step 2 in clause 5.16.4.

4)	Lack of Media Player configuration API on M7d to configure CMCD data collection and reporting, including the acknowledgement of the capabilities in step 5 and step 6 in clause 5.16.4.

5)	Missing functionalities in Media Player that reports CMCD data on M4d as part of the media requests. If the next segment request is included, then CMCD needs to be extended to add the timestamp when the Segment/media object is available in step 7c in clause 5.16.4.

6)	Missing functionalities in Media/5GMSd AS to extract and process CMCD data and provide information to the Media AF on M3d. This includes to proactively request media segments according to CMCD information in step 7f in clause 5.16.4.

7)	Missing functionalities in Media AF to process CMCD information received from the Media AS via M3d, to use this information to initiate and re-configure Media Session Handling functions as needed, and to aggregate the information for delivery to OAM and/or 5GMSd application provider in steps 8, 9, 10 in clause 5.16.4.

### 5.16.6	Candidate Solutions

Editor’s Note: Provide detailed semantics and syntax for the identified gaps in clause 5.16.6.

#### 5.16.6.1	Provisioning information on M1d

Provisioning information is needed to configure CMCD reporting, including delivery to the 5GMSd Application Provider and to the OAM Server on reference point M1d in step 1 in clause 5.16.4.

#### 5.16.6.2	Configuration Signaling and Data Collection on M3d

Configuration signaling for the 5GMSd/Media AS to collect CMCD information for specific sessions or all clients on reference point M3d in step 1a and 1b in clause 5.16.4.

Provide CMCD information to the Media AF on M3d.

#### 5.16.6.3	Service Access information to collect CMCD Data on M5d

Service Access information that contains CMCD configuration information and a session identifier on M5d in step 2 in clause 5.16.4.

#### 5.16.6.4	Media Player configuration API on M7d

Media Player configuration API on M7d to configure CMCD data collection and reporting, including the acknowledgement of the capabilities in step 5 and step 6 in clause 5.16.4.

#### 5.16.6.5	CMCD reporting on M4d

Media Player that reports CMCD data on M4d as part of the media requests. If the next segment request is included, then CMCD needs to be extended to add the timestamp when the Segment/media object is available in step 7c in clause 5.16.4.

#### 5.16.6.6	Functional Updates to 5GMSd/Media AF

Functionalities in Media AF to process CMCD information received from the Media AS via M3d, to use this information to initiate and re-configure Media Session Handling functions as needed, and to aggregate the information for delivery to OAM and/or 5GMSd application provider in steps 8, 9, 10 in clause 5.16.4.

#### 5.16.6.7	Functional Updates to 5GMSd/Media AS

Functionalities in Media/5GMSd AS to extract and process CMCD data and provide information to the Media AF on M3d. This includes to proactively request media segments according to CMCD information in step 7f in clause 5.16.4.

#### 5.16.6.8	Functional Updates to Media Player

Functionalities in Media Player that reports CMCD data on M4d as part of the media requests. If the next segment request is included, then CMCD needs to be extended to add the timestamp when the Segment/media object is available in step 7c in clause 5.16.4.

#### 5.16.6.9	Functional Updates to Media Session Handler

Functionalities in the Media Session Handler to process CMCD configuration information and to instruct media player to initiate CMCD collection reporting.

### 5.16.7	Summary and Conclusions

Editor’s Note: Needs to be completed.

## =====  CHANGE =====

## 6.16	Common Client Metadata

Editor’s Note: Needs to be completed.