3GPP TSG-SA WG4 Meeting # post 128	S4aI240111 is

revision of S4aI240070 is

revision of S4-241011

Adhoc,  6th June- 27th  July 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.942 | pCR |  | rev | 06 | Current version: | 0.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | ### Key Issue #2: Monitoring and measurement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_MediaEnergyGREEN |  |  |  |  |  | Date: |  |  | 2024-06-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The latest draft of 3GPP TR 26.942 contains clause 5.2 on Key Issue #2: Monitoring and Measurement. In this context, it is proposed to add the proposed content to the latest draft of TR 26.942 v 1.0.0 so that it is not left incomplete. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR proposes new text to be added in TR 26.942 on “Key Issue #2: Monitoring and Measurement” section. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Proposed objectives will not be met. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2 (new), 5.2.1 (new), 5.2.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 1st Change

# 2	References

[23288]	3GPP TS 22.882: "Study on Energy Efficiency as a service criteria".

[28310]	3GPP TS 28.310: "Management and orchestration; Energy efficiency of 5G".

## 2nd Change

## 5.2	Key Issue #2: Monitoring, measurement

### 5.2.1	Description

Advanced media services such as XR services, Split Rendered media services, etc. are expected to incur substantial energy consumption both at the device and network levels, presenting significant challenges for operators and service providers.

For instance, when a Mobile Network Operator (MNO) deploys a communication service to fulfill application service requirements, such as those of a gaming application, it's crucial for the customer – whether an Application Service Provider (ASP) or an industry vertical– to ensure that the application service minimises energy consumption for both end users and the data network. Failure to do so could necessitate short-notice application layer adaptations within the Application Service Provider's domain. This may result in adjustments to being made to service levels in response to anticipated high energy consumption in specific service areas or during peak hours that adversely affect the Quality of Exeprience for service users.

In a practical scenario, an ASP intends to deploy a gaming service within a designated service area, served by an MNO's 5G network. Various service levels may exist, each associated with specific Key Performance Indicators (KPIs), such as automation levels or video quality targets. To ensure energy efficiency the ASP  monitors the energy efficiency of its application service across specified service levels. Monitoring of application energy consumption may occur periodically or may be event-triggered, depending on the ASP's requirements, which are typically outlined in the Service Level Agreement (SLA). The MNO and ASP may  agree on a certain energy efficiency target for the application service and optionally for given service levels.

### 5.2.2	Potential new requirements

Based on TS 28.310 [28310], the following potential requirements need to be considered in this Key Issue:

- Based on the collected and/or predicted energy efficiency information exposed from the 5G System, the Application Service Provider should be able to adapt the application service parameters based on the 5GS feedback.

In order to perform such operations, the Application Service Provider requires relevant APIs to be exposed related to the usage of media applications in order to expose Energy efficiency related information.

This depends on the relationship between all the participating entities (5GMS Application Provider, 5GMSd-Aware Application, 5GMSd Client, 5GMSd AF, and 5GMSd AS) for the 5G Media Streaming session described in clause A.2 of TS 26.501 apply in this scenario. , .

Monitoring of application energy consumption may occur periodically or may be event-triggered, depending on the ASP's requirements, which are typically outlined in the Service Level Agreement (SLA).

## End of change