3GPP TSG-SA WG4 ad hoc Meeting post #129e	S4aI240135

Electronic, 25th September–23rd November 2024

Title:	Client API for Background Data Transfer

Source:	BBC

Agenda Item:	2.4 (5GMS_Pro_Ph2)

Document for:	Discussion and agreement

Contact:	Richard Bradbury <richard dot bradbury at bbc dot co dot uk>

# Abstract

This paper identifies gaps in the client API for Background Data Transfer specified in 3PGP Release 18.

# Background

## Background Data Transfer feature

In 3GPP Release 18, the Dynamic Policy feature defined in TS 26.501 [1] has been extended to support the instantiation of Policy Templates with an attached schedule of (possibly repeating) windows when a low-priority Background Data Transfer is permitted by the 5GMS System.

Stage-3 interactions to support this feature were specified in the new TS 26.510 [3] deliverable in Release 18, including enhancements to the provisioning API at reference point M1, to the media session handling API at reference point M5 and to the media session handling client API at reference point M6/M11.

## Documentation of procedures for Media Delivery

As part of a work item analysing the suitability of 3GPP specifications for media production and distribution, 5G-MAG has analysed the 5G Media Streaming System defined in TS 26.501 [1] and specified in TS 26.512 [2] and TS 26.510 [3].

Knitting together the procedures and call flows defined in TS 26.501 [1], it has produced sequence diagrams for four key concrete usage scenarios:

- Scenarios exercising the Dynamic Policy API

  - Selecting a Service Operation Point

  - Requesting a Background Data Transfer

- Scenarios based on the Network Assistance API

  - Obtaining a bit rate recommendation ("throughput estimation")

  - Requesting a delivery boost

A copy of the sequence diagrams for the above four activities is attached.

In the course of documenting the call flow for Background Data Transfer, a number of gaps have been identified in the client API specified in TS 26.510 [3]. These are highlighted in red in the second attached sequence diagram and are summarised below.

# Gap analysis

The following is a summary of the gaps identified in the client API to support the Background Data Transfer feature as specified in the relevant stage-3 specifications in Release 18.

## Gaps in the client media session handling API at reference point M6/M11

The following gaps have been identified in the client media session handling API exposed by the Media Session Handler at reference points M6 and M11, as specified in clause 11.3 of TS 26.510 [3]:

- The POLICY_ACTIVATED notification event sent by the Media Session Handler to the Media-aware Application (specified in clause 11.3.2 of TS 26.510 [3]) does not include a reference to the granted Background Data Transfer window as required by step 13 of the call flow presented in clause 5.7.8 of TS 26.501 [1].

- There is no method specified to enable the Media-aware Application to deactivate a Dynamic Policy when a Background Data Transfer completes normally.

- There is no means for the Media Session Handler to notify the Media-aware Application of new Background Data Transfer opportunities, per step 8 of the call flow presented in clause 5.7.8 of TS 26.501 [1] as a consequence of receiving an asynchronous MQTT NOTIFICATION_DYNAMIC_POLICY_INSTANCE at reference point M5.

For more details see https://github.com/5G-MAG/Standards/issues/148.

## Gaps in the client media session handling API at reference point M5/M3

The following gaps have been identified in the client media session handling API exposed by the Media AF at reference points M5 and M3, as specified in clauses 9.3 and 10.2 of TS 26.510 [3]:

- It is not clear how the ERROR_BACKGROUND_DATA_TRANSFER asynchronous notification from the Media Session Handler to the Media-aware Application is stimulated by an asynchronous MQTT NOTIFICATION_DYNAMIC_POLICY_INSTANCE received by the Media Session Handler at reference point M5 in case of modification or cancellation of an existing Background Data Transfer policy, as required by steps 24 and 27 respectively of the call flow presented in clause 5.7.8 of TS 26.501 [1].

For more details see https://github.com/5G-MAG/Standards/issues/148.

## Gaps in the client media stream handling API at reference point M7/M11

The following gaps have been identified in the client media stream handling API exposed by the downlink Media Player at reference points M7 and M11, as specified in clause 13.2 of TS 26.512 [2]:

- In TS 26.501 [1], the Media Player is the entity that interacts with the 5GMSd AS at reference point M4. The procedure for Background Data Transfer defined in clause 5.7.8 envisages that the Media Player acquires (step 17) and stores (step 18) the content. However, it is not clear which API is used to initiate the content acquisition in the case of Background Data Transfer.

  - The play() method specified in clause 13.2.3.5 passes downloaded media segments to the media playback and content decryption platform for immediate presentation.

  - The preload() method specified in clause 13.2.3.4 passes downloaded media segments to the media playback and content decryption platform for buffering and later presentation.

  - The attach() method specified in clause 13.2.3.3 downloads only the Media Entry Point resource, but also establishes a "media playback management and protection controller" in the media playback and content decryption platform, which is not required in the case of download.

- It is not clear how media assets acquired by Background Data Transfer are subsequently selected for presentation by a Media-aware Application.

For more details, see https://github.com/5G-MAG/Standards/issues/149.

# Proposal

It is proposed that SA4 agrees:

- That the gaps identified in section 3 of this Discussion Paper are valid in the context of the stage-2 design defined in Release 18 by TS 26.501 [1].

- To identify Individual Member(s) willing to draft a Change Request to clause 11 of TS 26.510 [3] to complete the stage-3 specification of the following APIs to fully support the scope of the Background Data Transfer feature defined in Release 18 by TS 26.501 [1]:

  - The client media session handling API at reference point M6/M11.

  - The network media session handling API at reference point M5/M3.

- To identify Individual Member(s) willing to draft a Change Request to clause 11 of TS 26.512 [2] to complete the stage-3 specification of the following APIs to fully support the scope of the Background Data Transfer feature defined in Release 18 by TS 26.501 [1]:

  - The client media stream handling API at reference point M7/M11.

# References

3GPP TS 26.501: "5G Media Streaming (5GMS); General description and architecture", Release 18.

- [2]	3GPP TS 26.512: "5G Media Streaming (5GMS); Protocols", Release 18.

- [3]	3GPP TS 26.510: "Media delivery; interactions and APIs for provisioning and media session handling", Release 18.