## 3GPP SA4 MBS SWG Post SA4#129-e	S4aI240145

# Online, 10th October 2024	revision of S4aI240138

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.804 | CR | 0006 | rev | 16 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_AMD ] WT 03a: Multi-CDN and Multi-Access Media Delivery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Dolby France SAS |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_AMD |  |  |  |  |  | Date: |  |  | 2024-05-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Document the FS_AMD key topic “c) Multi-Access and Multi-CDN Delivery” description and collaboration scenarios. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of new clause 5.x Multi-CDN and Multi-Access Media Delivery including sub-clause structure and headings. Addition of prose for Description and Collaboration Scenarios. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | FS_AMD objectives not achieved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 5.x (NEW) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S4-240591: SA4#127-bis-e, description and collaboration scenarios S4-240844: updates with comments received at SA4#127-e-bis S4aI240052: updated to match proposed template and address pending comments. S4al240894: S4al240052 endorsed at SA4 post 127-bis-e and comments/changes accepted for ease of reading. Updates with comments received at SA4 post 127-bis-e and introduction of candidate solutions. More background on candidate solution is contained in S4-240895. Multi-access media delivery overview and collaboration scenarios have been moved to S4-241082. S4-241230: updates to address pending comments. S4-241273: updates to address pending comments. S4al240095: S4-241273 endorsed at SA4#128. Comments/changes accepted for ease of reading. CMMF candidate solution architecture and reference point descriptions added. S4al240107: CMMF candidate solution architecture and reference point descriptions replaced with options to incorporate CMMF within the existing 5GMS architecture. S4al240114: Edits from BBC. Endorsed by MBS SWG. S4-241588: Addition of CMMF background information and addresses outstanding comments. S4-241665: Reorginization of content to fit within the existing template and updates to address received comments. S4aI240120: updates to CMMF architecture and high-level call flows. S4aI240138: Updates to CMMF’s introduction to include more details concerning the expected overhead CMMF induces as well as an example architecture and call flow used to deliver CMMF content for the trial where the included performance metrics were collected. Further updates to the CMMF architecture and high-level call flows to address outstanding comments. S4al240144: Incorporates edits and captures additional BBC comments. |  |  |  |  |  |  |  |  |

## FIRST CHANGE

## 2	References

…

[4]	Belshe, M., Peon, R., and M. Thomson, Ed., "Hypertext Transfer Protocol Version 2 (HTTP/2)", RFC 7540, May 2015, https://www.rfc-editor.org/info/rfc7540

…

[15]	3GPP TS 26.501: "5G Media Streaming (5GMS) General description and architecture (Release 18)".

[16]	3GPP TS 26.512: "5G Media streaming (5GMS) protocols (Release 18)".

…

[DASH9]	Draft Text of ISO/IEC FDIS 23009-9: "Information technology - Dynamic adaptive streaming over HTTP (DASH) – Part 9: Redundant Encoding and Packaging for segmented live media (REaP)", ISO/IEC JTC 1/SC 29/WG 3 NO 1165, Jan. 26, 2024. [Online]. Available: https://www.mpeg.org/standards/MPEG-DASH/9/

[UNPKG24]	Emma Roth, "A popular open-source content delivery network went down for hours", The Verge, Apr. 12, 2024. [Online]. Available: https://www.theverge.com/2024/4/12/24128276/open-source-unpkg-cdn-down (accessed May 9, 2024).

[NET23]	Sebastian Moss, "Cloudflare recovers from service outage after power failure at core North American data center", Data Center Dynamics, Nov. 3, 2023. [Online]. Available: https://www.datacenterdynamics.com/en/news/cloudflare-recovers-from-service-outage-after-power-failure-at-core-north-american-data-center/ (accessed May 9, 2024).

[FSLY21]	Brian Barrett, "How an Obscure Company Took Down Big Chunks of the Internet", Wired, Jun. 8, 2021. [Online]. Available: https://www.wired.com/story/fastly-cdn-internet-outages-2021/ (accessed May 9, 2024).

[AKAM21]	Josh Fomon, "CDN Provider Akamai Takes Down Popular Internet Services During Outage", Ookla, Jul. 22, 2021. [Online]. Available: https://www.ookla.com/articles/akamai-outage-july-22-2021 (accessed May 9, 2024).

[NET22]	Charlotte Trueman, "Cloudflare outage brings hundreds of sites, services temporarily offline", Computer World, Jun. 21, 2022. [Online]. Available: https://www.computerworld.com/article/1627967/cloudflare-outage-brings-hundreds-of-sites-services-temporarily-offline.html (accessed May 9, 2024).

[VZ19]	Jim Salter, "The Internet broke today: Facebook, Verizon, and more see major outages", Ars Technica, Jul. 3, 2019. [Online]. Available: https://arstechnica.com/information-technology/2019/07/facebook-cloudflare-microsoft-and-twitter-suffer-outages/ (accessed May 9, 2024).

[DEMX01]	Marc Hoppner, "A content owner, a CDN and a player walk into a bar", (Jan. 6, 2023). Accessed: May 9, 2024. [Online Video]. Available: https://www.youtube.com/watch?v=S9EdoQFOQ9I&list
=PLkyaYNWEKcOf98lZxnCcL6y7ZIVU3oSYO&index=12

[DEMX02]	Guillaume du Pantavice, "Improving streaming experience with Bayesian optimization, from AB to AZ test", (Dec. 25, 2021). Accessed: May 9, 2024. [Online Video]. Available: https://www.youtube.com/
watch?v=t4nRrLygVwo&list=PLkyaYNWEKcOfD1GYFxFbZXDP03XM-cZPg&index=19

[IEEE01]	E. Ghabashneh and S. Rao, "Exploring the interplay between CDN caching and video streaming performance", IEEE INFOCOM 2020 – IEEE Conference on Computer Communications, Toronto, ON, Canada, 2020, pp. 516-525.

[ACM01]	K. Vermeulen, L. Salamatian, S. H. Kim, M. Calder, and E. Katz-Bassett, "The central problem with distributed content: common CDN deployments centralize traffic in a risky way", In Proceedings of the 22nd ACM Workshop on Hot Topics in Networks (HotNets ’23). Association for Computing Machinery, New York, NY, USA, 70-78.

[MHV01]	A. Bentaleb, R. Farahani, F. Tashtarian, C. Timmerer, H. Hellwagner, and R. Zimmermann, "Which CDN to Download From? A Client and Server Strategies", (Jan. 6, 2024). Accessed: May 9, 2024. [Online Video]. Available: https://www.youtube.com/watch?v=xCZmCnWgQRE

[VAS01]	Will Law, "Content steering with MPEG DASH", (May 4, 2023). Accessed: May 9, 2024. [Online Video]. Available: https://www.youtube.com/watch?v=o9Pa5y-Usxw

[MWS23]	W. Law and Y. Reznik, "DASH content steering at scale", Media Web Symposium (MWS’23), June 2023.

[DIFCS]	ETSI TS 103 998: "DASH-IF: Content steering for DASH".

[CMMF]	ETSI TS 103 973: "Coded multisource media format (CMMF) for content distribution and delivery".

[TS26510]	3GPP TS 26.510: "Interactions and APIs for provisioning and media session handling (Release 18)".

[RAPTORQ]	IETF RFC 6330: "RaptorQ forward error correction scheme for object delivery", August 2011.

[RSFEC]	IETF RFC 5110: "Reed-Solomon forward error correction (FEC) schemes", April 2009.

## SECOND CHANGE

## 5.19	Multi-CDN media delivery

### 5.19.1	Description

#### 5.19.1.1	Introduction

Media streaming applications conventionally obtain content from a single source or endpoint over a single path within a network. This imposes several limitations:

1.	Performance is constrained to that of the source/endpoint and path chosen. Whatever the limits on network bandwidth and latency between the client and that source/endpoint are directly translated to the client’s achievable Quality of Service (QoS) and Quality of Experience (QoE).

2	Disruptions or degraded performance caused by the source/endpoint in use or on any of the network links between the client and source/endpoint can lead to poor user experience, often in the form of lower playback quality, rebuffering, or complete playback failure.

This study considers integration of different technologies into the 5G Media Streaming System that addresses these, and similar, issues by allowing media streaming applications to efficiently access content across multiple Content Delivery Networks (CDNs) and/or multiple access networks. Different client implementations may then beneficially use the content on these multiple sources or networks concurrently, potentially guided by service or network provider. In addition, formats and techniques for generating content for multiple CDN or multiple access network delivery such as MPEG-DASH Part 9 (ReAP) [DASH9] may be considered. Further extensions include the ability for a client to use multiple access networks at the same time to support media delivery. Study of integration of different technologies into the 5G Media Streaming System is of relevance to address content provisioning, content hosting, impacts on user plane reference points M2 and M4, and on media session handling at reference point M5 as well as potential benefits in terms of quality and resource usage.

#### 5.19.1.2	Challenges Multi-CDN deployments aim to address

CDNs are often used by content distributors to globally scale delivery of their content to end-users. These networks consist of a number of Points of Presence (PoPs) located at various locations around the networks’ edge. These PoPs help load-balance delivery of content as well as improve Quality of Service (QoS) by reducing the distance/latency between every client and the content they are accessing. In many cases, content distributors employ multiple CDNs to leverage the strengths of one CDN over another in every location those CDNs have a PoP. For example, a client experiencing degraded performance while using one CDN may switch to another that is offering better performance at that time and location. As another example, a content distributor may prefer one CDN over another at a given time to reduce delivery costs and/or meet monthly contractual commitments. These multi-CDN deployments aim to solve content delivery issues that exist when only one CDN is used; but the benefits they provide may not be fully realized because of the various challenges experienced and underlying methods used to stream content to every client.

Challenges multi-CDN deployments and architectures aim to address may include:

1.	Sustained CDN-/network-wide service disruptions where network access, connectivity or QoS is severely degraded. Examples may include cases where an entire CDN’s network is degraded because of a network-wide misconfiguration or power failure. The duration of these events may last minutes to hours and affect a majority of the client population. Examples of recorded instances can be found in [UNPKG24], [NET23], [FSLY21], [AKAM21], [NET22], and [VZ19].

2.	Intermittent or short-term disruptions affecting QoS for an individual or small group of clients. Examples include short periods of congestion within the network, isolated HTTP request/response failures or delays caused by application server congestion, etc. The following discusses these in greater detail: [DEMX01], [DEMX02], [IEEE01], [ACM01], [MHV01], [VAS01], and [MWS23]

3.	Augmentation of one CDN's performance with that of another to achieve a level of performance that neither can provide on its own. An example is a peer-to-peer CDN where each peer has limited uplink capacity and is unable to satisfactorily service client demand on its own.

#### 5.19.1.3	Coded Multi-source Media Format (CMMF)

##### 5.19.1.3.1	Functional description of CMMF

Coded Multi-source Media Format (CMMF) [CMMF] is an extensible container format designed to facilitate the management and interchange of audio-visual media and metadata in one or more coded representations (e.g., encoded with application-layer, forward error correction (FEC), linear, network, or channel codes). The coded media representations supported by CMMF enable the efficient use of multi-source, multi-path, and multi-access connectivity for network-delivered media applications. The use of CMMF does not replace the basic media streaming architectures and procedures already defined. Rather, it is intended to supplement them to provide additional capabilities.

To understand some of the advantages of using CMMF for streaming media, CMMF was implemented and trialed on a commercial streaming platform from approximately September 2022 through September 2023. This platform offers a large content library, streamed to a world-wide customer base where the majority of the content had a maximum bit rate of 5 Mbps or less (the median maximum bit rate available was approximately 3.5 Mbps while over 70% of all sessions had a maximum possible bit rate of 5 Mbps or less). Approximately 5% - 50% of the traffic on selected device types was streamed using CMMF while the remainder of the traffic was streamed using a popular conventional server-side switching/DNS-based multi-CDN implementation. Both the CMMF multi-source and the conventional multi-CDN approach used three tier 1 CDNs. CMMF clients downloaded content from each CDN in parallel, while the "conventional" clients switched between the three based on input from the multi-CDN switching platform. Performance measurements for all traffic were collected using an industry-leading performance measurement platform. This data includes session-level information about relevant QoE key performance indicators (KPIs). In addition, supplemental QoS information was collected for only those sessions using CMMF multi-source as a delivery method.

A summary of the amount of traffic measured for each delivery method during this trial is provided in Table 5.19.1.3-1. This and subsequent tables only show traffic measured for Android clients streaming over cellular networks from January 1 through July 26, 2023. Furthermore, only those sessions where the mean edge cache hit rate is greater than 50% are considered. For CMMF traffic, this was determined using the supplemental QoS information collected for each CMMF session. For conventional traffic, this supplemental information was not available on a session-by-session basis since this traffic bypassed the functionality added when implementing CMMF within the player. Rather, it was confirmed, via querying each CDN utilized, that the mean edge cache hit rates for all conventional traffic was greater than 95%. This estimate of the edge cache hit rate was also validated in a separate experiment where conventional traffic was routed through the new CMMF player functionality so that QoS metrics (including cache hit status) could be collected. Unfortunately, the volume of CMMF traffic and the diversity of the content streamed during the trial made it very difficult to keep CDN caches warm with CMMF encoded content. Trying to match multi-source and conventional edge cache hit rates on a one-to-one basis was not possible. As a result, the threshold established above provides sufficient data to provide statistically significant results; but it also implicitly favors conventional delivery since those sessions were more often served by the CDNs’ edge.

Table 5.19.1.3-1: CMMF real-world multi-CDN trial summary. Only sessions measured on cellular networks and running Android are shown

| Delivery method | Hours watched | Number of playback sessions | Number of unique devices | Number of unique countries | Minutes watched per unique device |
| --- | --- | --- | --- | --- | --- |
| Conventional | 25,026.92 | 120,269 | 23,752 | 178 | 63.22 |
| CMMF | 14,013.76 | 44,081 | 12,534 | 141 | 67.08 |

An overview of the performance improvements multi-source delivery provided over conventional multi-CDN switching for various QoE KPI’s is shown in Table 5.19.1.3-2. The table provides the mean value of the relevant KPI plus/minus one standard deviation. In general, double-digit gains were observed across all key QoE performance indicators showing that CMMF enabled multi-source delivery can drastically improve the quality of streamed media.

Table 5.19.1.3-2: Real-world multi-CDN QoE performance results

| Delivery method | Normalized average session bit rate (% of max session bit rate) (NOTE 1) | Playback start-up time (s) (NOTE 2) | Connection-induced rebuffering ratio (%) (NOTE 3) | Start-up failure rate (%) | Playback failure rate (%) |
| --- | --- | --- | --- | --- | --- |
| Conventional | 83.70 ± 28.08 | 3.40 ± 10.08 | 0.28 ± 1.78 | 0.51 | 1.22 |
| CMMF | 94.31 ± 16.23 | 1.83 ± 9.34 | 0.19 ± 1.17 | 0.07 | 0.59 |
| Difference | 10.61 ± 32.43 | -1.57 ± 13.74 | -0.09 ± 2.13 | -0.44 | -0.63 |
| Gain | + 12.68% | - 46.18% | - 32.14% | -86.27 % | - 51.64% |
| NOTE 1: The normalized average session bit rate is defined as the average bit rate measured during a session divided by the maximum bit rate listed in the session’s content manifest. Only sessions without a start-up or video playback failure, a playing time greater than or equal to 60 seconds, playback completed at least 10% of the content, and the maximum bit rate as defined by the sessions’ corresponding manifest was available. These statistics are weighted using the sessions’ duration. NOTE 2: Only sessions without a start-up or playback failure and a start-up time greater than 0 seconds. NOTE 3: Only sessions without a start-up or playback failure, a playing time greater than or equal to 60 seconds, and playback completed at least 10% of the content. These statistics are weighted using the sessions’ duration. |  |  |  |  |  |

The empirical CDFs for the content normalized average session bit rate, start-up time, and connection-induced rebuffing ratio are provided in figures 5.19.1.3-1, 5.19.1.3-2, and 5.19.1.3-3 respectively.

Figure 5.19.1.3-1 shows that 60% of the sessions, regardless of delivery method, experienced an average session playback bit rate close to the maximum possible based on the content being played. However, CMMF multi-source delivery was able to lift more of those clients that could not reach the highest bit rate further up the bit rate ladder than conventional delivery.

Figure 5.19.1.3-1: Empirical CDF of the content normalized average session bit rate.

Figure 5.19.1.3-2 shows that CMMF multi-source delivery was able to significantly reduce the playback start-up time as well. For example, only 10.4% of the CMMF sessions experienced a startup time greater than 3 seconds compared to 29.0% of the conventional sessions.

Figure 5.19.1.3-2: Empirical CDF of the video startup time.

Finally, figure 5.19.1.3-3 shows that CMMF multi-source delivery reduced the number of sessions that experienced a connection-induced rebuffering event from 22.4% to 14.9% in addition to reducing the total duration of rebuffering given a rebuffering event occurred.

Figure 5.19.1.3-3: Empirical CDF of the connection-induced rebuffering ratio (CIRR).

A typical Video-on-Demand (VoD) MPEG-DASH HTTP adaptive streaming system is set up similarly to the non-shaded blocks shown in figure 5.19.1.3.1-1. Source media (e.g. audio/video elementary streams) are segmented and encoded into multiple representations, each with a different quality and bit rate. These segments are packaged together using MPEG-DASH (or HLS) and stored on an origin server located within the network. One or more CDNs are set up to distribute and deliver this content to an OTT service provider’s customer base. These CDNs obtain every requested MPEG-DASH (or HLS) segment from an origin server, caches these segments at their respective network edges, and deliver these segments to clients.

Enabling multisource delivery using CMMF within this existing delivery architecture can be accomplished through the addition of a CMMF Bitstream Generator/Source before segments are delivered to the CDNs, and a CMMF Receiver on each client, as illustrated by the shaded boxes in figure 5.19.1.3.1-1.

Figure 5.19.1.3.1-1: MPEG-DASH with CMMF Delivery System Example

While Figure 5.19.1.3.1-1 shows the CMMF Bitstream Generator/Source between the Origin Server and the CDNs (i.e., CMMF bitstreams are created on demand), the CMMF Bitstream Generator/Source can just as easily be located between the MPEG-DASH Packager and the Origin Server. In the former case, the original MPEG-DASH segments are stored on the Origin Server and CMMF representations of those segments are cached on each CDN. In the latter case, the CMMF Bitstream Generator/Source creates multiple CMMF representations of each segment produced by the MPEG-DASH Packager and stores them on the Origin Server for later retrieval by a CDN.

In this system, the Application Service Provider is responsible for segmenting, encoding, and packaging the media. It is also responsible for creating a presentation manifest (MPD, shown in figure 15.19.1.3.1-2) that contains relative URLs to the files/segments (shown in a dotted outline) that make up the adaptation sets. This information is transferred from the Application Service Provider to the media player (using a method preferred by the Application Service Provider) and is used by the media player to determine which segments are to be downloaded (via the CMMF Receiver) and played.

Figure 15.19.1.3.1-2: Example MPEG-DASH MPD

The Application Service Provider in this example is also responsible for determining and setting up the delivery system (i.e., the CDNs) to distribute CMMF-encoded media. Information about this setup is captured within the CMMF Configuration Information as a list of host URLs to each of the CDNs. It is important to note that both the MPEG-DASH master manifest and the CMMF Configuration Information is required to download the media. In this example, it is assumed that the Application Service Provider is utilizing two CDNs and each segment listed in the MPEG-DASH MPD is encoded and packaged into two unique CMMF bitstreams/objects, one intended for the first CDN and the other for the second CDN according to the list of host URLs provided in the CMMF Configuration Information. An example of this is shown in figure 15.19.1.3.1-3. Furthermore, example bitstream/object constructions can be found in annex C of [CMMF].

Figure 15.19.1.3-3: CMMF bitstreams/objects generated
to deliver the MPEG-DASH packaged content

The process for streaming this content is shown in figure 15.19.1.3.1-4. An Application Provider provisions an origin server with the original source segments shown in figure 15.19.1.3.1-1. Upon client initialization, the CMMF Receiver requests the CMMF Configuration Information (in this case it is stored as a JSON file) from the Application Service Provider. This information communicates a list of host URLs specifying the locations for which the client can retrieve CMMF encoded content for every segment listed within the master manifest file. Upon media player/delivery session initialization, the media player retrieves and parses the presentation manifest (MPD) from the Application Service Provider and chooses the adaptation set(s) it wishes to stream. Once selected, the media player requests the appropriate segment from the CMMF Receiver using the relative URL communicated in the master manifest. The CMMF Receiver joins the relative URL with each of the host URLs from the CMMF Configuration Information file and requests two distinct CMMF bitstreams of that segment from the two available CDNs. Assuming the appropriate CMMF bitstream is cached, the CDN begins delivery. Otherwise, the CDN requests the segment from the CMMF Bitstream Generator/Source. At which point, the original segment is pulled from storage, encoded, and delivered to the CDN and to the client. The CMMF Receiver downloads the two CMMF bitstreams of that segment until it is capable of decoding. These downloads are performed using an appropriate strategy based on the underlying network protocols in use and network conditions. Once enough CMMF-encoded content has been received, the CMMF Receiver decodes the segment and delivers it to the media player. The media player selects the next segment to be downloaded and the process repeats.

Figure 15.19.1.3.1-4: CMMF request and content delivery example for MPEG-DASH

In the case of a third CDN being introduced, a new CMMF bitstream can be generated and cached without replacing or modifying the existing CMMF bitstreams already cached in the initial two CDNs. All that is required is an update to the host URL list managed by the Application Service Provider.

##### 5.19.1.3.2	CMMF requirements on multi-CDN delivery

Based on the description in the previous clause, multi-CDN and/or multi-access media delivery using CMMF requires preparation of the content to be served to a population of clients and clients that can access and download from multiple sources in parallel. Specifically,

1.	The ability to create CMMF-encoded media objects and distribute/stripe these (in addition to possibly distributing the original source media such as MPEG-DASH or HLS media segments) across multiple client-accessible network locations (e.g., 5GMS Application Servers, CDNs, etc.).

Accessing content from multiple sources/endpoints within the network simultaneously requires that each network source/endpoint be populated with a unique CMMF bitstream/object containing the content being requested. A CMMF network source/endpoint is one that can be individually addressable or reachable (i.e., it is recommended that there exists a one-to-one mapping between the set of individually addressable or reachable sources/endpoints and the set of CMMF bitstreams/objects for each CMMF encoded piece of content). Source/endpoint types may be entire CDN distributions, single points-of-presence (PoPs) within a single CDN distribution, or standalone servers. For example, a single CDN which replicates content across their PoPs and uses DNS or anycast to route traffic to the PoPs within their network would be considered one source/endpoint. Alternatively, a CDN that enables clients to reach individual PoPs within their network may allow for each PoP to be an CMMF source/endpoint assuming each PoP can be populated with a unique CMMF bitstream/object.

Various methods for creating unique CMMF bitstreams/objects for each CMMF network source/endpoint exist. The necessary CMMF bitstreams/objects can be created offline (e.g., at the time the video/audio is encoded and packaged) and stored on an origin server for later retrieval by the CMMF network sources/endpoint. They can also be created on demand using a cloud-based or edge-based just-in-time encoder as client requests are received.

Detailed examples for preparing original source media for delivery from multiple serving endpoints using CMMF are provided in [CMMF]. In general, the processing required to create CMMF bitstreams/objects is minimal (and scalable) allowing for a wide range of available implementation options.

2.	The capability for clients to access, efficiently download, and decode information collected from the partially received CMMF-encoded media bitstreams/objects obtained in parallel from multiple network endpoints where CMMF-encoded media objects (and possibly original source media) are stored/cached.

These capabilities can be implemented as a plug-in similar to the MBMS Client shown in clause 4.6.1 of [TS26510] to simplify integration into existing platforms and players, or they can be implemented directly within the streaming media player located on each client. When downloading content (e.g., a segment that is intended to be played), a CMMF client will connect to multiple sources/endpoints and request the CMMF bitstream/object associated with that content from each. Any one of these CMMF bitstreams/objects do not need to be obtained in their entirety, nor does any byte-level scheduling need to occur (e.g., each CMMF bitstream/object can be transmitted from their beginning to their end). Rather, a client only needs to obtain enough information from all of the transmitted CMMF bitstreams/objects so that it can decode the content those bitstreams/objects carry.

The methods used to efficiently download media using CMMF from multiple sources/endpoints are heavily dependent on the underlying network and transport protocols used to deliver CMMF-encoded bitstreams/objects, as well as the implementation of the CMMF-enabled client. For streaming use cases using either HTTP/1.1 or HTTP/2 over TCP, total overhead (i.e., total amount of data egressed from all of the sources/endpoints (including HTTP and CMMF headers) with respect to the size of the original source media object) has been demonstrated to be between 0.5-3% when downloading 1080p, 4-second MPEG-DASH segments encoded at approximately 5 Mbps (it should be highlighted that this is roughly on-par with the overhead induced by in-market media players related to downloading multiple bit rates of the same segment and (obviously) rendering only one during playback). Overhead when using other network/transport protocols or different implementations may differ.

Likewise, the number of requests sent by a CMMF-enabled client to each CMMF endpoint can also be considered overhead. Various strategies can be employed depending on the capabilities or limitations of the underlying transport protocols and network. Cases where the client is limited (e.g., it is using HTTP/1.1 [?] to obtain CMMF-encoded content where the client can only make requests serially and cancel them by closing the underlying TCP connection) may require a different strategy than cases where a more advanced protocol is used (e.g., HTTP/2 [4]).

-	In the case where HTTP/1.1 is used as the application protocol, one strategy to download CMMF-encoded content while limiting the amount of extra data downloaded as well as avoiding cancelling requests may be to make several byte range requests to each CMMF endpoint where these byte range requests are sized appropriately for the observed network conditions and the amount of data that is still required at that time. A current, state-of-the-art implementation of this strategy generates between three and six requests to each CMMF endpoint (depending on the network conditions) while downloading 1080p, 4-second MPEG-DASH segments encoded at approximately 5 Mbps.

-	In cases where the client is using an underlying protocol suite with more features (e.g., HTTP/2 over TCP or HTTP/3 [5] over QUIC [?]), those features may allow for different strategies. A strategy that uses flow control features available within HTTP/2 achieves the same download as mentioned above while only sending one request to each CMMF endpoint.

Of course, other strategies exist, and the approach they take to obtain CMMF-encoded content from multiple CMMF endpoints is dependent on the network protocols used, the underlying networks’ capabilities, and the overall use case.

The complexity and client device impacts of decoding received CMMF bitstreams/objects has also been demonstrated to be minimal. While the decode complexity is dependent on the CMMF code type used (CMMF [CMMF] supports a variety of different code types including general deterministic and random linear codes (RLC), the 3GPP Raptor code specified in RFC 5053 [RFC5053] as defined in TS 26.346 [26346], RaptorQ as specified in RFC 6330 [RAPTORQ], and Reed-Solomon as specified in RFC 5110 [RSFEC]), CMMF has been demonstrated on over 4,000 unique client device models without issue.

##### 5.19.1.3.3	CMMF performance

To understand some of the advantages of using CMMF for streaming media, CMMF was implemented and trialed on a commercial streaming platform from approximately September 2022 through September 2023. This platform offers a large content library, streamed to a world-wide customer base where the majority of the content had a maximum bit rate of 5 Mbps or less (the median maximum bit rate available was approximately 3.5 Mbps while over 70% of all sessions had a maximum possible bit rate of 5 Mbps or less). Approximately 5% - 50% of the traffic on selected device types was streamed using CMMF while the remainder of the traffic was streamed using a popular conventional server-side switching/DNS-based multi-CDN implementation. Both the CMMF multi-source and the conventional multi-CDN approach used three tier 1 CDNs. CMMF clients downloaded content from each CDN in parallel, while the "conventional" clients switched between the three based on input from the multi-CDN switching platform. Performance measurements for all traffic were collected using an industry-leading performance measurement platform. This data includes session-level information about relevant QoE key performance indicators (KPIs). In addition, supplemental QoS information was collected for only those sessions using CMMF multi-source as a delivery method.

A summary of the amount of traffic measured for each delivery method during this trial is provided in table 5.19.1.3.3-1. This and subsequent tables only show traffic measured for Android clients streaming over cellular networks from January 1 through July 26, 2023. Furthermore, only those sessions where the mean edge cache hit rate is greater than 50% are considered. For CMMF traffic, this was determined using the supplemental QoS information collected for each CMMF session. For conventional traffic, this supplemental information was not available on a session-by-session basis since this traffic bypassed the functionality added when implementing CMMF within the player. Rather, it was confirmed, via querying each CDN utilized, that the mean edge cache hit rates for all conventional traffic was greater than 95%. This estimate of the edge cache hit rate was also validated in a separate experiment where conventional traffic was routed through the new CMMF player functionality so that QoS metrics (including cache hit status) could be collected. Unfortunately, the volume of CMMF traffic and the diversity of the content streamed during the trial made it very difficult to keep CDN caches warm with CMMF encoded content. Trying to match multi-source and conventional edge cache hit rates on a one-to-one basis was not possible. As a result, the threshold established above provides sufficient data to provide statistically significant results; but it also implicitly favors conventional delivery since those sessions were more often served by the CDNs’ edge.

Table 5.19.1.3.3-1: CMMF real-world multi-CDN trial summary. Only sessions measured on cellular networks and running Android are shown

| Delivery method | Hours watched | Number of playback sessions | Number of unique devices | Number of unique countries | Minutes watched per unique device |
| --- | --- | --- | --- | --- | --- |
| Conventional | 25,026.92 | 120,269 | 23,752 | 178 | 63.22 |
| CMMF | 14,013.76 | 44,081 | 12,534 | 141 | 67.08 |

An overview of the performance improvements multi-source delivery provided over conventional multi-CDN switching for various QoE KPI’s is shown in table 5.19.1.3.3-2. The table provides the mean value of the relevant KPI plus/minus one standard deviation. In general, double-digit gains were observed across all key QoE performance indicators showing that CMMF enabled multi-source delivery can drastically improve the quality of streamed media.

Table 5.19.1.3.3-2: Real-world multi-CDN QoE performance results

| Delivery method | Normalized average session bit rate (% of max session bit rate) (NOTE 1) | Playback start-up time (s) (NOTE 2) | Connection-induced rebuffering ratio (%) (NOTE 3) | Start-up failure rate (%) | Playback failure rate (%) |
| --- | --- | --- | --- | --- | --- |
| Conventional | 83.70 ± 28.08 | 3.40 ± 10.08 | 0.28 ± 1.78 | 0.51 | 1.22 |
| CMMF | 94.31 ± 16.23 | 1.83 ± 9.34 | 0.19 ± 1.17 | 0.07 | 0.59 |
| Difference | 10.61 ± 32.43 | -1.57 ± 13.74 | -0.09 ± 2.13 | -0.44 | -0.63 |
| Gain | + 12.68% | - 46.18% | - 32.14% | -86.27 % | - 51.64% |
| NOTE 1: The normalized average session bit rate is defined as the average bit rate measured during a session divided by the maximum bit rate listed in the session’s content manifest. Only sessions without a start-up or video playback failure, a playing time greater than or equal to 60 seconds, playback completed at least 10% of the content, and the maximum bit rate as defined by the sessions’ corresponding manifest was available. These statistics are weighted using the sessions’ duration. NOTE 2: Only sessions without a start-up or playback failure and a start-up time greater than 0 seconds. NOTE 3: Only sessions without a start-up or playback failure, a playing time greater than or equal to 60 seconds, and playback completed at least 10% of the content. These statistics are weighted using the sessions’ duration. |  |  |  |  |  |

The empirical CDFs for the content normalized average session bit rate, start-up time, and connection-induced rebuffing ratio are provided in figures 5.19.1.3.3-1, 5.19.1.3-2, and 5.19.1.3.3-3 respectively.

Figure 5.19.1.3.3-1 shows that 60% of the sessions, regardless of delivery method, experienced an average session playback bit rate close to the maximum possible based on the content being played. However, CMMF multi-source delivery was able to lift more of those clients that could not reach the highest bit rate further up the bit rate ladder than conventional delivery.

Figure 5.19.1.3.3-1: Empirical CDF of the content normalized average session bit rate.

Figure 5.19.1.3.3-2 shows that CMMF multi-source delivery was able to significantly reduce the playback start-up time as well. For example, only 10.4% of the CMMF sessions experienced a startup time greater than 3 seconds compared to 29.0% of the conventional sessions.

Figure 5.19.1.3.3-2: Empirical CDF of the video startup time.

Finally, figure 5.19.1.3.3-3 shows that CMMF multi-source delivery reduced the number of sessions that experienced a connection-induced rebuffering event from 22.4% to 14.9% in addition to reducing the total duration of rebuffering given a rebuffering event occurred.

Figure 5.19.1.3.3-3: Empirical CDF of the connection-induced rebuffering ratio (CIRR).

These results were collected using a system similar to that shown in annex B of [CMMF].

### 5.19.2	Collaboration scenarios

#### 5.19.2.1	Multi-CDN media delivery

In this scenario, the 5GMSd Client requests adaptive media streaming content from two or more 5GMSd Application Servers. The Client may choose one 5GMSd AS or use multiple simultaneously. This allows the client to distribute network load across Application Servers and M4 downlink transports, optimize costs, as well as improve QoS.

The client’s Media Session Handler discovers the URLs of these Application Servers from the 5GMSd Application Function (AF), either through a Media Entry Point or from a separate piece of metadata. QoE metrics from the client may be used by the AF to determine the best Application Server(s) for each client to use when streaming media.

Figure 5.19.2.1-1 shows the client communicating with multiple Application Servers. Each AS has no direct communication with its peers; rather it communicates (minimally) with the Application Provider and with the 5GMSd AF (not depicted) via reference point M3d.

Figure 5.19.2.1-1: Multi-CDN media delivery within 5G system

#### 5.19.2.2	Joint multi-CDN and multi-access media delivery

In this scenario, the 5GMSd Client is directly connected to multiple data, or access, networks (e.g., an unmanaged Wi-Fi network and the 5G network), as described in clause 5.18. The client requests adaptive media streaming content from two or more 5GMSd Application Servers. The Client may choose one or use multiple simultaneously. This allows the client to distribute network load across access networks and Application Servers, optimize costs, as well as improve QoS.

The client’s Media Session Handler discovers the URLs of these Application Servers from the 5GMSd Application Function (AF), either through a Media Entry Point or from a separate piece of metadata. QoE metrics from the client may be used by the AF to determine the best Application Server(s) for each client to use when streaming media.

Figure 5.19.2.2-1 shows the client communicating with multiple Application Servers through different data networks. Neither data network nor AS has direct communication with its peers. Rather each 5GMSd AS communicates (minimally) with the 5GMSd Application Provider at reference point M2 and with the 5GMSd AF (not depicted) via reference point M3d.

Figure 5.19.2.2-1: Multi-access media delivery within 5G system

### 5.19.3	Architecture mappings

#### 5.19.3.1	Server-side CDN switching

Editor’s Note:	Inclusion and expansion on these sets of solutions is dependent on interest from working group.

#### 5.19.3.2	Client-side CDN switching

Editor’s Note:	Inclusion and expansion on these sets of solutions is dependent on interest from working group.

#### 5.19.3.3	Concurrent CDN access using CMMF

Several options on the network side, client side and configuration exist when implementing CMMF within the 5GMS System. These are expanded upon further below.

##### 5.19.3.3.1	CMMF content preparation

The use of CMMF for delivering content from multiple sources/CDNs requires that the content be encoded into multiple coded representations (one per source/CDN). This requires the source content to be prepared (i.e., encoded within CMMF) somewhere within the network prior to a client attempting to access it. Various options exist for where these encoding procedures take place. These options include:

1.	5GMSd Application Provider Content Preparation. In this option, it is the responsibility of the 5GMS Application Provider to encode and package source content within CMMF bitstreams/objects prior to delivery of that content separately to each 5GMSd AS instance via reference point M2d or to each external 5GMSd AS instance. This is illustrated in figures 5.19.2.1-1 and 5.19.2.2-1 above.

2.	Centralized 5GMSd Content Preparation. In this option, a single, primary 5GMSd AS instance encodes and packages source content that has been ingested at reference point M2d into CMMF bitstreams/objects according to a (yet to be defined) configuration provided by a Content Preparation Template(s). The CMMF bitstreams/objects created during this media processing task may be delivered directly to the 5GMSd Client (via reference point M4d), delivered to another (secondary) 5GMSd AS instance (via reference point M10), or delivered to a 5GMSd AS located in an external, possibly untrusted, Data Network. These CMMF bitstreams/objects may then be cached and/or forwarded onward. This primary 5GMSd AS instance is responsible for creation of all CMMF encoded representations used to deliver content from multiple sources. This is illustrated in figure 5.19.3.3.1-1 below. The secondary 5GMSd AS instances may be deployed either in the Trusted DN, in an Edge DN or in an External DN.

Figure 5.19.3.3.1-1: Option #2 for deploying CMMF within 5GMS
where a single, primary 5GMSd AS performs all CMMF content preparation.

3.	Decentralized 5GMSd Content Preparation. The possibility also exists to distribute the CMMF media processing across 5GMSd Application Servers such that each 5GMSd AS instance is only responsible for creation of a single CMMF representation for which it intends to cache and/or deliver to a 5GMSd Client via reference point M4d. In this option, each 5GMSd AS instance provisioned with the Content Preparation Template may receive original source content or CMMF-encoded content from either the 5GMSd Application Provider at reference point M2d or from another 5GMSd AS instance at reference point M10d. This received content is then processed to create a new, unique CMMF-encoded representation which can be used in conjunction with others during a multi-source download. Similarly, an externally deployed 5GMSd AS may be configured by the 5GMSd Application Provider (by private means) to perform a similar media processing task to create its own CMMF-encoded representation. This is illustrated in figure 5.19.3.3.1-2 below.

Figure 5.19.3.3.1-2: Option #3 for deploying CMMF within 5GMS
where each 5GMSd AS instance performs independent CMMF content preparation.

A combination of options 1-3 is also possible where some aspect of all three exist within a physical realization of the network.

##### 5.19.3.3.2	CMMF 5GMS client architecture

Implementing multi-source/CDN delivery using CMMF requires modifications to the 5GMSd Client. At a minimum, a 5GMSd Client must be able to download CMMF bitstreams/objects from multiple 5GMSd Application Server instances simultaneously and decode the received bitstreams/objects. Options for implementing multi-source/CDN delivery using CMMF within the 5GMSd Client include:

1.	CMMF Client Proxy. This option implements multi-source/CDN using CMMF within the client as a proxy between the Media Player and each 5GMSd AS. The proxy consists of a CMMF Client and a Media Server. Once the Media Session Handler of the 5GMSd Client has configured the CMMF Client via reference point CMMF-2, the Media Player may request source content via the Media Server using reference point CMMF-3. Once a request is received, the CMMF Client downloads different CMMF encoded representations of the requested content via reference point(s) CMMF-1 (this reference point is functionally equivalent to reference point M4 despite terminating on a different logical function in the 5GMSd Client), decodes the received CMMF bitstreams/objects, and replies to the Media Player with the requested source content via CMMF-3. This option is illustrated in figure 5.19.3.3.2-1.

Figure 5.19.3.3.2-1: Option #1 for integration of CMMF within the 5GMS Client where CMMF is implemented as a client proxy.

2.	CMMF decoder integrated in Media Player. This option implements CMMF within the Media Player itself. An example is provided in figure 5.19.3.3.2-2 depicting CMMF integrated within the DASH-based 5GMSd Client specified in clause 13.2 of TS 26.512 [16]. The architecture and operation of the 5GMS Client is similar to that in [16] with the following exceptions:

a.	Download: Downloads source content objects and/or CMMF bitstreams/objects from one or more 5GMSd AS instances in parallel.

b.	Request Scheduling: Performs the same function as defined in clause 13.2 of [16] with the addition of managing the concurrent requests sent over reference point M4 during the download of content encoded within CMMF.

c.	Throughput Estimation: Estimates the throughput from each individual 5GMSd AS instance in addition to estimating the aggregated throughput from all 5GMSd AS instances.

d.	CMMF Receiver/Decoder: Temporarily stores and jointly decodes CMMF bitstreams/objects as they are downloaded. Once decoded, the source content objects are moved to the Media Playback Management and Protection Controller. The CMMF Receiver/Decoder also provides status updates containing decode progress to each active download function for the purposes of managing/terminating in-process downloads.

Figure 5.19.3.3.2-2: Option #2 for integration of CMMF within the 5GMS Client where CMMF is integrated directly within the Media Player.

##### 5.19.3.3.3	CMMF configuration, provisioning, hosting, and Service Access Information

###### 5.19.3.3.3.0	General

CMMF configuration, provisioning, and hosting is the overall responsibility of the 5GMSd Application Provider. The 5GMSd Application Provider may configure and provision resources to deliver media using CMMF across both external and trusted data networks. Options for configuring CMMF media delivery include:

1.	CMMF-encoded media delivery is transparent to the 5GMS System. In these cases, the 5GMSd Application Provider prepares all CMMF bitstreams/objects intended to be distributed across every Application Server instance (whether located within an external or trusted Data Network). Hosting of CMMF encoded media within the 5GMS System is performed according to established downlink 5G Media Streaming architectures (see clause 4.2 of TS 26.501 [15]) and procedures (see clause 5.2 of [15]). CMMF-enabled 5GMSd Clients accessing CMMF-encoded media hosted on one or more 5GMSd Application Server instances where the media is replicated rather than different CMMF-encoded representations of that media will treat the set of 5GMSd AS instances as a single CMMF endpoint. CMMF endpoints established in external Data Networks configured and provisioned by the 5GMSd Application Provider can be used to supplement 5GMS-delivered media. In this case, a CMMF-enabled 5GMSd Client is configured to use CMMF via reference point M8d.

2.	CMMF content preparation and/or hosting is performed by the 5GMS System. In these cases, the 5GMSd Application Provider configures and provisions 5GMS resources (e.g., 5GMSd AF, 5GMSd AS instances, etc.) via reference point M1d to prepare and/or deliver CMMF-encoded media. The 5GMSd Application Provider may specify CMMF content preparation (e.g., CMMF encoding, etc.) and/or hosting by network-side components of the 5GMS System according to a provisioned CMMF Content Preparation Template. In such cases, individual 5GMSd AS instances may operate as separate CMMF endpoints for the purposes of CMMF-enabled media delivery assuming the system has been appropriately configured and provisioned to ensure CMMF-encoded media is not replicated between available 5GMSd AS instances. 5GMSd Client configuration information is provided by the 5GMS AF via reference point M5d.

The following clauses provide additional details for option 2 above, where configuration, provisioning and hosting CMMF media streaming is supported by the 5GMS System.

###### 5.19.3.3.3.1	CMMF provisioning and service information

The provisioning step allows a 5GMSd Application Provider to configure information about its CMMF content preparation and hosting requirements for media streaming sessions using 5GMS resources. The following information may be made available and/or configurable over reference point M1d:

-	Endpoint locations or base URLs outside of the 5GMS system (i.e., within external data networks) that support CMMF delivery.

-	Information necessary for configuring the preparation of CMMF-encoded media on 5GMSd AS instances within Trusted DNs.

-	Necessary CMMF bitstream/object preparation parameters as defined in [CMMF] required to properly encode and package media within CMMF bitstreams/objects. Examples may include CMMF code type, CMMF profile, etc.

-	Information necessary for configuring the hosting of CMMF-encoded media on 5GMS Application Servers within trusted DNs.

-	This may include Edge Computing requirements and EAS profile information as defined in clause 4.5.3 of TS 26.501 [15] for each 5GMSd EAS instance that will be used to create CMMF bitstreams/objects.

Once provisioned, the 5GMSd AF allocates and manages the set of 5GMSd AS instances (including Edge Computing resources, if required) needed for the creation and/or hosting of CMMF bitstreams/objects generated from media provided by the 5GMSd Application Provider at reference point M2d. The 5GMSd AF ensures that each 5GMSd AS instance that is hosting CMMF-encoded content distributes a unique CMMF bitstream/object created from the original source media.

The 5GMSd AF is also responsible for communicating necessary configuration information (see clause 5.19.4.3.1) with the Media Session Handler in the 5GMSd Client via reference point M5d.

###### 5.19.3.3.3.2	Service Access Information for downlink media streaming using CMMF

CMMF supplements existing media streaming architectures (e.g., MPEG-DASH, etc.) to enable clients to obtain media from multiple endpoints in parallel. Enabling this functionality requires, at a minimum, information concerning where CMMF-encoded media can be accessed on one or more endpoints within the network(s). This information may be conveyed to the Media Player in the 5GMSd Client (assuming client architecture 2 in clause 5.19.3.3.2) either:

1.	Privately by the 5GMSd Application Provider as Service Access Information provided via a 5GMSd-Aware Application at reference point M8d followed by M7d. Alternatively, the 5GMSd-Aware Application may provide this information to the Media Session Handler via reference point M6d, and the Media Session Handler makes this information available to the Media Player via reference point M11d.

2.	By the 5GMSd AF to the Media Session Handler as Service Access Information made available at reference point M5d and then passed via reference point M11d.

3.	Contained within a Media Entry Point specified by an existing media streaming framework that is understood by the Media Player (e.g., as additional XML elements or attributes within an MPEG-DASH MPD).

Other CMMF configuration information that are outside the scope of the present document may also be provided and/or required based on implementation. This may include additional configurable parameters such as load-balancing policies that influence client behaviour when downloading from multiple CMMF endpoints, etc.

The CMMF Service Access Information is the set of parameters and addresses which are needed by a 5GMSd Client to activate and control the reception of a CMMF-enabled media downlink media streaming session. Minimal additions to the Service Access Information defined in clause 4.2.3 of TS 26.501 [15] are needed to enable the use of CMMF, primarily the communication of pointers (e.g., base URLs) to each endpoint where CMMF-encoded content is hosted. These pointers can be used with existing Service Access Information (e.g., a Media Entry Point document such as a MPEG-DASH MPD) to construct proper URLs that enable the download of CMMF-encoded content from each CMMF endpoint (5GMSd AS instance). For example, the base URLs provided by the additional Service Access Information can be combined with relative URLs communicated within an MPD to construct a URL that points to a CMMF bitstream/object constructed from the original DASH content or video clip file referenced within the MPD. In some implementations, additional modifications to the requests or URLs used by a CMMF-enabled 5GMSd Client may be required to access CMMF-encoded media. For example, CMMF bitstreams/objects may be located within an adjacent directory to those provided within the MPD or they may be accessible using specific HTTP request headers or URL query parameters. These modifications are left to the implementation and are outside the scope of this Key Issue.

### 5.19.4	High-level call flows

#### 5.19.4.1	Server-side CDN switching

Editor’s Note:	Inclusion and expansion on these sets of solutions is dependent on interest from working group.

#### 5.19.4.2	Client-side CDN switching

Editor’s Note:	Inclusion and expansion on these sets of solutions is dependent on interest from working group.

#### 5.19.4.3	Concurrent CDN access using CMMF

##### 5.19.4.3.1	Overview

The intent of CMMF is to supplement existing downlink streaming procedures, rather than replace them entirely. As such, minimal changes to the procedures provided in clause 5 of TS 26.501 [15] are necessary. Enabling multi-CDN media delivery using CMMF within existing downlink media delivery workflows can generally be realised through the following:

1.	CMMF-encoded media objects, and possibly original source media (e.g., MPEG-DASH or HLS media segments), are striped across multiple 5GMSd Application Server instances. The set of multiple 5GMSd AS instances containing the same CMMF-encoded media object (or stripe) of the media (i.e., the CMMF-encoded media object is replicated across multiple 5GMSd AS instances) is considered a single CMMF endpoint rather than multiple CMMF endpoints. The 5GMSd Application Provider may make the CMMF-encoded media objects, and possibly original source media (e.g., MPEG-DASH or HLS media segments), available at reference point M2d or they may be created by a 5GMSd AS instance performing content preparation on either regular media objects (e.g., MPEG-DASH or HLS media segments) or already created CMMF-encoded media objects.

2.	Upon initialization of a playback session, the 5GMSd Media Client’s Media Session Handler obtains relevant Service Access Information from the 5GMSd AF at reference point M5. At a minimum, this includes details concerning the location of each set of 5GMSd AS instances from which a stripe of CMMF-encoded and possibly original media (e.g., MPEG-DASH or HLS media segments) may be obtained, as well as appropriate signalling to indicate whether the media at each location is CMMF-encoded. Each of these locations should be considered a CMMF endpoint as defined above.

3.	The 5GMSd Media Client connects to and downloads CMMF-encoded media objects, and possibly the original source media (e.g., MPEG-DASH or HLS media segments), from each CMMF endpoint simultaneously via reference point M4d, terminating the download from each 5GMSd AS instance early upon obtaining enough of the CMMF-encoded objects to recover the source media (e.g., MPEG-DASH or HLS media segment). Once decoded, the source media is delivered to the Media Player in the 5GMSd Media Client for presentation.

The procedures for enabling multi-CDN or multi-5GMSd AS instance delivery using CMMF for MPEG-DASH streaming sessions and provisioning the 5GMS network to deliver CMMF-encoded media objects are provided in the following clauses.

##### 5.19.4.3.2	CMMF provisioning procedure

The workflow for provisioning network resources and/or hosting CMMF-encoded media is dependent on the 5GMSd Application Provider requirements and network configuration. Assuming the 5GMSd Application Provider creates CMMF-encoded media, provides that media over reference point M2d, provisions CMMF endpoints external to the 5GMS System, and manages the configuration of the 5GMSd Client over reference point M8d, existing procedures as outlined in clause 5 of TS 26.501 [15] are sufficient to host and deliver CMMF-encoded media. However, cases exist where the 5GMS System may take a more active role in delivering CMMF-encoded media. These cases are outlined in the procedure below. Differences from the baseline procedure are highlighted in boldface.

Figure 5.19.4.3.2-1: Media processing procedures for downlink media streaming using CMMF

Steps:

1.	Upon setting up a CMMF Provisioning and Content Hosting Configuration, the 5GMSd Application Provider requests media processing, provisioning, and hosting to be set up. The 5GMSd Application Provider provides a description of the type and placement of the processing required to create CMMF-encoded media, the CMMF endpoint URL(s), the caching rules of the CMMF-encoded media, CMMF endpoint media ingest configuration, etc. Various options for configuring the 5GMS System for delivering CMMF-encoded media are described in clause 5.19.3.3.1. For example,

a.	The 5GMSd Application Provider chooses to configure the 5GMS System similarly to figures 5.19.2.1-1 or 5.19.2.2-1 where CMMF Content Preparation is performed by the 5GMSd Application Provider. In this case, one or more 5GMSd AS instances are configured to function as different CMMF endpoints (each accessible by 5GMS Clients) where the CMMF-encoded media cached and/or delivered by each 5GMSd AS instance is obtained directly from the 5GMS Application Provider. The CMMF Provisioning and Content Hosting Configuration assigns URLs to each 5GMSd AS instance acting as a CMMF endpoint, as well as configuring each 5GMSd AS instance to obtain and cache CMMF-encoded media directly from the 5GMSd Application Provider.

b.	The 5GMSd Application Provider chooses to configure the 5GMS System similarly to figure 15.19.3.3-1 where CMMF Content Preparation is performed by a single 5GMSd AS instance before distribution of that content via reference point M10d to 5GMSd AS instances serving as CMMF endpoints. In this case, the CMMF Provisioning and Content Hosting Configuration configures the single 5GMSd AS assigned to perform CMMF Content Preparation (which may include providing an Edge Resource Configuration and defining the location where media is ingested), as well as configuring each 5GMSd AS instance acting as a CMMF endpoint (which may include assigning URLs and configuring each 5GMSd AS instance to obtain and cache CMMF-encoded media created by the 5GMSd AS instance assigned to perform CMMF Content Preparation).

c.	The 5GMSd Application Provider chooses to configure the 5GMS System similarly to figure 5.19.3.3.1-2 where CMMF Content Preparation is performed by each set of 5GMSd AS instances configured as a CMMF endpoint. In this case, the CMMF Provisioning and Content Hosting Configuration may include an Edge Resource Configuration for each set of 5GMSd AS instances for the purposes of preparing CMMF-encoded media, assigning URLs to each of the 5GMSd AS instances, defining where each 5GMSd AS instance can obtain media (whether it is the original source media or already created CMMF objects of that media), the caching policies of each 5GMSd AS instance, etc.

2.	The 5GMSd AF provisions via reference point M3d a selected set of 5GMSd AS instance(s) to fulfil the requested media processing in the appropriate placement by means of one or more Content Preparation Templates referenced by a Content Hosting Configuration. Depending on the configuration, one or multiple 5GMSd AS instance(s) may be involved. The Content Hosting Configuration(s) may reference Server Certificates and Content Preparation Templates, as required.

3.	The 5GMSd AS instance(s) confirm successful provisioning to the 5GMSd AF.

4.	The 5GMSd AF confirms the successful creation of the CMMF Provisioning and Hosting Configuration with the requested media processing to the 5GMSd Application Provider.

##### 5.19.4.3.3	Procedures for CMMF downlink media streaming

The procedure depicted below illustrates how CMMF can supplement downlink media delivery using MPEG-DASH as defined in clause 5.2.3 of TS 26.501 [15]. Differences from the baseline procedure are highlighted in boldface.

Figure 5.19.4.3.3-1: High-level procedure for delivering DASH content using CMMF

Prerequisites:

-	The 5GMSd Application Provider has provisioned the 5G Media Streaming System and has set up content ingest. In the case where the 5G Media Streaming System is responsible for CMMF media object preparation, the 5GMSd Application Provider has also provisioned 5GMS resources to encode CMMF media objects and host those objects on 5GMSd Application Server instances.

-	The 5GMSd-Aware Application has received the service announcement from the 5GMSd Application Provider.

Steps:

1.	The 5GMSd-Aware Application triggers the Service Announcement and Service and Content Discovery procedure. The Service and Content Discovery procedure only involves the App and the external Application Server. The Service Announcement includes either the whole Service Access Information (i.e. details for Media Session Handling (M5d) and for Media Streaming access (M4d)) or a reference to the Service Access Information.

2.	A media content item is selected.

3.	The 5GMSd-Aware Application triggers the 5GMSd Client to start media playback. The Media Player Entry is provided to the 5GMSd Client.

4.	When the 5GMS-Aware Application has received only a reference to the Service Access Information (see step 1), the Media Session Handler interacts with the 5GMSd AF to acquire the whole Service Access Information. Pointers to the 5GMSd AS instances serving as CMMF endpoints are included within this Service Access Information. An example of a pointer may include the base URLs of each set of 5GMSd AS instances hosting the same stripe of CMMF-encoded media objects.

5.	Once the information about the pointers (e.g., base URLs) to each CMMF endpoint has been parsed from the Service Access Information, the Media Session Handler communicates these to the CMMF-Enabled Media Player to be used while accessing media in subsequent steps.

6.	In parallel, the Media Player is invoked to start media access and playback.

7.	The Media Player establishes the transport session for acquiring the MPD referenced by the Media Player Entry.

8.	The Media Player requests the MPD.

9.	The Media Player receives the MPD.

10.	The Media Player processes the MPD. Based on the information contained within the MPD as well as the relevant CMMF information contained in the Service Access Information, the Media Player determines, for example, the number of needed transport sessions for media acquisition. The Media Player should use the MPD information to initialize the media pipelines for each media stream. The MPD should also contain information to initialize the DRM client, when DRM is used.

11.	The Media Player notifies the Media Session Handler about the MPD. The notification may contain parameters from the MPD.

12.	Optional: the Media Player acquires the necessary DRM information, for example a DRM License.

NOTE:	CMMF does not impede the use of DRM as long as DRM is applied to the original source media prior to creation of CMMF-encoded objects of that media.

13.	The Media Player configures the media playback pipeline.

14.	The Media Player establishes the necessary transport sessions for the CMMF-encoded content. For example, the Media Player may establish one transport session for each media component (audio, video, etc) and possibly additional transport sessions for other media representations to each CMMF endpoint communicated by the Service Access Information.

15.	The Media Player notifies the Media Session Handler that it is ready to commence playback and optionally provides transport session parameters.

16.	The Media Player requests initialization information. In the case where this initialization information has been encoded within CMMF objects, the Media Player requests the CMMF-encoded initialization information objects from each CMMF endpoint in parallel. Figure 15.19.4.3.3-2 provides the procedures for how the CMMF-encoded initialization information objects are located and/or distributed to the 5GMS AS upon receiving the request from the Media Player. The Media Player repeats this step for each required initialization segment.

17.	The Media Player receives the initialization information. In the case where this initialization information has been encoded within CMMF objects, the Media Player downloads these multiple CMMF-encoded initialization information objects from each CMMF endpoint in parallel until such time as the CMMF decoder has received enough information to successfully decode. At which time, the download of any incompletely acquired CMMF objects is terminated.

18.	The received information from the CMMF objects containing the initialization information is decoded by the CMMF decoder.

19.	The Media Player requests media segments according to the MPD. In the case where these media segments have been encoded within CMMF objects, the Media Player requests the CMMF-encoded media segment objects from each CMMF endpoint in parallel. Figure 15.19.4.3.3-2 provides the procedures for how the CMMF-encoded media segment objects are located and/or distributed to the 5GMS AS upon receiving the request from the Media Player.

20.	The Media Player receives media segments. In the case where these media segments have been encoded within CMMF objects, the Media Player progressively downloads these multiple CMMF-encoded media segment objects from each CMMF endpoint until such time as the CMMF decoder has received enough information to successfully decode. At which time, the download of any incompletely acquired CMMF objects is terminated.

21.	The received information from the CMMF objects containing the media segments is decoded by the CMMF decoder and the decoded information is placed into the appropriate media rendering pipeline.

22.	Previous steps are repeated according to the MPD information.

##### 5.19.4.3.4	CMMF content retrieval procedure

The workflow provided within figure 5.19.4.3.4-1 expands upon the process that occurs between steps 16 and 17, as well as steps 19 and 20, within figure 5.19.4.3.3-1. This workflow focuses on the processes a 5GMSd AS instance follows to locate and distribute CMMF objects upon receiving a request from the 5GMSd Client’s Media Player. The steps of this workflow are dependent on how the 5GMS System was configured and provisioned by the 5GMSd Application Provider. For the purposes of this procedure, it is assumed that each 5GMSd AS instance is configured as a different CMMF endpoint. In the case where multiple 5GMSd AS instances belong to a single CMMF endpoint, the procedure provided below can be modified accordingly.

Figure 5.19.4.3.4-1: Media processing procedures for downlink media streaming using CMMF

Steps:

1.	The 5GMSd Client sends request(s) for CMMF-encoded media to one or more 5GMSd AS instances where each 5GMSd AS instance is configured as a different CMMF endpoint.

2.	Assuming the following conditions are met:

a.	The 5GMSd AS CMMF endpoint does not have a copy the requested CMMF-encoded media cached,

b.	The 5GMSd AS instance is configured to obtain already prepared CMMF-encoded media from the 5GMSd Application Provider,

the 5GMSd AS instance requests and fetches a unique CMMF-encoded representation of the requested media from the 5GMSd Application Provider. In this case, it is the responsibility of the 5GMSd Application Provider to ensure that the CMMF-encoded representations of the requested media provided to each CMMF endpoint are unique. Upon receipt of the requested unique CMMF-encoded representation of the requested media, the 5GMS AS instance may cache the CMMF object locally to support subsequent requests for that content.

3.	Assuming the following conditions are met:

a.	The 5GMS AS CMMF endpoint does not have a copy of the requested CMMF-encoded media cached,

b.	The 5GMSd AS instance is configured to perform CMMF Content Preparation (i.e., encode and create CMMF objects),

c.	The 5GMSd AS instance is configured to obtain media directly from the 5GMSd Application Provider,

the 5GMSd AS instance requests and fetches the requested original source media from the 5GMSd Application Provider.

4.	Assuming the conditions listed in step 3 are met and the 5GMSd AS instance has obtained the requested media from the 5GMSd Application Provider, the 5GMSd AS instance creates a unique CMMF-encoded representation of the requested media. Once the CMMF-encoded representation has been created, either the original source media and/or the newly created CMMF-encoded media may be cached by the 5GMS AS instance to support subsequent requests for that content.

5.	Assuming the following conditions are met:

a.	The 5GMSd AS CMMF endpoint does not have a copy of the requested CMMF-encoded media cached, and

b.	The 5GMSd AS instance is configured to obtain already prepared CMMF-encoded media from another 5GMSd AS instance via reference point M10d,

the 5GMSd AS instance requests and fetches a unique CMMF-encoded representation of the requested media from the 5GMS AS instance configured to perform CMMF Content Preparation. In this case, it is the responsibility of the 5GMS AS instance configured to perform CMMF Content Preparation to ensure that the CMMF-encoded representation of the requested media provided to the requesting 5GMS AS instance is unique to the CMMF endpoint for which the requesting 5GMS AS instance belongs. Upon receipt of the unique CMMF-encoded representation of the requested media, the requesting 5GMS AS instance may cache the CMMF object locally to support subsequent requests for that content.

6.	Assuming the following conditions are met:

a.	The 5GMSd AS CMMF endpoint does not have a copy of the requested CMMF-encoded media cached,

b.	The 5GMSd AS instance is configured to perform CMMF Content Preparation (i.e., encode and create CMMF objects), and

c.	The 5GMS AS instance is configured to obtain media (whether original source media or CMMF-encoded media) from another 5GMS AS instance via reference point M10d,

the 5GMSd AS instance requests and fetches the requested media from an upstream 5GMS AS instance. In this case, the media obtained from an upstream 5GMS AS instance may be the original source media or an already CMMF-encoded representation of that media. Note: If the latter, the CMMF-encoded representation of the requested media does not have to be unique to the CMMF endpoint for which the requesting 5GMS AS instance belongs since a unique CMMF-encoded representation will be created within the next step.

7.	Assuming the conditions listed in step 6 are met and the 5GMS AS instance has obtained the requested media from an upstream 5GMSd AS instance, the requesting 5GMSd AS instance creates a unique CMMF-encoded representation of the requested media. In the case where the original source media is obtained from the upstream 5GMS AS instance, the requesting 5GMS AS instance encodes the original source media and creates a CMMF-encoded representation of that media that is unique to the CMMF endpoint for which the requesting 5GMS AS instance belongs. In the case where a CMMF-encoded representation of the requested media is obtained from an upstream 5GMS AS instance, the requesting 5GMS AS instance may first need to decode back to the original source media and then re-encode the media into a CMMF-encoded representation of the requested media that is unique to the CMMF endpoint for which the requesting 5GMS AS instance belongs. Once the CMMF-encoded representation has been created, the newly created CMMF-encoded media may be cached by the requesting 5GMS AS instance to support subsequent requests for that content.

8.	Assuming the requested CMMF-encoded media is cached, the 5GMSd AS CMMF endpoint pulls the content from cache.

9.	Once the CMMF-encoded representation of the requested media has been obtained by the 5GMS AS instance, the 5GMS AS instance responds to the 5GMSd Clients’ request with a CMMF-encoded representation of the requested media that is unique to the CMMF endpoint for which the 5GMS AS belongs.

Different variants of these procedures may be possible, depending on the type of processing, the placement of the processing, and the characteristics of the Content Hosting Configuration. Furthermore, the introduction of CMMF to supplement the download of media in other scenarios (e.g., downlink streaming to Media Players with different presentation manifests per clause 5.2.4 of TS 26.501 [15]) can be applied in a similar fashion to that shown above.

### 5.19.5	Gap analysis and requirements

#### 5.19.5.1	Server-side CDN switching

Editor’s Note:	Inclusion and expansion on these sets of solutions is dependent on interest from working group.

#### 5.19.5.2	Client-side CDN switching

Editor’s Note:	Inclusion and expansion on these sets of solutions is dependent on interest from working group.

#### 5.19.5.3	Concurrent CDN access using CMMF

Editor’s Note:	Gap analysis and requirements are left for future study.

### 5.19.6	Candidate solutions

#### 5.19.6.1	Server-side CDN switching

These candidate solutions include approaches where a media streaming client or population of clients changes or switches between two or more CDNs based on recommendations from a remote server. An example includes the DASH Industry Forum’s content steering architecture [DIFCS].

Editor’s Note:	Inclusion and expansion on these sets of solutions is dependent on interest from working group.

#### 5.19.6.2	Client-side CDN switching

These candidate solutions include approaches where a media streaming client changes or switches between two or more CDNs based on decisions made locally.

Editor’s Note:	Inclusion and expansion on these sets of solutions is dependent on interest from working group.

#### 5.19.6.3	Concurrent CDN access using CMMF

This candidate solution includes approaches where a 5GMSd Client accesses and downloads, via reference point M4, CMMF-encoded media objects [CMMF], and possibly original source media (e.g., MPEG-DASH or HLS media segments), from two or more 5GMSd Application Servers simultaneously. Additionally, the 5GMSd Client may access these 5GMSd Application Servers over different access networks (such as 3GPP and non-3GPP access networks).

### 5.19.7	Summary and conclusions

## END OF CHANGES