3GPP TSG-SA4 Meeting #0-e (AH) MBS SWG post 129e	S4aI240151

# Online, , 26th Sep - 24th Oct 2024											revision of S4aI240124

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.802 | CR | 0002 | rev | 5 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_AMD] MBS User Service and Delivery Protocols for eMBMS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Germany |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_AMD |  |  |  |  |  | Date: |  |  | 2024-10-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The MBS User Service architecture and protocol follows the modern design philosophies of the 5G System with separation of user services from transport, a service-based architecture and RESTful APIs. At the same time, eMBMS and enTV as used for LTE-based 5G Broadcast support a transparent delivery mode. While interworking in between MBMS and MBS is addressed in TS 23.247, interworking between these two systems at the User Service level is not addressed. In order for MBMS and LTE-based 5G broadcast to leverage MBS User Service technologies, a study is warranted to identify the gaps to fully support this functionality. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addresses the work item objectives for this key issue - Documents the key issue in more detail, in particular how they relate to the 3GPP Media Delivery architecture and/or the MBS User Service architecture - Studies collaboration scenarios between the Application Service Provider and the 5G System and for each of the key topics. - Based on existing architectures, provides one or more deployment architectures that address the key topics and the collaboration models. - Maps the key topics to basic functions and develop high-level call flows. - Identifies the issues that need to be solved. - Provides candidate solutions including call flows, protocols and APIs for each of the identified issues. Identifies gaps and recommend potential normative work for stage-2 and stage-3, including which existing specifications would be impacted and/or if any new specifications would preferably be developed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.10 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TR 26.802 CR 0001 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Updated scope, references and abbreviations are in CR 0001 References [103720] ETSI TS 103 720, 5G Broadcast System for linear TV and radio services; LTE-based 5G terrestrial broadcast system [23247] 3GPP TS 23.247, 3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services; Stage 2". [26502] 3GPP TS 26.502, 5G multicast-broadcast services; User service architecture Abbrevations: XXX XXXX |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S4aI240124; [FS_AMD] MBS User Service and Delivery Protocols for eMBMS; Qualcomm Germany; Thomas Stockhammer. E-mail Discussion: Revisions: none Presenter: Thomas Stockhammer Online Discussion: October 10, 2024 Decision: - Richard: The diagram is mixing reference architecture and deployment architecture. This a quizzy. - Thomas: Maybe I should do the call flow to make this more comprehensive. Any offline comments are appreciated. S4aI240124 is noted. |

## =====  CHANGE =====

## 2	References

[103720]	ETSI TS 103 720: "LTE-based 5G Broadcast System".

## =====  CHANGE (new – presented w/o revision marks) =====

## 5.10	Key Issue #9: MBS User Service and Delivery Protocols for eMBMS

### 5.10.1	Description

The MBS User Service architecture and protocol follows the modern design philosophies of the 5G System with separation of user services from transport, a service-based architecture and RESTful APIs. At the same time, eMBMS and enTV (as used for LTE-based 5G Broadcast) support transparent delivery mode and group communication. While interworking between MBMS and MBS is addressed in clause 5.2 of TS 23.247 [23247] and clause 4.9 of TS 26.502 [26502], interworking between these two systems at the User Service level is not addressed. In order for MBMS and LTE-based 5G broadcast as defined in ETSI TS 103 720 [103720] to leverage MBS User Service technologies, a study is warranted to identify the gaps to fully support this functionality.

Figure 5.10.1-1 provides the MBS–eMBMS interworking system architecture as documented in TS 26.502 [26502], figure 4.9-1. The functional elements that fall within the scope of [26502] are highlighted in green.

Figure 5.10.1-1: MBS–eMBMS interworking system architecture (see TS 26.502 [26502], figure 4.9-1)

The interworking architecture defined in clause 4.9 of [26502] addresses the following functionalities:

1.	Using MBS northbound interfaces at reference point Nmb10 for MBS, and using eMBMS northbound interfaces at reference point xMB-C or MB2-C for eMBMS.

2.	Potential dynamic switching between MBS and eMBMS reception, if a UE implements both an MBS Client and an eMBMS Client.

3.	Common ingest of content through reference point Nmb8/xMB-U, if these reference points are compatible.

4.	Common MBS User Services distribution and eMBMS delivery methods such that the same ingested content can be delivered to an MBS Client and to an eMBMS Client. UEs supporting only eMBMS are served by this architecture as well.

However, there is no guarantee that 3 and 4 can generally be achieved in practice.

### 5.10.2	Collaboration scenarios and architecture mapping

A more common interest is the ability to deploy a system for which MBS User Services are distributed via eMBMS. This would allow a single, common User Service specification for MBS and eMBMS/5G Broadcast to be maintained going forward. A modification of the architecture is shown in figure 5.10.2-1, in which:

-	Only the MBS northbound reference points Nmb10 and Nmb8 are exposed respectively by the MBSF and MBSTF. These are extended as required to support eMBMS transport (as those are extended, they are marked with an asterik).

-	The UE in the 5G System is extended to support eMBMS reception, for example an LTE-based 5G Broadcast profile as defined in ETSI TS 103 720 [103720]. Such an approach permits a single middleware client with unified APIs, etc. to be deployed in the UE that is capable of both MBS User Service reception and eMBMS User Service reception.

Figure 5.10.2-1: MBS User Services on top of eMBMS

A further variant of the architecture is shown in figure 5.10.2-2, in which case MBS radio delivery is not even in scope, but the MBS User Service is used to deliver only eMBMS traffic.

Figure 5.10.2-2: MBS User Services on top of eMBMS

Another possible implementation architecture is shown in Figure 5.10.2-3 where the MB2-U is used southbound of MBSTF and keep the BM-SC basics to communicate with the EPS. Such a deployment architecture may be of interest in order to address a combination of MBS User services with eMBMS radio delivery. Also, the MBSF implements a southbound MB2-C interface for control information communication.

According to TS 26.346, the Group Communication Service (GCS) AS as defined by 3GPP TS 23.468 [23468] uses the MBMS Group Communication delivery method on top of the MBMS bearers for MBMS delivery. However, in general, the MBMS Group Communication delivery method is available for any application. In this case, the application interfaces to the BM-SC using the MB2 interface. MB2 carries control plane signalling (via MB2-C) and user plane data (via MB2-U) between the Application Server and BM-SC. The data transferred via MBMS bearer(s) is delivered from the BM-SC using the Group Communication delivery method as defined in TS 26.346. Description of the stage 2 procedures on MB2 between the GCS AS and the BM-SC is provided in TS 23.468. The stage 3 specification of the MB2 procedures and the protocol aspects of MB2-C and MB2-U are given in TS 29.468 [29468].

In this deployment scenario, the MBS User Service is treated as an application on top of the group communication delivery method. The UE would implement the relevant MBS User Service functionalities to support the reception of the MBS user service video the group communicaton API as defined in TS 23.479 [23479].

Figure 5.10.2-3: MBS User Services on top of eMBMS with Group Communication

Yet another possible architecture is shown in Figure 5.10.2-4 where the xMB-U is used southbound of MBSTF and keep the BM-SC basics to communicate with the EPS. Also, the MBSF implements a southbound xMB-C interface for control information communication. The UE would implement the relevant MBS User Service functionalities to support the reception of the MBS user service video the transparent delivery mode API.

Figure 5.10.2-3: MBS User Services on top of eMBMS with Transparent Delivery

### 5.10.3	High-level call flows

#### 5.10.3.1	Joint BM-SC and MBSF Functionality

The extended high-level baseline procedures for MBS User Services for the architecture showing in Figure 5.10.2-2 are shown in figure 5.10.3.1-1, highlighting in bold the extensions to the call flow in TS 26.502 [26502], clause 5.2.1.

Figure 5.10.3.1-1: MBS User Service high-level baseline procedures with Joint BM-SC and MBSF Functionality

The same procedures as defined in TS 26.502, clause 5.2 apply, but the distribution is via eMBMS as shown in step 9 and 11. However, the detailed procedures documented in the remainder of clause 5 in TS 26.502 need considerations to support eMBMS distribution.

In clause 5.3, 5.3A, and 5.5 of TS 26.502, the communication with the MB-SMF to allocate TMGIs, create sessions, update sessions, or delete sessions, is extended with a communication with the MBMS-GW using the SGmb reference point and the procedures defined in TS 23.246 [26246], clause 8 apply. In particular

-	the BM-SC sends a Session Start Request message to MBMS GW to indicate the impending start of the transmission and to provide the session attributes (TMGI, Flow Identifier, QoS, MBMS service Area, list of cell IDs if available, Session identifier, estimated session duration, list of MBMS control plane nodes (MMEs, SGSNs) for MBMS GW, time to MBMS data transfer, MBMS data transfer start, access indicator, ...).

-	The MBMS GW responds with a Session Start Response message with information for BM-SC to send MBMS data to the MBMS GW.

According to TS 23.247, a common TMGI for MBS and eMBMS is used towards the AF/AS and the TMGI is also used as identifier for transport over E-UTRAN/EPC. The MBSTF distributes the received data to the MB-UPF at reference point Nmb9 and/or the MBMS-GW at reference point SGi-mb, when supported by operator network configuration.

On stage-3 as defined in TS 26.517 [26517], the session description document as defined in TS 26.517, clause 5.2.5 defines the MBS distribution session, either using

-	The session description for the MBS Object Distribution Method as specified in clause 6.2.2 of TS 26.517, or

-	The session description for the MBS Packet Distribution Method as specified in clause 7.2.3. of TS 26.517.

In either case, the service type is restricted to Multicast MBS and Broadcast MBS as shown in clause 6.2.2.2. The use of the mbms-mode as defined in TS 26.346 [26346] is not permitted in TS 26.517.

#### 5.10.3.2	MBSF/MBSTF interface with MB2 to BM-SC

According to TS 23.468 [23468], MB2 offers access to the MBMS bearer service from an application. MB2 carries control plane signalling (MB2-C) and user plane (MB2-U) between the Application Server and the BM-SC. Some relevant properties of MB2 are summarized as follows:

-	MB2 is used by the AS to interact with the BM-SC for MBMS bearer management.

-	The data transferred via MBMS bearer(s) by the AS is transparent to the BM-SC.

-	MB2 is a standardized secured interface to an AS.

-	The AS needs to be configured with the IP addresses or a FQDN of the contact points of MB2-C. The MB2-C contact points need to be configured per PLMN ID.

-	The user plane transport information (e.g. IP address/UDP port) for delivering group communication data flow from the AS to the BM-SC over MB2-U is exchanged over MB2-C.

The MB2 interface provides the ability for the application to use the functionality of the MBMS system to deliver data to group members over MBMS. The procedures supported include:

-	allocation of a set of TMGIs (TS 23.246 [3]) by the BM-SC at the request of the AS, (see clause 5.1.2.2.2 of TS 23.468),

-	deallocation of a set of TMGIs by the BM-SC at the request of the AS, (see clause 5.1.2.2.3 of TS 23.468),

-	activating an MBMS bearer, (see clause 5.1.2.3.2 of TS 23.468):

-	this may include request related to BM-SC applying FEC or RoHC or both to a set of media.

-	deactivating an active MBMS bearer, (see clause 5.1.2.3.3 of TS 23.468),

-	modifying characteristics of an active MBMS bearer, (see clause 5.1.2.4 of TS 23.468), and

-	reporting of MBMS delivery status from the BM-SC to the AS, (see clause 5.1.2.5 of TS 23.468).

The MB2 interface between the AS and the BM-SC is established before any MB2 messages are sent between these two entities, and carries all MB2 messages between the two entities for all MBMS bearers used by the AS. The TMGI/FlowID is the unique identifier used by the AS and BM-SC to refer to the MBMS bearer.

The extended high-level baseline procedures for MBS User Services for the architecture showing in Figure 5.10.2-3 are shown in figure 5.10.3.2-1, highlighting in bold the extensions to the call flow in TS 26.502 [26502], clause 5.2.1.

Figure 5.10.3.2-1: MBS User Service high-level baseline procedures using Group Communication enablers and APIs

The core extensions are:

-	The distribution session provisioning, TMGI allocation and MBMS bearer allocation in steps 2, 3 and 4 need to be updated to address the allocation of the bearers on MBMS distribution. The variant shown in figure let’s MBSF handle the communicaton with MBSTF and BM-SC. In a variant, also the MBSTF may directly communicate with the BM-SC.

-	In step 10, the MBSF client provides information to the MBMS client in order to establish the bearer using the MC-MBMS-API, involving also the MBSTF client.

-	In step 11, the MBSTF client and the MBMS client activate the session to receive content.

-	In step 13, the information is received through the MBMS bearer and the media content is directly provided to the MBSTF client for relevant processing, for example FEC decoding, unicast repair determination and so on.

#### 5.10.3.3	MBSF/MBSTF interface with xMB to BM-SC

Editor’s Note: Still to be completed.

### 5.10.4	Gap analysis and requirements

#### 5.10.4.1	Joint BM-SC and MBSF Functionality

For the joint BM-SC and MBSF Functionality, no architectural gaps are identified. However, the following stage-3 gaps have been identified:

-	The session description for the MBS Object Distribution Method in TS 26.517 is restricted to MBS sessions.

-	The session description for the MBS Packet Distribution Method in TS 26.517 is restricted to MBS sessions.

To support the functionality, the removal of the above restrictions is needed.

#### 5.10.4.2	MBSF/MBSTF interface with MB2 to BM-SC

In order to support the extended implementation in clause 5.10.3.2, in addition to the extensions documented in clause 5.10.4.1, the following extensions are needed:

-	The MBSF and MBSTF need to be able to communicate with BM-SC using MB2 interfaces based on what is presented in clause 5.10.3.2, figure 5.10.3.2-1, steps 2, 3 and 4.

-	The MBSF client needs to be extended in order to provide information to the MBMS client in order to establish the bearer using the MC-MBMS-API, involving also the MBSTF client as shown in figure 5.10.3.2-1, step 10.

-	The MBSTF client and the MBMS client need to be extended to activate the session to receive content via MBMS bearer using the MC-MBMS-API  as shown in figure 5.10.3.2-1, step 11.

-	The establishment of the data from MBMS client to MBSTF client needs to be established to receive content via MBMS bearer using the MC-MBMS-API  as shown in figure 5.10.3.2-1, step 13.

#### 5.10.4.3	MBSF/MBSTF interface with xMB to BM-SC

Editor’s Note: Still to be completed.

Editor’s Note: Identify the issues that need to be solved.

### 5.10.5	Candidate solutions

Editor’s Note: Provide candidate solutions including call flows, protocols and APIs for each of the identified issues.

### 5.10.6	Summary and conclusions