3GPP TSG-SA4 MBS SWG post 129e	 S4aI240157

# Online, 16-18th October 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PSEUDO CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.942 | CR |  | rev |  | Current version: | 0.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_MediaEnergyGREEN] Solution for KI3 based on French regulators study |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_MediaEnergyGREEN |  |  |  |  |  | Date: |  |  | 2024-10-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Add a potential solution for KI#3 Evaluation Framework |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a potential solution for KI#3 based on work done by the French regulators |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No solution for KI#3 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2 and 7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

# 2	References

[…]

[ARCOM]	ARCEP, “Arcom and Arcep, in tandem with ADEME, publish an unprecedented study on the environmental impact of audiovisual media consumption in France in 2022, and up to 2030”, https://en.arcep.fr/fileadmin/user_upload/57-24-english-version.pdf

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

It is preferred that the reference to TR 21.905 be the first in the list.

2nd Change

# 7	Potential Solutions

[Editor’s note: Description of potential solutions]

## 7.1 	Mapping of Solutions to Key issues

## 7.2	Solution #1: Evaluation framework based on French regulators work

### 7.2.1	Key issue mapping

This solution addresses partially the Key Issue#3 Evaluation Framework described in clause 6.3.

### 7.2.2	Functional Description

Arcom and Arcep (French regulators) published a study on the environmental impact of audiovisual media consumption in France in 2022, and up to 2030 [ARCOM]. The study assesses the environmental impact of consuming audiovisual mass media in 2022, taking all the component parts into account: hardware (user devices), networks (fixed broadband and superfast broadband, digital terrestrial, and satellite) and datacentres. It covers the main systems used to access audiovisual media: linear and time-shifted TV and radio, video on demand, audio and video streaming, and video sharing platforms. Every type of impact has been assessed (carbon footprint, consumption of mineral and metal resources, final energy consumption) including energy usage, the target for the framework of this report.

In this study, energy usage (named “final energy consumption”) is measured in kWh and refers to the amount of electricity consumed during the usage phase of the three tiers of the digital value chain (user devices, networks and datacenters). Only the usage stage of terminals, networks, and datacenters is concerned (upstream electricity consumption for the manufacturing phase is not addressed by this indicator).

A comparative assessment of 9 audiovisual usage scenarios (on the scale of one hour of audio or video content consumption in France in 2022) have been used:

- A1: Listening to live FM radio on a radio set

- A2: Listening to live FM radio on a car radio

- A3: Listening to live radio via the internet on a smartphone connected to the fixed network

- A4: Listening to music/podcast on a streaming platform (app) on a smartphone connected to the internet via mobile network

- V1: Watching a TV channel in HD on a television via integrated DTT access

- V2: Watching a TV channel in HD on a television connected to the internet via a TV decoder linked to an ISP box (managed IPTV)

- V3: Watching catch-up TV in HD on a smart TV connected to the internet via a TV decoder linked to an ISP box

- V4: Watching subscription video-on-demand (SVOD) in HD on a smart TV connected to the internet via fixed network

- V5: Watching online videos on a video-sharing platform in HD on a smartphone connected to the internet via mobile network

To estimate energy consumption of devices, 4 differents devices have been evaluated in lab (2 smartphones, 1 PC and 1 TV). For the TV and the PC, a measurement module (digital wattmeter) is integrated between the device and the electrical outlet. This module measures energy consumption in alternating current. The module is connected to a computer to retrieve the measurements. For smartphones, measurements are taken using software probes to record energy and data consumption. Energy is measured in mWh/s or mAh. The measurements are initiated for 1 minute. Several iterations are performed (a minimum of 3) to ensure relevance and limit artifacts related to the measurement itself. Testing conditions are noted for traceability of the measurements.

Two measurement modes are possible:

- Systematic content change between iterations. This has the advantage of avoiding user-side CDN caching strategies but has the disadvantage of introducing variability with different content. This measurement also provides stronger representativeness of user behavior.

- Iterations are conducted on a continuously played video. This has the advantage of freezing the content but the disadvantage of potentially underestimating consumption due to caching technologies.

The systematic content change solution is favored in the campaign for video-sharing platforms. The continuous video strategy is used to freeze the content's impact and study certain parameters (such as video codecs). Given the diversity of hardware, it was agreed that the user journey would not be automated. Thus, each measurement is independent and can be compared to another, regardless of the platforms studied.

The data measured in lab are very specific. They are conducted on a single device (two for smartphones), which performs a single precise usage. This allows, for example, to differentiate consumption during content playback and browsing a catalog. However, these measurements are not necessarily representative of the entire equipment landscape. Thus, comprehensive and representative data from the literature on a more diverse equipment pool were preferred over certain laboratory measurements for the quantification of audio and video usage at the national level in France.

The method to estimate the energy consumption of the mobile network will not be reusable for this study as it is using a theoretical calculation based on the total amount of energy consumed by the mobile network, the mobile network usage duration per subscriber and a formula allocating energy consumption per subscriber/year and per data volume.

The method to estimate the energy consumption of datacenters will not be reusable either as it is based on external estimations.

This solution does not allow to define an evaluation framework including the energy consumption during the usage phase of the three tiers of the digital value chain (user devices, networks and datacenters). No metric or API are available today for the network and datacenters parts. Their scopes would be too broad to be addressed. These parts will be for further study outside the scope of this document. This is not the case for the user device part. We can see that required metrics and API are already available, at least on major smartphones OS, and are already used by regulators for evaluating the impact of some specific parameters. The ARCOM/ARCEP [ARCOM] study demonstrated their uses for evaluating the environmental impact of video codecs, resolutions or frame rate. But this could easily be extended to other parameters like network types (i.e., Wifi, 5G, LTE) or delivery modes (i.e., unicast, MBS, 5G Broadcast).

### 7.2.3	Procedures

- Describe test conditions:

  - Network (connection type, upload and download bandwith, latency)

  - User devices (type, model, SoC, OS version, video player). At least 2 devices should be used.

  - Test environment (number of devices, number of iterations, …)

- Launch the application including energy-related information reporting including discharge speed or, discharge and measure duration.

- The measurements are initiated for 1 minute. Several iterations are performed (a minimum of three) to ensure relevance and limit artifacts related to the measurement itself.

NOTE: Two measurement modes are possible, selection is made according to the influence of the caching on the test:

- Systematic content change between iterations. This has the advantage of avoiding user-side CDN caching strategies but has the disadvantage of introducing variability with different content. This measurement also provides stronger representativeness of user behavior.

- Iterations are conducted on a continuously played video. This has the advantage of freezing the content but the disadvantage of potentially underestimating consumption due to caching technologies.

- Extract data from the data collector for analysis.

### 7.2.4	Impacts on existing services, entities and interfaces

Direct Data Client:

- Collect and expose energy-related information reporting including discharge speed or, discharge and measure duration.

Data collection AF:

- Expose expose energy-related information reporting for a given UE.

End of Changes