3GPP TSG-SA4 Meeting #129 post MBS AHG-e	S4aI240169

# Online, , 26th Sep 2024 - 24th Oct 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.841 | CR | pseudo | rev | - | Current version: | 0.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_MeMe] IETF MIMI and 3GPP Messaging |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Germany |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_MeMe |  |  |  |  |  | Date: |  |  | 2024-10-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The study item asks for advanced media formats A) Integration of TS 26.143 Capabilities and Profiles into IETF MIMI: TS 26.143 only provides containers based on Multi-part MIME. MIMI defines a similar framework, but a mapping of the MMBP data model from TS 26.143 needs to be completed. For this the progress of IETF MIMI needs to be monitored, and integration of TS 26.143 profiles and capabilities need to be studied. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR studies the requirements and gaps to integrated 3GPP media into IETF MIMI The document also does some bugfixes and general enhancements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.3, 4.1, 4.2.3, 5.1.1, 5.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | No updates were done do the IETF Draft draft-ietf-mimi-content-04, so no new information is provided |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## =====  CHANGE =====

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[26143]	3GPP TS 26.143: "Messaging Media profiles".

[26857]	3GPP TR 26.857: "5G Media Service Enablers".

[RFC2045]	IETF RFC 2045, "Multipurpose Internet Mail Extensions (MIME) Part One: Format of Internet Message Bodies", November 1996

[RFC2046]	IETF RFC 2046, "Multipurpose Internet Mail Extensions (MIME) Part Two: Media Types"

[RFC2387]	IETF RFC 2387, "The MIME Multipart/Related Content-type"

[RFC6381]	IETF RFC 6381, "The 'Codecs' and 'Profiles' Parameters for "Bucket" Media Types"

[14496-12]	ISO/IEC 14496-12: "Information technology - Coding of audio-visual objects -Part 12: ISO base media file format".

[IETF-MIMI]	IETF Draft draft-ietf-mimi-content-04: "More Instant Messaging Interoperability (MIMI) message content", June 2024.

[RFC8610]	IETF RFC 8610, Concise Data Definition Language (CDDL)

[RFC8949]	IETF RFC 8949, Concise Binary Object Representation (CBOR)

[23000-24]	ISO/IEC 23000-24: Messaging media application formatNOTE: The latest working draft is available in document WG03 N01286.

[26265]	3GPP TS 26.265: "Video Capabilities and Operating Points".

[26956]	3GPP TR 26.956: "Evaluation and Characterization of Beyond 2D Video Formats and Codecs".

[26966]	3GPP TR 26.966: "Evaluation of new HEVC coding tools".

[AMVHEVC]	Apple HEVC Stereo Video - Interoperability Profile (Beta), Version 0.9, June 21, 2023, https://developer.apple.com/av-foundation/HEVC-Stereo-Video-Profile.pdf

[SPATIAL]	Mike Swanson, "Spatial Video", March 7 2024, https://blog.mikeswanson.com/spatial-video/

[CONTOUR]	Video Contour Map Payload, Version 0.9, June 21, 2023, https://developer.apple.com/av-foundation/Video-Contour-Map-Metadata.pdf

[26118]	3GPP TS 26.118: "Virtual Reality (VR) profiles for streaming applications".

## =====  CHANGE =====

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

API	Application Programming Interface

CBOR	Concise Binary Object Representation

CDDL	Concise Data Definition Language

DASH	Dynamic Adaptive Streaming over HTTP

DRM	Digital Rights Management

HEVC	High Efficiency Video Coding

HMD	Head-Mounted Display

IANA	Internet Assigned Numbers Authority

JSON	JavaScript Object Notation

MIME	Multipurpose Internet Mail Extensions

MIMI	More Instant Messaging Interoperability

MLS	Messaging Layer Security

MMBP	multimedia messaging body part

MMS	Multimedia Messaging Service

RCS	Rich Communication Services

## =====  CHANGE =====

## 4.1	Background from TS 26.143

Editor’s Note:	TS 26.143 defines capabilities and profiles for messaging services. The profiles in TS 26.143 are aligned with TS 26.140.  The container format is based on IETF RFC 2045 as the format for the MMBPs. In addition, the profiles permits to encapsulate encapsulation of real-time video into the 3GP file format using the baseline profile. The container does not support external bodies, i.e. the MMBP is expected to be delivered as a single message. The media types for the player address basic text, audio/speech, images, video, text/subtitle, 3D scenes including AR as well as simple HTML-5 presentations. For the content generator, the supported media types in this profile are basic text, audio/speech, images, video, and text.

TS 26.143 [26143] addresses the usability of 3GPP defined media types and formats into messages as part of a message body within message containers. The focus of the specification is the definition of parts of message body that carry multimedia content, referred to as multimedia messaging body part (MMBP).  The term media type is used as short to refer to the IANA media type, subtype, and parameters as defined in IETF RFC 2046 [RFC2046] and provides defined properties of a content. For example, it may tell if the content is video or audio, it provides the encapsulation format, and it may provide parameters such as the codec in use.  Figure 4.1-1 provides an example system for a messaging services and highlights scope of this specification, namely the definition of a multimedia messaging body part (MMBP) and the associated metadata aligned with TS 26.143.

Figure 4.1-1 Example system for Messaging multimedia message exchange

A Messaging Service Sender instructs a MMBP generator to generate an MMBP, for example using an API. This for example allows to define configurations on codecs, size, experiences or other attributes of the MMBP. In TS 26.143, details of the API are not yet specified. The sender adds the MMBP to a Container Message (either included as a body part or by reference), together with MMBP metadata parameters that provide information about the MMBP.  The client of the messaging service receives the container message that includes the above information. The client communicates with a MMBP player its capabilities whether the MMBP can be played back, and if multiple alternatives are present, which of those are to be selected. Then the messaging service client instructs the MMBP player to playback the MMBP as part of the messaging service, based on the processing requirements and instructions. Playback may be combined with additional instructions for a player, including play, pause, seek, etc.

The design of the formats defined in TS 26.143 is based on the player model as shown in Figure 4.1-2. The figure illustrates the logical components of a conceptual MMBP Player model. In this figure, the MMBP parser receives the MMBP, and playback instructions. The Messaging Service Client may use metadata provided in a container message for playback selection. Such metadata may for example include codec capability information, language codes, accessibility information and other information for the selection of alternative parts in the MMBP.  The client then provides the sub-parts for processing and decoding to the related sub-part processors, and controls those for playback. The rendered message output may be handed back to the Messaging Service client for inband rendering or may be rendered directly.

Figure 4.1-2 MMBP Player Model

TS 26.143 defines media capabilities for both, MMBP generators as well as MMBP players. The media capabilities provide requirements for content generation as well as playback instructions, respectively.  This specification also defines profiles for content generators and players. Profiles are a collection of media capability requirements and recommendations. For detailed capabilities and profiles, refer to TS 26.143 or specific key issues in clause 5.

TS 26.143 does not include any high-level flow chart. Figure 4.1-3 provides a generic call flow based on the architecture shown in Figure 4.1-1 and the player model shown in Figure 4.1-2.

Figure 4.1-3 High-Level Call flow for messaging

The following high-level call flow is executed to address messaging:

1) 	A Message Service Sender instructs and MMBP generator to generate a message with the requested formats where the format is defined through a set of parameters handed over on an MMBP-GEN-API.

2) 	The MMBP generator creates an MMBP with relevant media data, and adds relevant static and dynamic metadata.

3) 	The generated file with associated relevant static and dynamic metadata is provided in a container message that is uploaded to a messaging server, together with a signaling in the media type and associated parameters. The media signaling may include that the file is suitable for different clients.

4)  A messaging service client is informed about the availability of a message in this format using the media type signaling together with appropriate sub-parameters.

5)	A messaging service client uses the MMBP-PLAY-API to query the MMBP player with it can process, decode and render the message.

6)	If the MMBP player indicates that it supports the capability, the messaging service clients selects and downloads the message and hands it to the MMBP player for playback.

7)	The MMBP player processes, decodes and renders the message based on its decoding and rendering capabilities.

## =====  CHANGE =====

### 4.2.3	IETF MIMI

Editor’s Note:	IETF MIMI will continue the work to develop More Instant Messaging Interoperability (MIMI) with the charter here: https://datatracker.ietf.org/wg/mimi/about/.

The IETF draft for the "More Instant Messaging Interoperability (MIMI) message content" [IETF-MIME] describes content semantics common in Instant Messaging (IM) systems and describes a profile suitable for instant messaging interoperability of messages end-to-end encrypted inside the MLS (Message Layer Security) Protocol. Commonly found features commonly found in IM group chat shared files/audio/videos and possibly even calling or conferencing.  Specifically, [IETF-MIME] describes the semantics of a message container in as sense as defined in TS 26.143. The container typically carries one or more body parts with the actual message content (for example, an emoji used in a reaction, a plain text or rich text message or reply, a link, an inline image, or in general a multimedia content).

The MIMI Content format is encoded in Concise Binary Object Representation (CBOR) [RFC8949].  The Working Group chose a binary format in part because:

-	we do not want to scan body parts to check for boundary marker collisions.  This rules out using multipart MIME types.

-	we do not want to base64 encode body parts with binary media types (ex: images).  This rules out using JSON to carry the binary data. All instance documents validate using Concise Data Definition Language (CDDL) [RFC8610] schemas for the MIMI Content Container.

According to clause 4.1 of [IETF-MIMI], MIMI content is structured as documented in Listing 4.2.3-1 using CDDL.

Listing 4.2.3-1 MIMI content structure according to [IETF-MIMI]

mimiContent = [ replaces: null / MessageId, ; {1} topicId: bstr, ; {2} expires: uint .size 4, ; {3} inReplyTo: null / InReplyTo, ; {4} lastSeen: [* MessageId], ; {5} extensions: {* name => value }, ; {6} nestedPart: NestedPart ; {7} ]

All fields are explained in [IETF-MIMI], clause 4. The core subject in context of this Technical Report is {7}, the NestablePart body. Details are defined in clause 4.4 of [IETF-MIMI], the syntax is provided in Listing 4.2.3-2.

Every MIMI content message has a body {7} which can have multiple, possibly nested parts. A body with zero parts is permitted when deleting or unliking. External body parts are also supported. When there is a single (inline) part or a (single) externally reference part, its IANA media type, subtype, and parameters are included in the contentType field {8}.

Listing 4.2.3-2 Syntax of NestablePart body

NestedPart = [ disposition: baseDispos / $extDispos / unknownDispos, ; {10} language: tstr, ; {11} partIndex: uint .size 2, ; {12} ( NullPart // SinglePart // ExternalPart // MultiPart) ] NullPart = ( cardinality: nullpart ) SinglePart = ( cardinality: single, contentType: tstr, ; {8} content: bstr ) ExternalPart = ( cardinality: external, contentType: tstr, url: uri, expires: uint .size 4, size: uint .size 8, encAlg: uint .size 2, key: bstr, nonce: bstr, aad: bstr, hashAlg: uint .size 1, contentHash: bstr, description: tstr ) MultiPart = ( cardinality: multi, partSemantics: chooseOne / singleUnit / processAll, parts: [2* NestedPart] ) ; cardinality nullpart = 0 single = 1 external = 2 multi = 3 ; part semantics {9} chooseOne = 0 ; receiver picks exactly one part to process singleUnit = 1 ; receiver processes all parts as single unit processAll = 2 ; receiver processes all parts individually

With some types of message content, there are multiple media types associated with the same message which need to be rendered together, for example a rich-text message with an inline image. With other messages, there are multiple choices available for the same content, for example a choice among multiple languages, or between two different image formats. The relationship semantics among the parts is specified as an enumeration {9}.

-	The chooseOne part semantic is roughly analogous to the semantics of the multipart/alternative media type, except that the ordering of the nested body parts is merely a preference of the sender. The receiver can choose the body part among those provided according to its own policy.

-	The singleUnit part semantic is roughly analogous to the semantics of the multipart/related media type, in that all the nested body parts at this level are part of a single entity (for example, a rich text message with an inline image). If the receiver does not understand even one of the nested parts at this level, the receiver should not process any of them.

-	The processAll part semantic is roughly analogous to the semantics of the multipart/mixed media type. The receiver should process as many of the nested parts at this level as possible. For example, a rich text document with a link, and a preview image of the link target could be expressed using this semantic. Processing the preview image is not strictly necessary for the correct rendering of the rich text part.

As an example, an ExternalPart is a convenient way to present both "attachments" and (possibly inline rendered) content which is too large to be included in a Messaging Layer Security (MLS)application message. The disposition data field is set to inline if the sender recommends inline rendering, or attachment if the sender intends the content to be downloaded or rendered separately. Listing 4.2.3-3 provides an example with a large file attachment in annotated Extended Diagnostic Notation, and pretty printed CBOR.

Listing 4.2.3-3 Example with a large file attachment

[ null, / replaces / h'', / topicId / 0, / expires = never / null, / inReplyTo / [ / lastSeen (1 item) / / Expiring message / h'5c95a4dfddab84348bcc265a479299fbd3a2eecfa3d490985da5113e5480c7f1' ], {}, / extensions / [ / body (NestedPart) / 6, / dispostion = attachment / "en", / language = en / 0, / partIndex = 1st part / 2, / cardinality = external part / "video/mp4", / contentType / 32("https://example.com/storage/bigfile.mp4"), / url / 0, / expires / 708234961, / size / 1, / encAlg = AES-128-GCM / h'21399320958a6f4c745dde670d95e0d8', / key / h'c86cf2c33f21527d1dd76f5b', / nonce / h'', / aad / 1, / hashAlg = sha256 / / content hash / h'9ab17a8cf0890baaae7ee016c7312fcc080ba46498389458ee44f0276e783163', "2 hours of key signing video" / description / ] ]

The MIMI required and recommended media types are documented in section 6.1 of [IETF-MIMI], but those are restricted to basic text and image formats (jpeg and png).

In addition, MIMI permits clients to send and receive proprietary formats among themselves. Using the functionality in the MIMI Content container, clients can send a message using the basic functionality described in [IETF-MIMI] and a proprietary format.

## =====  CHANGE =====

### 5.1.1	Description

TS 26.143 [26143] in Rel-18 provides containers based on Multi-part MIME as defined in IETF RFC 2045 [RFC2045]. IETF MIMI [IETF-MIMI], as introduced in clause 4.2.3 defines a similar framework, but a mapping of the MMBP data model from TS 26.143 is outstanding. The support of MMBPs, media capabilites and profiles defined in TS 26.143 in IETF MIME based containers needs to be studied.

Clause 4.2.3 provides and overview of IETF MIMI work.

Editor’s note:	Document the key topic in more detail, in particular how they relate to the system and data models in TS 26.143 and collect additional industry requirements according to F) Additional industry requirements as above.

## =====  CHANGE =====

### 5.5.2	Gap Analysis and Requirements

In order to identify the gaps to add 3D video to advanced messaging, a high-level flow chart is provided in Figure 5.5.2-1 and also eloborated further below based on the architecture shown in Figure 4.1-1 and the player model shown in Figure 4.1-2. The call flow is based on the generic description in clause 4.1.

Figure 5.5.2-1 High-Level Call flow for 3D video messaging

The following high-level call flow is executed to address the 3D video messaging and gaps are indicated in bold:

1) 	A Message Service Sender instructs and MMBP generator to generate a message with 3D format where the format is defined through a set of parameters handed over on an MMBP-GEN-API.

2) 	The MMBP generator creates an MMBP that includes 3D video formats encoded with MV-HEVC, and adds relevant static and dynamic metadata.

3) 	The generated file with associated relevant static and dynamic metadata is provided in a container message that is uploaded to a messaging server, together with a signaling in the media type and associated parameters. The media signaling may include that the file is suitable for different clients, for example it includes a 2D and a Beyond 2D experience.

4)  A messaging service client is informed about the availability of a message in this format using the media type signaling together with appropriate sub-parameters.

5)	A messaging service client uses the MMBP-PLAY-API to query the MMBP player with it can process, decode and render the message, possibly in 2D legacy or as 3D video.

6)	If the MMBP player indicates that it supports the capability, the messaging service clients selects and downloads the message and hands it to the MMBP player for playback.

7)	The MMBP player processes, decodes and renders the 3D message based on its decoding and rendering capabilities together with other media types.

Based on the analysis in the call flow, the following aspects are missing:

a)	the definition of the 3D formats that are supported

b)	a generator for a 3D video message including API parameters

c)	a 3D video messaging format including MV-HEVC with relevant metadata, including depth, alpha. projection formats, disparity information – for details refer to TR 26.956 [26956].

d)	the signaling of the media type and the associated parameters, typicaly a MIME type and the codecs parameter.

e)	an API to query the MMBP player that it can process, decoder and render the message

f)	an MMBP player capability that processes, decodes and renders the message.

g)	an MMBP player capability renders the video message together withother media components in a synchronized and spatially aligned matter.