3GPP TSG-SA WG4 MBS SWG Meeting Ad hoc post #129-e	S4aI240183

# Online, 26th September 2024 - 24th October 2024			                    revision of S4aI240139

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.804 | CR | 19 | rev | 3 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_AMD] WT #13: New clause 5.24 QUIC-based Media Delivery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_AMD |  |  |  |  |  | Date: |  |  | 14-10-2024 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | FS_AMD includes the topic about opportunities with QUIC for segmented streaming (topic “m)”). The corresponding new clause “QUIC-based segmented media delivery” needs to be developped. |  |  |  |  |  |  |  |  |
| Summary of change: |  | All the following changes against the baseline CR are proposed: - Updating clause 5.24 QUIC-based Media Delivery |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.24 [New] |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | - CR-19rev2: [N/A] - CR-19rev3: Populate clause 5.24 |  |  |  |  |  |  |  |  |

Change #1

## 2	References

[2]	Akamai Blog, "A QUICk Introduction to HTTP/3", April 2020, https://www.akamai.com/blog/developers/a-quick-introduction-http3

[5]	IETF RFC 9114: "HTTP/3", June 2022

[W3C-WT]	W3C Working Draft: "WebTransport", May 2024,
https://www.w3.org/TR/webtransport

Change #2

## 5.24	QUIC-based segmented media delivery

### 5.24.1	Description

#### 5.24.1.1	General

QUIC, specified in RFC 9000 [32], is a secure, reliable, multiplexed, connection-oriented transport protocol built on top of UDP. It is widely available and its impact on media streaming requires further study. In this clause we study QUIC-specific media streaming, not necessarily based on HTTP/3 [5] that is studied in clause 5.4.

A QUIC client establishes a connection with a server, and within this connection multiple concurrent streams can transport data . Thanks to a more efficient implementation of the TLS initial handshake, a QUIC connection is typically established faster than a TCP + TLS connection, therefore reducing initialization time. Additionally, by allowing QUIC streams to be multiplexed into a single QUIC connection, they can operate independently of each other, each with its own separate congestion window. Because packet retransmission occurs within a stream, a stall in one stream does not block the progress in the other streams. Similarly, packet loss in one stream does not affect the progress of data transfer in other streams multiplexed in the same QUIC connection. Finally, QUIC, like HTTP/2 [4], enables prioritisation capabilities at a stream level. However, QUIC does not provide the means to signal this prioritisation when multiplexing streams and it is up to the client implementation and the application to exchange and set the appropriate priorities.

Even though QUIC solves many issues compared to TCP e.g. faster connection establishing and non-blocking multiplexing (using streams), there are some open issues and shortcomings when it comes to media content delivery.

A 5GMS Application Provider runs an adaptive media streaming service between a 5GMS AS and a 5GMS Client running on a UE using 5G Media Streaming protocols conveyed at reference points M2 and M4. However, only M4 is relevant for this key topic since it focuses on the media delivery to the UE and not on the ingest of the media itself. Also, since M5 is not meant to transport media, this reference point is also excluded from this key topic.

#### 5.24.1.2	Application access to QUIC protocol features

In a common scenario, QUIC is not directly exposed to the application but it is mediated through a QUIC-enabled application protocol such as HTTP/3 [5] or WebTransport [W3C-WT]. For that reason, the set of QUIC features available to an application is the subset exposed by the chosen application protocol (typically invoked via the public API of a client or server library). This approach facilitates the development of QUIC-based communication but at the cost of limiting the level of control by the application on the connection and stream management.

#### 5.24.1.3	Connection and stream management

Applications are encouraged to keep QUIC connections alive when it makes sense to do so, and HTTP/3 client libraries typically facilitate connection keep-alive behaviour for efficiency reasons. In addition, QUIC's "0-RTT" connection establishment procedure allows a client to reconnect to a server it has previously connected to, and to reuse a security context negotiated during a previous connection and cached by both parties to send application payload data to the server in the first UDP datagram of the new QUIC connection.

In the context of HTTP/3 [5], the mechanism for handling streams is tightly specified by the HTTP/3 protocol that is each HTTP request-response transaction consumes one stream in the QUIC connection.

In contrast, WebTransport [W3C-WT] allows the application to manage the connections and the streams. However, WebTransport for segmented media delivery is not a well-studied area.

Editor's Note: Some publications about WT and streaming were recently published. Further input needed.

WebTransport allows both the client and the server to request opening streams. Finally, WebTransport supports different congestion control algorithms and unreliable connections.

#### 5.24.1.4	Stream prioritisation

Even though QUIC supports prioritisation at a stream level, it does not provide the means to exchange the prioritisation information. Additionally, stream priority is nominated by the sender, and as such neither the network nor the recipient are aware of the applied mechanism. Thus, prioritization is typically handled on the higher application layers. Existing standardization efforts include an extensible prioritization scheme for HTTP 3 defined in RFC 9218, and prioritization scheme for Media over QUIC (MOQ) Transport protocol currently being developed by IETF.

#### 5.24.1.5	Key Issue objectives

Editor's Note: Focus the preceding set of open issues into a set of clear questions that this Key Issue aims to address.

##### 5.24.1.5.1	Introduction

The objectives listed below are targeting QUIC-specific delivery aspects;.

- Application access to QUIC connections (e.g. via QUIC libraries)

- Variability of QUIC implementations

Editor's Note: Further content to be provided.

### 5.24.2	Collaboration scenarios

#### 5.24.2.1	General

For the purpose of describing the following scenarios, it is only assumed that the 5GMS Client supports the QUIC protocol. Additionally, the 5GMS Client may support higher level protocols based on QUIC (for instance HTTP/3 or WebTransport) but this is not required and the analysis of those collaboration scenarios is still applicable to any of those cases.

#### 5.24.2.2	QUIC-agnostic 5GMS Client

In this scenario, the Media Stream Handler of the 5GMS Client operates a QUIC session over reference point M4 but the 5GMS Client has no specific feature regarding QUIC. This has the advantage that 5GMS Client is generic and implements the same logic whether or not QUIC is used for the delivery of the media.

Figure 5.24.2.2-1: QUIC-agnostic 5GMS Client

#### 5.24.2.3	Media-independent QUIC-aware 5GMS Client

In this scenario, the Media Stream Handler of the 5GMS Client operates a QUIC session over reference point M4 and the 5GMS Client is able to detect whether QUIC is used and, in case it is used, the 5GMS Client can apply different logic.

Figure 5.24.2.3-1: Media-independent QUIC-aware 5GMS Client

In this case, the QUIC client implementation is not specifically optimised for media transport (e.g. a generic off-the-shelf QUIC client library) and the set of QUIC protocol features exposed to the Media Stream Handler is limited by the richness of its API.

With some limited control over the QUIC streams, such a 5GMS Client would typically be able to:

- Set relative priorities between the different QUIC streams. Relative stream priorities can be useful to differentiate audio and video, base layer and enhancement layer, etc.

- Receive updates sent proactively by the 5GMS AS. For example, a 5GMSd AS could send MPD updates to a Media Player using this mechanism.

Editor’s note: More possible general features to be added.

#### 5.24.2.4	Media-optimised QUIC-aware 5GMS Client

In this scenario, the Media Stream Handler of the 5GMS Client operates a QUIC session over reference point M4 and the 5GMS Client is able to control the delivery of the media within the QUIC session.

Figure 5.24.2.4-1: Media-optimised QUIC-aware 5GMS Client

In this case, the QUIC client implementation is optimised for media transport and the set of QUIC protocol features exposed to the Media Stream Handler is therefore unlimited. Hence, the media-optimised QUIC-aware 5GMS Client provides the finest control over the delivery of media within the QUIC session.

With fine control over the QUIC streams, such a 5GMS Client would typically be able to:

- Set relative priorities between the different QUIC streams. Relative stream priorities can be useful to differentiate audio and video, base layer and enhancement layer, etc.

- Receive updates sent proactively by the 5GMS AS. For example, a 5GMSd AS could send MPD updates to a Media Player using this mechanism.

- Use one QUIC stream for all the media segments of a given component (e.g. per CMAF Track).

Editor’s note: More possible special features to be added.

### 5.24.3	Architecture mapping

#### 5.24.3.1	General

In the 5GMS architecture, the Media Stream Handler in the 5GMS Client is connected to the 5GMS Application Server via the M4 interface. Therefore, both the client and the server on that interface need to be compatible in terms of protocols and functionalities. In the clause, the QUIC Client module which is part of the 5GMS Client will thus be connected to a QUIC Server connected or part of the 5GMS Application Server. Three mappings will be considered, one for each collaboration scenario.

#### 5.24.3.2	Mapping with a QUIC-agnostic 5GMS Client

In this mapping, both 5GMS Client and Application Server are agnostic to the QUIC protocol and the usage of QUIC as a network protocol is thus transparent for system. The integration of the QUIC Client in the UE and the QUIC Server with the 5GMS Application Server is considered to be out of scope of the 5GMS scope.

Figure 5.24.3.2-1: Architecture with QUIC-agnostic 5GMS Client and 5GMS AS

#### 5.24.3.3	Mapping with a Media-independent QUIC-aware 5GMS Client

In this mapping, the 5GMS Client and the 5GMS Application Server are integrated with respectively a QUIC Client and QUIC Server. The QUIC Client and the QUIC Server are external to the systems which means that any software implementation may be used. The integrations of the QUIC Client in the UE and the QUIC Server with the 5GMS Application Server are achieved via the exposed APIs by both the QUIC Client and QUIC Server. However, since the QUIC Client and Server are external to the 5GMS system, the APIs exposed by the QUIC Client and the QUIC Server may thus also differ in capabilities. In addition, the QUIC Client and the QUIC Server needs to support a QUIC version that both parts supports. The QUIC protocol does not provide the means for negotiating the version when establishing the connection but rather enables a QUIC Server to rejects and proposes alternative versions to a QUIC Client. This step of version selection may be influenced by Media Stream Handler and the 5GMS AS through those APIs.

Figure 5.24.3.3-1: Architecture with QUIC-aware 5GMS Client and 5GMS AS

#### 5.24.3.4	Mapping with a Media-optimised QUIC-aware 5GMS Client

In this mapping, the 5GMS Client and the 5GMS Application Server are integrated with respectively a QUIC Client and QUIC Server. The QUIC Client and the QUIC Server are part of the 5GMS which means that their software implementation follows requirement and functionalities supporting the 5GMS Client’s needs. The integrations of the QUIC Client in the UE and the QUIC Server with the 5GMS Application Server are achieved via the exposed APIs by both the QUIC Client and QUIC Server. Since the QUIC Client and Server belong to the 5GMS system, the APIs exposed by the QUIC Client and the QUIC Server are compatible in terms of functionalities. In addition, the QUIC Client and the QUIC Server support a commonly agreed QUIC version to guarantee the establishment of the QUIC connection. In addition, those client and server APIs enables the 5GMS Client to optimise the media delivery.

Figure 5.24.3.4-1: Architecture with media-optimised QUIC-aware 5GMS Client and 5GMS AS

### 5.24.4	High-level call flow

#### 5.24.4.1	General

Editor's Note: Further content to be provided.

### 5.24.5	Gap analysis and requirements

#### 5.24.5.1	General

Editor's Note: Further content to be provided.

#### 5.24.5.2	Computational overhead of QUIC clients

Adoption of QUIC-based application protocols such as HTTP/3 can have impact on the UEs due to the processing required for the underlying QUIC protocol itself. In TCP, most functions are executed in kernel space, which over the years have been heavily optimised including, in some hosts, hardware offload. In QUIC, by contrast, most protocol functions are executed in user space.

There are two identified sources for this computational overhead [x6]:

1.	The in-kernel UDP stack issues many packet reads; this is because each datagram arriving in the link layer is forwarded for processing to the transport layer i.e., there are no offload mechanisms used like the sender-side Generic Segmentation Offload (GSO), or the generic receive offload mechanism (GRO) used by the link layer module to combine datagrams into a mega datagram before forwarding it to the transport layer.

2.	There is increased overhead of processing packets (and, as a result, in generating responses) due to increased user space processing required as a result of 1 above (i.e. all the packets need to be processed individually) combined with managing QUIC acknowledgements in user space (instead of kernel space as with TCP in HTTP/2).

These potential performance effects should be taken in consideration because they can negatively impact the overall Quality of Service that can be obtained from a QUIC connection, and can increase the energy consumption of the UE.

### 5.24.6	Candidate Solutions

Editor’s Note: Provide candidate solutions (including call flows) for each of the identified issues.

### 5.24.7	Summary and Conclusions

The study of this Key Issue has explored the ways in which QUIC can be deployed to support the 5G Media Streaming architecture, and the potential open issues arising from this deployment.

Editor's Note: Further content to be provided.

END OF CHANGES