3GPP TSG-S4 Meeting  	S4aI250006

# SA4-e (AH) MBS SWG post 130

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.510 | CR | 0014 | rev |  | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [5GMS_Pro_Ph2] Editorial correction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS_Pro_Ph2 |  |  |  |  |  | Date: |  |  | 2024-12-17 |
| Category: | D |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Editorial modifications to correct wrong clause and table numbers in TS 26.510. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Editorial correction in clause 5.4.4, 5.6.2, and 11.4.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misleading by wrong clause and table numbers. |  |  |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.4, 5.6.2, and 11.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

### 5.4.4	Network Assistance invocation

At the start of a media delivery session, the Media Access Function shall determine an initial bit rate recommendation by invoking an appropriate API method on the Media Session Handler at reference point M11. The Media Session Handler shall invoke the operation specified in clause 5.3.4.2 to create a new Network Assistance Session and shall return to the Media Access Function the bit rate recommendation information in the response from the Media AF. The Media Access Function may use this information to set an initial bit rate for the media delivery session (e.g., by selecting an initial MPEG-DASH Representation or contribution media encoding bit rate). The Media Access Function may also use this bit rate recommendation to select a different Service Operation Point, resulting in a change to the current Dynamic Policy (see clause 5.4.2).

The Media Session Handler shall subscribe to receive notifications from the Media AF at reference point M11 of bit rate recommendation updates. When such an update is received from the Media AF, the Media Session Handler shall send a notification to the Media Access Function at reference point M11 providing the new bit rate recommendation. As a result, the Media Access Function may modify the bit rate of the media delivery session (e.g., by switching to a different MPEG-DASH Representation or by varying the rate controller of a contribution media encoder). The Media Access Function may also use this bit rate recommendation to select a different Service Operation Point, resulting in a change to the current Dynamic Policy (see clause 5.4.2).

During the course of the media delivery session the Media Access Function may request a delivery boost by invoking an appropriate API method on the Media Session Handler at reference point M11 (see clause 11.4.1.2).

Next change

### 5.6.2	ANBR-based Network Assistance

If ANBR-based Network Assistance is supported, the Media Session Handler may use an interface to the Medium Access Control entity in the UE modem to send and receive bit rate recommendation messages. This interface may be based on the AT commands +CGBRRREQ and +CGBRRREP as defined in TS 27.007 [21].

Furthermore, messaging across that interface corresponds to the logical translations of the Bit Rate Recommendation and/or Bit Rate Recommendation Query messages, carried by the Recommended bit rate MAC CE, exchanged between the RAN Modem and the RAN, as specified in TS 38.321 [22] for 5G NR and TS 36.321 [22] for LTE. The association between the LCID for which the recommendation applies and the actual flow (including the intermediate RLC channel) is performed by the modem.

NOTE:	The AT command +C5GQOSRDP=? may be used to obtain a list of CID values that are associated with QoS flows (both network and MT/TE initiated). When used to request a bit rate boost, the query shall not request a bit rate that may exceed the MFBR for the corresponding QoS Flow. Failure to ensure this may result in unexpected congestion-induced packet delays and packet dropping.

The Bit Rate Recommendation Query shall indicate the bit rate desired by the application, as described by [22] and [23]. This request may be used by the Media Session Handler to request for a temporary increase in bit rate for the corresponding flow ("delivery boost"). The RAN responds with a Bit Rate Recommendation message that confirms the recommended bit rate after the boost grant. Once the bit rate drops again after a boost grant, the network shall inform the Media Session Handler about the new recommended bit rate by means of an ANBR message.

Whenever the Media Session Handler receives a message from the UE modem corresponding to the logical translation of the Bit Rate Recommendation message for the associated RAN uplink or downlink, it shall notify the Media Access Function (via a suitable reference point M11 notification – see table 11.4.2-2) of the new bit rate recommendation associated with the indicated PDU session.

Furthermore, whenever the Media Session Handler receives a request for a delivery boost from the Media Access Function (via a suitable reference point M11 method invocation – see clause 11.4.1.2) relating to a PDU session in use for Media Delivery, it may send a delivery boost message to the UE modem. That delivery boost request is logically translated by the modem to the Bit Rate Recommendation Query message which is then sent to the RAN on the associated RAN uplink or downlink.

It is left to the implementer of the Media Access Function to decide how to best use the bit rate recommendation and the bit rate recommendation query information for individual media delivery sessions.

Next change

### 11.4.2	Network Assistance information

Table 11.4.2-1 specifies the status information that can be obtained from the Media Session Handler.

Table 11.4.2-1: Status Information relating to Network Assistance

| Status | Type | Parameter | Definition |
| --- | --- | --- | --- |

Table 11.4.2-2 provides a list of general notification events exposed by the Media Session Handler.

Table 11.4.2-2: Notification Events relating to Network Assistance

| Event | Definition | Payload |
| --- | --- | --- |
| BIT_RATE_RECOMMENDATION | Triggered when a bit rate recommendation is received from the Media AF or from the UE modem. The source of the recommendation is indicated as a parameter of the payload. | Media delivery session identifier, Recommendation source, Recommended downlink bit rate, Recommended uplink bit rate. |

End of changes