3GPP TSG-SA4 Meeting #0-e (AH) MBS SWG post 130	S4aI250046

# Online, , 19th Dec 2024 - 6th Feb 2025	revision of S4aI250030

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.501 | CR | 0102 | rev | 2 | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [AMD-ARCH-MED] Distributing encrypted and high-value content |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Germany |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AMD-ARCH-MED |  |  |  |  |  | Date: |  |  | 2025-01-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | DRM and Conditional Access: DRM and Conditional Access are commonly used by third-party streaming services. However, in case streaming is done through MBS or MBMS, a more careful management of the keys needs to be checked. Scalability of key delivery is an issue. The support for -encrypted content in Unicast/Multicast and Broadcast is relevant. Integration of Content Protection interfaces in the provisioning, for example using CPIX back-end interfaces is of high relevance for the industry and should accordingly be studied. The impacts of these on media plane (reference points M2 and M4) as well as the media session handling APIs (reference points M3, M5) should also be studied. For details see TR 26.804 clause 5.10 and the conclusions in clause 6.10. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For distributing encrypted and high-value content as introduced in clause 5.10 of TR 26.804: i. Functional updates to the definition of the 5GMS AS to support: 1. Ingest, delivery, and contribution of encrypted content 2. Content preparation tasks for: 3. Decrypting content ingested at reference point M2d. 4. (Re-)encrypting content prior to distribution at reference point M4d. ii. Updates to the definitions of reference points to support: 1. Carriage of Content Protection information at reference point M2d. 2. Delivery of Content Protection information in presentation manifests at reference point M4d. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Feature not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This revision 1 is updating the initial version to add substance. It is submitted for endorsement as basis for future work S4aI250030; [AMD-ARCH-MED] Distributing encrypted and high-value content; Qualcomm Germany; Thomas Stockhammer. E-mail Discussion: none Revisions: - S4aI250030_BBC.docx Presenter: Thomas Stockhammer Online Discussion: (January 8 2025) - Richard: Do you see anything further being required than this? - Thomas: I need to check in 501. - Richard: Is the only mapping you think is relevant? - Thomas: Obviously that is the case. - Richard: What about the case, where you decrypt, decode and re-encrypt? - Thomas: I'm not aware of such a case. - Fred: will be there a formal revision? - Thomas: lets park it and then we will see and lets do a r02. Decision: - January 8, 2025: parked, expect a revision. S4aI250030 is parked. S4aI250030; [AMD-ARCH-MED] Distributing encrypted and high-value content; Qualcomm Germany; Thomas Stockhammer. E-mail Discussion: none Revisions: - S4aI250030_BBC.docx - S4aI250030r01.docx - S4aI250030r02.docx - S4aI250030r02_BBC.docx - S4aI250030r03.docx - S4aI250030r03_BBC.docx - S4aI250030r04.docx - S4aI250030r04_huawei.docx Presenter: Thomas Stockhammer Online Discussion: (January 8 2025) - Richard: Do you see anything further being required than this? - Thomas: I need to check in 501. - Richard: Is the only mapping you think is relevant? - Thomas: Obviously that is the case. - Richard: What about the case, where you decrypt, decode and re-encrypt? - Thomas: I'm not aware of such a case. - Fred: will be there a formal revision? - Thomas: lets park it and then we will see and lets do a r02. (January 10 2025) - Thomas: Surprised to get comments from Huawei at this late stage. - Rufael: Clarifies indicating inconsistencies. For example in the TR key request is done by DASH player and not media client. - Thomas: It is unfortunate because this has not been indicated during the 1st presentation. It is very difficult. - Rufael: Yes, sorry but we only had one day to review. Decision: - January 8, 2025: parked, expect a revision. - January 10, 2025:. R04_huawei is revised and the revised doc is noted. S4aI250030 is revised to S4aI250046 and S4aI250046 is noted. |

## =====  CHANGE =====

### 4.0.4	Content preparation

The content preparation feature is applicable to both downlink media streaming (where is provisioned as part of the content hosting feature introduced in clause 4.0.2) and uplink media streaming (where is provisioned as part of the content publishing feature introduced in clause 4.0.3). The content preparation feature enables a 5GMS Application Provider to specify content manipulation by network-side components of the 5GMS System according to provisioned Content Preparation Templates.

When a 5GMSd Application Provider has provisioned the content preparation feature for downlink media streaming:

1.	Network-side components of the 5GMS System may manipulate ingested media content and may cache the manipulated content prior to serving it to the 5GMSd Client in the UE. Content Preparation may include encoding, transcoding, packaging, encryption and protecting content using DRM.

When a 5GMSu Application Provider has provisioned the content preparation feature for uplink media streaming:

1.	Network-side components of the 5GMS System may manipulate the media content ingested from the 5GMSu Client in the UE and may cache the manipulated content prior to egesting it to the 5GMSu Application Provider.

## =====  CHANGE =====

## 5.14	Content Preparation of DRM-protected content

### 5.14.1	Scenario

There are cases for which the 5GMSd System runs a content encoding, encryption and/or packaging service. This may be needed in case the content is provided under restricted licence. In a typical deployment scenario, the License Server is external in the 5GMSd Application Provider domain, but the encoding, encryption and packaging is completed in the 5GMSd AS, for example to adapt the content to the needs of mobile distribution. In order to complete the encryption tasks and manifest generation, the 5GMSd AS needs to communicate with the License Server for content encoding and packaging.

### 5.14.2	Procedure

A typical example workflow of encrypted content is shown in figure 5.14.2-1.

Figure 5.14.2-1: Procedure for encoding, packaging and encrypting content

The steps are as follows, highlighting in bold relevant information exchanged via 5GMSd reference points:

Initialisation:

1.	The Encryptor/Packager, License Server, and Authorization Server exchange public keys via reference point M2d.

Content Protection Information construction:

2.	The Packager constructs content protection information.

3.	The Encryptor retrieves keys and adds them to the content protection information.

Content Protection Information distribution:

4.	The Encryptor/Packager sends the content protection information to the License Server via reference point M2d

and the License Server sends the content protection information to the Authorization Server.

5.	The Authorization Server decrypts the keys and adds data to the content protection information.

6.	The Authorization Server sends the updated content protection information to the License Server.

7. 	The License Server decrypts the keys and adds data to the content protection information.

8.	The License Server sends the updated content protection information to the Encryptor/Packager and the Manifest Creator via reference point M2d.

Presentation manifest and media segment generation:

9.	The Manifest Creator generates the presentation manifest (e.g. DASH MPD) and adds the content protection information.

10.	The Manifest Creator uploads the presentation manifest to the Content Hosting.

11.	The Encryptor/Packager generates encrypted segments and adds the content protection information.

12.	The Manifest Creator uploads the encrypted segments to the Content Hosting.

Client requests and authorisation:

13.	The DASH Client requests the presentation manifest including content protection information from the Content Hosting via reference point M4d.

14.	The DASH Client requests authorisation tokens from the Authorization Server via reference point M13d.

15.	The DASH Client requests a DRM licence from the License Server via reference point M13d, possibly using the authorisation tokens.

16.	The DASH Client provides the DRM licence to the DRM Client.

Content delivery and decryption:

17.	The DASH Client requests encrypted segments from the Content Hosting via reference point M4d.

18.	The DASH Client provides the encrypted segments to the Media Platform.

19.	The DASH Client provides the encrypted samples to the DRM Client.

20.	The DRM System decrypts the samples using the DRM licence and content keys.

21.	The DRM System provides the decrypted samples to the Media Platform.