3GPP TSG-SA WG4 Adhoc Telcos post 130	S4aI250051

Adhoc eMeeting,  19th Dec '24- 6th Feb'25

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.942 | pCR |  | rev |  | Current version: | 1.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | ### Solution #7: Potential solution to Key Issue #2: QMC-based monitoring and measurement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_MediaEnergyGREEN |  |  |  |  |  | Date: |  |  | 2025-02-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The latest draft of 3GPP TR 26.942 contains clause 6.2 Key Issue #2: Energy-related monitoring and measurement. In this context, under KI #2; the following questions were defined: In this context, the subsequent analysis by this Key Issue will consider the following questions: - Which UE energy-related information will be collected to measure, correlate, and optimize energy usage across the entire streaming distribution chain? - Can existing methods be leveraged to measure/monitor the identified UE energy-related information? - Which UE entity is appropriate to measure this UE energy-related information? It is proposed to add the proposed content to the latest draft of TR 26.942 v1.0.0 as one of the potential solutions to KI #2 so that it is not left incomplete. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR proposes new text to be added in TR 26.942 on “Clause 7 Potential Solutions”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Proposed objectives will not be met. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S4-242024 S4aI250008 S4aI250044 |  |  |  |  |  |  |  |  |

## 1st Change

# 2	References

[TS 26.114] 3GPP TS 26.114 IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction

[TS 26.247] 3GPP TS 26.247 Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)

## 2nd Change

## 7	Potential solutions

## 7.1	Mapping of solutions to Key Issues

Table 7.1-1: Mapping of solutions to Key Issues

| Solutions |  |  |  |
| --- | --- | --- | --- |
|  | KI#1 | KI#2 | KI#3 |
| #1 |  |  |  |
| #2 |  |  |  |
| #3 |  |  |  |
| #4 |  |  |  |
| #5 |  |  |  |
| #6 |  |  |  |
| #7 | X |  |  |
| #8 |  |  |  |
| #9 |  |  |  |

## 3rd Change (all new text)

## 7.x	Solution #7: QMC-based monitoring and measurement.

### 7.x.1	Key Issue mapping

This solution addresses Key Issue #2.

### 7.x.2	Functional description

#### 7.x.2.1	Introduction

There is currently no solution enabling to monitor energy saving actions at application layer and in the RAN access stratum based on information provided by the application layer on the UE. To this end, we propose an energy consumption aware mechanism for media handling and delivery (both UL and DL) based on QoE metrics collection, configuration and reporting for different types of media services. The mechanism for media handling and delivery includes UE side and network side operations according to the reported energy consumption information.

This candidate solution focuses on the energy consumption monitoring. As a result of collecting and evaluating energy-related measurements on the UE, energy consumption in the network may be reduced, thus triggering network energy savings. A typical use case is for the network (potentially acting on behalf of an application) to initiate a campaign of UE energy-measurements in order to evaluate the impacts of a specific action taken (e.g. updating some parameters of a media session). In particular, when contextual to QoE measurement, the network, or an application, can appreciate the relationship between QoE and energy consumption on the UE, that is to look for an optimum configuration that would save most energy on the UE whilst preserving the target QoE (trade-off).

In this context, this candiate solution proposes a method leveraging on energy consumption information, to monitor and measure the way the media content is handled and delivered to the users, and to provide better quality of experience (QoE) of users. Specifically, this candidate solution focuses on extending the UE QoE reporting with energy-related information.

#### 7.x.2.2	MTSI Quality of Experience (QoE) metrics

MTSI Quality of Experience (QoE) metrics is a relevant background for this candidate solution. As defined in TS 26.114 [TS 26.114], the metrics are valid for speech, video and text media, and are calculated for each measurement resolution interval "Measure-Resolution”. They are reported to the server according to the measurement reporting interval "Sending-Rate" and after the end of the session. The metrics defined in TS 26.114 includes: corruption duration metric, successive loss of RTP packets, frame rate, jitter during, sync loss duration, average codec bitrate, codec information as well as call setup time, etc. But the specified metrics don’t include energy consumption related information.

Furthermore, the QoE configuration and reporting can optionally be specified by the QoE Measurement Collection (QMC) functionality. In this case the QoE configuration is received via specific RRC messages for UMTS, RRC messages for LTE, and RRC messages for NR over the control plane, and the QoE reporting is also sent back via RRC messages over the control plane. Accordingly, examples signaling diagram for NR is attached:

Figure 16.5.1-3: Example signalling diagram for NR

#### 7.x.2.3 	DASH Quality of Experience (QoE) metrics

In TS 26.247 [26.247], 3GPP has defined QoE metrics and procedures for progressive download and DASH media streaming. Configuration and reporting can be based on the same mechanisms (QMC) as for MTSI, or via MPD or OMA-DM.

### 7.x.3	Procedures

This Candidate Solution proposes a new metric; procedures for reporting this metric from the UE to an external entity are described in solution #4 in clause 7.5.

#### 7.x.3.1	Network triggered QoE configuration

1) QoE Measurement Collection (QMC) functionality can be reused as follows (Following signalling diagram is based on TS 26.114 for the MTSI use cases)

- Option 1 (adding EC as a new flag in MTSI QoE reporting, related to a specific media session):

  - Step 0: When UE starts/registers, the QMC handler of the UE indicates “qoe-EC-MeasReport” capability via UE Access Stratum when supported.

  - Step1a: OAM sends QoE configuration requests with EC flag (energy consumption) inside MTSI QoE reporting request which is associated with media session ID, time stamp, etc.

  - Step 1b: QMC Handler within the UE triggers the MTSI Client to collect MTSI QoE metrics;

  - Step 2: UE (MTSI Client) collects Energy-related QoE metrics related to the media session. This may be done e.g. based on new AT commands between the UE Application Layer and the UE Access Stratum.

    - UE may rely on the “UE energy information handler” defined in NC332464, including via the QMC handler entity.

  - Step 3: the new QoE report is created and sent to OAM via QMC Handler, including the requested EC information in the MTSI QoE container;

  - After OAM received UE energy consumption status report (step 3c of the figure below), OAM may forward this information to an MnS Consumer (e.g. AF), the AF can accordingly propose an optimized network configuration (e.g. different QoS) or slice to the UE via the 5GC to fit the UE energy consumption status.

Figure 7.x.3.1: Example signalling diagram for option 1

- Option 2 (as a dedicated QoE configuration for energy reporting only):

  - Step 0: UE indicated to gNB that it supports energy consumption measurement via “capability = qoe-EC-MeasReport” ;

  - Step 1: OAM requests energy consumption reporting via a dedicated QoE configuration request “qoe-EC-MeasReport” (step 1a of the figure below), UE (Energy Handler) will forward this request to the client;

  - Step 2: UE collects QoE metrics including the requested EC information and create the new QoE report.

  - Step3: UE sends the QoE report (periodically) to the OAM via UE (Energy Handler) with the requested EC information.

  - After OAM received UE QoE status report (step 3c of the figure above), OAM may forward this information to an MnS Consumer (e.g. AF), the AF can accordingly propose an optimized network configuration (e.g. different QoS) or slice to the UE via the 5GC to fit the UE energy consumption status.

Figure 7.x.3.2: Example signalling diagram for option 2

- The same mechanisms apply to DASH and XR use cases using QMC: similarly to MTSI, the energy consumption information mentioned above can be incorporated to the “qoe-Streaming-MeasReport” QoE configuration request and reporting using QMC.

### 7.x.4	Summary

This candidate solution introduces:

- New QoE metrics definition related to UE energy consumption at device, processing and application level, which can be associated to a dedicated media session;

- UE side configuration to optimize user experience based on the collected energy and media session QoE-related metrics, including requesting the network or AF for session optimization or split-rendering support

- Network triggered mechanisms for UE energy metrics configuration and reporting, either inside or outside media session QoE metrics collection, as well as RAN visible QoE reporting of UE energy consumption information.

These mechanisms will enable the network (CN or RAN) to trigger energy saving actions. According to some examples OAM may forward the UE energy consumption information to a Consumer (e.g. AF), the AF can accordingly propose an optimized network configuration (e.g. different QoS) or slice to the UE via the 5GC to fit the UE energy consumption status. According to some other examples, the UE energy consumption information received via RAN visible QoE reporting, corresponding to energy consumption for UE radio transmission and reception, may trigger actions in the RAN e.g. in terms of addition/removal of SCells for Carrier Aggregation, or e.g. use of Dual Connectivity, e.g. addition/release of SCG (Secondary Cell Group) or SCG activation/deactivation.

## End of change