3GPP TSG-SA4 Meeting #0-e (AH) MBS SWG post 130	S4aI250058

# Online, , 19th Dec 2024 - 6th Feb 2025	revision of S4aI250039

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.502 | CR | 0033 | rev | 3 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [AMD-ARCH-MED] In-session Unicast Repair for MBS Object Distribution |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Germany |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AMD-ARCH-MED |  |  |  |  |  | Date: |  |  | 2025-02-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In-session Unicast Repair for MBS Object Distribution: For live and low-latency live services using the Object Distribution Method in MBS, in certain cases the transmission of an object is not successful. In this case, unicast repair for individual MBS Clients can improve the service quality. However, the timing of such requests needs to be carefully studied in order to avoid network overloads or significant latencies in the delivery. A study to extend MBS User Services and object streaming to address in-session repair is of relevance. For details refer to TR 26.802. In TR 26.802, In-session unicast repair for MBS Object Distribution as introduced in clause 5.9 and based on the conclusions in clause 5.9.7 to address Gap#1 in clause 5.9.5 by the candidate solution in clause 5.9.6 has been motivated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In-session unicast repair for MBS Object Distribution as introduced in clause 5.9 of TR 26.802: i. Define a new reference point in TS 26.502 between the MBSTF and the MBS AS. ii. Document call flows and procedures for both post-session and in-session unicast repair. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Feature not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 1, 2, 4.2.2, 4.2.6, 4.3.1, 4.3.3.1, 4.3.3.2, 4.3.4, 4.4.1, 4.9, 5.2, 5.3, 5.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S4aI250022; [AMD-ARCH-MED] In-session Unicast Repair for MBS Object Distribution; Qualcomm Germany; Thomas Stockhammer. E-mail Discussion: none Revisions: - S4aI250022_BBC.docx - S4aI250022r01.docx Presenter: Thomas Stockhammer Online Discussion: (January 10 2025) - r01_BBC version presented. - Richard: Clause 7.4 has been deleted. It is interesting but I prefer to manage it in a future release. - Richard: My recommendation is to not include procedure in clause 5.6.2. - Thomas: OK this clause will be deleted and incorporated in 5.3-2. - Richard: The parallelisation in a procedure is difficult. Less is more. We should illustrate only one object in the diagram and manage the parallelisation in the text. - Thomas: MSC generator does the interleaving for you. Maybe the layout is not great. But I would like to see it in the diagram. - Richard: But the current diagram is not good. - Thomas: We can check this. Decision: - January 10, 2025: Revised to XXX corresponding to 022_r01_BBC. XXX is endorsed and a new revision is expected during next call. S4aI250022 is revised to S4aI250039 and endorsed. This revision addresses cleaning to the extent possible. It also changes call flows from OLE to ALT+TEXT |

## =====  CHANGE =====

# 1	Scope

The present document defines an architecture and high-level procedures for User Services conveyed using the 5G multicast–broadcast capabilities of the 5G System defined in TS 23.501 [2], TS 23.502 [3] and TS 23.247 [5].

The design of the system is primarily motivated by the conclusions of TR 26.802 [20].

## =====  CHANGE =====

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[3]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[4]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[5]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services; Stage 2".

[6]	3GPP TS 26.348: "Northbound Application Programming Interface (API) for Multimedia Broadcast/Multicast Service (MBMS) at the xMB reference point".

[7]	3GPP TS 26.501: "5G Media Streaming (5GMS); General description and architecture".

[8]	IETF RFC 3550: "RTP: A Transport Protocol for Real-Time Applications".

[9]	IETF RFC 2250: "RTP Payload Format for MPEG1/MPEG2 Video".

[10]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[11]	3GPP TS 26.531: "Data Collection and Reporting; General Description and Architecture".

[12]	3GPP TS 23.468: "Group Communication System Enablers for LTE (GCSE_LTE)".

[13]	Void.

[14]	3GPP TS 23.468: "Group Communication System Enablers for LTE (GCSE_LTE)".

[15]	3GPP TS 29.522: "5G System; Network Exposure Function Northbound APIs; Stage 3".

[16]	OMA: "OMNA BCAST Service Class Registry", https://technical.openmobilealliance.org/OMNA/bcast/bcast-service-class-registry.html.

[17]	IANA: "Reliable Multicast Transport (RMT) FEC Encoding IDs and FEC Instance IDs", https://www.iana.org/assignments/rmt-fec-parameters/rmt-fec-parameters.xhtml#rmt-fec-parameters-1.

[18]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[19]	3GPP TS 33.246: "Security of Multimedia Broadcast/Multicast Service (MBMS)".

[20]	3GPP TR 26.802: "Multicast Architecture Enhancement for 5G Media Streaming".

## =====  CHANGE =====

### 4.2.2	User Services network architecture

MBS User Services enable high-level applications to make use of the low-level features of the MBS System. An MBS User Service is provided by the MBSF and MBSTF working in combination to support configuration option 2 and configuration option 3 defined in annex A of TS 23.247 [5]. In addition to the Network Functions defined in [5]:

-	The MBS AF provides unicast User Service Announcements to the MBSF Client in the user plane and to the MBSTF.

-	The MBS AS provides unicast services such as Object Repair to the MBSTF and MBSTF Client.

-	The MBSSF supports the User Plane security procedure according to clause W.4.1.3 of TS 33.501 [18] and provides a user plane authentication service to the MBSF Client.

Together, these functions enable a complete service offering to an end-user, via a set of APIs that allows the MBS Client to activate or deactivate reception of MBS User Services.

The MBS User Services architecture depicted in figure 4.2.2-1 shows the MBS-related entities involved in providing MBS User Services delivery and control. These are described in the following clauses. The MBS Application Provider plays the role of the AF/AS.

Figure 4.2.2-1: MBS User Services network architecture

## ===== CHANGE =====

### 4.2.6	Object Repair

When using the Object Distribution Method, Object Repair procedures may be provisioned by the MBS Application Provider. This feature allows an MBS Client to retrieve from the MBS AS (via reference point MBS-4-UC) missing portions of objects not received intact from the MBSTF at reference point MBS-4-MC.

The following Object Repair procedures are defined in clause 5.6 of the present document:

-	In-session Object Repair during the MBS Distribution Session, including the ability to repair objects in streaming distribution.

-	Post-session Object Repair after the completion of the MBS Distribution Session, including a randomized back-off period for different MBS Clients in order to avoid overloading the MBS AS.

The functions of the MBS AS are defined in clause 4.3.4.

## =====  CHANGE =====

### 4.3.1	General

The MBSF and MBSTF offer service layer functionality for sending data via MBS Sessions. The MBSF (clause 4.3.2) offers control plane functionality while the MBSTF (clause 4.3.3) offers user plane functionality. The MBSTF acts as a User Plane anchor when it sources IP multicast traffic. Reference point Nmb2 provides the means for the MBSF to configure the delivery methods in the MBSTF, and also supports push-based ingest of object manifests. Reference point MBS-11 supports pull-based ingest by the MBSTF of object manifests and pull-based ingest of User Service Announcement objects for delivery to the MBS Client via reference point MBS-4-MC (as described in clause 4.2.4).

Figure 4.3.1-1 shows the complete set of functional entities involved in supporting MBS User Services when the MBS Application Provider is deployed in the Trusted DN, including client functions in the UE.

NOTE:	When the MBS Application Provider is deployed outside the Trusted DN, it interacts with the MBSF via the NEF at reference point N33, as shown in figure 4.2.2-1, instead of via Nmb10.

Figure 4.3.1-1 MBS User Service reference architecture

In the above architecture, MBS-specific functions such as the MBS AS and MBSF are shown as independent and standalone. In deployments, they may be co-located on physical devices with other functions. As an example, the MBS AS may be hosted in the MBS Application Provider domain, or it may be hosted in a 5GMS AS.

## =====  CHANGE =====

#### 4.3.3.1	General

The functionality of the MBSTF is defined in clause 5.3.2.12 of TS 23.247 [5]. It receives User Plane data traffic at reference point Nmb8 and sends MBS data packets to the MB-UPF via reference point Nmb9.

NOTE:	The MBSTF may not be present in all deployments of the MBS System. However, the MBSTF is an integral and necessary component of MBS User Services, and the present document is not concerned with deployments in which it is absent.

The present document defines additional User Plane functionalities of the MBSTF to support MBS User Services as follows:

1.	Receiving MBS Distribution Session configurations from the MBSF at reference point Nmb2.

2.	Sending notification events to the MBSF, e.g., data ingest failure, session terminated, delivery started via reference point Nmb2.

3.	Based on the MBS Distribution Session configuration:

a)	MBS delivery of ingested objects or sequences of objects to the MBS Client using the Object Distribution Method (see clause 6.1). This may be used to support real-time distribution of media segments (as special objects) including CMAF segments.

b)	MBS delivery of ingested packet streams to the MBS Client using the Packet Distribution Method (see clause 6.2).

c)	Delivery of MBS User Service Announcement information (including updates as necessary) within the MBS Session.

4.	In the case of 3a), potentially forwarding ingested objects to the MBS AS to support object repair.

## =====  CHANGE =====

#### 4.3.3.2	MBSTF subfunctions to support Object Distribution Method

The MBSTF subfunctions supporting the Object Distribution Method are depicted in figure 4.3.3.2-1 below.

Figure 4.3.3.2-1: MBSTF architecture overview for Object Distribution Method

The Object ingest subfunction supports:

-	Pull-based ingest at reference point MBS-11: The Object ingest subfunction in this case fetches one or more objects from the MBS AF using HTTPS for inclusion in the MBS User Service Announcement Channel. (The set of objects to be included is listed in an object manifest previously fetched from the MBS AF at the same reference point.)

-	Push-based ingest at reference point Nmb2: The Object ingest subfunction receives an object manifest from the MBSF that describes a set of objects for inclusion in the MBS User Service Announcement Channel. The MBSTF goes on to fetch the objects listed in the object manifest from the MBS AF via reference point MBS-11.

-	Pull-based ingest at reference point Nmb8: The Object ingest subfunction in this case fetches one or more objects from the MBS Application Provider (AF/AS).

-	Push-based ingest at reference point Nmb8: The Object ingest subfunction receives one or more objects from the MBS Application Provider (AF/AS).

-	Pull-based egest at reference point MBS-12: The MBS AS fetches one or more objects from the Object ingest subfunction that have been ingested from the MBS Application Provider (AF/AS).

-	Push-based egest at reference point MBS-12: The Object ingest subfunction forwards one or more objects from the MBS Application Provider (AF/AS) to the MBS AS.

Object ingest procedures at reference point Nmb2 and MBS-11 shall be a subset of those at reference point Nmb8.

The Object segmentation subfunction supports the partitioning of an object into payload units suitable for MBS transmission.

The optional Application Layer FEC subfunction supports object recovery when some packets are not received by the MBMS Client.

The Packetisation subfunction places the payload units (and, optionally, the FEC data) into Nmb9 transmission packets according to clause 6.1.

The Packet scheduling subfunction schedules the outgoing packet stream according to target bit rate configuration.

The Control subfunction offers support for MBSTF service configuration, status query and notifications at reference point Nmb2.

## =====  CHANGE =====

### 4.3.4	MBS AS

The MBS AS is an optional entity that performs the following functions to support MBS User Services:

-	Providing a byte-range file repair service to the MBSTF Client (via reference point MBS-4-UC) for use with the Object Distribution Method.

The MBS AS is configured by the MBSF at reference point MBS-9. This interaction is not further defined by the present document.

The MBS AS may acquire content from the MBSTF via MBS-12 by one of the two methods:

-	Pull-based ingest at reference point MBS-12: The MBS AS fetches one or more objects from the MBSTF.

-	Push-based ingest at reference point MBS-12: The MBS AS receives one or more objects from the MBSTF.

The MBS AS may be deployed as a standalone entity, or its functions may be co-located with other Network Functions such as the MBSTF (see clause 4.3.3) or the 5GMS AS defined in TS 26.501 [7].

## =====  CHANGE =====

### 4.4.1	Overview

The following reference points defined in clause 5.1 of TS 23.247 [5] are relevant to MBS User Services architecture: Nmb1, Nmb2, Nmb5, Nmb8, Nmb9, Nmb10 and Nmb12.

The following additional reference points are defined by the present document:

-	MBS-3: Used by the MBSF to configure the MBS AF and to publish User Service Announcements to it. This reference point is not described further in the present document.

-	MBS-4-MC: Unidirectional multicast distribution of content from the MBSTF to the MBS Client.

-	MBS-4-UC: User Plane interactions between the MBSTF Client and the MBS AS for the purpose of file-based unicast repair.

-	MBS-5: User Plane interactions between the MBSF Client and the MBS AF for the purpose of MBS control plane and service handling.

-	MBS-6: API exposed by the MBSF Client and used by the MBS-Aware Application to manage and control MBS User Services.

-	MBS-7: API exposed by the MBSTF Client and used by the MBS-Aware Application to receive user data information distributed using MBS User Services.

-	MBS-8: Announcement of MBS User Services to the MBS-Aware Application by the MBS Application Provider. The procedures at this reference point are beyond the scope of 3GPP specification.

-	MBS-9: Used by the MBSF to configure the MBS AS. This reference point is not described further in the present document.

-	MBS-10: User Plane interactions between the MBSF Client and the MBSSF for the purpose of authorising access to security-protected MBS data by means of the User Plane security procedure specified in clause W.4.1.3 of TS 33.501 [18].

-	MBS-11: Used by the MBSTF to retrieve object manifests and User Service Announcements listed in object manifests from the MBS AF.

-	MBS-12: Used by the MBS AS to ingest objects from the MBSTF to support unicast repair.

In addition, the following reference points are defined inside the MBS Client function:

-	MBS-6′: API exposed by the MBSTF Client and used by the MBSF Client to (de)activate reception of an MBS Session by the MBSTF. The reception parameters are supplied by the MBSF Client.

This reference point is outside the scope of MBS User Services and is not described further in the present document.

-	MBS-7′: API exposed by the MSTF Client and used by the MBSTF to supply MBS Session configuration information that has been received from reference point MBS-4-MC.

This reference point is outside the scope of MBS User Services and is not described further in the present document.

## =====  CHANGE =====

## 4.9	Interworking with eMBMS

Interworking between MBS and eMBMS is described in clause 5.2 of TS 23.247 [5] and applies at the service layer in cases where the same Multicast/Broadcast service is provided simultaneously via eMBMS and MBS. Figure 4.9-1 depicts a combined network architecture based on figure 4.2.1-1 in the present document and figure 5.2-1 in TS 23.247 [5].

Figure 4.9-1: MBS–eMBMS interworking system architecture

To support MBS-eMBMS interworking according to the description in clause 5.2 of TS 23.247 [5], the MBS User Services architecture supports the following functionality:

1.	MBS User Services are provisioned in the MBSF via reference point Nmb10 per the present document. eMBMS User Services are separately provisioned in the MBSF at reference point xMB-C per TS 26.348 [6] or at reference point MB2-C per TS 23.468 [12]. A common TMGI is used in the MBS System and in the eMBMS System. The two User Services should be provisioned to ingest the same content if service continuity between the MBS System and the eMBMS System is required.

2.	Signalling from the MBSF to the MBS Client is present in the MBS User Service Announcement indicating that the equivalent User Service is also available via eMBMS. By this, a UE that implements both an MBS Client and an eMBMS Client may choose to receive the service from either network, and may dynamically switch between them according to varying reception conditions. Such an architecture is shown in figure 4.9-2.

3.	Content for the provisioned MBS User Service is ingested logically by the MBTSF at reference point Nmb8 per the present document. Content for the provisioned eMBMS User Service is ingested logically by the MBSTF at reference point xMB-U per TS 26.348 [6] or at reference point MB-2 per TS 23.468 [12]. If these reference points are compatible, the content is ingested once to satisfy both logical ingests.

4.	MBS User Services distribution methods are compatible with eMBMS delivery methods such that the same ingested content can be delivered to an MBS Client and to an eMBMS Client. UEs supporting only eMBMS are served by this architecture as well.

Figure 4.9-2: MBS–eMBMS interworking reference architecture

## =====  CHANGE =====

## 5.2	High-level baseline procedures

The high-level baseline procedures for MBS User Services are shown in figure 5.2-1.

NOTE:	In the interests of brevity, the prefix MBS is omitted from the numbered steps in the figure.

Figure 5.2-1: MBS User Service high-level baseline procedures

The basic procedures are as follows:

1.	The MBS Application Provider provisions an Application Service as an MBS User Service by invoking the Nmbsf service on the MBSF via reference point Nmb10.

2.	The MBSF provisions one or more MBS Distribution Session by invoking the Nmbstf service on the MBSTF via reference point Nmb2.

2a.	Optionally, the MBSF provisions the Object Repair feature for the MBS Distribution Session on the MBS AS via reference point MBS-9. This interaction is not further defined in this release.

3.	The MBSF provides an MBS User Service Announcement document that may be accessed by the MBS Client of interested UEs.

4.	The MBS Application Provider informs the MBS-Aware Application via MBS-8 that the specific Application Service can be accessed via an MBS User Service by means of an Application Service Announcement.

[Alternatively, the MBSF Client synthesises the Application Service Announcement from the User Service Announcement received in step 3 and informs the MBS-Aware Application via MBS-6 that the Application Service can be accessed via an MBS User Service.]

5.	The MBS Application Provider creates an MBS User Data Ingest Session with the MBSTF (via the MBSF) and the latter ingests the user data via Nmb8.

5a.	Optionally, if object repair is provisioned, the MBSTF establishes a repair object ingest session with the MBS AS and the latter ingests the relevant user data via reference point MBS-12.

6.	The MBS-Aware Application requests the MBS Client to access a specific MBS User Service by invoking procedures at MBS-6.

7.	Optional: If it has not already been received in step 3 above, the MBSF Client may discover additional access information about this MBS User Service by querying the MBSF via MBS-5.

8.	The MBSF Client provides the MBS Distribution Session information acquired as part of the User Service Announcement information to the MBSTF Client via MBS-6′.

9.	The MBSTF Client activates reception of the MBS Distribution Session.

10.	The MBSF Client continuously handles the MBS Distribution Session data based on updates received via MBS-5.

11.	The MBSTF Client receives MBS Distribution Session data via MBS-4-MC as part of the MBS Distribution Session.

11a.	If unicast repair is provisioned, then the MBSTF Client may use reference point MBS-4-UC to accomplish post-session or in-session Object Repair as part of the MBS Distribution Session.

12. The MBSTF provides the MBS Distribution Session data to the MBS-Aware Application via MBS-7 in an MBS Application Data Session.

13.	The MBS-Aware Application controls the MBS User Service by invoking MBS Application Service Control procedures on the MBSF Client via MBS-6.

## =====  CHANGE =====

## 5.3	Procedures for User Service provisioning

The procedure begins with the MBS Application Provider provisioning an MBS User Service and, within its scope, a set of MBS User Data Ingest Sessions, as shown in figures 5.3-1 and 5.3.-2 below.

Figure 5.3-1: Call flow for MBS User Service provisioning by MBS Application Provider

First, the MBS Application Provider provisions a new MBS User Service Session in the MBS System:

1.	To support Use Cases (e.g. Group Communication) where there is a requirement for TMGI allocation to be managed outside the MBS System, the MBS Application Provider may pre-allocate a TMGI for some or all of the MBS Distribution Sessions declared in step 3 below by invoking the Nmbsmf_TMGI_Allocate service operation on the MB-SMF at reference point Nmb13 (or N33+N29mb, if invoked via the NEF), as specified in clause 9.1.2.2 of TS 23.247 [5].

2.	The MBS Application Provider invokes the Nmbsf_MBSUserService_Create service operation at reference point Nmb10 (or N33+Nmb5 if invoking via the NEF) to create a new MBS User Service, as defined in clause 4.5.3.

Immediately, or at some later time, the MBS Application Provider creates at least one MBS User Data Ingest Session (as defined in clause 4.5.5) within the scope of the MBS User Service created in step 2 above:

3.	The MBS Application Provider creates an MBS User Data Ingest Session by invoking the Nmbsf_MBSUserDataIngestSession_Create service operation at reference point Nmb10 (or N33+Nmb5, if invoked via the NEF).

The MBS User Data Ingest Session optionally includes a schedule of start and end times referred to as active periods.

The MBS User Data Ingest Session comprises the details of one or more MBS Distribution Session(s), as defined in clause 4.5.6. Each such MBS Distribution Session fully specifies one of the distribution methods defined in clause 6 and may optionally nominate a TMGI to be used if one was pre-allocated in step 1 above.

4.	The MBS Application Provider subscribes to status events from the MBSF relating to the MBS User Data Ingest Session just created by invoking Nmbsf_MBSUserDataIngestSession_StatusSubscribe service operation at reference point Nmb10 (or N33+Nmb5, if invoked via the NEF).

5.	Void.

Shortly before a provisioned MBS User Data Ingest session is scheduled to become active (see clause 4.5.5), or immediately if no schedule of active periods is provisioned, the MBSF establishes in the MBSTF all MBS Distribution Sessions comprising that MBS User Data Ingest Session as shown in figure 5.3-2 below.

Figure 5.3-2: Call flow for MBS User Service internal provisioning

For each such MBS Distribution Session of a parent MBS User Data Ingest session:

6.	If no MBS Session ID was nominated by the MBS Application Provider in step 1 above, the MBSF may allocate a TMGI for it at this point by invoking the Nmbsmf_TMGI_Allocate service operation on the MB-SMF at reference point Nmb1, as specified in clause 9.1.2.2 of TS 23.247 [5].

7.	The MBSF creates an MBS Session to reserve resources in the MBS System for the MBS Distribution Session by invoking the Nmbsmf_MBSSession_Create service operation on the MB-SMF at reference point Nmb1, as specified in clause 9.1.3.6 of TS 23.247 [5]). The MBS Session ID provided for the MBS Distribution Session in step 1 or reserved in step 6 above is provided as an input parameter, if available. If needed, the MB-SMF allocates a TMGI as a side-effect of this operation. The MBSF determines the other input parameters as specified in clause 4.5.9.

8. 	For MBS Distribution Sessions of Service type broadcast, the MB-SMF notifies the MBSF upon successful establishment of the corresponding MBS Session(s).

9.	Once it is satisfied that the Broadcast MBS Session has been successfully established, the MBSF notifies the MBS Application Provider about the successful establishment of the Broadcast MBS Session(s) corresponding to the MBS Distribution Session using one or more Session started events.

The following steps do not need to wait for the completion of step 8:

10.	The MBSF creates the MBS Distribution Session in the MBSTF by invoking the Nmbstf_‌MBS‌Distribution‌Session_‌Create service operation at reference point Nmb2. This is a mirror of the entity in the MBSF (see clause 4.5.6).

-	In the case of the Object Distribution Method with push-based Object acquisition method, the response includes additional content ingest parameters chosen by the MBSTF for this MBS Distribution Session (see Object ingest base URL in table 4.5.6-2).

-	In the case of the Packet Distribution Method, the response includes additional content ingest parameters chosen by the MBSTF for this MBS Distribution Session (see MBSTF ingest endpoint addresses in table 4.5.6-3).

11.	The MBSF invokes the Nmbsf_MBSUserDataIngestSession_‌StatusNotify service operation at reference point Nmb10 (or Nmb5+N33, if invoked via the NEF) to inform the MBS Application Provider of the content ingest parameters that have been chosen for this MBS Distribution Session using the Distribution Session starting event (see MBSTF ingest endpoint addresses in table 4.5.6-3).

12.	The MBSF subscribes to status events from the MBSTF relating to the MBS Distribution Session just created by invoking the Nmbstf_MBSDistributionSessionStatusSubscribe service operation at reference point Nmb2.

Optionally, if object repair is provisioned

12a:	The MBSF creates an ingest session for repair objects on MBS AS.

NOTE: 	This step 12a is outside scope of this document.

13.	In the case of the Packet Distribution Method and pull-based content ingest using the Object Distribution Method, the MBSTF attempts to establish content ingest from the MBS Application Provider at reference point Nmb8 according to the ingest parameters and distribution method provisioned for the MBS Distribution Session in question (see table 4.5.6-1).

On success, the state of the MBS Distribution Session in the MBSTF becomes ESTABLISHED; on failure, it remains INACTIVE (see step 2 in clause 4.6.1).

NOTE:	Success of this step varies according to the provisioned distribution method and its configuration. Success may, for example, be defined as establishing a network association with the MBS Application Provider (using the additional parameters defined in table 4.5.6-3), or it may require successful ingest of an initial object from the MBS Application Provider (using the additional parameters defined in table 4.5.6-2).

14.	The MBSTF invokes the Nmbstf_MBSDistributionSession_StatusNotify callback service operation at reference point Nmb2 to inform the MBSF of the (un)successful establishment of content ingest using the Distribution Session established event.

On success, the state of the MBS Distribution Session in the MBSF becomes ESTABLISHED; on failure, it remains INACTIVE (see step 2 in clause 4.6.1).

Optionally, if object repair is provisioned

14a:	The MBSTF and the MBS AF establish a repair content ingest session via MBS-12.

14b:	The MBS AS informs the MBSF that it has successfully created an ingest session with the MBSTF.

NOTE: 	This step 14b is outside scope of this document.

15.	The MBSF invokes the Nmbsf_MBSUserDataIngestSession_StatusNotify callback service operation at reference point Nmb10 (or Nmb5+N33, if invoked via the NEF) to inform the MBS Application Provider of the (un)successful establishment of content ingest for the MBS Distribution Session in the context of its parent MBS User Data Ingest Session using either the Distribution Session established event or Distribution Session establishment failure event, as appropriate.

16.	If content ingest was established successfully in step 11 above, the MBSF compiles the metadata relating to this MBS Distribution Session into an MBS Distribution Session Announcement, as defined in clause 4.5.8.

When the MBSF has completed its attempt to configure all MBS Distribution Sessions required by the parent MBS User Data Ingest Session:

17.	The MBSF invokes the Nmbsf_MBSUserDataIngestSession_StatusNotify callback service operation at reference point Nmb10 (or Nmb5+N33, if invoked via the NEF) to inform the MBS Application Provider of the (un)successful establishment of all MBS Distribution Sessions of the parent MBS User Data Ingest Session using either the User Data Ingest Session established event or User data ingest failure event, as appropriate.

## =====  CHANGE =====

## 5.6	Procedure for User Service data repair

### 5.6.1	General

In the case of the Object Distribution Method (as defined in clause 6.1), the MBSTF Client may collaborate with the MBS AS at reference point MBS-4-UC to recover lost portions of content corresponding to MBS data that was not successfully received by the MBSTF Client at reference point MBS-4-MC (see step 9 in clause 5.5).

The high-level procedure for data repair is illustrated in figure 5.6.1-1 below:

Figure 5.6.1-1: High-level call flow for MBS User Service data repair

In the following clauses, more detailed procedures are provided for:

-	In-session Object Repair during the MBS Distribution Session, including the ability to repair objects in streaming distribution, defined in clause 5.6.2.

-	Post-session Object Repair after the completion of the MBS Distribution Session defined in clause 5.6.3.

### 5.6.2	In-session Object Repair

For in-session object repair, the procedures in clause 5.5 are extended as shown in figure 5.6.2-1. In particular, after the steps described in clause 5.5.2 and figure 5.5-2, in case in-session object repair is configured for the MBS Client, the following steps are carried out.

Figure 5.6.2-1: Call flow for in-session object repair

In particular, starting with an extended step 13 for each object:

13a0:	The MBSTF Client receives content for object 1.

13a1:	The MBSTF Client determines the end of object 1 delivery.

13a2:	The MBSTF Client determines if any data is missing for object 1.

13b0:	The MBSTF Client receives content for object 2.

13b1:	The MBSTF Client determines the end of object 2 delivery.

13b2:	The MBSTF Client determines if any data is missing for object 2.

In parallel to 13b0–13b2:

13a3: 	If missing, it the MBSTF Client requests missing data.

13a4:	The MBSTF Client recovers the object.

13a5:	The MBSTF Client informs the application on the recovered object.

13a6:	The application acquires object 1 from the MBSTF Client.

Then:

13c0:	The MBSTF Client receives content for object 3.

13c1:	The MBSTF Client determines the end of object 3 delivery.

13c2:	The MBSTF Client determines if any data is missing for object 3.

In parallel to 13c0–13b3:

13b3: 	If missing, it the MBSTF Client requests missing data.

13b4:	The MBSTF Client recovers the object.

13b5:	The MBSTF Client informs the application on the recovered object.

13b6:	The application acquires object 2 from the MBSTF Client.

### 5.6.3	Post-session Object Repair

For post-session object repair, the procedures in clause 5.5 are extended as shown in figure 5.6.3-1. In particular, after the steps described in clause 5.5.2 and figure 5.5-2, in case post-session object repair is configured for the MBS Client, the following steps are carried out.

Figure 5.6.3-1: Call flow for Post Session Object repair

In particular, after step 13, when the content is received:

13a:	The MBSTF determines the end of session for each MBS distribution session.

13b:	The MBSTF determines if any data is missing.

13c:	If missing, it determines some random back-off time.

13d:	After back-off time has elapsed, the MBSTF requests missing data from the MBS AS.

13e:	The MBSTF recovers the object.

13f:	Once recovered, the MBSTF notifies the application on the available object.

13g:	the application acquires the object from the MBSTF.