Source:	Huawei

Title:	AR media types and transport discussion for MeCAR

Document for:	Discussion and Agreement

Agenda Item:	5.6

1	Introduction

At the 3GPP SA4#118-e, the Permanent Document for MeCAR v1.0  and the draft TS of MeCAR were approved.

As part of the MeCAR work, the AR/MR media types and formats need to be defined. This contribution provides AR/MR media type definitions according to AR/MR conversational use cases, and defines these AR/MR media transmission requirements and solutions.

2	AR/MR Call Use Cases and Media Types

2.1	AR/MR Call Use Cases

The use cases defined in Annex A of 3GPP TR 26.998[1] and Annex A of 3GPP TR 26.928[2] can be categorized into AR/MR two-party call use cases and AR/MR conferencing use cases.

The AR/MR two-party call use cases include:

-	UC#7(3GPP TR 26.928): Real-time 3D Communication

-	UC#8(3GPP TR 26.928): AR guided assistant at remote location (industrial services)

-	UC#11(3GPP TR 26.928): Real-time communication with the shop assistant

-	UC#13(3GPP TR 26.928): 3D shared experience

-	UC#17(3GPP TR 26.928): AR animated avatar calls

-	UC#16(3GPP TR 26.998): AR remote cooperation

-	UC#17(3GPP TR 26.998): AR remote advertising

-	UC#19(3GPP TR 26.998): AR conferencing

The AR/MR conferencing use cases include:

-	UC#12(3GPP TR 26.928): 360-degree conference meeting

-	UC#15(3GPP TR 26.928): XR meeting

-	UC#16(3GPP TR 26.928): Convention / Poster Session

-	UC#18(3GPP TR 26.928): AR avatar multi-party calls

-	UC#19(3GPP TR 26.928): Front-facing camera video multi-party calls

2.2	AR/MR Media Types

AR/MR media types relevant to the use cases are as follows:

| Use Cases | AR/MR Media Type |
| --- | --- |
| UC#7: Real-time 3D Communication | 1. User’s fov 2. User’s header 3D model 3. User’s header action 4. User facial expression |
| UC#8: AR guided assistant at remote location (industrial services) | 1.AR mark 2.AR mark’s position 3.Spatial description |
| UC#11: Real-time communication with the shop assistant | 1.Furniture’s 3D model 2.Furniture’s position |
| UC#13: 3D shared experience | 1.Scene description 2.User’s 3D model 3.User’s position 4.User’s fov 5. User’s body action 6. User’s facial expression 7. User’s gesture |
| UC#17: AR animated avatar calls | 1.User’s 3D model 2.User’s body action |
| UC#16: AR remote cooperation | 1.AR mark 2.AR mark’s position |
| UC#17: AR remote advertising | 1.Furniture’s 3D model 2.Furniture’s position 3.AR mark 4.AR mark’s position 5.House’s 3D model 6.User’s fov 7.User’s position |
| UC#19: AR conferencing | Overlay |
| UC#12: 360-degree conference meeting | 1.Scene description 2.User’s 3D model 3.User’s position 4.User’s fov 5.User’s body action 6.User’s facial expression 7.User’s gesture 8.Overlay |
| UC#15: XR meeting | 1.Scene description 2.User’s 3D model 3.User’s position 4.User’s fov 5.User’s body action 6.User’s facial expression 7.User’s gesture |
| UC#16: Convention / Poster Session | 1.Scene description 2.User’s 3D model 3.User’s position 4.User’s fov 5.User’s body action 6.User’s facial expression 7.User’s gesture |
| UC#18: AR avatar multi-party calls | 1.User’s 3D model 2.User’s position |
| UC#19: Front-facing camera video multi-party calls | 1.User’s 3D model 2.User’s position 3.User’s body action 4.User’s facial expression |

Here are the media types summarized as below which are included in above table:

1. Scene description

2. Spatial description

3. 3D model

4. AR mark

5. Position

6. Fov

7. Gesture

8. Body action

9. Facial expression

However, there are still some media types not mentioned above, but they are described in 3GPP TR 26.998 and 3GPP TR 26.928[2], shown as below:

1. Sensor information, which is defined in clause 6.2.7 of 3GPP TR 26.928[2].

2. Camera information, which is defined in clause 4.4.7.1 of 3GPP TR 26.998[2].

3. Projection information, which is defined in clause 4.6.2.4 of 3GPP TR 26.928[2].

3	AR/MR media transport requirement

For each AR/MR media, they have different definition and transport requirement, the table below shows how and where the AR/MR media are defined and the real-time transport level we recommended according to the use case.

| AR/MR Media | Data Definition | Definition Source | Dependency on Real-Time Data Transport |
| --- | --- | --- | --- |
| Scene Description | Type: glTF2.0, JSON format Size: XX MB Organization: MPEG-I and Khronos | Clause 4.4.2 of 3GPP TR 26.998[1] | Low |
| Spatial Description | Type: For example: https://www.khronos.org/registry/OpenXR/specs/1.0/html/xrspec.html#XR_FB_spatial_entity Size: 1X-XXX MB Organization: OpenXR | Clause 4.4.7.3 of 3GPP TR 26.998[1] | Low |
| 3D Model | Type: PoLYgon Size: Several MB or GB Organization: None | Clause 4.6.3.5.2 of 3GPP TR 26.928[2] | Low |
| AR Mark | Type: A image includes color, width, and position list of each point. Size: XX B/KB Organization: None | None | Low |
| Position | Type: It consists of 4 quaternions for orientation and 3 vectors for position. Timestamp is represented by a 64 bit monotonically increasing nano-second-based integer. Frequency: at least 1kHz Size: XX B Organization: Khronos OpenXR | Clause 4.4.3.1 of 3GPP TR 26.998[1] | High |
| FOV | Type: It consists of vertical fov and horizontal fov. Size: XX B Organization: None | Y.6.2.3 of 3GPP TS 26.114[3] | High |
| Viewport | Type: It shall contain all of the parameters Viewport_azimuth, Viewport_elevation, Viewport_tilt, Viewport_azimuth_range and Viewport_elevation_range Size: XX B Organization: None | Y.7.2 of 3GPP TS 26.114[3] | High |
| Gesture | Type: A array of finger joint position. For example: https://www.khronos.org/registry/OpenXR/specs/1.0/html/xrspec.html#XR_EXT_hand_tracking Size: XX B Organization: OpenXR | None | High |
| Body action | Type: bvh format. Frequency: at least 1kHz Size: XX B Organization: BioVision company | None | High |
| Facial expression | Type: A array of key point position. For example: https://www.khronos.org/registry/OpenXR/specs/1.0/html/xrspec.html#XrSystemFacialTrackingPropertiesHTC Size: XX B Organization: None | OpenXR | High |
| Sensor information | Type: None Size: XX B Organization: None | None | Low |
| Camera information | Type: The camera parameters such as focal length, principal points, calibration parameters and the pose of the camera all contribute in understanding the relevance between points in the volumetric scene and pixels in the captured image. Size: XX B Organization: None | None | Low |
| Projection information | Type: Projection mapping information (indicating the projection format in use, e.g., Equirectangular projection (ERP) or Cubemap projection (CMP)), for the projection sample location remapping process as specified in clauses 7.5.1.3 and 5.2 of ISO/IEC 23090-2 [179] Size: XX B Organization: None | Clause Y.3 of 3GPP TS 26.114 and H264 protocol | Low |

4 Solutions for AR/MR media transmission

In line with clause 3, there are different transmission requirements for each AR/MR media, we consider two solutions, RTP and IMS data channel.

4.1 RTP Solution

RTP over UDP is used for real-time media transmission. Currently, RTP is mainly used for audio/video media transmission, it is very mature.

RTP extensions to support AR/MR media transmission have the following three solutions:

1. RTP header extension: Use RTP header extension part to transport AR/MR media.

According to clause 5.3.1 of RFC3550[4], 16 bits can be used for extension, the maximum length used for AR/MR media can be 65535 bytes. AR/MR media can be carried together with audio RTP packet because video may not be available.

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      defined by profile       |           length              |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                        header extension                       |

|                             ....                              |

2.RTCP extension: Use RTCP packet to transport AR/MR media.

According to clause 6.7 of RFC3550[4], RTCP packet with PT=APP can be used to send AR/MR media, which specifies subtype with the AR/MR media type. The maximum length can be 65535 bytes.

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|V=2|P| subtype |   PT=APP=204  |             length            |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                           SSRC/CSRC                           |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                          name (ASCII)                         |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|                   application-dependent data                ...

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

3. New RTP media connection: Use a new m=application line in SDP with a new payload type to connect and transport AR/MR media.

All AR/MR medias transmission will use this new connection. To distinguish AR/MR media type, an extension on RTP header extension is needed. Here is an example:

SDP:

m=application 3238 UDP 100

a=rtpmap:100 xr-media

RTP:

0                   1                   2                   3

0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1 2 3 4 5 6 7 8 9 0 1

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|      defined by profile       |           length              |

+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+

|             header extension: AR/MR media type                |

|                             ....                              |

A comparison for these three solutions is shown as below:

|  | Solution1: RTP header extension | Solution2: RTCP extension | Solution3: New RTP media connection |
| --- | --- | --- | --- |
| Adv. | Easy to extend | 1.Easy to extend 2. No need extra transmission path. | 1.Independent channel beyond audio and video. 2.All AR/MR media data transmission can reuse the new connection. |
| Dis. | 1.It will cause the RTP header too big, which may bring new problems to audio packet transmission. 2. Some AR/MR media data such as scene description and 3D model are too large to fit into one RTP packet, and needs to be divided into multiple pieces for transmission. 3. The application layer needs to add a flag in front of the AR/MR media data content to indicate the AR/MR media type. 4. The AR/MR media transmission frequency should be the same with audio, which can’t meet the requirements for AR/MR media. | 1. RTCP packet will send with a limited interval (0.5s to 1.5s), can’t meet the requirement of AR/MR media transmission, for example the tracking frequency for gesture should be at least 1000Hz according to clause 4.5.1 of 3GPP TR26.998. 2. Some AR/MR media data such as scene description and 3D model are too large to fit into one RTP packet, and needs to be divided into multiple pieces for transmission. 3. RTCP packet will stop sending when the network is in congestion. | 1. Application level needs to map RTP header extension tag to AR/MR media type. |

From the comparison above, it is easily to achieve a conclusion the only solution over RTP is using a new RTP media connection for AR/MR media transmission.

4.2 IMS Data Channel Solution

According to clause 6.5.5 of 3GPP TR 26.998[1], IMS data channel can be used for AR/MR media (AR-specific data) transmission. IMS data channel over SCTP is derived from WebRTC data channel which is widely used web real-time communication.

Application can use stream id for different AR/MR media type identification. Data channel can set flexible reliability modes for different streams id, by setting “max-retr” and “max-time” parameters in each a=dcmap during data channel establishment. Here is an example:

m=application 1002 UDP/DTLS/SCTP webrtc-datachannel

a=max-message-size:100000

a=sctp-port:5000

a=setup:actpass

a=fingerprint:SHA-1 \

4A:AD:B9:B1:3F:82:18:3B:54:02:12:DF:3E:5D:49:6B:19:E5:7C:AB

a=tls-id:abc3de65cddef001be82

a=dcmap:1001 subprotocol="http"; max-retr=3; max-time=10

a=dcmap:1002 subprotocol="http"; max-retr=5; max-time=15

4.3 Summary

From the analysis above, both RTP and IMS data channel solutions for AR/MR media transmission are available, a comparison is shown as below:

|  | RTP Solution: New RTP media connection | DC over SCTP Solution |
| --- | --- | --- |
| Adv. | RTP protocol is mature for real time media transmission with low latency and high reliability mechanism | 1. The application can be decoupled from the terminal architecture, and the application can customize the transmitted data and data format, which can be expanded flexibly without terminal changes; 2. The application does not need to be pre-installed or built-in on the terminal. |
| Dis. | RTP extension is difficult, it needs chip support and to be open to upper layer applications. | 1. The delay of SCTP link establishment is large due to its 4RTTs interaction. 2. The forced SCTP over DTLS encryption and decryption performance is poor. 3.SCTP has no FEC mechanism. |

Due to rapid progress of AR technology, AR/MR media data will expand frequently. In order to avoid the continuous expansion of RTP protocol, considering that SA2 is studying IMS data channel architecture, it appears advantageous to use IMS data channel to transmit AR/MR media.

5 Proposal

We propose to include the Clause 2.2, Clause 3 and Clause 4 of this contribution in the PD.

References

- 3GPP TR 26.998: "Support of 5G Glass-type Augmented Reality / Mixed Reality (AR/MR) devices ".

- 3GPP TR 26.928: "Extended Reality (XR) in 5G"

- 3GPP TS 26.114: " IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

- RFC3550: "A Transport Protocol for Real-Time Applications".