Agenda item: 	4.5

Source: 	InterDigital Communications

Title: 	[GA4RTAR] iRTCW client architecture

Document for:	Discussion and Agreement

# Introduction

In this contribution, we discuss the architecture and the detailed functional components in an iRTCW client. More specifically we discuss the additional functional components that extend the basic architecture defined in GA4RTAR item to enable WebRTC based real-time communication.

# iRTCW Client architecture and functional components

The proposed functional components of an iRTCW client using 5G System are shown in Figure 1. An iRTCW client with these functional components is capable of supporting the four collaboration scenarios discussed in the permanent document [2].

Figure 1 Functional components of an iRTCW client in UE

Media session handler:

The MSH is an entity running on the UE, which assists with the 5G integration of the WebRTC application. It exchanges, on behalf of the application, information about the WebRTC sessions with the network.

The MSH receives information about a new WebRTC session from the application. It relays the information to the Support Function. It also receives events and other network information about the WebRTC session from the Support Function, which it may relay to the application.

The following additional subfunctions (highlighted in yellow) are identified as part of a more detailed breakdown of Media Session Handler:

Metrics Collection and Reporting: executes the collection of QoE metrics measurement logs from the iRTC Framework and sending of metrics reports to the 5G-RTC AF for the purpose of metrics analysis or to enable potential transport optimizations by the network.

Consumption Collection and Reporting: executes the collection of content consumption measurement logs from the iRTC Framework and sending of consumption reports to a 5G-RTC AF about the currently consumed media within the available presentation, and about the environment of the media session for potential transport optimizations by the network or consumption report analysis.

iRTC framework:

An open framework for an iRTC client that enables real-time communications capabilities for transport of immersive media in browser based and native WebRTC applications.

iRTC media channel:

This functional component represents a network channel for an established media delivery session and is used for unidirectional or bidirectional media transfer between iRTCW peers using RTP/SRTP protocols. This component directly interacts with the media encoders and decoders which support 3GPP media codecs. In case of native and web-based WebRTC applications, this component supports access to next generation 3GPP media codecs which are capable of encoding and decoding 3D/immersive media content.

WebRTC Data channel:

This component represents a network channel which can be used for unidirectional or bidirectional peer-to-peer transfers of arbitrary data which may include non-real time media data in some use-cases. Support for transmission of real-time media data using data channels may be further studied. Each Data channel is associated with a WebRTC session. When the SDP message contains a scene description URL, a WebRTC data channel is established to retrieve scene descriptions, including scene updates, over the network. In some use cases, the data channel may also be used for downloading some of the AR media in the scene over the network.

WebRTC Media channel:

A component in the WebRTC framework that is capable of encoding and decoding the real-time media and represents a network channel for an established media delivery session. This component is used for unidirectional or bidirectional media transfer between iRTCW peers using RTP/SRTP protocols.

In the case of browser-based WebRTC applications, the WebRTC media channels only have access to media codecs supported by the browser implementation of the WebRTC stack. To support immersive media transport, the WebRTC stack implemented in the browser needs to support 3GPP media codecs. A fallback solution using WebRTC data channels and 3GPP media codecs may be further studied.

Scene Manager:

The Scene Manager is a software component that can create an immersive scene graph representation after parsing the scene description information. When an SDP message contains a URL for a scene description, the WebRTC session handler instantiates and initializes a Scene Manager.

AR media access:

A generic interface module on the client device that enables access to AR media and other related data that is needed by the scene manager or AR Runtime to provide an AR experience. The AR Media access module typically uses 5G System functionalities to access AR media and other related data.

AR Runtime:

The AR Runtime creates a local AR/MR session and performs registration with the local environment. AR runtime provides the functionalities such as allowing applications to discover capabilities of AR devices, managing an AR session and its state, receiving information about user’s actions, Spatial Computing and synchronizing the display and rendering the composited frame onto the AR displays. For spatial computing activities such as mapping, AR Runtime communicates with cameras to generate information about the world 3D space surrounding the AR user. AR Runtime also takes input from sensors for getting current position and predicting tracking positions, spatial computing activities such as relocalazation to estimate the pose when tracking is lost. The Scene Manager requests the creation of a local AR/MR session from the AR Runtime interface module present in an iRTCW client.

3D media encoder:

A generic encoder module capable of encoding the 3D media representation content in real-time. Appropriate codecs, media encoding formats and profiles will be defined by the MeCAR work item. For example, a 3D media representation may be a point cloud or a 3D mesh.

3D media decoder:

A 3D media decoder is a decoding module capable of decoding the 3D media representation in real-time.

Spatial audio decoder and encoder:

An audio decoder capable of decoding spatial audio content. An audio encoder capable of encoding multiple microphones audio signals into spatial audio content.

# Definitions

AR Media:

Media objects that are part of the scene rendered on the iRTCW client.

# Proposal

We propose to agree the baseline functional components of an iRTCW client in clause 2, definitions in clause 3 and document them in the GA4RTAR TS.

# References

- 3GPP TS 26.114: “IP Multimedia Subsystem (IMS) Multimedia telephony, media handling and interaction”.

- S4aR220010, “iRTCW Permanent Document”