Source:	Meta Ireland (Rapporteur)

Title:	3D Video Types for iRTC Client in the Terminal

Agenda Item:	4.3

Document for:	Discussion and Agreement

- Introduction

In this document, we compare the two types of 3D video iRTC client supports, holographic (or volumetric) and avatar-type, which address different use cases and available channel capacity.

- Holographic/volumetric-type

This conventional type corresponds to a representation of 3D objects using point cloud or mesh-based compression techniques. Several standard formats such as V-PCC are discussed in [1], [2]. This type intends to compress and restore the captured 3D video data as closely as possible, and do not make any assumptions on the structure of 3D objects being captured.

- Avatar-type

As outlined in [3], this type assumes the 3D objects being captured are a human head or an upper body, and extracts and transmits a set of features to calibrate models for the objects, which need to be shared by the sender and the receiver. Simple avatar can be constructed with a single image [4], but more realistic ones need a large amount of data and processing for training.

- Comparison

Relationship of holographic/volumetric and avatar-types may be compared with that of waveform coding and linear predictive coding, in that the former does not assume the nature of input signals while the latter assumes the input signals are generated in human vocal tract. In a similar fashion, the two types of 3D video can be compared as follows:

|  | Holographic/volumetric-type | Avatar-type |
| --- | --- | --- |
| Bit-rate |  | Low |
| Flexibility | Applicable to any objects | Applicable to modelled objects (excluding areas, e.g., hair, clothing) |
| Training | Not required | Required |
| Lowest quality |  | Bounded by models |

- Proposal

It is requested to include clauses 2-4, and the references of this document in the permanent document and take them into account in the discussion of 3D video types.

- References

[1] 3GPP TR 26.928 Extended Reality (XR) in 5G

[2] 3GPP TR 26.998 Support of 5G Glass-type Augmented Reality / Mixed Reality devices

[3] S4-221012 Functional Requirements for Avatar Driven, China Mobile Com. Corporation

[4] https://www.facebook.com/help/278747370042382